3GPP TSG RAN WG1 Meeting #109-e	R1-2204750

May 9th – May 20th, 2022

Agenda Item:	9.3.1

Source:	CEWiT, IIT-M, IIT-K, Reliance Jio, Saankhya Labs

Title:	Discussion on evaluation on NR duplex evolution

Document for:	Discussion

# Introduction

A new study item on “Study on evolution of NR duplex operation” was approved in RAN#94-e [1] with following objectives:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

To identify the relevant deployment scenarios for non-overlapping sub-band full duplexing (SBFD), an evaluation methodology needs to be developed. This contribution discusses the various important aspects of evaluation methodology that need to be defined to have consistent comparison of results across different scenarios.

# Evaluation methodology

A node capable of SBFD, can transmit and receive simultaneously in different sub-bands in frequency domain. Thus, a gNB capable of SBFD can perform DL and UL at the same time but different sub-bands within the same carrier. As discussed in our companion contribution [2], Fig. 1 show how SBFD can be implemented.

Fig. 1 SBFD in different non-overlapping sub-bands

For evaluation of SBFD, the sub-band partitioning for DL and UL resources can be fixed or dynamic. A fixed sub-band partitioning would ensure fixed number of UL and DL resources in frequency domain per slot. The fixed number can be in terms of DL to UL ratio. A fixed partitioning would also ensure a fair comparison  with half duplex (HD) system which has fixed DL to UL ratio, e.g., 2:1 and 1:1.

Proposal 1: Sub-band partitioning for DL and UL resources based on fixed DL to UL ratio is supported for evaluations.

Proposal 2: Baseline TDD system with same DL to UL ratio can be used as reference for comparing the performance of SBFD system.

One of the main advantages of implementation of SBFD at the gNB is reduction of latency as compared to HD TDD systems in UL or DL. For latency evaluations, FTP 3 traffic model can be considered where the parameters should be commonly defined or provided as a part of the simulation parameters.

Proposal 3: Define the various parameters (e.g., inter packet delay) for FTP 3 to be used in the evaluations.

In SBFD, CLI will be present in the network unlike TDD scenario. Network has to manage the CLI to minimise its impact on the system throughput. E.g., UL throughput might get heavily affected by gNB-to-gNB CLI if not managed properly. However, the additional UL resources available in SBFD compared to conventional TDD can be used to implement schemes like repetition of UL signal, reduced code rate, longer sequences for UL data/reference signal/control signal which will provide an improvement in the SINR and helps to combat the CLI. Besides that, SINR improvement also provides coverage enhancement in UL. Thus, SBFD can also be exploited to improve the SINR of UL signals.

Proposal 4: Define the use cases for SBFD evaluation, a. latency reduction, b. UL SINR improvement.

Proposal 5: The following evaluation metrics are supported:

a. Average latency for DL and UL

b. Average SINR in UL

Proposal 6: Evaluation metric “latency” is defined as the time taken from the generation of a packet to the complete transmission of the packet (including HARQ transmission).

Another important aspect of implementing SBFD at the gNB is the self-interference (SI) due to adjacent sub-bands of DL and UL. The gNB performs DL and UL simultaneously in different sub-bands within the carrier. This will result in leakage from the transmission in DL sub-band to the reception in UL sub-band. The leakage will be directly proportional to the transmit power in DL. The SI will degrade the performance in UL depending on its strength. Further, the gNB can use certain techniques, e.g., filtering, beamforming, etc., to reduce the SI. which can be implementation specific. As per the SID [1], the details of SI will be studied in RAN4. However, for evaluation of SBFD, the amount of SI at the gNB will have an impact on the UL reception. Thus, a common model of SI should be agreed upon which can be used for the evaluation. E.g., SI model can be as simple as defining the amount of SI suppression that is possible at the gNB.

Proposal 7: Define the SI model to be used for SBFD evaluations.

# Conclusion

The following observations and proposals are made in this contribution,

Proposal 1: Sub-band partitioning for DL and UL resources based on fixed DL to UL ratio is supported for evaluations.

Proposal 2: Baseline TDD system with same DL to UL ratio can be used as reference for comparing the performance of SBFD system.

Proposal 3: Define the various parameters (e.g., inter packet delay) for FTP 3 to be used in the evaluations.

Proposal 4: Define the use cases for SBFD evaluation, a. latency reduction, b. UL SINR improvement.

Proposal 5: The following evaluation metrics are supported:

a. Average latency for DL and UL

b. Average SINR in UL

Proposal 6: Evaluation metric “latency” is defined as the time taken from the generation of a packet to the complete transmission of the packet (including HARQ transmission).

Proposal 7: Define the SI model to be used for SBFD evaluations.

# References

- RP-213591, New SI: Study on evolution of NR duplex operation, RAN#94-e, Dec. 2021

- R1-2204751, Discussion on subband non-overlapping full duplex, RAN#109-e, May 2022