3GPP TSG RAN WG1 #109-e			R1-2204753

e-Meeting, May 9th – 20th, 2022

Source: 	CEWiT, Reliance Jio, IITM, IITK, Saankhya Labs

Title:	Discussion on sidelink based positioning requirements & scenarios

Agenda:	9.5.1.1

Document for:	Discussion

### Introduction:

In RAN #94-e, the study item on expanded and improved NR positioning [1] is approved. This study will focus on further improvement in Rel 16 and 17 positioning techniques considering the requirements from different verticals viz. V2X, public safety (first responder), industrial IoT etc. In the same meeting RAN plenary level study is concluded [4] which includes the inputs/ recommendations/ requirements from automotive groups like 5GAA and SAE which emphasize on the extension of RAN based positioning solutions over sidelink. Based on various inputs following objectives for positioning over sidelink are agreed,

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

  - Scenario/requirements

    - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

    - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

    - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

    - Spectrum: ITS, licensed

  - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

  - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

  - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

    - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

    - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

    - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

    - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

  - Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution provides our view on sidelink (SL) positioning scenarios and requirements to be considered for the study and eventually in Rel 18 standards.

### Discussion:

- Use cases to be considered for sidelink positioning:

In the 3GPP specification, detailed positioning requirements are studied for various use cases and presented in TS 22.261, TS 22.186, TS22.280 and TS22.104. TS 38.261, TS 38.186 and TR 38.845 provides the different use cases where positioning is important. These specifications provide requirements for the KIPs as accuracy, service availability, latency, coverage/service area, etc.

Vehicle-to-Everything (V2X) promotes automated driving and enhances the traffic experience. The connection between cars, infrastructure, networks as well as pedestrians enables a much more efficient and safer transport system due to V2X. 5GAA in its TR [5] mentioned that positioning information is indispensable to V2X use cases, such as assisted driving, automated driving, traffic monitoring & management, autonomous parking, etc. These positioning requirements change across different V2X use cases. It changes from a few metres level to sub-metre level.

TR 38.845 captures the detailed use cases and requirements related to V2X positioning and positioning for public safety.  This has created enough traction to include V2X related use cases in further enhancement of positioning solutions over sidelink. Similarly, TS 22.261 provides the use cases for the 1st responders and requirements related to it. It is recommended that this service should be provided indoor and outdoor with few meter level accuracy. To get the required accuracy, latency and positioning coverage, it is necessary to consider public safety use cases in sidelink positioning solutions. In some of the factory environments getting LOS path is difficult. But to track the mobile devices as well as mobile assets is becoming increasingly critical to achieve metre to sub-metre level of accuracy in a factory environment. TS 22.104 provided sub-metre level accuracy requirements with 99 % availability with a max 30 Km/hr UE speed for future factory scenarios. To achieve this level of accuracy sidelink positioning is important where adjacent UEs will assist the target UE along with the gNBs installed inside the factory. Therefore, for such use case, positioning of target UE can be performed using Uu link as well as sidelink as co-operative localization.

Proposal 1: Rel 18 study should include V2X, public safety, and IIoT as target use cases and requirements should be derived considering them.

For V2X use case, three network scenarios are considered as per TR 38.845 [4] viz. in-coverage scenario where both communicating sidelink UEs are in coverage of network (connected to at least one gNB), partial coverage where one UE will be in the coverage and other UE will be out of coverage, and out-of-coverage scenario where both UEs are not connected to any network. It is found that partial coverage and out-of-coverage are equally important as the in-coverage scenarios for many V2X use cases. If V2X deployment is being operated on the ITS band, it will be considered an out-of-coverage scenario. Therefore, in Rel 18, all three scenarios should be considered for study and specification.

Proposal 2: In Rel 18, in-coverage, partial coverage, and out-of-coverage scenarios should be considered for V2X and public safety use cases.

- KPIs and requirements for use case:

Before deriving the solutions for positioning over sidelink it is necessary to set the key performance indices (KPI)s and requirements for selected KPIs for each use case. The following KPIs can be selected for sidelink positioning purposes.

Position accuracy: The accuracy can be either of an absolute position or a relative position. It can be further derived into a horizontal position accuracy and into a vertical position accuracy.

Availability: Percentage of time when a positioning system is able to provide the required position-related data within the performance targets or requirements.

Latency: Time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the positioning system interface. At the initialization of the positioning system, the latency is also defined as the ‘time to first fix’.

Time to first fix (TTFF): Time elapsed between the first event triggering the determination of the position-related data and the availability of the position-related data at the positioning system interface. TTTF is greater or equal to latency.

Reliability: It measures the ability of a positioning system to provide the position-related data under stated conditions for a specified period.

Continuity: It is the likelihood that the positioning system functionality will be available during the complete duration of the intended operation if the positioning system is functioning at the beginning of the operation.

Integrity: It measures the trust in the accuracy of the position-related data provided by the positioning system and the ability to provide timely and valid warnings to the UE and/or the user when the positioning system does not fulfill the condition for the intended operation.

Positioning Coverage Improvement: It is the measurement of average additional links/nodes which can be brought under positioning coverage using sidelink positioning. It can measure the links which are in partial coverage and out of coverage whose position can be estimated using sidelink which is otherwise not able to localize using Uu link only.

For use case V2X, public safety, and V2X, we can agree upon the following table as requirements for respective KPI.  The table provides tentative values which can be deliberated in the meeting for finalization for evaluation purposes.

Proposal 3: Positioning accuracy availability, latency, TTFF, reliability, continuity, integrity, and positioning coverage improvement should be the set of KPIs for partial and out of coverage scenarios.

Proposal 4: In the release 18 study, the following table should be agreed upon as a requirement set for different use cases.

| Use cases |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | V2X |  |  | Public safety | IIoT |  |
|  | Set 1 | Set 2 | Set 3 |  | Set 1 | Set 2 |
| Absolute/Relative | Both | Both | Both | Both | Both | Both |
| Scenarios | In coverage, partial coverage, out of coverage | In coverage, partial coverage, out of coverage | In coverage, partial coverage, out of coverage | In coverage, partial coverage, out of coverage | In coverage | In coverage |
| Speed (km/hr) | <250 | <250 | <250 | 3 | 3-30 | 3-30 |
| Accuracy (H-Horizontal V-Vertical) | <50 H, <3m V | <1.5m H <2m V | <0.2m H <2m V | <1m H, 2m V | <1m H | <30cm |
| Latency | 10 | 1 | 1 | 5 | 5 | 1 |
| Availability | 95 | 99 | 99 | 98 | 90 | 99 |
| TTFF | [] | [] | [] | [] | [] | [] |
| Reliability | [] | [] | [] | [] | [] | [] |

Table 1: Requirement table for the different use cases for the sidelink-based positioning enhancement study.

Proposal 5: Both relative and absolute positioning should be studied for all the use cases and scenarios within.

- Frequency bands for Study:

For the best accuracy of positioning estimate, the sampling rate at which PRS is transmitted is an important factor. Therefore, positioning estimate accuracy will be proportional to the bandwidth over which PRS is transmitted. Some of the use cases in V2X, public safety, and IIoT need sub-metre level of accuracy which needs large bandwidth. ITS band which is available for V2X communication is limited (75 MHz) further allowed ITS band in some of the regions is limited to 20 MHz. In this case, achieving sub-metre level accuracy is a challenging task. Similarly using a licensed band for V2X communication is feasible when it shares spectrum with the cellular network. This is supported in the V2X standard. In RAN plenary few companies proposed to use the unlicensed band for sidelink positioning where the unlicensed spectrum can be used for only PRS transmission. The use of an unlicensed band will definitely help in improving positioning estimate accuracy but sidelink has extended yet to use sidelink band for communication. Therefore, careful study is required to analyze the feasibility to use the unlicensed spectrum for sidelink position. Rel 18 sidelink work item agreed to bring enhancement to support NR-U over sidelink. We can wait till sufficient progress in this agenda and take up related development at the later stage of the study.

Proposal 6: Rel 18 study should support ITS and licensed band for sidelink positioning. The unlicensed band should be taken up later depending on the progress in the sidelink item.

### Conclusion:

The contribution draws the following conclusions,

Proposal 1: Rel 18 study should include V2X, public safety, and IIoT as target use cases and requirements should be derived considering them.

Proposal 2: In Rel 18, in-coverage, partial coverage, and out-of-coverage scenarios should be considered for V2X and public safety use cases.

Proposal 3: Positioning accuracy availability, latency, TTFF, reliability, continuity, integrity, and positioning coverage improvement should be the set of KPIs for partial and out of coverage scenarios.

Proposal 4: In the release 18 study, the following table should be agreed upon as a requirement set for different use cases,

| Use cases |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | V2X |  |  | Public safety | IIoT |  |
|  | Set 1 | Set 2 | Set 3 |  | Set 1 | Set 2 |
| Absolute/Relative | Both | Both | Both | Both | Both | Both |
| Scenarios | In coverage, partial coverage, out of coverage | In coverage, partial coverage, out of coverage | In coverage, partial coverage, out of coverage | In coverage, partial coverage, out of coverage | In coverage | In coverage |
| Speed (km/hr) | <250 | <250 | <250 | 3 | 3-30 | 3-30 |
| Accuracy (H-Horizontal V-Vertical) | <50 H, <3m V | <1.5m H <2m V | <0.2m H <2m V | <1m H, 2m V | <1m H | <30cm |
| Latency | 10 | 1 | 1 | 5 | 5 | 1 |
| Availability | 95 | 99 | 99 | 98 | 90 | 99 |
| TTFF | [] | [] | [] | [] | [] | [] |
| Reliability | [] | [] | [] | [] | [] | [] |

Proposal 5: Both relative and absolute positioning should be studied for all the use cases and scenarios within.

Proposal 6: Rel 18 study should support ITS and licensed band for sidelink positioning. The unlicensed band should be taken up later depending on the progress in the sidelink item.

### References:

- RP-213588 - Revised SID on Study on expanded and improved NR positioning

- R1-2204754 - Discussion on evaluation methods and results of sidelink based positioning

- R1-2204755 - Discussion on potential solutions for sidelink based positioning

- TS 38.845 V 2.0.0- Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases

- 5GAA: System Architecture and Solution Development; High-Accuracy Positioning for C-V2X