3GPP TSG RAN WG1 #109-e 			R1-2204755

e-Meeting, May 9th – 20th, 2022

Source: 	CEWiT, Reliance Jio, Saankhya Labs, IITM, IITK

Title:	Discussion on potential solutions for sidelink based positioning

Agenda: 	9.5.1.3

Document for:	Decision

### Introduction:

In RAN #94-e, the study item on expanded and improved NR positioning [1] was approved. This study will focus on further improvement in Rel 16 and 17 positioning techniques considering the requirements from different verticals viz. V2X, public safety (first responder), industrial IoT etc. In the same meeting, RAN plenary level study was concluded [4] which includes the inputs/ recommendations/ requirements from automotive groups like 5GAA and SAE that emphasize on extension of RAN based positioning solutions over sidelink. Based on these various inputs, the following objectives for positioning over sidelink are agreed upon,

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

  - Scenario/requirements

    - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

    - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

    - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

    - Spectrum: ITS, licensed

  - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

  - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

  - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

    - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

    - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

    - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

    - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

  - Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution provides our views on potential enhancement areas to be considered to enable positioning over sidelink for in coverage, partial coverage, and out of coverage scenarios for use cases including V2X, public safety and IIoT.

### Enhancements for side link-based positioning

Our companion contribution [2] on requirement and scenario provides the view on use cases, scenarios and the requirements to consider for Rel. 18 sidelink positioning study pertaining to enabling RAN based positioning over sidelink. In another companion contribution [3] on evaluation, initial results are provided for V2X and IIoT scenarios which emphasize the need for consideration of bandwidth, associating UE group selection, LOS-NLOS condition of channel, etc. Based on these, the following are the potential areas for enhancement in standards to support sidelink-based positioning.

### Bandwidth Consideration:

It is widely known that the bandwidth of positioning reference signals used for positioning parameter estimation is the governing factor for positioning accuracy. The higher the bandwidth, the higher will be the positioning accuracy for both time-based (DL-TDOA, UL-TDOA, M-RTT) and angle-based (DL-AoD, UL-AOA) positioning methods. In Rel. 16 and Rel. 17 studies, it is shown by simulation that the bandwidth should be configured for positioning measurements according to the simulation accuracy expected for scenario or use case. For example, to achieve an accuracy of few meters for outdoor and sub-meter in indoor, bandwidth of more than 100 MHz is necessary. Further, LOS–NLOS property of the link between gNB/TRP to UE impacts the quality of measurements.

The current specification supports sidelink over intelligent transport system (ITS) band and cellular licensed band. The spectrum band 5850-5925 MHz has been allocated for ITS services with maximum bandwidth being utilized as 20 MHz. In the FR2 spectrum, bandwidth up to 1 GHz is available in some regions. In licensed bands, NR systems provide flexibility to choose the bandwidth with the corresponding numerology which will help in achieving desired positioning accuracy. Therefore, study should be carried out on a per use case basis to fix the required bandwidth to get the desired accuracy.

Observation 1: Achievable positioning accuracy is proportional to the PRS bandwidth configured. Therefore, appropriate bandwidth should be allocated for each use case.

Figure 1: Positioning accuracy CDF for V2X Highway layout with the impact of bandwidth on SL-based co-operative positioning using the TDOA method.

.

### Positioning Methods:

NR supports round-trip time-based methods and angle-based methods for positioning. They are DL-Time difference of arrival (DL-TDOA), enhanced cell-ID (E-CID), Uplink Angle of Arrival (UL-AoA), Uplink Relative Time of Arrival (UL-RTOA), Uplink Time Difference of Arrival (UL-TDOA), Multi-Round Trip Time (M-RTT). These methods are configured by the location management function (LMF) server to gNB/TRP and UE using NRPPa and LPP protocol respectively. Signaling configuration depends on the type of method configured for positioning which in turn depends on the direction of the transmission of PRS over Uu link. For sidelink, both time-based methods and angle-based methods should be considered. TDOA method should be defined to measure RSTD over sidelink. Since over sidelink, PRS transmission will be between UE-to-UE, sidelink dependent parameters should be studied that will be configured along with relevant legacy parameters per positioning method. It includes PRS for sidelink allocation, density of PRS, repetition of PRS etc. Multi-RTT technique can be considered for sidelink positioning. However, the necessity of tight synchronization between Tx UEs should be carefully studied to check the feasibility and signaling impact.

For angle-based method, accuracy is governed by the number of transmitting and receiving antennas available at UE. Therefore, it is necessary to consider the type of antenna configuration that UE will have, and accordingly, angle-based method can be defined. For example, if a roadside unit (RSU) is used to transmit the PRS to UEs on the road, it can be assumed that the RSU has a sufficient number of antennas and sidelink AOD and AOA of signal from RSU can be used for positioning estimation. Therefore, by considering such appropriate assumptions and practical UE capabilities, the angle-based method should be defined. Architecture considered for sidelink positioning will also have an impact on methods and signalling.

Observation 2: For angle-based methods, antenna consideration at UE is an important factor. This needs to be studied considering UE capabilities and possible sidelink positioning architecture.

Observation 3: For M-RTT based method, tight synchronisation is required which will be a big challenge to enable it over sidelink for positioning.

Proposal 1: For Rel 18 study, both time-based (TDOA, M-RTT) and angle-based (AOD, AOA) positioning methods should be considered.

Proposal 2: Time difference of arrival (TDOA) method should be defined for sidelink to measure RSTD for positioning over sidelink.

Proposal 3: For angle-based methods for sidelink positioning, antenna configuration consideration should be studied using practical UE capabilities and sidelink positioning architecture.

### Positioning reference signal over sidelink

For positioning and localization, channel tap estimation with sufficient accuracy is very important to get more exact estimate of location. Therefore, the configuration of reference signal (RS) for positioning measurements (like RSTD, AoD and AOA) should be spread over time and frequency to capture the channel behaviour properly. For RAT based Uu positioning, a special positioning RS (PRS) is defined which has a staggered pattern with sufficient RB allocation and repetition. Further, for beam management, more than one PRS configuration can be configured to have measurements over different beams. This gives flexibility to LMF to choose better measurements from different beams, which increases the position estimate accuracy.

Over sidelink, SL-SSB and CSI-RS are the two-reference signals that are defined so far. SL-SSB is transmitted by the UEs which can become a synchronization reference for other UEs and not by all UEs. Whereas CSI-RS is limited to unicast transmission only.  Further, present CSI-RS will not be sufficient to get better measurements over sidelink. Therefore, new PRS for sidelink should be defined for sidelink positioning having similar characteristics as PRS over Uu link. CSI-RS design can be considered as a baseline for it.

Observation 4: SL-SSB over sidelink is transmitted by sync reference UEs only. Whereas CSI-RS over sidelink has a density lower than what is required for better positioning measurement estimation.

Proposal 4: New positioning reference signal should be designed for sidelink positioning. CSI-RS can be used as the baseline.

### Architectural enhancements

In Uu link positioning, LMF acts as a controlling server which configures gNB/TRP for positioning estimation and provides necessary assisting data to them depending on the positioning method configured. The sidelink positioning is localizing target UE with the help of more than one neighbouring UEs over radio links between target UE and assisting neighbouring UEs. Here, sidelink can be used as co-operative link along with Uu link in cases where Uu link is limited in terms of NLOS, low RSRP, shadow or greater GDOP. On other hand, for few cases like link out of coverage scenarios where the target UE in not connected with any gNB/TRP, sidelink can act as the only link for positioning estimation. In the scenario where sidelink is acting as a cooperative link to Uu, present architecture will be sufficient. The following figures show the possible two cases. In figure 1, the target UE is in coverage of gNB and performing additional measurements along with Uu measurements. Whereas figure 2 shows that the target UE is connected to LMF through anchor UE (may be a UE relay). In both cases, the target UE can be connected with LMF and so present positioning architecture will be sufficient. But for out of coverage case, this will not be sufficient. In out of coverage case, there is need of LMF equivalent node which will carry out LMF function for positioning.

Figure 2:  In coverage architecture- The position of the target UE is limited by its coverage and so the neighbouring UE participates in positioning under the control of NG-RAN/gNB.

Figure 3: Partial coverage scenario architecture- The target UE is out of coverage but can connect to gNB via a UE relay/ Anchor UE. Anchor UE manages the assisting UE (A-UEs) under the control of NGRAN/BS.

Observation 5: For in coverage and partial coverage scenarios, present positioning architecture should be sufficient.

Observation 6: For out-of-coverage, architecture enhancement is necessary for positioning.

Proposal 5: Out-of-coverage case should be considered for sidelink-based positioning architecture enhancements.

### Conclusion:

The contribution draws the following observations and conclusions,

Observation 1: Achievable positioning accuracy is proportional to the PRS bandwidth configured. Therefore, appropriate bandwidth should be allocated for each use case.

Observation 2: For angle-based methods, antenna consideration at UE is an important factor. This needs to be studied considering UE capabilities and possible sidelink positioning architecture.

Observation 3: For M-RTT based method, tight synchronisation is required which will be a big challenge to enable it over sidelink for positioning.

Observation 4: SL-SSB over sidelink is transmitted by sync reference UEs only. Whereas CSI-RS over sidelink has a density lower than what is required for better positioning measurement estimation.

Observation 5: For in coverage and partial coverage scenarios, present positioning architecture should be sufficient.

Observation 6: For out-of-coverage, architecture enhancement is necessary for positioning.

Proposal 1: For Rel 18 study, both time-based (TDOA, M-RTT) and angle-based (AOD, AOA) positioning methods should be considered.

Proposal 2: Time difference of arrival (TDOA) method should be defined for sidelink to measure RSTD for positioning over sidelink.

Proposal 3: For angle-based methods for sidelink positioning, antenna configuration consideration should be studied using practical UE capabilities and sidelink positioning architecture.

Proposal 4: New positioning reference signal should be designed for sidelink positioning. CSI-RS can be used as the baseline

Proposal 5: Out-of-coverage case should be considered for sidelink-based positioning architecture enhancements.

### References:

- RP-213588 - Revised SID on Study on expanded and improved NR positioning

- R1-2204753- Discussion on sidelink based positioning requirements & scenarios

- R1-2204754 - Discussion on evaluation methods and results of sidelink based positioning

- TS 38.845 V 2.0.0- Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases