3GPP TSG RAN WG1 Meeting #109-e	R1-2204756

May 9th – May 20th, 2022

Agenda Item:	9.7.2

Source:	CEWiT, IIT-M, IIT-K, Reliance Jio, Saankhya Labs

Title:	Network energy saving techniques

Document for:	Discussion

# Introduction

A new study item on “Study on network energy savings for NR” was approved in RAN#94-e [1] with following objective:

The objectives of the study are the following:

- Definition of a base station energy consumption model [RAN1]

- Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations.

- Definition of an evaluation methodology and KPIs [RAN1]

- The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed.

Note: WGs will decide KPIs to evaluate and how.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2]

- Information exchange/coordination over network interfaces [RAN3]

Note: Other techniques are not precluded

The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed.

In this contribution, we focus on third objective of the study item and present techniques required to improve network energy savings (NES).

# Techniques for NES at gNB

In NR, irrespective of the load and number of connected UEs, the gNB has to perform set of mandatory operations (e.g., transmission of SSB) to ensure the availability of the cell to the UEs and for maintenance of the link. These mandatory operations consume energy at the gNB even when gNB is not serving any active UE. The gNB cannot avoid or skip these operations otherwise it will adversely affect the initial access, synchronization procedures, and user performance. For e.g., increasing the periodicity of SSB transmission can cause delay at UE in accessing the cell. Simplifying the signals associated with the mandatory set of operations and optimizing the operations in terms of energy consumption and performance helps in power saving at the gNB without creating much effect in performance. Assistance information defined in the standards or provided by the neighboring gNBs or connected UEs helps in reducing the impact of simplified set of operations on performance.

Observation 1: Mandatory set operations consume energy at the gNB irrespective of the load.

Proposal 1: Reducing the complexity of signals associated with mandatory set of operations at the gNB and optimizing the operations in terms of energy consumption is supported.

The energy saving at the gNB can be achieved by enabling sleep states without any transmission and reception or by deactivating some components e.g., RF components or cells or beams. The gNB can activate sleep state for a cell requiring no communication in UL or DL for certain duration of time in dynamic and/or semi-static way based on UE traffic and arrival rate. Adaption of sleep state can be in symbol, slot, sub-frame, frame level. The longer sleep state provides dormancy for more RF components and operations, thereby providing more energy saving. However, it causes delay and degradation in system performance, whereas and the shorter sleep states have quick activation/de-activation of the cell causing lower delay and less impact in the user experience and system performance. Therefore, it is necessary to study and define sleep states for gNB. Further, the sleep states will have an impact on the  legacy operations e.g., mobility measurements. Therefore, it is essential to study the impact of sleep mode on legacy operation.

Proposal 2: Define sleep state for gNB and study the specification impact.

In NR, with the usage of higher frequencies and requirement for narrow beams the usage of number of antenna, TRX chains and antenna elements has increased which is a dominant contributor for energy consumption at the gNB. The energy consumption at gNB can be reduced by adapting number of the antenna components used for transmission or receptions based on parameters like Channel condition, QoS requirement and traffic load. For e.g., the gNB can deactivate some of antenna elements and transmit or receive with less number of antenna ports as shown in Figure 1. The adaptation of antenna components based on channel condition, UE arrival rate, user density etc can reduce the network energy consumption in an optimal way. The adaptation of antenna components will have an impact on the transmission/reception of channels/signals. For e.g., CSI-RS.

a) 32 Ports

b) 16 Ports

Active antenna elements

De-active Antenna elements

Figure 1: Antenna panel with 32 and 16 ports in (a) and (b) respectively

Proposal 3: Adapting the active antenna components is supported for NES and study the impact on specifications.

In NR, the EPRE of downlink signals and channels is derived from the downlink transmit power provided by higher layers to the UE. The EPRE is same across all the resource elements and bandwidth of the signal/channel. The semi static updation of the downlink transmit power which is provided to the UE causes unnecessarily power consumption at gNB in certain cases. For example, in a good channel condition or in a case of UEs near to the gNB, dynamic reduction in the transmit power will help to conserve energy at the gNB. Thus, dynamic adaptation of downlink transmission power is supported for NES. The adaptation should take into account the dynamic variations like variable channel condition, UE density, traffic arrival rate, etc. to achieve an optimized transmission power. Some assistance information from the UE, e.g., location information, helps in the dynamic adaptation of DL power at gNB.

Proposal 4: Dynamically adapting the DL transmission power at gNB utilizing assistance information from the UE is supported.

# Conclusion

In this contribution, we discussed the techniques required for NES. According to the technical analysis, the following proposals are made,

Observation 1: Mandatory set operations consume energy at the gNB irrespective of the load.

Proposal 1: Reducing the complexity of signals associated with mandatory set of operations at the gNB and optimizing the operations in terms of energy consumption is supported.

Proposal 2: Define sleep state for gNB and study the specification impact.

Proposal 3: Adapting the active antenna components is supported for NES and study the impact on specifications.

Proposal 4: Dynamically adapting the DL transmission power at gNB utilizing assistance information from the UE is supported.

# References

- RP-213554, New SI: Study on network energy savings for NR, RAN#94-e, Dec. 2021

- RP-213488, Moderator’s summary for [94e-08-R18-NetworkEnergy] Network energy savings.