3GPP TSG RAN WG1 #109-e			R1-2204760

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Discussion on HARQ-ACK multiplexing on PUSCH

Agenda item:	7.1

Document for:	Decision

Introduction

In RAN1 108-e meeting, RAN1 further discussed HARQ-ACK multiplexing on PUSCH without PUCCH due to PDCCH miss-detection. Unfortunately, RAN1 failed to make any conclusion for Rel-15/Rel-16 [1]. In this contribution, we discussed potential solutions focusing on Rel-17 unified solution.

# Discussion

## 2.1 Unified solution for HARQ-ACK multiplexing on PUSCH

After several round discussion in several meetings, companies almost converge to a unified solution for cases with single or more than one PUSCH with UL-TDAI not equal to 4 (for type-2 codebook) or equal to 1 (for type 1 codebook), though details are still FFS.

Proposal 4-1: For Rel-17 UEs, for a unified design, the following should be specified: - Selection of the candidate PUSCH for multiplexing - Candidate PUSCHs - Alt-1: All the PUSCHs within the PUCCH slot are candidates - Alt-2: PUSCHs without UL-TDAI=4 in case Type 2 CB, and without UL-TDAI n.e. 1 in case of Type 1 CB within the PUCCH slot are candidates - N/W sets all TDAI values that overlap with PUCCH to the [same/different] value with TDAI n.e. 4 - Prioritization rules to select PUSCH for multiplexing. Prioritization rules are identical to 38.213 - Limitations for multiplexing - UE expects to multiplex HARQ-ACK on only 1 PUSCH selected based on step 2 in the PUCCH slot. - All the PUSCHs in the determined candidate set after step 1 have to satisfy Rel-15 UCI multiplexing timeline, defined with respect the starting symbol of the earliest PUSCH transmission in the candidate set. - gNB will not schedule any additional PUSCH in the PUCCH slot that does not satisfy the multiplexing timeline

FFS points for step 1 include whether candidate PUSCHs include PUSCHs with UL-TDAI equal to 4 (type-2 codebook) and equal to 1 (type-1 codebook), and whether UL-TDAI values for PUSCHs overlapping with PUCCH should be the same.

Alt-2 with same T-DAI values for PUSCHs overlapping with PUCCH would a natural solution, if main purpose of whole discussion is to handle the case of miss-detected DL assignment for PDSCH (PUCCH). gNB could set proper UL-TDAI for a PUSCH which is expected to be chosen to carry UCI. To ensure some scheduling flexibility at gNB side, e.g., gNB may schedule a PUSCH1 overlapping with a PUCCH and later schedule a PUSCH2 also overlapping a PUCCH and the PUSCH2 would be the PUSCH selected for UCI multiplexing, it is reasonable to support multiple PUSCHs with UL-TDAI not equal to 4 for type-2 codebook or not equal to 1 for type-1 codebook, but there is no clear motivation to set different UL-TDAI value for these PUSCHs, because all HARQ-ACKs to be multiplexed with a PUSCH are available before UL grant for any of the overlapped PUSCHs.

Proposal 1: To select the candidate PUSCH for multiplexing, PUSCHs without UL-TDAI=4 in case Type 2 CB, and without UL-TDAI n.e. 1 in case of Type 1 CB within the PUCCH slot are candidates, and NW sets all TDAI values that overlap with PUCCH to the same value with TDAI n.e. 4.

A PUCCH slot can be shorter or longer than a slot for PUSCH transmission, e.g., due to different SCS or due to sub-slot based PUCCH resource configuration. Typically, SCS for a Pcell is smaller than Scells. Thus, PUCCH slot duration shorter than a slot for PUSCH can be considered as a corner case, though the specification still supports such case. For PUCCH sub-slot, it can be considered as a typical configuration for latency sensitive traffic. Therefore, RAN1 should consider how to handle HARQ-ACK multiplexing on PUSCH without PUCCH, as well as for the case of PUCCH with sub-slot configuration.

It seems straightforward to consider all PUSCHs overlapping with a PUCCH slot as candidate PUSCHs for the PUCCH slot regardless of PUCCH slot duration, and UE can select one PUSCH from candidate PUSCHs for HARQ-ACK multiplexing according to UL T-DAI as mentioned above. However, due to uncertainty of PUCCH location in a PUCCH slot, UE may select a wrong PUSCH which is different from the PUSCH expected by gNB.

Figure 1-1 provides an example of PUSCH overlapping with two PUCCH slots. Assuming PUCCH1 and PUCCH2 in sub-slot 0 and sub-slot 1 respectively, PUSCH 1 on Scell 1 cross PUCCH sub-slot boundary while PUSCH 2 on Scell 2 is only within sub-slot 1. If UE correctly identifies PUCCH1 and PUCCH2, UE multiplexes PUCCH1 with PUSCH1 and PUCCH2 with PUSCH2 respectively. However, if UE fails to identify PUCCH2 in sub-slot 1, it is unclear whether PUSCH candidates within a PUCCH sub-slot (sub-slot1) include both PUSCH1 and PUSCH2, or only PUSCH 2.

For example, if PUSCH candidates include any PUSCH overlapping with the PUCCH sub-slot, then, PUSCH1 should be one of PUSCH candidates. Unfortunately, UE selects PUSCH1 for sub-slot 1 that leads to more than one HARQ-ACK PUCCH multiplexed into one PUSCH, which is error case and unexpected by gNB. Note that UE behaviour is not specified for the error case, which leads to potential drop of PUCCH1.

Furthermore, for some cases, UE may not identify whether a PUCCH is missed in a PUCCH slot. As shown in Figure 1-2, gNB may or may not schedule a PUCCH2 in sub-slot 1. Further, based on the rule as mentioned in Proposal 1, UE may not identify whether gNB schedules a PUCCH2 in sub-slot 1. If UE assumes a PUCCH2 in sub-slot 1 and UE selects PUSCH1 to multiplex PUCCH2, PUSCH1 would multiplex with two PUCCHs, which leads to same issue as mentioned above.

To avoid impact on HARQ-ACK multiplexing on PUSCH due to a potentially miss-detected PUCCH, UE behaviour should be specified for this case. For simplicity, if a UE identifies more than one PUCCH with HARQ-ACK to be multiplexed in the same PUSCH, UE only multiplexes one PUCCH onto the PUSCH, e.g., the one PUCCH is associated with detected DL assignment or SPS PDSCH. As shown in Figure 1-1 and Figure 1- 2, UE only multiplexes HARQ-ACK from PUCCH1 onto PUSCH1, and UE does not multiplex HARQ-ACK from PUCCH2 onto any PUSCH.

| Figure 1-1 PUSCH1 across two PUCCH sub-slots | Figure 1-2 PUSCH1 and 2 across two PUCCH |
| --- | --- |

Proposal 2: RAN1 to clarify that PUSCHs within a PUCCH slot include PUSCHs overlapping with the PUCCH slot.

- If the UE identifies more than one PUCCH with HARQ-ACK to be multiplexed in the same PUSCH due to uncertainty of miss-detected PUCCH, UE only multiplexes HARQ-ACK from one PUCCH onto the PUSCH.

## 2.2 PUSCH repetition

According to the current specification as captured below, if UE would transmit a PUCCH and the PUCCH overlaps with multiple PUSCH repetitions, UE would multiplex UCI on every PUSCH repetitions overlapping with the PUCCH.

TS 38.213 Clause 9 If a UE transmits a PUSCH over multiple slots and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots.

If UE miss-detects all PDCCHs for a PUCCH, UE behaviour for HARQ-ACK multiplexing on PUSCH without PUCCH should be defined for PUSCH repetition case. In our view, a unified solution is preferred for all different cases including single PUSCH, multiple PUSCHs and multi-slot PUSCH, including PUSCH repetitions and TB processing over multiple slots (TBoMS).  Therefore, at least one PUSCH should be selected to multiplex HARQ-ACK for PUSCH repetition case rather than not to multiplex HARQ-ACK into any PUSCH repetition.

For one or more PUSCH repetitions for HARQ-ACK multiplexing, on one hand, multiplexing HARQ-ACK on all the PUSCHs in the PUCCH slot is more aligned with UE behaviour when UE correctly identifies PUCCH. On the other hand, if UE fails to identify PUCCH, the overhead of all NACKs in every PUSCH repetition in a PUCCH slot would lead to worse PUSCH resource efficiency if the intended PUCCH only overlaps with some of PUSCH repetitions in the PUCCH slot. Therefore, multiplexing HARQ-ACK in 1st PUSCH repetition within a PUCCH slot would be a reasonable solution.

Proposal 3: For multi-slot PUSCH case, 1st PUSCH repetition within a PUCCH slot is selected for HARQ-ACK multiplexing.

## 2.3 Effect of CSI

Within a PUCCH slot, if a PUCCH with CSI overlaps with a PUCCH with HARQ-ACK, both CSI and HARQ-ACK are multiplexed into one PUSCH. The PUSCH might be different from a PUSCH for CSI multiplexing, if the PUCCH for HARQ-ACK is not present. As shown in Figure 2-1, gNB expects PUSCH1 for HARQ-ACK and CSI multiplexing, while gNB expects PUSCH1 for HARQ-ACK and PUSCH2 for CSI respectively in Figure 2-2. If UE miss-detects PDCCH for PUCCH2, it is unclear how UE determines PUSCH for CSI multiplexing.

For example, whether UE chooses same PUSCH for HARQ-ACK and CSI in a PUCCH slot, or UE chooses a PUSCH for CSI multiplexing as if no PUCCH for HARQ-ACK, if UE miss-detects PDCCH for HARQ-ACK. For simplicity, UE can multiplex CSI into same PUSCH for HARQ-ACK, though gNB may need to blindly detect PUSCH for HARQ-ACK and CSI multiplexing respectively. For example, in the Figure 2-2UE multiplexes HARQ-ACK and CSI in PUSCH1, and gNB needs to detect with the assumption of HARQ-ACK and CSI in PUSCH1 or HARQ-ACK in PUSCH1 and CSI in PUSCH2.

| Figure 2-1 CSI + HARQ-ACK multiplexing on same PUSCH | Figure 2-2 CSI and HARQ-ACK multiplexing on different PUSCH |
| --- | --- |

Proposal 4: If a UE fails to receive any PDCCH for a PUCCH with HARQ-ACK in a PUCCH slot, UE multiplexes CSI in the same PUSCH for HARQ-ACK.

Conclusions

In summary, we have the following list of proposals:

Proposal 1: To select the candidate PUSCH for multiplexing, PUSCHs without UL-TDAI=4 in case Type 2 CB, and without UL-TDAI n.e. 1 in case of Type 1 CB within the PUCCH slot are candidates, and N/W sets all TDAI values that overlap with PUCCH to the same value with TDAI n.e. 4.

Proposal 2: RAN1 to clarify that PUSCHs within a PUCCH slot includes PUSCHs overlapping with the PUCCH slot.

- If the UE identifies more than one PUCCH with HARQ-ACK to be multiplexed in the same PUSCH due to uncertainty of miss-detected PUCCH, UE only multiplexes HARQ-ACK from one PUCCH onto the PUSCH.

Proposal 3: For PUSCH repetition case, 1st PUSCH repetition within a PUCCH slot is selected for HARQ-ACK multiplexing.

Proposal 4: If a UE fails to receive any PDCCH for a PUCCH with HARQ-ACK in a PUCCH slot, UE multiplexes CSI in the same PUSCH for HARQ-ACK.

References

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #108e-Meeting, February, 2022.