3GPP TSG RAN WG1 #109-e	R1-2204762

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Discussion on gap symbol collision handling for antenna switching

Agenda item:	7.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide some discussion on the gap symbols collision handling for SRS antenna switching operation in Rel-15.

# Discussion on SRS antenna switching

## 2.1 Collision handling for the gap symbols

According to the Rel-15 spec of 38.214 [1], the gap symbols for antenna switching are also used for collision handling. When performing collision handling, the priority of the gap symbols is the same as SRS.

However, in the section of SRS antenna switching, it is also specified that the UE should not transmit any other uplink signal during the gap period.

The corresponding spec text from Rel-15 38.214 [1] is copied as below.

## 6.2.1 UE sounding procedure … When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured according to clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured. … ## 6.2.1.2 UE sounding procedure for DL CSI acquisition … The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. …

It can be observed that the description regarding the collision handling for gap period is contradictory. For example, there is PUCCH for ACK which collides with the gap symbol. According to the description in Section 6.2.1 of 38.214, the gap symbols should follow the priority handling for SRS, i.e., the PUCCH for ACK should be transmitted. But according to the description in Section 6.2.1.2 of 38.214, any other uplink signal should not be transmitted over the gap period, i.e., the PUCCH for ACK should be dropped.

Therefore, the collision handling for gap symbols for antenna switching is inconsistent in the current Rel-15 spec and should be clarified.

In RAN1 #108-e meeting, for antenna switching in Rel-17, it was agreed that for inter-set guard period, if the interval between SRS resource sets is Y symbols, when both the SRS resource prior to the gap and the SRS resource after the gap are dropped, then the gap symbols are also dropped and can be used for UL transmission.

The corresponding spec text in Rel-17 38.214 [2] is copied as below.

## 6.2.1.2 UE sounding procedure for DL CSI acquisition … For the inter-set guard period, the UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. …

Hence, the similar solution could be used to address the issue of intra-set gap period in Rel-15 spec, since the gap period in Rel-15 is always Y symbols.

Proposal 1:

- For Rel-15 SRS antenna switching, when both the SRS resource prior to the gap and the SRS resource after the gap are dropped due to collision handling, the gap symbols are also dropped and can be used for UL transmission.

Correspondingly, we have the following proposal for the spec change.

Proposal 2:

- Adopt the following text change for Rel-15 38.214

## 6.2.1.2 UE sounding procedure for DL CSI acquisition … The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. The UE does not transmit any other signal on any symbols of the guard period. - When both the SRS resource on all of the corresponding symbols prior to the guard period and the SRS resource on all of the corresponding symbols after the guard period are dropped due to collision handling, the guard period is also dropped with same priority and can be used for UL transmission. …

# Conclusion

In conclusion, we have the following proposals on SRS antenna switching operation in Rel-15.

Proposal 1:

- For Rel-15 SRS antenna switching, when both the SRS resource prior to the gap and the SRS resource after the gap are dropped due to collision handling, the gap symbols are also dropped and can be used for UL transmission.

Proposal 2:

- Adopt the following text change for Rel-15 38.214

## 6.2.1.2 UE sounding procedure for DL CSI acquisition … The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set. The UE does not transmit any other signal on any symbols of the guard period. - When both the SRS resource on all of the corresponding symbols prior to the guard period and the SRS resource on all of the corresponding symbols after the guard period are dropped due to collision handling, the guard period is also dropped with same priority and can be used for UL transmission. …

# References

- 3GPP TS 38.214 v15.16.0, Physical layer procedures for data

- 3GPP TS 38.214 v17.1.0, Physical layer procedures for data

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #108-e, February 21st – March 3rd, 2022