3GPP TSG RAN WG1 #109-e	R1-2204765

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Remaining details on SRS enhancements

Agenda item:	8.1.3

Document for:	Discussion and Decision

# Introduction

In RAN1 #108-e meeting, the following agreements on SRS enhancement were reached.

Agreement The following TPs in Section 5 of R1-2201201 are endorsed for the editors CR - TP 2-1 – Part 1: For TS38.214 subclause 6.2.1 on AP SRS triggering - TP 4-1: For TS38.211 to include SRS repetition with {10, 14} consecutive OFDM symbols - TP 4-3: For TS38.211 subclause 6.4.1.4.3 to correct an error on SRS resource mapping formula Agreement When RPFS is configured, UE expects the length of the SRS sequence to be a multiple of 6. Agreement Confirm the following working assumption To support 4 ports with Max CS = 6, - Port 0 and Port 2 locate in n_CS and (n_CS+3) mod 6 in comb offset k_TC, respectively. - Port 1 and Port 3 locate in n_CS and (n_CS+3) mod 6 in comb offset (k_TC + 4) mod 8, respectively. - Note: n_CS and k_TC are the configured CS and comb offset values. - Note: This working assumption can be revisited if Max CS = 12 is agreed. Agreement FL Proposal 3-1: Support N = 1 for aperiodic SRS configuration for 1T4R - This new configuration is UE optional. Agreement TP 3-1A and TP 4-2 in Section 3 of R1-2202625 are endorsed for the editor’s CR on TS38.214. Agreement RPFS is applicable for both frequency hopping and non-frequency hopping cases, where support of RPFS for non-FH case is an optional UE feature for UEs supporting RPFS. Agreement For inter-set guard period, UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. - The above is the only collision handling rule to be introduced in Rel-17 for antenna switching guard period

In this contribution, we provide our views on the remaining details of SRS enhancements for Rel-17.

# Remaining details on SRS enhancements for Rel-17

## 2.1 Antenna switching for up to 8 antennas

### 2.1.1 Guard period

In RAN1 #108-e meeting, it was agreed that for inter-set guard period, if the interval between SRS resource sets is Y symbols, when both the SRS resource prior to the gap and the SRS resource after the gap are dropped, then the gap symbols are also dropped.

One remaining issue is how to handle the gap symbols when the inter-set interval is larger than the required Y symbols for gap operation.

Since the gap symbols are treated with the same priority as SRS for collision handling, the gap symbol position should be determined, otherwise the gNB and the UE may have different understanding on whether some symbols are dropped or not.

Therefore, it is preferred that if the interval between SRS resource sets is larger than Y symbols, the first Y symbols are used as gap symbols.

Proposal 1:

- For antenna switching, the gap symbol position should be determined when the interval between two SRS resource sets is larger than the gap period, i.e., the first Y symbols in the interval are used as gap symbols

When performing collision handling for the gap period, if the SRS resource prior to the gap symbols is dropped, then the gap symbols should also be dropped.

Proposal 2:

- For antenna switching, when the interval between two SRS resource sets is larger than the gap period, if the SRS resource prior to the gap is dropped due to collision handling, the gap symbols are also dropped and can be used for UL transmission

# Conclusion

In conclusion, we have the following proposals on SRS enhancements in Rel-17.

Proposal 1:

- For antenna switching, the gap symbol position should be determined when the interval between two SRS resource sets is larger than the gap period, i.e., the first Y symbols in the interval are used as gap symbols

Proposal 2:

- For antenna switching, when the interval between two SRS resource sets is larger than the gap period, if the SRS resource prior to the gap is dropped due to collision handling, the gap symbols are also dropped and can be used for UL transmission

# References

- 3GPP TS 38.214 v17.10, Physical layer procedures for data

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #108-e, February 21st – March 3rd, 2022