3GPP TSG RAN WG1 Meeting #109-e	R1-2204769

e-Meeting, May 09 – 20, 2022

Source:	Intel Corporation

Title:	Discussion on Channel Access Mechanism for extending NR up to 71 GHz

Agenda item:	8.2.4

Document for:	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #86, a new SID on studying the support of NR from 52.6 GHz to 71 GHz (RP-193228) [1] was approved. Updated working item (WI) has been approved with the aim to extending NR up to 71 GHz [2] in the 3GPP TSG RAN Meeting #93-e. As part of the objectives of the WI, the following aspects were included:

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement - Radio interface protocol architecture and procedures [RAN2]: - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified. - Core specifications for UE, gNB and RRM requirements [RAN4]: - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range. - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - Specify RRM/RLM/BM core requirements. Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives. Note 2: UEs supporting a band in the range of 52.6GHz-71GHz are not required to support 480kHz SCS and 960kHz SCS. Note 3: The maximum FFT size required to operate the system in 52.6GHz-71GHz frequency is 4096, and the maximum of RBs per carrier is 275 RBs. Note 4: the system is designed to support both single-carrier and multi-carrier operation. Note 5: RAN plenary will decide whether new FR (e.g. FR3) shall be defined for the frequency range from 52.6GHz-71GHz or the existing FR2 shall be extended to cover frequency range from 52.6GHz-71GHz. Similar to regular NR and NR-U operations below 52.6GHz, NR/NR-U operation in the 52.6GHz to 71GHz can be in stand-alone or aggregated via CA or DC with an anchor carrier.

In this contribution, in the context of channel access to extend NR up to 71 GHz, and based on the agreements and conclusions made during the previous RAN1 meetings along this WI [2-10], the following aspects will be discussed:

- Channel access indication within Fall-back DCIs;

- Cyclic prefix extension for CG UL transmissions;

- Definition of Pout within the EDT calculation;

- Details related to the short signaling exemption;

- Details related to multi-beam channel access procedure;

- Details Related to COT sharing procedure;

- Correction of editorials.

# Channel Access Indication within Fall-back DCIs

In RAN1 #107 meeting [8], it was agreed that for FR2-2 the field ChannelAccess-CPext carried in the non-fallback DCIs is reinterpreted to carry information indicating which channel access type to use. In particular, the following was agreed:

Agreement For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

However, no consensus was reach regarding the details of the channel access indication for both the fallback DCI format 0-0 and 1-0, and the RAR UL grant. In this matter, even though type 2 channel access mechanism may be only supported based on the UE’s capability, the gNB should be still allowed to configure/indicate this channel access type when supported.

Proposal 1: For fallback DCI formats 0_0/1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field indicates one of the entries of a table which entries are “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

In order to capture the above intent, the following text should be captured in Sec. 7.3.1.1.1, and 7.3.1.2.1 of TS 38.212:

Reasons for change: Currently no indication related to which channel access type should be used are included in the fall-back DCIs for FR2-2, which means this DCIs cannot be used for unlicensed operation in this band, given that a UE is unaware of this information and whether it may operate within out outside of gNB’s COT, and more importantly compliance with the regional regulatory requirements cannot be always guaranteed. In order to solve this issue, the following CR is proposed.

Summary of change: To indicate channel access types within fall-back DCIs.

------------------------------ TP#1: TS 38.212 ----------------------------------- *** Unchanged text is omitted *** ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: *** Unchanged text is omitted *** - ChannelAccess-CPext –2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bit otherwise. *** Unchanged text is omitted *** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: *** < Unchanged parts are ommitted> *** - ChannelAccess-CPext –2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bit otherwise. *** Unchanged text is omitted *** Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 in frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0. 3; -; -. Table 7.3.1.1.1-4B: Channel access type in frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. 3; Reserved. *** Unchanged text is omitted *** ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: *** Unchanged text is omitted *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bits otherwise *** Unchanged text is omitted *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: *** Unchanged text is omitted *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bits otherwise *** Unchanged text is omitted ***

Furthermore, the following text should be captured in Sec. 8.2 and 8.2A of TS 38.213:

Reasons for change: Currently no indication related to which channel access type should be used are included in the RAR UL grant for FR2-2, which means that compliance with the regional regulatory requirements cannot be always guaranteed, and COT sharing during random access procedure cannot be performed. In order to solve this issue, the following CR is proposed.

Summary of change: To indicate channel access types within RAR UL grant.

------------------------------ TP#2: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 8.2 Random access response - Type-1 random access procedure *** Unchanged text is omitted *** The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided for frequency range 1. For operation in a cell in frequency range 2-2 if LBT-mode is provided and indicates that channel access procedures would need to be performed [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212. Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 0, for operation without shared spectrum channel access, or for operation with shared spectrum access in frequency range 2-2 when LBT-mode indicates that channel access procedures would not need to be performed or LBT-mode is not provided, 2, for operation with shared spectrum channel access in frequency range 1, or in frequency range 2-2 when LBT-mode is provided and indicates that channel access procedures would need to be performed.. ## *** Unchanged text is omitted *** ## 8.2A Random access response - Type-2 random access procedure *** Unchanged text is omitted *** If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer - an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or - transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where - a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon - a slot for the PUCCH transmission is indicated by a HARQ Feedback Timing Indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆+2μ∙Kcell,offset, where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], μ is the SCS configuration of the active UL BWP, and Kcell,offset is provided by Koffset in ServingCellConfigCommon; otherwise, if not provided, Kcell,offset=0 - the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214] - for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided for frequency range 1. For operation in a cell in frequency range 2-2 if LBT-mode is provided and indicates that channel access procedures would need to be performed [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212. - the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission ## *** Unchanged text is omitted ***

Proposal 2: TP#1 and TP#2 should be supported.

In prior RAN#1 meetings [3-10], the following was agreed:

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed Agreement Support gNB as the initiating device to resume transmission within maximum COT without a Cat 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device - Note: This is motivated by regions where LBT is not required before each transmission (say outside Japan)? - Note: This should only be used when allowed by local regulation Agreement Support gNB as the initiating device to resume transmission with a Cat 2 LBT if there is gap longer than Y us from the previous transmission from initiating device or responding device - Note this is motivated by regions where LBT is required before each transmission (say Japan) - Y is left for initiating device implementation and should comply with local regulation but no less than 8us

While these agreements imply that either an initiating or responding device may resume a transmission within an MCOT, it is still unclear how a UE performing an UL and indicated or configured to use Type 1 channel access, would decide which LBT to use if that UE later finds out, through DCI 2_0 detection, that the transmission falls in a gNB COT. In this matter, it is important to remind that there are regions, such as Japan, that mandate the use of LBT on any transmission, and therefore in this case type 2 channel access should be used, instead of type 3 channel access. In this matter, an explicit indication is needed and a gNB through a cell-specific RRC parameter in SIB1 should provide such information.

Proposal 3: The gNB indicates through a cell-specific RRC parameter in SIB1 whether Type 2 channel access or Type 3 channel access should be used (subject to UE capability) by a UE within a gNB’s shared COT.

# Cyclic Prefix Extension for CG UL Transmissions

In RAN1 #107 meeting [8], it was concluded that no cyclic prefix (CP) extension would be used for F2-2 for DCI scheduled UL transmissions, but it was left for further study whether a CP extension should be supported for CG-PUSCH transmissions:

Conclusion Rel.16 NR-U style Cyclic Prefix extension is not supported for FR2-2 at least for DCI scheduled UL transmission - FFS: If CP extension is supported for CG-PUSCH in FR2-2

In this matter, it is important to clarify that a CP extension applied in FR2-2 to a CG-PUSCH may not be intended to create a suitable gap for COT sharing, but may be still used to mitigate mutual interference among UEs so that UEs may start their transmission with a proper offset between them, but could also be used so that to prioritize UL scheduled transmissions over UL CG transmissions.

It is worth recalling that in Rel.16 NR-U a CP extension is applied to the first symbol of the configured resources based on a defined set of the offset values {16, 25, 34, 43, 52, 61, k=1CTsymb, (l-k)mod 7∙2μμ}, which is used both when a UE performs a CG PUSCH transmission within and outside of a gNB-initiated COT with the aim to mitigate mutual interference. Furthermore, a grant based PUSCH is prioritized over a CG PUSCH through proper scheduling, but also through proper configuration of the aforementioned offset. In this regards, the offset values were picked taking in mind that: i) the starting position offsets are not necessarily applied within symbol #0, ii) multiple subcarrier spacings are supported, iii) the slot granularity of 11ac is 9 us, and the offset values can be aligned across the same granularity, and iv) a gap of at least 16us must be left between a scheduled and a CG transmission to perform a 16us LBT. In this matter, the CP extension and the starting offsets were derived as follows:

Text=k=12μTsymb,  l-kmod 7∙2μ μ-Δi

where Δi is selected from the following set of values {16, 25, 34, 43, 52, 61, k=12μTsymb,  l-kmod 7∙2μ μ}.

With that said, similar approach can be used for FR2-2, with the difference that the slot granularity of 11ad/ay is 5us and for FR2-2 at most a gap of 8us is required given that this is the observation length for the type 2 LBT procedure. Therefore, while the small starting offset value should not be 8 us, the granularity among the values within the set should be 5us.

Proposal 4: Support cyclic prefix extension for CG-PUSCH transmissions in the FR2-2 frequency range using the same design principle as NR-U.

- The first starting offset value should be equal to 8us and the granularity among the set of starting offsets should be equal to 5us.

# Definition of Operating BW and Pout within the EDT Calculation

In prior RAN#1 meetings [3-10], the EDT threshold to use to determine whether a channel is busy or not has been defined for FR2-2 as per the following agreement:

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

While all the details have been discussed, as for the definition of Pout, the following agreements have been made during the prior RAN1 meetings [3-10]:

Working assumption: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT. Agreement Confirm the WA with some clarifications Working assumption: - For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT. - The node is not expected to transmit in the COT with higher Pout than the Pout used to determine the EDT used to acquire the COT Agreement For gNB initiated COT, for Pout in EDT determination at the initiating device (gNB), the Pout of the responding device (UE) is not considered

However, in case an initiating device may perform a per-beam LBT procedure, it is not yet clear how the Pout is calculated in case the maximum EIRP in different beams may be different. In this case, our view is that for the matter of the individual calculation of the ED threshold the Pout should be accounting only for the maximum EIRP in the transmission beams that are covered by that specific sensing beam, and should not account for all other beams.

Proposal 5: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during the COT.

# Details Related to the Short Signaling Exemption

Short signal exemption (SSE) is a mechanism to send relatively short signaling transmissions (compared to regular data transmissions) without sensing the channel for presence of other signals. For frequency ranges from 52.6 GHz up to 71 GHz, SSE is described in the most recent draft of EN 302 567 v2.2.0 [10] with application to short management and control frames. The total duration of the short control signaling transmissions is required to be constrained to less than 10 msec within 100 msec observation period per device. While this exemption is supported and can be applied in CEPT/EU regions, this may not be used in other regions even when LBT may be used as an adaptivity method. As an example while in Japan ARIB mandates the use of LBT, this hasn’t defined the concept of SSE. Therefore, this relaxation cannot be applied in the related regions. In this matter, it may be necessary to introduce a new RRC parameter to allow the network to indicate to a UE whether the SSE should be applied or not.

Proposal 6: The gNB indicates through a cell-specific RRC parameter in SIB1 whether the short signal exemption should be applied or not.

In previous RAN1 meetings [3-10], it has been concluded that the SSE rules can apply to the transmission of mg1 for the 4-step RACH and MsgA for the 2-step RACH for all supported SCS as long as the 10% duty cycle is met. However, it was argued how to actually interpret this constraint, and whether the 10% duty cycle is applied per cell or from the perspective of a UE. As indicated along this section, the EN 302 567 v2.2.0 [11] is written from the point of view of the device (equipment), and the limitations imposed for a SSE are meant to be applied per device. Therefore, when applying this to UL channels, the maximum 10 % transmission over an observation of 100ms should be measured from the UE’s perspective.

Proposal 7: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective.

In order to capture the above intent, the following text should be captured in Sec. 4.4.5 of TS 37.213:

Reasons for change: Currently it is unclear from the specification text, whether the short control signaling exemption should apply per transmitting device or per cell, and also how to disable this exemption when this is not supported based on regional regulatory requirements. In order to solve this issue, the following CR is proposed.

Summary of change: To specify that the short control signaling exemption applies per UE and to indicate to the UE when this exemption should be applied.

------------------------------ TP#4: TS 37.213 ----------------------------------- ### *** Unchanged text is omitted *** ### 4.4.5 Exempted transmissions from sensing In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, if the higher layer parameter ShortControlSignalling-r17 is configured a gNB/UE may transmit the following transmission(s) without sensing the channel: - Transmission(s) of the discovery burst by the gNB; - Transmission(s) of the first message in a random access procedure by the UE. When the gNB/UE transmits the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/UE shall not occupy the corresponding channel more than 10ms over any 100ms interval from the perspective of the transmitting device. *** Unchanged text is omitted ***

Proposal 8: TP#4 should be supported.

# Details Related to Multi-beam Channel Access Procedure

In the context of multi-beam operation, in prior RAN1 meetings [3-10], the following agreements were made:

Agreement For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below: - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, if the node can perform simultaneous sensing in different beams Note: On UE side, no UE capability will be introduced for this purpose. Agreement Within a COT with TDM of beams with beam switching, at least support Alt 1 - Alt 1 (from previous agreement): Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT Agreement Within a COT with TDM of beams with beam switching, Alt 2 is supported if the node has the capability to perform simultaneous sensing in different beams. Alt 3 is allowed as node implementation choice if the node also supports Cat 2 LBT. The use of Alt 2 or Alt 3 is based on node’s implementation. - Alt 2 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch Agreement When independent per-beam LBT sensing is performed at gNB, a transmission is allowed to occur on a beam if the corresponding LBT procedure has been successful before the channel occupancy start time. - Note: For multi-beam transmission, channel occupancy start time corresponding to all Tx beams is aligned. - FFS: When independent per-beam LBT sensing is performed at UE

Given that counters for the per-beam LBT procedure may be independently maintained per each beam, one aspect that must be yet clarified in this matter is on how and when to declare the channel clear or idle for a beam and when to reinitialize each counters for each beam. Additionally, another aspect that should be jointly clarified is on how to handle the likely situation that the independent per beam LBT procedures may terminate in different instances of time. In this sense, in order to initiate a COT over multiple beams, independent type 1 channel access procedure should be performed, where each counter is drawn and re-initialized per beam, while the COT should be aligned across all beams. Furthermore, when acquiring a new COT, type 1 should never start before the end of a previous COT.

Proposal 9: When independent per-beam LBT sensing is performed, each time a device attempts to acquire a COT:

- It applies independent Type 1 channel access to each beam;

- The gNB/UE shall re-initialize the counter for each beam;

- The initial value of the counter is independently determined for each beam;

- The count-down process is independent for each beam;

- The start of the channel occupancy time in all beam is aligned.

- To acquire a new COT, the applied Type 1 channel access process for a new COT to each beam shall not start before the end of the previous COT.

Together with an independent per-beam LBT sensing where the sensing is done simultaneously across the beams, it should be also supported an LBT procedure where the LBT sensing is done in a sequential manner one at the time.

Proposal 10: When time-domain switching across beams within the same COT is supported, the per-beam LBT for different beams is also performed in a sequential manner. In particular, the initiating device may sense on a beam before either transmitting on that beam or switching to a separate beam to perform sensing.

# Details Related to COT Sharing Procedure

In Rel.16, the CG UE’s COT sharing procedure has two modes of operation:

- Mode A: If the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is provided, then the UE is allowed to share its COT with the gNB. In this case, the DL transmission should contain a transmission to the UE that initiated the channel occupancy and can also include non-unicast  and /or unicast transmissions, where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. Under this mode of operation, the UE is configured by cg-COT-SharingList-r16 where cg-COT-SharingList-r16 provides a table configured by higher layer, where each row of the table provides jointly the following information:

  - Channel access priority class used by the UE when acquiring the COT

  - Slot from where the DL transmission could start, which is identified as x+O, where x is the current slot and O is an offset indicated in terms of slots

  - Maximum duration of the DL transmission, which is indicated by D, and provided in terms of slots.

In addition, cg-COT-SharingList-r16 also includes an entry indicating that no COT sharing would be allowed:

cg-COT-SharingList-r16 SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16 CG-COT-Sharing-r16 ::= CHOICE { noCOT-Sharing-r16 NULL, cot-Sharing-r16 SEQUENCE { duration-r16 INTEGER (1.. 39), offset-r16 INTEGER (1.. 39), channelAccessPriority-r16 INTEGER (1..4) } }

An illustration of this modality of operation is provided in Figure 1.

Figure 1 – High level illustration of mode A

- Mode B: If the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is not provided, then if 'COT sharing information' in CG-UCI indicates '1' the UE is allowed to share its COT only for the purpose of DL control transmission for the length of 2/4/or 8 OFDM symbol for 15/30 or 60 KHz subcarrier spacing, respectively. In this case, the gNB can share the UE channel occupancy and start the DL transmission X symbols from the end of the slot where CG-UCI is detected, where cg-COT-SharingOffset-r16 is provided by higher layer.

An illustration of this modality of operation is provided in Figure 2.

Figure 2 – High level illustration of mode B.

Moving forward to Rel.17 for 52.6 GHz band, the following agreement was made:

Agreement In Rel-17, the same ED threshold determination mechanism is used for UL to DL COT sharing and for UL transmission without COT sharing with UE as initiating device. - FFS: Spec impact for UL to DL COT sharing mechanism

However, based on the description of the Rel.16 UE’s COT sharing procedure, it is not clear whether both modes of operation will be supported or only mode B will be supported, which is in line with the agreement above and implies that ul-toDL-COT-SharingED-Threshold-r16 is not applicable for NR operated above 52.6 GHz band. Our view is that for this frequency range, there is no need to support mode B, and only mode A should be supported, given that mode B was defined specifically for sub-6 GHz band to coexist with 11ac.

Proposal 11: When a UE performing a CG transmission shares its COT with a gNB, the gNB is always allowed to perform both unicast and broadcast transmissions without any constrains, and cg-UCI may always indicate one entry of cg-COT-SharingList-r16.

# Editorial Corrections

Along TS 37.213 a few editorials have been identified:

- Along the description of the general channel access procedure in Sec. 4.4, it would be preferrable to use “that” before “channel(s)” in order to indicate that we are referring to the channel over which sensing should be done before transmission(s) should be performed;

- In Sec. 4.4.1, “performing as least” should be substituted with “performing at least”;

- The measurement window definition is the same between type 1 and type 2 channel access procedure. Therefore, when describing independently the two procedure same language should be used when indicating how the measurement is done within the defer duration Td=8μs .

In order to capture the above intent, the following text should be captured in Sec. 4.4, 4.4.1 and 4.4.2 of TS 37.213:

Reasons for change: Along the text a few editorials have been identified: - Along the description of the general channel access procedure in Sec. 4.4, it would be preferrable to use “that” before “channel(s)” in order to indicate that we are referring to the channel over which sensing should be done before transmission(s) should be performed; - In Sec. 4.4.1, “performing as least” should be substituted with “performing at least”; - The measurement window definition is the same between type 1 and type 2 channel access procedure. Therefore, when describing independently the two procedure same language should be used when indicating how the measurement is done within the defer duration Td=8μs .

Summary of change: To correct the aforementioned editorials.

------------------------------------ TP#5: Sec. 4.4, 4.4.1 and 4.4.2 of TS 37.213 ----------------------------------- ### *** Unchanged text is omitted *** ## 4.4 Channel access procedures for frequency range 2-2 When a gNB/UE(s) is required by regulations to sense channel(s) for availability for performing transmission(s) on that channel(s) or when a gNB provides UE(s) with higher layer parameters ChannelAccessMode2-r17 by SIB1 or dedicated configuration indicating that the channel access procedures would be performed before transmission(s) on that channel(s), the channel access procedures described in this clause, for accessing the channel(s) on which the transmission(s) are performed by the gNB/UE(s), are applied. *** Unchanged text is omitted *** 4.4.1 Type 1 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by the sensing slots that are sensed to be idle before a transmission(s) is random based on a fixed contention window size. The clause is applicable to any transmission initiating a channel occupancy by the gNB/UE. *** Unchanged text is omitted *** The defer duration is Td=8μs that ends with a sensing slot of a Tsl=5μs for performing at least a single measurement to determine whether the channel is idle. A gNB/UE shall not transmit on a channel for a Channel Occupancy Time that exceeds 5ms. ### 4.4.2 Type 2 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by sensing slots that are sensed to be idle before a DL/UL transmission(s) is deterministic. A gNB/UE may transmit a transmission(s) on a channel immediately after Td that ends within a sensing slot of duration Tsl=5μs for performing at least a single measurement to determine whether the channel is sensed to be idle. *** Unchanged text is omitted ***

Proposal 12: TP#5 should be supported.

# Conclusions

In this contribution, we discussed several aspects related to channel access enhancements for extending NR up to 71 GHz, and we derived the following proposals, and observations:

Proposal 1: For fallback DCI formats 0_0/1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field indicates one of the entries of a table which entries are “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

Proposal 2: TP#1 and TP#2 should be supported.

Proposal 3: The gNB indicates through a cell-specific RRC parameter in SIB1 whether Type 2 channel access or Type 3 channel access should be used (subject to UE capability) by a UE within a gNB’s shared COT

Proposal 4: Support cyclic prefix extension for CG-PUSCH transmissions in the FR2-2 frequency range using the same design principle as NR-U.

- The first starting offset value should be equal to 8us and the granularity among the set of starting offsets should be equal to 5us.

Proposal 5: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during the COT.

Proposal 6: The gNB indicates through a cell-specific RRC parameter in SIB1 whether the short signal exemption should be applied or not.

Proposal 7: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective.

Proposal 8: TP#4 should be supported.

Proposal 9: When independent per-beam LBT sensing is performed, each time a device attempts to acquire a COT:

- It applies independent Type 1 channel access to each beam;

- The gNB/UE shall re-initialize the counter for each beam;

- The initial value of the counter is independently determined for each beam;

- The count-down process is independent for each beam;

- The start of the channel occupancy time in all beam is aligned.

- To acquire a new COT, the applied Type 1 channel access process for a new COT to each beam shall not start before the end of the previous COT.

Proposal 10: When time-domain switching across beams within the same COT is supported, the per-beam LBT for different beams is also performed in a sequential manner. In particular, the initiating device may sense on a beam before either transmitting on that beam or switching to a separate beam to perform sensing.

Proposal 11: When a UE performing a CG transmission shares its COT with a gNB, the gNB is always allowed to perform both unicast and broadcast transmissions without any constrains, and cg-UCI may always indicate one entry of cg-COT-SharingList-r16.

Proposal 12: TP#5 should be supported.

# References

- RP-193228, “New SID Study on supporting NR above 52.6 GHz”, Intel Corporation, 3GPP TSG RAN Meeting #86, Dec. 2019.

- RP-212637, “Revised WID: Extending current NR operation to 71GHz,” Qualcomm, Intel, 3GPP TSG RAN Meeting #93-e, Dec. 2020.

- RAN1Chairman’s notes, 3GPP TSG RAN WG1 #104 e-Meeting, February 2021.

- RAN1Chairman’s notes, 3GPP TSG RAN WG1 #104b e-Meeting, April 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #105 e-Meeting, May, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106 e-Meeting, August, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106-bis e-Meeting, October, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107 e-Meeting, November, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107b e-Meeting, January, 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #108 e-Meeting, March, 2022.

- Draft ETSI EN 302 567 V2.2.0, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,” Dec. 2020.