3GPP TSG RAN WG1 Meeting #109-E     	                                         	 R1-2204771

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Remaining details on UE complexity reduction for Rel-17 RedCap

Agenda item:	8.6.1

Document for:	Discussion and Decision

# Introduction

While the Rel-17 work item (WI) for the support of Reduced Capability (RedCap) NR devices was mostly completed in RAN1 by the end of RAN1 #107-e meeting, some details remain to be addressed. Some of these were captured as part of the Status Report (SR) on Rel-17 WI on RedCap approved at RAN #94-e [2].

In this contribution, we present our views on such remaining aspects for support of reduced bandwidth (BW) for RedCap UEs that include the following:

- UE behavior when SIB1-configured initial DL BWP for non-RedCap UEs is larger than max RedCap UE BW but separate initial DL BWP is not configured

- Center frequency alignment between initial UL and DL BWPs associated with RACH/Msg3 transmission and Type 1 CSS monitoring respectively

- Presence of SSB in separate initial DL BWP in connected mode for BWP#0 configuration option 1

- RAN2 LS on time offsets between CD- and NCD-SSB.

# UE behavior when SIB1-configured initial DL BWP for non-RedCap UEs is larger than max RedCap UE BW but separate initial DL BWP is not configured

It has been agreed that a RedCap UE can be optionally provided with a separate initial DL BWP via SIB1 such that the BW of the separate initial DL BWP does not exceed the maximum RedCap UE BW in the corresponding band. For initial UL BWP, it was also agreed that in case the initial UL BWP for non-RedCap UEs is larger than max RedCap UE BW, a RedCap UE expects a separate configuration of initial UL BWP with BW that is no larger than max RedCap UE BW.

For initial DL BWP, when a separate configuration is not provided, a RedCap UE is expected to use the one defined by CORESET#0 to receive common control in Idle/inactive modes, etc. However, for non-RedCap UEs, a gNB may configure a value of locationAndBandwidth for the initial DL BWP that is larger than the one defined by CORESET#0 as per Rel-15 operations. In this case, a non-RedCap UE applies configurations provided as part of the initial DL BWP in SIB1 except for the locationAndBandwidth parameter, which it applies upon RRC connection establishment. This functionality should be preserved with the introduction of RedCap UEs as well. For instance, the larger initial DL BWP can be provided to non-RedCap UEs while RedCap UEs could continue in the initial DL BWP defined by CORESET#0 upon RRC connection establishment until reconfigured. This would preserve the existing mechanism of initial DL BWP configuration for non-RedCap UEs while not mandating additional signaling by SIB1 (thereby increasing SIB1 payload) for RedCap UEs when the gNB may decide to serve the RedCap UEs within the DL BWP defined by CORESET#0.

This approach allows a gNB to have the full flexibility to configure a separate initial DL BWP whenever the SIB1-indicated initial DL BWP for non-RedCap UEs is larger than max RedCap UE BW, while also allowing another gNB implementation that prefers to avoid an unnecessary increase in the SIB1 payload which is already a cause for concern regarding SIB1-coverage.

During RAN1 #108-e meeting, there were discussions regarding potential impact to requirements/expectations on center frequency alignment between initial DL BWP (CORESET#0 in this case) and initial UL BWP in TDD deployments. This led to discussions amongst the following options:

High Priority Proposal 2-1-2b: For the case that the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth, the UE behavior is up to RAN2, e.g., according to one of the following options down select between the following options: - Option 1: A separate initial DL BWP is always configured for RedCap if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth. - Note: For TDD, the center frequencies of the separate initial DL BWP and the initial UL BWP are aligned (in accordance with earlier agreement). - Option 2a: If a separate initial DL BWP is not configured for RedCap, the RedCap UE continues to use at least the location, bandwidth, SCS, and cyclic prefix of the MIB-configured CORESET#0. - For TDD, the total frequency span of MIB-configured CORESET#0 and the initial UL BWP does not exceed the RedCap UE maximum bandwidth. - Option 2b: If a separate initial DL BWP is not configured for RedCap, the RedCap UE continues to use at least the location, bandwidth, SCS, and cyclic prefix of the MIB-configured CORESET#0. - For TDD, the center frequencies of the MIB-configured CORESET#0 and the initial UL BWP are aligned.

Regarding the above options, our first preference is Option 2b. Option 2a would have been considerable as well. However, Option 2b is well-aligned with the decision made during RAN1 #108-e on center frequency alignment [1]:

Agreement For FR1 and FR2, for TDD, when a (separate or shared) initial DL BWP includes CD-SSB (for FR1 and FR2) and the entire CORESET#0 (for FR1), the center frequencies for the (separate or shared) initial DL BWP and the (separate or shared) initial UL BWP are assumed to be the same.

As can be seen from the above, this agreement applies to both idle/inactive and connected modes. Considering UEs in idle/inactive mode, the above agreement means that, for TDD, initial DL BWP (which, when separate initial DL BWP is not configured, is same as MIB-indicated CORESET#0) and (shared or separate) initial UL BWP should have aligned center frequencies.

Therefore, in effect, the center frequency alignment between initial DL BWP defined by MIB-indicated CORESET#0 and (separate/shared) initial UL BWP is already expected by the UE even for Option 1. Thus, Option 2b does not lead to any “new UE behavior” nor require “additional efforts” from the gNB.

Proposal 1:

- For the case that the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth, if a separate initial DL BWP is not configured for RedCap, the RedCap UE continues to use at least the location, bandwidth, SCS, and cyclic prefix of the MIB-configured CORESET#0.

  - Note: For TDD, the center frequencies of the MIB-configured CORESET#0 and the initial UL BWP are aligned based on earlier decision during RAN1 #108-e.

# Center frequency alignment between initial UL and DL BWPs associated with RACH/Msg3 transmission and Type 1 CSS monitoring respectively

The following have been agreed in past RAN1 meetings.

Agreement For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

Agreement For FR1 and FR2, for TDD, when a (separate or shared) initial DL BWP includes CD-SSB (for FR1 and FR2) and the entire CORESET#0 (for FR1), the center frequencies for the (separate or shared) initial DL BWP and the (separate or shared) initial UL BWP are assumed to be the same.

that, in TDD deployments, center frequencies of initial UL and initial DL BWPs are expected to be the same for RedCap UEs.

However, this has not yet been captured in the latest version of TS 38.213. As part of the post-meeting CR review discussions after RAN1 #107-e meeting, the specification editor had indicated the reason for not capturing it since the center frequency alignment is captured since Rel-15 in Section 12 of TS 38.213. However, in our understanding, the current text in Section 12 of TS 38.213 is not sufficient since it does not capture the decision pertinent to RedCap accurately.

The current spec-text (since Rel-15) says the following:

However, the above fails to capture the decision for RedCap accurately since, with the possible configuration of one or more of separate initial DL BWP and separate initial UL BWP for RedCap UEs the notion of “same BWP-Id” becomes ambiguous. For instance, when configured with separate initial DL BWP for RedCap UE for monitoring of Type1-PDCCH CSS set while Types 0/0A/2 PDCCH CSS sets are configured to MIB-configured CORESET#0, the UE should expect the BWP associated with Type1-PDCCH CSS set should have aligned center frequency with initial UL BWP in which the UE is expected to transmit Msg1/Msg3 or MsgA and not necessarily the initial DL BWP defined by MIB-configured CORESET#0 which the UE would use for receiving CD-SSB, SIB, or paging.

Thus, it would be necessary to capture in the RAN1 specifications that a RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set (separate or shared with non-RedCap UEs) is different than the center frequency for an initial UL BWP (separate or shared with non-RedCap UEs) in which the RedCap UE may transmit Msg1/Msg3 or MsgA.

Proposal 2:

- Capture the following UE behavior in TS 38.213, subclause 17.1:

  - A RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set (separate or shared with non-RedCap UEs) is different than the center frequency for an initial UL BWP (separate or shared with non-RedCap UEs) in which the RedCap UE may transmit Msg1/Msg3 or MsgA.

# Presence of SSB in separate initial DL BWP in connected mode for BWP#0 configuration option 1

During the RAN1 #106-e meeting, RAN1 made the following agreement:

Agreements: Confirm the following working assumptions from RAN1#105-e: - After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

Further, during RAN1 #107-e meeting, the following were agreed (only parts of the agreement relevant to current discussion quoted below):

Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. … - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. Agreement: - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. … - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities.

For BWP#0 configuration option 1, the initial DL BWP is not considered to be an RRC-configured BWP, although a UE may be configured to operate using the initial DL BWP after RRC connection setup. In such a case using BWP#0 configuration option 1, if a RedCap UE is configured with a separate initial DL BWP for random access-related DL reception, it would be expected to remain on the initial DL BWP as its active DL BWP upon RRC connection setup until reconfigured. On the other hand, as the above agreements indicate, it is not clear if the RedCap UE may expect SSB within the initial DL BWP that serves as the active DL BWP in this case. Further, for a UE supporting mandatory FG 6-1, it may not be possible to operate in the active DL BWP without any SSB.

The following was part of the alternative proposals (Option 2) discussed during RAN1 #108-e.

- For BWP#0 configuration option 1, upon successful completion of the random access procedure, a RedCap UE supporting FG 6-1 only (but not FG 6-1a) in connected mode is not required to receive any DL signals except for RACH-related messages and RRC-based BWP switch signal operate on a separate initial DL BWP that does not contain SSB.

However, how the above can be realized in practice remains unclear. For instance, UE’s support of FG 6-1a or not may not be known until capability report reception at the gNB, and this occurs after completion of the random access procedure. Thus, the UE cannot be moved to another BWP from separate initial DL BWP (w/o SSB) until after UE capability reporting, and the timeline for this depends on gNB implementation and transparent to RAN1 specifications.

Option 1 from the alternatives discussed during RAN1 #108-e could be a “fallback option” at the cost of significant restriction on separate initial DL BWP configuration (makes the use case of separate initial DL BWP in BWP#0 configuration 1 impractical).

- Option 1: - For FR1, for BWP#0 configuration option 1, - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0. - For FR2, for BWP#0 configuration option 1, - A RedCap UE in connected mode does not expect to operate in a separate initial DL BWP that does not include CD-SSB and the entire CORESET#0.

On the other hand, for BWP#0 configuration option 1, a RedCap UE could simply be provided with Non-Cell Defining-SSB (NCD-SSB) in the separate initial DL BWP if it is used in connected mode and if the separate initial DL BWP does not include the CD-SSB.

From a signalling perspective, there is no fundamental issue in providing NCD-SSB configuration by SIB signalling in case of BWP#0 configuration option 1. It should be noted that the decision in RAN2 related to NCD-SSB and paging in idle mode has to do with cell selection/re-selection measurements (that includes neighbor cells) prior to paging monitoring and not about the signalling itself via SIB. Thus, the option of configuring NCD-SSB in the separate initial DL BWP via SIB signaling is preferred.

Proposal 3:

- For BWP#0 configuration option 1, a RedCap UE can expect to be provided with Non-Cell Defining-SSB (NCD-SSB) transmission in the separate initial DL BWP when it is used in connected mode if the separate initial DL BWP does not include the CD-SSB.

# RAN2 LS on time offsets between CD- and NCD-SSB

RAN1 received an LS from RAN2 indicating the following [2]:

1. Overall Description: In RAN2#117-e, it was concluded that, from RAN2 signalling standpoint, CD-SSB and NCD-SSB(s) may be transmitted at different times by configuring an offset. RAN2 would like to ask RAN4 and RAN1 whether such offset is feasible/needed. 2. Actions: To RAN4 and RAN1 ACTION: RAN2 kindly asks RAN4 and RAN1 to take the information above into consideration and provide feedback.

Configuring non-zero values of time offsets between CD-SSB and an NCD-SSB configured within a RRC-configured DL BWP is feasible. Furthermore, if same Tx power level is to be maintained between CD- and NCD-SSB, then, in some cases (e.g., considering possible application of power boosting), it may be beneficial to be able to multiplex them in time from the gNB perspective.

Proposal 4:

- Send a reply LS to RAN2 indicating that, from the perspective of RAN1, it is feasible and could even be beneficial to support non-zero time offsets between CD-SSB and NCD-SSB(s).

# Conclusions

In this contribution, we presented our views on the remaining details for efficient support of RedCap UEs with reduced UE BW in existing and future NR deployments with minimal impact to non-RedCap UEs.

Based on the presented discussion, our views can be summarized via the following proposals.

Proposal 1:

- For the case that the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth, if a separate initial DL BWP is not configured for RedCap, the RedCap UE continues to use at least the location, bandwidth, SCS, and cyclic prefix of the MIB-configured CORESET#0.

  - Note: For TDD, the center frequencies of the MIB-configured CORESET#0 and the initial UL BWP are aligned based on earlier decision during RAN1 #108-e.

Proposal 2:

- Capture the following UE behavior in TS 38.213, subclause 17.1:

  - A RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set (separate or shared with non-RedCap UEs) is different than the center frequency for an initial UL BWP (separate or shared with non-RedCap UEs) in which the RedCap UE may transmit Msg1/Msg3 or MsgA.

Proposal 3:

- For BWP#0 configuration option 1, a RedCap UE can expect to be provided with Non-Cell Defining-SSB (NCD-SSB) transmission in the separate initial DL BWP when it is used in connected mode if the separate initial DL BWP does not include the CD-SSB.

Proposal 4:

- Send a reply LS to RAN2 indicating that, from the perspective of RAN1, it is feasible and could even be beneficial to support non-zero time offsets between CD-SSB and NCD-SSB(s).

# References

- Chairman’s Notes, 3GPP RAN1 #108-e.

- R1-2203046 (R2-2204115), “LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times,” RAN WG2.