3GPP TSG RAN WG1 Meeting #109-e                                                                R1-2204772

e-Meeting, May 9th – 20th, 2022

Agenda Item:		8.6.2

Source: 	Intel Corporation

Title:	Remaining details on support of HD-FDD for Rel-17 RedCap

Document for: 	Discussion/Decision

# Introduction

In this contribution, we provide our further views on the remaining issues on HD-FDD operations of RedCap UE.

# Handling overlaps between a DL reception and a UL transmission

Case 5: Configured SSB vs. Msg3 (re)transmission and PUCCH for Msg4/MsgB

For the case of overlap between SSB and Msg3 (re)transmission and PUCCH for Msg4/MsgB, a working assumption was made in last meeting that SSB is prioritized, which provides a unified solution for all cases of overlap between SSB and UL transmissions. In last meeting, the moderator tried to confirm the WA as an agreement. However, it is failed mainly due to lack of time. In fact, the only remaining concern during the email discussion is for the clarification on the collision between CD-SSB and other DL channels during random access. From our understanding, since both SSB and other DL channels/signals are under control of gNB, it should be up to gNB configuration/scheduling to avoid the collision. No specification impact is expected. Regarding overlap between SSB an Msg3 (re)transmission and PUCCH for Msg4/MsgB, we believe it is reasonable to confirm the WA since there is essential no concern on the WA now.

Working Assumption - For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

Proposal 1: For Case 5, if a SSB overlaps with Msg3 (re)transmission and PUCCH for Msg4/MsgB,

- Confirm the WA as agreement that SSB is prioritized.

- No specification work regarding the overlap between SSB and other DL channels during random access.

Available slots determination for PUSCH or PUCCH repetitions

In last RAN1 meeting, the available slot determination for PUCCH repetitions for HD-FDD UEs was discussed. Above all, there is no objection to define a TDD-like rule for available slot determination. Since tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are not applicable to HD-FDD operation, the available slot determination is only based on collisions with SSB symbols. Two factors may be considered in the determination of available slots: 1) a PUSCH or PUCCH repetition cannot overlap with an SSB; 2) the gap between the SSB and the PUSCH or PUCCH repetition must be not less than the necessary switching gap. Further, a unified solution for PUSCH or PUCCH repetition is commonly preferred.

We would like to first clarify an assumption on handling the case of “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and PUSCH or PUCCH repetitions. Based on the existing agreements, the allowed cases only include

- between cell specific configured DL and cell-specific configured UL

- between cell-specific configured DL and dedicated configured UL

- between dedicated configured DL and cell-specific configured UL

Agreement - For the case of the “back-to-back” non-overlapping UL/DL without sufficient gap between cell specific configured DL and cell-specific configured UL, e.g., SSB or PDCCH in CSS vs. valid RO, it is up to UE implementation to ensure that the switching time is satisfied Agreement  The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., SSB vs. CG PUSCH, PUCCH or SRS - Configured UL transmission is cancelled (as in the overlapping case) - The “back-to-back” non-overlapping UL/DL without sufficient gap between dedicated configured DL and cell-specific configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS vs. valid RO - Leave it to UE implementation to cancel either DL reception or UL transmission to ensure sufficient switching time

The question is whether to define a behaviour to handle “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamically scheduled PUSCH or PUCCH repetitions. If it is allowed, we prefer to explicitly clarify the allowance and the following FL proposal from last meeting would be a good start point. Since the available slot determination for PUSCH or PUCCH repetition is only introduced in CovEnh WI when the number of repetitions is larger than 1, and this FL proposal is not intended to challenge such assumption, the FL proposal applies to the case K>1 by default. On the other hand, since both PUSCH repetition Type A and B are supported in NR and the available slot determination is only applicable for PUSCH repetition Type A, it is preferred to clearly reflect such restriction in the proposal.

FL7 High Priority Proposal 2.6-1 [1] - For PUCCH repetition in RRC connected state, - HD-FDD UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition starts or ends at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with synchronization signal block (SSB) transmission - For PUSCH repetition with a configured grant or scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled - HD-FDD UE determines a slot as an available slot for PUSCH repetitions when a PUSCH repetition starts or ends at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with synchronization signal block (SSB) transmission

On the other hand, to stick to the existing agreement on allowed cases of “back-to-back” non-overlapping UL/DL without sufficient gap, two options can be considered based on the above FL proposal. we prefer Option 2 for simplicity.

- Option 1: revise the above FL proposal to exclude the case of back-to-back non-overlap without sufficient switching gap between SSB and dynamic PUSCH or PUCCH repetition.

- Option 2: revise the above FL proposal to exclude all cases of back-to-back non-overlap without sufficient switching gap between SSB and semi-statically configured or dynamically scheduled UL transmission.

Proposal 2: Regarding available slots determination for PUSCH or PUCCH repetitions, it is preferred to firstly clarify whether “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic PUSCH or PUCCH repetition is allowed or not

Proposal 3: If “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic PUSCH or PUCCH repetition is allowed, the FL proposal from last meeting is preferred with a clarification that the proposal only applies to PUSCH repetition Type A

- For PUCCH repetition in RRC connected state,

  - HD-FDD UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition starts or ends at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with synchronization signal block (SSB) transmission

- For PUSCH repetition Type A with a configured grant or scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled

  - HD-FDD UE determines a slot as an available slot for PUSCH repetitions when a PUSCH repetition starts or ends at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, from the last or first symbol, respectively, in the set of symbols with synchronization signal block (SSB) transmission

Proposal 4: If “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic PUSCH or PUCCH repetition is NOT allowed, we prefer to exclude back-to-back non-overlap without sufficient switching gap between SSB and semi-statically configured or dynamically scheduled UL transmission in available slot determinaton.

- For PUCCH repetition in RRC connected state,

  - HD-FDD UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition is not overlapped with the set of symbols with synchronization signal block (SSB) transmission

- For PUSCH repetition Type A with a configured grant or scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled

  - HD-FDD UE determines a slot as an available slot for PUSCH repetitions when a PUSCH repetition is not overlapped the set of symbols with synchronization signal block (SSB) transmission

Invalid symbols for PUSCH repetition Type B transmission

In the current specification, it is only specified that, for PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	For a reduced capability half-duplex UE in paired spectrum and for PUSCH repetition Type B transmission, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

It is preferred to clarify how to handle “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and the PUSCH repetition Type B transmission. Two options can be considered

- Option 1: to be considered in invalid symbol determination. That is, symbols that are not at least NTx-Rx⋅Tc before the first symbol or not at least NRx-Tx⋅Tc after the last symbol indicated for a SSB are considered as invalid symbols for PUSCH repetition Type B transmission.

- Option 2: not considered in invalid symbol determination. That is, a symbol of the PUSCH repetition Type B transmission is invalid only if it is overlaps with a SSB. Then, if there is no sufficient gap between the PUSCH repetition Type B transmission and the SSB, the PUSCH repetition Type B transmission is cancelled, i.e., SSB is prioritized.

We prefer Option 1 for a unified solution handling all PUSCH/PUCCH repetitions. Option 1 can also provide more opportunities for PUSCH repetition Type B transmission. Then, based on the clarification in Proposal 2, such invalid symbol determination may apply both semi-statically configured and dynamically scheduled PUSCH repetition Type B transmissions, or only semi-statically configured PUSCH repetition Type B transmissions.

Proposal 5: for PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows

- Symbols that are not at least NTx-Rx⋅Tc before the first symbol or not at least NRx-Tx⋅Tc after the last symbol indicated for a SSB are considered as invalid symbols for PUSCH repetition Type B transmission

Collision between NCD-SSB and UL transmissions

One more remaining issue is how to handle the collision between NCD-SSB and a semi-statically configured or dynamically scheduled UL transmission. For FG 6-1, the NCD-SSB would be configured on the RRC-configured active DL BWP if it doesn’t contain the CD-SSB. To avoid additional standardization effort in the maintenance phase, we prefer to define that NCD-SSB is handled in the same way as CD-SSB,

- If a NCD-SSB is overlapped with a semi-statically configured or dynamically scheduled UL transmission, the NCD-SSB is prioritized

- The “back-to-back” non-overlapping UL/DL without sufficient gap between NCD-SSB and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., SSB vs. CG PUSCH, PUCCH or SRS

  - Configured UL transmission is cancelled (as in the overlapping case)

- Available slots determination for PUSCH or PUCCH repetitions considering CD-SSB, once concluded, is applicable to NCD-SSB too.

Proposal 6: To handle the collision between NCD-SSB and a semi-statically configured or dynamically scheduled UL transmission

- If a NCD-SSB is overlapped with a semi-statically configured or dynamically scheduled UL transmission, the NCD-SSB is prioritized

- The “back-to-back” non-overlapping UL/DL without sufficient gap between NCD-SSB and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., SSB vs. CG PUSCH, PUCCH or SRS

  - Configured UL transmission is cancelled (as in the overlapping case)

- Available slots determination for PUSCH or PUCCH repetitions considering CD-SSB, once concluded, is applicable to NCD-SSB too.

# Conclusion

In this contribution, we provide our views on open issues for HD-FDD operation. we make the following proposals.

Proposal 1: For Case 5, if a SSB overlaps with Msg3 (re)transmission and PUCCH for Msg4/MsgB,

- Confirm the WA as agreement that SSB is prioritized.

- No specification work regarding the overlap between SSB and other DL channels during random access.

Proposal 2: Regarding available slots determination for PUSCH or PUCCH repetitions, it is preferred to firstly clarify whether “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic PUSCH or PUCCH repetition is allowed or not

Proposal 3: If “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic PUSCH or PUCCH repetition is allowed, the FL proposal from last meeting is preferred with a clarification that the proposal only applies to PUSCH repetition Type A

- For PUCCH repetition in RRC connected state,

  - HD-FDD UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition starts or ends at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol in the set of symbols with synchronization signal block (SSB) transmission

- For PUSCH repetition Type A with a configured grant or scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled

  - HD-FDD UE determines a slot as an available slot for PUSCH repetitions when a PUSCH repetition starts or ends at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, from the last or first symbol, respectively, in the set of symbols with synchronization signal block (SSB) transmission

Proposal 4: If “back-to-back” non-overlapping UL/DL without sufficient gap between SSB and dynamic PUSCH or PUCCH repetition is NOT allowed, we prefer to exclude back-to-back non-overlap without sufficient switching gap between SSB and semi-statically configured or dynamically scheduled UL transmission in available slot determinaton.

- For PUCCH repetition in RRC connected state,

  - HD-FDD UE determines a slot as an available slot for PUCCH repetitions when a PUCCH repetition is not overlapped with the set of symbols with synchronization signal block (SSB) transmission

- For PUSCH repetition Type A with a configured grant or scheduled by DCI format 0_1 or 0_2, when AvailableSlotCounting is enabled

  - HD-FDD UE determines a slot as an available slot for PUSCH repetitions when a PUSCH repetition is not overlapped the set of symbols with synchronization signal block (SSB) transmission

Proposal 5: for PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows

- Symbols that are not at least NTx-Rx⋅Tc before the first symbol or not at least NRx-Tx⋅Tc after the last symbol indicated for a SSB are considered as invalid symbols for PUSCH repetition Type B transmission

Proposal 6: To handle the collision between NCD-SSB and a semi-statically configured or dynamically scheduled UL transmission

- If a NCD-SSB is overlapped with a semi-statically configured or dynamically scheduled UL transmission, the NCD-SSB is prioritized

- The “back-to-back” non-overlapping UL/DL without sufficient gap between NCD-SSB and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., SSB vs. CG PUSCH, PUCCH or SRS

  - Configured UL transmission is cancelled (as in the overlapping case)

- Available slots determination for PUSCH or PUCCH repetitions considering CD-SSB, once concluded, is applicable to NCD-SSB too.

# References

[1] R1-2112601, “FL summary #3 on other aspects for RedCap” Moderator (Qualcomm)

# ANNEX A: Overlap handling between DL and UL channels/signals in NR TDD

Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols. - If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from Clauses 9 and 9.2.5 or Clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols. - The UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the UE detects the DCI format. The UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise μr=0.

Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.

Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

For a set of symbols of a slot that are indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot.

Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

Case 8: Dynamic or semi-static DL vs. valid RO

For a set of symbols of a slot corresponding to a valid PRACH occasion and symbols before the valid PRACH occasion, as described in Clause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

# ANNEX B: All RAN1 agreements

Agreements:

- For HD-FDD, for cases (if any) where collision handling needs to be specified, then the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum are used as a starting point if deemed applicable.

Agreements:

- (Working assumption) For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

- Sending an LS to RAN4 to inform the above working assumption, and to ask for feedback if any

  - The LS will not include the two FFS bullets

Agreements:

- For HD-FDD operation for RedCap UEs, consider at least the following DL/UL collision cases collisions may be addressed or alleviated with proper scheduling. The following cases of potential collisions can be further studied to see if any change to the current specs is necessary:

  - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

    - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH, or RO

  - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

    - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

  - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

  - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

  - Case 5: Configured SSB vs. dynamically scheduled or configured UL transmission

    - e.g., PUSCH, PUCCH, PRACH, SRS

  - Case 6: Monitoring for UL cancellation indication (if supported) while transmitting in UL

  - Case 7: Collision due to BWP switching (if supported)

  - Case 8: Dynamic or semi-static DL vs. valid RO

  - Case 9: Collision due to direction switching

Agreements:

For Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum.

- FFS whether the timeline is extended to include the RX/TX switching time for HD-FDD

For Case 4: dynamically scheduled DL reception vs. dynamic scheduled UL transmission, reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum

- That is, it is considered as an error case if a dynamically scheduled DL reception overlaps with a dynamically scheduled UL transmission

For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), reuse the existing collision handling principles in Rel-15/16 NR for operation on a single carrier/single cell in unpaired spectrum

- The semi-statically configured DL reception may include PDCCH (excluding ULCI), SPS PDSCH, CSI-RS or PRS.

  - FFS on PDCCH carrying ULCI, including whether or not it is supported by RedCap UEs (including potential difference between HD vs. FD RedCap UEs)

- The dynamically scheduled UL transmission may include PUSCH, PUCCH, SRS or PRACH triggered by PDCCH order

Agreements:

For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

- A HD-FDD UE does not expect to receive both cell specific higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

- FFS on cell-specifically configured DL reception vs. cell-specifically configured UL transmission

- FFS: whether or not there are conditions that need to be considered

Working assumption: For HD-FDD, no additional UE behavior for switching position determination is specified as compared to the existing specification.

Conclusion: Enhancement for potential UL and DL collision handling due to TA misalignment is not considered for Type-A HD-FDD operation of RedCap UEs

Working Assumption: For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

- A HD-FDD UE is not expected to transmit in the uplink earlier than [NRX-TX Tc] after the end of the last received downlink symbol in the same cell

- A HD-FDD UE is not expected to receive in the downlink earlier than[NTX-RX Tc] after the end of the last transmitted uplink symbol in the same cell

- FFS NTX-RX and NRX-TX

- FFS: how it jointly works with the agreement for other collision cases

Working assumption:

- If a dynamically scheduled UL transmission overlaps with an SSB, down-select one of the following options:

  - Option 1: Follow the handling of case 2 that dynamic UL is prioritized over SSB

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamic UL

  - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- If a semi-static configured UL transmission overlaps with an SSB, down-select one of from the following options

  - Option 1: Up to gNB configuration to avoid such collision and if it happens it is an error case

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over semi-static UL

  - Option 3: Leave to UE implementation (e.g. UE can receive the SSB if UE needs to receive the SSB; otherwise, UE can transmit the UL transmission) whether to receive the SSB or transmit the UL transmission

  - Other options are not precluded

- FFS: whether/how to account for Tx/Rx switching time before and after the set of SSB symbols

- FFS: whether or not the semi-static configured UL transmission includes a valid RO

Agreement:

- For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI

  - No special handling on the priority rule for PDCCH carrying ULCI

Conclusion:

- No consensus of specification support of semi-static UL/DL pattern to HD-FDD RedCap UEs in Rel-17.

Agreement:

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

  - Option 2: Leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO

  - Option 3: If configured PDCCH is in a Type-2 CSS set, then PDCCH is prioritized; otherwise the valid RO is prioritized

  - Option 4: Configured PDCCH is prioritized over valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - FFS: whether or not the set of symbols overlapping with PDCCH in CSS set includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

  - FFS whether a valid RO follows TDD’s or FDD’s definition, and if so, the corresponding impact

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured DL

  - Option 2: Leave to UE implementation whether to receive the configured DL or transmit the PRACH on the valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - Other options are not precluded.

  - FFS: whether or not the set of symbols overlapping with configured DL includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD for operation on a single carrier /single cell in unpaired spectrum

  - Option 2: Leave to UE implementation whether to receive the DL or transmit the PRACH on a valid RO

  - Option 3: Follow the handling of Case 1 to cancel PRACH based on a timeline that when the cancellation timeline is satisfied, the UE cancels the PRACH transmission and receives the DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 2 in R1-2103809)

  - Option 4: Valid RO is prioritized over dynamic DL that UE performs PRACH transmission and does not perform the DL receptions (Interpretation 3 in R1-2103809)

  - Option 5: When the cancellation timeline is satisfied, the UE neither performs transmission nor receives any DL signal/channels on the symbols overlapping with PRACH occasion (Interpretation 1 in R1-2103809)

  - FFS: whether or not the set of symbols overlapping with dynamic DL reception includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

Agreement:

- For Case 5 of SSB overlaps with in configured UL transmission, re-use the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over configured UL transmission

  - The configured UL transmission includes CG-PUSCH, or SRS

  - FFS: Confirm that PUCCH is included

Agreement

- For Case 5 of SSB overlaps with configured UL transmission, the configured UL transmission includes PUCCH transmission configured by higher layers

- Note:  The UL transmission indicated by DCI is supposed to be dynamic UL transmission.

Working Assumption

- For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid (same as FD-FDD RedCap UEs), and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH

- No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs

Working Assumption

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH

- FFS: whether or not there are conditions (e.g., exception for valid RO not intended for PRACH transmission) that need to be considered.

- Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2.

Agreement

Confirm this Working Assumption.

Working Assumption

- For Type-A HD-FDD UEs, all ROs applicable to RedCap UEs are valid (same as FD-FDD RedCap UEs), and for the case of SSB overlapping with valid RO from cell specific point of view, leave it to UE implementation whether to receive SSB or transmit PRACH

- No support of differentiating of ROs for Type-A HD-FDD Redcap UEs and FD FDD RedCap UEs

Agreement

Confirm this Working Assumption.

Working Assumption

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, leave it to UE implementation whether to receive configured PDCCH or transmit PRACH

- FFS: whether or not there are conditions (e.g., exception for valid RO not intended for PRACH transmission) that need to be considered.

- Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2.

Agreement

- For Case 8 of valid RO overlapping with UE-dedicated configured DL reception (e.g. PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS), leave it to UE implementation whether to receive the DL or transmit PRACH

- Note: For valid RO intended for PRACH triggered by PDCCH order, it has been covered in Case 2.

Agreement

- For Case 5 of dynamically scheduled UL transmission vs. SSB, one or both of the following options to be determined till next meeting:

  - Option 1: Dynamically scheduled UL transmission is prioritized over SSB

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

- FFS: whether or not the same UE behavior is applied to Msg3 (re)transmission and PUCCH for msg4

Agreement

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, downselect one of following options in next meeting

  - Option 2: Leave to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH

  - Option 3: Follow the handling of Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission)

  - Option 4: Valid RO is prioritized over dynamic DL reception

Agreement

For Case 1, the existing timeline in Rel-15/16 NR for operation on a single carrier /single cell in unpaired spectrum is reused for HD-FDD

Agreement

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

Note: With this agreement, no need to confirm below Working Assumption(From RAN1#104e)

Working Assumption (FromRAN1#104e )

- For HD-FDD switching time, reuse existing switching times for UE not capable of full duplex in TS 38.211, Table 4.3.2-3.

  - FFS: whether to define the guard times in symbol units

  - FFS: the switching positions

Conclusion:

- No consensus on defining a guard time in symbol units for HD-FDD Type A operation in Rel-17

Agreement

Revise the RAN1#104bis-e agreement for Case 3 as the following

- For Case 3, semi-statically configured DL reception vs. semi-statically configured UL transmission

  - A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception in the set of symbols of the slot

  - A HD-FDD UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and cell specific higher layer parameters configuring reception in the set of symbols of the slot

    - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set

  - FFS: whether or not there are conditions that need to be considered

Agreement

- For Type-A HD-FDD, no additional UE behaviour for UL/DL collision handling based on a priority indicator is specified as compared to the existing specification

Agreement

- Whether or not to account for the Tx/Rx switching time before and after the set of SSB symbols can be further discussed under Case 9

Agreement

- For Case 8 of valid RO overlapping with dynamically scheduled DL reception, leave it to UE implementation whether to receive the dynamically scheduled DL or transmit PRACH

Agreement

- The same validation rules of MsgA PUSCH occasions and RO/Preamble-to-PRU mapping rules for FDD can be reused for HD-FD

Agreement

- For HD-FDD, reuse the same principle as Rel-15/16 UE not capable of full-duplex communication

  - A HD-FDD UE is not expected to transmit in the uplink earlier than NRX-TX Tc after the end of the last received downlink symbol in the same cell

  - A HD-FDD UE is not expected to receive in the downlink earlier than NTX-RX Tc after the end of the last transmitted uplink symbol in the same cell

  - NRX-TX Tc and NTX-RX Tc are the same as the transition time for FR1 in Table 4.3.2-3, TS 38.211 for a UE not capable of full-duplex communication

- (Working Assumption) The “back-to-back” non-overlapping UL/DL without sufficient gap between RRC configured UL and DL may happen, i.e., are allowed for HD-FDD UEs.

  - RRC configured DL/UL includes at least cell specific higher layer parameters configured DL/UL

  - Discuss further whether to specify a clear UE behavior, or leave it to UE implementation to ensure that the switching time is satisfied

  - Note: This does not mean a HD-FDD UE is required to support the back-to-back UL/DL switching without sufficient gap

Agreement

- For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4

  - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

Agreement

- For MsgA PUSCH occasion overlapping with dynamic or semi-static DL reception, leave it to UE implementation to prioritize the DL reception or MsgA PUSCH transmission

Agreement

- For the case of the “back-to-back” non-overlapping UL/DL without sufficient gap between cell specific configured DL and cell-specific configured UL, e.g., SSB or PDCCH in CSS vs. valid RO, it is up to UE implementation to ensure that the switching time is satisfied

Agreement

      The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., SSB vs. CG PUSCH, PUCCH or SRS

  - Configured UL transmission is cancelled (as in the overlapping case)

- The “back-to-back” non-overlapping UL/DL without sufficient gap between dedicated configured DL and cell-specific configured UL may happen, i.e., allowed for HD-FDD UEs

  - E.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS vs. valid RO

  - Leave it to UE implementation to cancel either DL reception or UL transmission to ensure sufficient switching time

Agreement

      No additional UE behavior for DL/UL collision handling is specified in Rel-17 if SFI monitoring is supported for HD-FDD RedCap UEs.

Agreement

- The following text proposal to TS 38.213, clause 17.2 is endorsed

===================== Unchanged parts are omitted=====================

If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a CORESET where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS38.214], or the PRACH transmission in the set of symbols.

-	the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a CORESET where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols.

Tproc,2 is the PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH or μr, where μr corresponds to the SCS configuration of the PRACH if it is 15 kHz or larger; otherwise μr=0.

===================== Unchanged parts are omitted =====================

Agreement

- The following text proposal to TS 38.214 is endorsed.

======TP to TS 38.214 V17.0.0 ====== #### 5.1.2.1 Resource allocation in time domain #### < unchanged text omitted> A PDSCH reception in a slot of a multi-slot PDSCH reception is omitted according to the conditions in Clause 11.1 and Clause 17.2 of [6, TS38.213]. #### < unchanged text omitted> #### 6.1.2.1 Resource allocation in time domain #### < unchanged text omitted> For PUSCH repetition Type A and TB processing over multiple slots, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1, and Clause 11.2A and Clause 17.2 of [6, TS 38.213]. < unchanged text omitted> For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1, and Clause 11.2A and Clause 17.2 of [6, TS 38.213]. The UE shall repeat the TB across actual repetitions. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2, where N=1. For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a ‘CSI request’ field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a ‘CSI request’ field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a ‘CSI request’ field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1, and Clause 11.2A and Clause 17.2 of [6, TS 38.213]. < unchanged text omitted> ##### 6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant < unchanged text omitted> A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1, and Clause 11.2A and Clause 17.2 of [6, TS 38.213]. < unchanged text omitted> ======TP to TS 38.214 V17.0.0 ======

Working Assumption

- For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB

  - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

Agreement

- DL-UL collision handling for HD-FDD operation is applied after intra-UE multiplexing/prioritization, and the following TP to Clause 9 of TS 38.213 is endorsed

===================== Unchanged parts are omitted===================== In the remaining of this clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH before considering limitations for UE transmission as described in clause 11.1, and clause 11.1.1 and clause 17.2. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ===================== Unchanged parts are omitted =====================

Agreement

- The following text proposal to TS 38.213, clause 17.2 is endorsed

===================== Unchanged parts are omitted===================== If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or PRACH, or SRS based on a detected DCI format and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols. ===================== Unchanged parts are omitted =====================

Agreement

- The following text proposal to TS 38.213, clause 17.2 is endorsed

===================== Unchanged parts are omitted ===================== If a HD-UE would transmit a PRACH or MsgA PUSCH triggered by higher layers in a set of symbols and would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS, or is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in symbols that include any symbol from the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the PL RS, or the PDCCH, or the SS/PBCH blocks. If a HD-UE would receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS based on a configuration by higher layers or is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, and the HD-UE would transmit PRACH or MsgA PUSCH triggered by higher layers starting or ending at a symbol that is earlier or later than or, respectively, from the last or first symbol in the set of symbols, the HD-UE can select based on its implementation whether to either transmit the PRACH or the MsgA PUSCH or receive the PDSCH, or the CSI-RS, or the DL PRS, or the PDCCH, or the SS/PBCH blocks ===================== Unchanged parts are omitted =====================

Conclusion

- For collision handling, HD-FDD RedCap UEs do not support [partialCancellation] in Rel-17

  - Note: No specification impact is expected