3GPP TSG RAN WG1 #109-e																		    R1-2204776

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	Remaining issues on joint channel estimation for PUSCH and PUCCH

Agenda item:	8.8.2

Document for:	Discussion and Decision

# Introduction

At the RAN1#108-e meeting, the following conclusions and agreements were made regarding joint channel estimation for PUSCH and PUCCH [1]:

Agreement

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where a dropping or cancellation of a PUSCH transmission according dropping rules in [17.2, TS 38.213].

Agreement

- For HD-FDD RedCap UEs configured with DMRS bundling, an event is constituted for a case where the gap between two consecutive PUSCH transmissions overlaps with any symbol of downlink reception or downlink monitoring even if neither of the repetitions overlaps with it.

Conclusion

   PUSCH repetitions with different sets of power control parameters in multi-TRP operation is regarded as a semi-static event.

Note: No additional specification impact is expected

Agreement

   Gap between two PUSCH/PUCCH transmissions exceeds 11 symbols for extended CP is regarded as an event.

Agreement

- The value range of PUSCH-TimeDomainWindowLength is INTEGER (2 .. 32).

- The value range of PUCCH-TimeDomainWindowLength is INTEGER (2 .. 8).

- Note: the value shall not exceed the maximum duration.

Agreement

Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

Agreement

Inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling is determined based on down-selection (in RAN1#108e) option 2

- Option 2: relative slot index – option A

Note: Option A/B for relative slot index are defined as the following

- Option A: frequency hopping pattern is determined based on relative slot index. Relative slot 0 is the slot where PUCCH/PUSCH repetition starts. Each of the subsequent slots are counted and relative slot index increase by one regardless the slot is used to transmit PUCCH/PUSCH or not.

- Option B: frequency hopping pattern is determined based on relative slot index. Relative slot 0 is the slot where PUCCH/PUSCH repetition starts. One of the subsequent slots are counted and the slot index increase by one only if the slot is used to transmit PUCCH/PUSCH.

Conclusion

For frequency hopping for PUCCH/PUSCH repetitions with DMRS bundling, in Rel-17, there is no consensus to increase the number of frequency offset over what are supported in Rel-15/16.

Agreement (Excerpted from RAN1#107bis-e)

PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a [semi-static] event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions.

Agreement

Update the above agreement made in RAN1 107bis-e as below

PUCCH repetitions with different sets of power control parameters in multi-TRP operation should be regarded as a semi-static event that causes power consistency and phase continuity not to be maintained across PUCCH repetitions.

Agreement

- Values for PUSCH-Frequencyhopping-Interval are “{2, 4, 5, 6, 8, 10, 12, 14, 16, 20}”

Note: For unpaired spectrum, UE is not expected to be configured the value of 6, 8, 12, 14, 16

Agreement

- Values for PUCCH-Frequencyhopping-Interval are “{2, 4, 5, 10}”

Agreement

When both inter-slot frequency hopping and DMRS bundling are enabled for PUCCH/PUSCH repetitions, UE is expected to be configured with at least one of frequency hopping interval and TDW length.

Note: Conclusion is to added to RRC parameter agreement and sent to RAN2 in LS.

In the contribution, we discuss remaining issues on joint channel estimation for PUSCH and PUCCH. Our views on remaining issues for PUSCH enhancements are described in our companion contribution [2].

# Discussion on action of TPC command

At the RAN1#107-e meeting, the following working assumption was made regarding the action of group common TPC commands for DMRS bundling [3].

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

In our view, when DMRS bundling is configured and employed for PUSCH repetitions, for UEs in coverage enhancement mode, it is expected that UE would apply maximum transmit power for the transmission of PUSCH repetitions. In this case, power control accumulation mechanism may not be necessary for DMRS bundling. Further, the working assumption is only applied for FDD system. For TDD scenario, when UE receives the group command TPC command in a PDCCH, this indicates that UE cannot maintain power consistency and phase continuity.

Note that RAN1 could not reach consensus on UE behavior in Rel-15/16 for action of TPC command of PUSCH/PUCCH repetitions including timeline KPUSCHi, support of absolute TPC command, etc. For instance, if one of the interpretations, i.e., Interpretation 1 is considered, UE needs to maintain same transmit power during DG-PUSCH repetitions, which indicates that at least the working assumption for DG-PUSCH repetitions cannot be confirmed.

In our view, for DMRS bundling, if gNB realizes that a transmit power adjustment is essential for coverage enhancement of uplink transmission, it may be more desirable to treat this as an event so that UE can react quickly and dynamically adjust the transmit power as early as possible. Based on the discussions above, the working assumption for action of group common TPC commands is not confirmed. Further, action of group common TPC commands is considered as an event for DMRS bundling.

The following TP is proposed for action of group common TPC commands for DMRS bundling:

------------------------------ TP#1: TS 38.214----------------------------------- ### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged text omitted > Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, exceeds 13 symbols for normal cyclic prefix or exceeds 11 symbols for extended cyclic prefix. - The gap between any two consecutive PUSCH transmissions, or the gap between any two consecutive PUCCH transmissions, does not exceed 13 symbols but other uplink transmissions are scheduled between the two consecutive PUSCH transmissions or the two consecutive PUCCH transmissions. - For PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B or TB processing over multiple slots, a dropping or cancellation of a PUSCH transmission according to clause 9, clause 11.1 and clause 11.2A of [6, TS 38.213]. - For PUCCH transmissions of PUCCH repetition, a dropping or cancellation of a PUCCH transmission according to clause 9, clause 9.2.6 and clause 11.1 of [6, TS 38.213]. - For any two consecutive PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, and when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', a different SRS resource set association is used for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, according to Clause 6.1.2.1. - For any two consecutive PUCCH transmissions of PUCCH repetition, and when a PUCCH resource used for repetitions of a PUCCH transmission by a UE includes first and second spatial relations or first and second sets of power control parameters, as described in [10, TS 38.321] and in clause 7.2.1 of [6, TS 38.213], different spatial relations or different power control parameters are used for the two PUCCH transmissions of PUCCH repetition, according to Clause 9.2.6 of [6, TS 38.213]. - Uplink timing adjustment in response to a timing advance command according to clause 4.2 of [6, TS 38.213]. - Frequency hopping. - For reduced capability half-duplex UEs, - a dropping or cancellation of a PUSCH transmission according to clause 17.2 of [6, TS 38.213] or - an overlapping of the gap between two consecutive PUSCH transmissions and any symbol of downlink reception or downlink monitoring. - Transmit power adjustment in response to a TPC command in DCI format 2_2 according to Clause 7.1 and 7.2 of [6, TS 38.213]. < Unchanged text omitted >

Proposal 1

- The working assumption for action of group common TPC commands is not confirmed.

  - Action of group common TPC commands is considered as an event for DMRS bundling

- Agree on TP#1 for action of group common TPC commands for DMRS bundling.

# Discussion on enhanced inter-slot frequency hopping for PUSCH repetitions

It was agreed at the RAN1#108-e meeting that physical slot index is used to determine the inter-slot frequency hopping pattern with DMRS bundling for PUSCH repetitions [1]. However, in case when the number of repetitions and inter-slot frequency hopping interval are relatively large, the frequency hopping may cross the frame boundary, which may lead to unbalanced frequency hopping pattern if only physical slot index is used to determine the inter-slot frequency hopping pattern with DMRS bundling.

Several options can be considered for frequency hopping pattern determination:

- Option 1: using physical slot index only

- Option 2: using SFN and physical slot index

- Option 3: using relative SFN and physical slot index.

Figure 1 illustrates comparison of these options for inter-slot frequency hopping pattern. In the examples, it is assumed FDD system with 15kHz SCS, frequency hopping interval of 4 slots and 16 PUSCH repetitions, where the repetitions span within SFN = 1 and 2. Further, the starting physical slot for repetition is slot 0 in SFN 1.

From the figure, it can be observed that Option 1 using physical slot only for the determination of hopping pattern may lead to unbalanced frequency hopping pattern when repetitions cross the frame boundary. Similarly, for Option 2, unbalanced frequency hopping pattern may be observed at the beginning and end of the PUSCH repetitions. On the contrary, for Option 3, balanced frequency hopping pattern can be achieved when relative SFN and physical slot index are employed for the determination of hopping pattern.

Figure 1. Frequency hopping pattern determination

Based on the above discussions, it is evident that Option 3 can achieve balanced frequency hopping pattern compared to Option 1 and 2. In our view, inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index and relative system frame number.

The following TP is proposed for inter-slot frequency hopping with DMRS bundling for PUSCH repetitions.

------------------------------ TP#2: TS 38.214----------------------------------- ### 6.3.1 Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots < Unchanged text omitted > In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, the starting RB during slot nsμ is given by: [ RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1 ] where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, has value 0 for the system radio frame containing the first slot determined for the PUSCH transmission, and is increased by 1 for each subsequent system radio frame, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μ of the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval, if provided; otherwise, NFH is the value of PUSCH-TimeDomainWindowLength, RBstart is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and RBoffsetis the frequency offset in RBs between the two frequency hops. < Unchanged text omitted >

Proposal 2

- Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index and relative system frame number.

- Agree TP#2 for inter-slot frequency hopping with DMRS bundling for PUSCH repetitions.

# Discussion on enhanced inter-slot frequency hopping for PUCCH repetitions

At the RAN1#108-e meeting, it was agreed that inter-slot frequency hopping pattern for PUSCH and PUCCH repetitions with DMRS bundling is determined based on physical slot index and relative slot index, respectively, with the motivation of extending existing inter-slot frequency hopping mechanism as defined in Rel-15/16 [1].

Note that in case of PUCCH cancellation or dropping due to dynamic SFI, UL CI, high priority uplink transmission, UE would not further postpone the PUCCH repetitions. Following this design principle, the description for inter-slot frequency hopping pattern with DMRS bundling for PUCCH repetition in Clause 9.2.6 in TS38.213 is not accurate [4].

The following TP is proposed for inter-slot frequency hopping with DMRS bundling for PUCCH repetitions.

------------------------------ TP#3: TS 38.213----------------------------------- ### 9.2.6 PUCCH repetition procedure < Unchanged text omitted > - if the UE is configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is provided PUCCH-DMRS-Bundling = ‘enabled’ - the UE performs frequency hopping per interval of NPUCCHinterval consecutive slots, that start from a slot indicated to the UE and where the UE would transmit a first repetition of the PUCCH, where NPUCCHinterval is the value of PUCCH-Frequencyhopping-Interval, if provided; otherwise, NPUCCHinterval is the value of PUCCH-TimeDomainWindowLength - the UE transmits the PUCCH over intervals until the UE transmits the PUCCH in NPUCCHrepeat slots, where the first interval has number 0 and each subsequent interval is counted regardless of whether or not the UE transmits the PUCCH in a slot - the UE transmits the PUCCH starting from a first PRB, provided by startingPRB, in intervals with even number and starting from a second PRB, provided by secondHopPRB, in intervals of frequency hopping intervals with odd number - the UE does not expect to be configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot - if the UE is not configured to perform frequency hopping for repetitions of a PUCCH transmission across slots and the UE is configured to perform frequency hopping for a repetition of the PUCCH transmission within a slot, the frequency hopping pattern between the first PRB and the second PRB is same within each slot < Unchanged text omitted >

Proposal 3

- Agree TP#3 for inter-slot frequency hopping with DMRS bundling for PUCCH repetitions.

# Conclusions

In this contribution, we discussed remaining issues on joint channel estimation for PUSCH and PUCCH. Further, we summarize the proposals as follows:

Proposal 1

- The working assumption for action of group common TPC commands is not confirmed.

  - Action of group common TPC commands is considered as an event for DMRS bundling

- Agree on TP#1 for action of group common TPC commands for DMRS bundling.

Proposal 2

- Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index and relative system frame number.

- Agree TP#2 for inter-slot frequency hopping with DMRS bundling for PUSCH repetitions.

Proposal 3

- Agree TP#3 for inter-slot frequency hopping with DMRS bundling for PUCCH repetitions.

# References

- Chairman’s notes, RAN1#108 e-Meeting, February 2022

- R1-2204775, “Remaining issues on PUSCH enhancements”, Intel Corporation, e-Meeting, May 2022

- Chairman’s notes, RAN1 #107 e-Meeting, November 2021

- TS38.213, V17.1.0, “NR: Physical layer procedures for control”