3GPP TSG RAN WG1 #109-e	R1-2204777

e-Meeting, May 9th – May 20th, 2022

Agenda item:	8.10

Source: 	Intel Corporation

Title: 	Remaining Details on Frequency-domain Resource Multiplexing for IAB

Document for:	Discussion and decision

# Introduction

RAN1#108e has made the following conclusions for deferred discussion.

- Defer discussion (contribution driven) about potential specification impact for conflict resolution between parent and child IAB nodes in case the Rel-17 H/S/NA configuration is provided but the IAB node needs to instead operate according to the Rel-16 H/S/NA configuration (based on its implementation as agreed previously in RAN1#107-e) in the slot until RAN#109-e.

- Defer discussion (contribution driven) about potential specification impact if the configured RB sets of an IAB-DU HSNA resource configuration do not cover the entire carrier bandwidth until RAN1#109-e.

- Defer discussion (contribution driven) about extending the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for soft resources until RAN1#109-e.

In this contribution, we will focus on those remaining controversial issues on frequency-domain resource multiplexing for IAB.

# Rel-17 H/S/NA Configuration vs. Rel-16 H/S/NA Configuration

Regarding the relationship between Rel-16 H/S/NA configuration and Rel-17 H/S/NA configuration, RAN1#105e-RAN1#107e have made the following agreements.

- For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA.

- For a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied.

- Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17.

RAN1#106bis-e has the following working assumption.

- Working Assumption: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected:

  - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

It has been very controversial about the potential specification impact for conflict resolution between parent and child IAB nodes in case the Rel-17 H/S/NA configuration is provided but the IAB node needs to instead operate according to the Rel-16 H/S/NA configuration (based on its implementation as agreed previously in RAN1#107-e) in the slot. Regarding always mandating Rel-17 H/S/NA configuration when provided, we have the following observations.

Firstly, it has been agreed that whether an IAB-node is operating in TDM/FDM is left for IAB-node’s implementation. When an IAB-DU has both Rel-17 H/S/NA and Rel-16 H/S/NA semi-static configuration, it is not reasonable to always semi-statically mandate Rel-17 FDM configuration for the IAB-node since an IAB-node may need to operate in TDM by its implementation. Always semi-statically mandating Rel-17 FDM configuration eliminates an IAB-node’s dynamic option of operating in TDM and reverses RAN1#107e agreement of TDM/non-TDM operations by IAB implementation.

Secondly, it is not possible for an IAB-node and its parent node semi-statically have the same understanding on the selected H/S/NA configuration (if both Rel-17 H/S/NA and Rel-16 H/S/NA are configured) since an IAB-node will dynamically choose to operate in TDM or FDM by its implementation.

Thirdly, since Rel-17 H/S/NA configurations are provided separately in addition to the Rel-16 H/S/NA, an IAB-DU can have both Rel-17 H/S/NA and Rel-16 H/S/NA semi-static configuration and its parent can be aware of both configurations. An IAB node and its parent node may need to remain both Rel-17/Rel-16 H/S/NA configuration and the parent node may be dynamically informed from an IAB-node which will be applied.

Hence, we need to provide some signaling mechanism to dynamically achieve synchronization between IAB-node and its parent node with TDM/FDM implementation and Rel-16/Rel-17 H/S/NA application. Note that the TDM/FDM dynamic switch (based on channel condition or hardware update) is expected in a relative slow time scale.

RAN1#108-e has provided the following alternatives for TDM/FDM conflict resolution between parent and child IAB nodes in case the Rel-17 H/S/NA configuration is provided but the IAB node (based on its implementation as agreed previously in RAN1#107-e) is not currently operating in a non-TDM multiplexing mode (i.e. one of the supported cases in the multiplexing info in 38.473) in the slot:

- Alt. 1: TDM fallback / FDM resume indication from child node (e.g. MAC CE)

- Alt. 2: An IAB node does not expect conflicting Rel-16 and Rel-17 H/S/NA configurations for a given RB set

- Alt. 3: A prioritization rule for TDM H/S/NA vs. FDM H/S/NA configurations for a given RB set is specified

- Alt. 4: No additional specification needed

Alt.1 is consistent with RAN1#105e and RAN1#107e agreements that Rel-17 H/S/NA configuration is provided separately in addition to the Rel-16 H/S/NA and whether or not an IAB-node can operate on TDM/FDM is left to IAB implementation. In other words, an IAB node and its parent node may need to remain both Rel-17/Rel-16 H/S/NA configuration and the parent node will be dynamically informed from an IAB-node which will be applied.

Alt.2 is very restrictive respective to Rel-17 H/S/NA configuration and it is questionable whether an IAB-node under Alt.2 can fulfil FDM operation. Rel-16 H/S/NA for TDM is configured in time-domain per D/U/F resource type and applies to the whole IAB-DU bandwidth, while Rel-17 H/S/NA for FDM is configured in frequency-domain per RB set. No conflicting between Rel-16 and Rel-17 H/S/NA for a given RB set actually means Rel-16 H/S/NA configuration is always valid if provided and the IAB-node may not be capable of FDM operation at those slots by its implementation.

Alt.3 is to semi-statically decide whether Rel-16 TDM H/S/NA or Rel-17 FDM H/S/NA will apply based on pre-defined prioritization rule. However, semi-statically mandating Rel-16 TDM H/S/NA or Rel-17 FDM H/S/NA configuration eliminates an IAB-node’s dynamic option of operating in TDM/FDM and reverses RAN1#107e agreement of TDM/non-TDM operations by IAB implementation.

Alt.4 does nothing and the conflict issue is not resolved between parent and child IAB nodes in case the Rel-17 H/S/NA configuration is provided but the IAB node needs to instead operate according to the Rel-16 H/S/NA configuration in the slot.

Based on those observations and discussion, Alt.1 is supported.

Observation 1: Regarding always mandating Rel-17 H/S/NA configuration when provided, we have the following observations.

        - Always semi-statically mandating Rel-17 FDM configuration eliminates an IAB-node’s dynamic option of operating in TDM and reverses RAN1#107e agreement of TDM/non-TDM operations by IAB implementation.

        - It is not possible for an IAB-node and its parent node semi-statically have the same understanding on the selected H/S/NA configuration (if both Rel-17 H/S/NA and Rel-16 H/S/NA are configured) since an IAB-node will dynamically choose to operate in TDM or FDM by its implementation.

        - An IAB node and its parent node may need to remain both Rel-17/Rel-16 H/S/NA configuration and the parent node may be dynamically informed from an IAB-node which will be applied.

Proposal 1: TDM fallback / FDM resume indication from child node (e.g. MAC CE) is supported.

# Rel-17 H/S/NA RB Set Configuration

Regarding Rel-17 H/S/NA RB set configuration, RAN1#106e-RAN1#108e have made the following agreements.

- N is a configured number of PRBs, where the CU configures N

  - N = {2, 4, 8, 16, 32, 64}

  - FFS: Value(s) of N in case of multiple configured BWPs at the IAB-MT

  - This agreement does not revert any existing RAN1 agreement

- A single value for the RB set size, N, is configured for a given IAB-DU cell’s Rel-17 frequency domain H/S/NA configuration.

- The maximum number of non-overlapping RB sets configurable per DU cell is M

- The value of the maximum number of contiguous and non-overlapping RB sets configurable per DU cell, M is 8.

- DU frequency configuration information should be provided to the parent node.

- The start RB index of the first RB set for the Rel-17 IAB-DU HSNA resource configuration is the lowest index of RB of the IAB-DU cell.

There are discussion and controversial opinions regarding the scenario that the configured Rel-17 H/S/NA RB sets do not cover the entire carrier bandwidth. Especially for the example of carrier bandwidth of 100MHz with SCS120kHz, there are maximum 66 RBs. To make the number of non-overlapping RB sets less or equal to M=8, the RB set size has to be chosen from N=8, 16, 32 or 64 and none of those choices is dividable by 66. There will be two RBs either belongs to the last RB set (with RB set size not equal to N) or not covered by the RB sets configuration.

Hence, there are two RB set configuration options for some scenarios.

- Option 1: RB sets can be non-equal-sized allowing the last RB set to have a different size than N.

- Option 2: RB sets configuration may not cover the entire bandwidth.

For Option 1, since it has already been agreed in RAN1#106e-RAN1#106bis-e that a single value of RB set N is configured with N = {2,4,8,16,32,64}, the last RB set with different RB set size may not fulfil those agreements requirements. Hence, we need to consider the scenarios in Option 2 that RB sets configuration may not cover the entire bandwidth.

When RB sets configuration does not cover the entire bandwidth, there are two scenarios

- Scenario 1: RBs which are non-overlapping with the IAB-MT’s active BWPs

- Scenario 2: RBs which are overlapping with the IAB-MT’s active BWPs

For Scenario 1, since Rel-17 frequency-domain H/S/NA configuration is to FDM IAB-DU/IAB-MT, for RBs not overlapping with the IAB-MT’s active BWPs, they can be directly treated as Hard resources for the IAB-DU.

For Scenario 2, if some leftover RBs are not configured with Rel-17 H/S/NA, there are some discussions on whether to let those resources follow the Rel-16 H/S/NA configuration. However, this will become very confusing and complicated to fulfil TDM/FDM since for one resource type there will be both Rel-16 H/S/NA in some RBs and Rel-17 H/S/NA in some RBs. Hence, we think it is better to assume that RBs which are overlapping with the IAB-MT’s active BWPs will be configured as RB sets for Rel-17 H/S/NA configuration.

Observation 2: There are two RB set configuration options for some scenarios.

- Option 1: RB sets can be non-equal-sized allowing the last RB set to have a different size than N.

- Option 2: RB sets configuration may not cover the entire bandwidth.

Option 1 does not fulfil the agreement requirements of a single value of RB set N configured with N = {2,4,8,16,32,64}.

Observation 3: When RB sets configuration does not cover the entire bandwidth, there are two scenarios

- Scenario 1: RBs which are non-overlapping with the IAB-MT’s active BWPs

- Scenario 2: RBs which are overlapping with the IAB-MT’s active BWPs

For Scenario 2, if some leftover RBs are not configured with Rel-17 H/S/NA but follow the Rel-16 H/S/NA configuration, it will become very confusing and complicated to fulfil TDM/FDM since for one resource type there will be both Rel-16 H/S/NA in some RBs and Rel-17 H/S/NA in some RBs.

Proposal 2: The scenario that RB sets configuration may not cover the entire bandwidth needs to be considered.

Proposal 3: When RB sets configuration does not cover the entire bandwidth:

- RBs which are non-overlapping with the IAB-MT’s active BWPs are treated as Hard resources.

- RBs which are overlapping with the IAB-MT’s active BWPs will be configured as RB sets for Rel-17 H/S/NA configuration.

# DCI format 2_5 Configuration

Regarding DCI format 2_5 for frequency-domain soft availability indication, RAN1#105e-RAN1#108e have made the following agreements.

- DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources.

- A single DCI format 2_5 can be received indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell.

- For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell:

  - AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group

  - One RB set group consists of one or multiple RB sets

- For the RRC signaling for the configuration of DCI Format 2_5

  - Number of RB set groups has a max value of [8] within a slot.

  - Number of RB sets for each group has a max value of [8].

- RB set with Rel-17 H/S/NA configuration                        (b) RB set group configuration

Figure 1: RB set and RB set group configuration illustration

In Figure 1, we show an example of RB set and RB set group configuration in one slot, where each RB set group is configured as continuous soft RB sets. As the maximum number of RB sets per DU cell is M=8, it is straightforward that the number of RB sets for each group has a max value of 8.

However, regarding the maximum number of RB set groups within a slot, we don’t see the needs of as many as 8 RB set groups per slot for soft availability indication. Comparing with time-domain soft availability indication with 3-bitmap indication within a slot, 8 RB set groups configuration in frequency-domain requires 8-bitmap indication per slot, which will cause a big DCI format 2_5 payload size increase.  Hence, to make DCI format 2_5 payload size comparable for time-domain/frequency-domain soft availability indication, the maximum number of RB set groups within a slot should be 3 and then there is no need to extend the maximum payload size of DCI format 2_5.

Proposal 4: For the RRC signalling for the configuration of DCI Format 2_5

- Number of RB set groups has a max value of 3 within a slot.

- Number of RB sets for each group has a max value of 8.

Proposal 5: No need to extend the maximum payload size of DCI format 2_5.

# Conclusion

In this contribution, we discussed remaining details on frequency-domain multiplexing for IAB.  It is summarized by the following observations and proposals.

Observation 1: Regarding always mandating Rel-17 H/S/NA configuration when provided, we have the following observations.

        - Always semi-statically mandating Rel-17 FDM configuration eliminates an IAB-node’s dynamic option of operating in TDM and reverses RAN1#107e agreement of TDM/non-TDM operations by IAB implementation.

        - It is not possible for an IAB-node and its parent node semi-statically have the same understanding on the selected H/S/NA configuration (if both Rel-17 H/S/NA and Rel-16 H/S/NA are configured) since an IAB-node will dynamically choose to operate in TDM or FDM by its implementation.

        - An IAB node and its parent node may need to remain both Rel-17/Rel-16 H/S/NA configuration and the parent node may be dynamically informed from an IAB-node which will be applied.

Observation 2: There are two RB set configuration options for some scenarios.

- Option 1: RB sets can be non-equal-sized allowing the last RB set to have a different size than N.

- Option 2: RB sets configuration may not cover the entire bandwidth.

Option 1 does not fulfil the agreement requirements of a single value of RB set N configured with N = {2,4,8,16,32,64}.

Observation 3: When RB sets configuration does not cover the entire bandwidth, there are two scenarios

- Scenario 1: RBs which are non-overlapping with the IAB-MT’s active BWPs

- Scenario 2: RBs which are overlapping with the IAB-MT’s active BWPs

For Scenario 2, if some leftover RBs are not configured with Rel-17 H/S/NA but follow the Rel-16 H/S/NA configuration, it will become very confusing and complicated to fulfil TDM/FDM since for one resource type there will be both Rel-16 H/S/NA in some RBs and Rel-17 H/S/NA in some RBs.

Proposal 1: TDM fallback / FDM resume indication from child node (e.g. MAC CE) is supported.

Proposal 2: The scenario that RB sets configuration may not cover the entire bandwidth needs to be considered.

Proposal 3: When RB sets configuration does not cover the entire bandwidth:

- RBs which are non-overlapping with the IAB-MT’s active BWPs are treated as Hard resources.

- RBs which are overlapping with the IAB-MT’s active BWPs will be configured as RB sets for Rel-17 H/S/NA configuration.

Proposal 4: For the RRC signalling for the configuration of DCI Format 2_5

- Number of RB set groups has a max value of 3 within a slot.

- Number of RB sets for each group has a max value of 8.

Proposal 5: No need to extend the maximum payload size of DCI format 2_5.