3GPP TSG RAN WG1 Meeting #109-e	R1-2204780

e-Meeting, May 09 – 20, 2022

Source:	Intel Corporation

Title:	Discussion on UE capabilities for extending NR up to 71 GHz

Agenda item:	8.16.2

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss aspects related to UE capability signaling for NR extension up to 71 GHz. The following table is copy of latest endorsed RAN1 UE feature list from [1].

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-1 | Basic FR2-2 DL support | 1. Support reception of 120kHz subcarrier spacing for DL data and control channels, SSB, and reference signals in FR2-2 for non-initial access |  | per band | A UE that supports FR2-2 must indicate this FG is supported | Optional with capability signalling |
| 24-1a | Basic FR2-2 UL support | 1. PRACH with 120KHz SCS and length 139 2. Support transmission of 120kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2 | 24-1 | per band |  | Optional with capability signalling |
| 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Per band | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] | Optional withcapability signalling |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Per band | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | 24-1 | Per band |  | Optional with capability signalling |
| 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI |  | Per band |  | Optional with capability signalling |
| 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | Per band |  | Optional with capability signalling |
| 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | Per band |  | Optional with capability signalling |
| 24-2 | 120KHz SSB support for initial access in FR2-2 | 1. Support 120KHz SSB for initial access in FR2-2 | 24-1, 24-1a | per band |  | Optional with capability signalling |
| 24-3 | 480KHz SSB support for initial access in FR2-2 | 1. Support 480KHz SSB for initial in FR2-2 | 24-2, 24-4, 24-4a | per band |  | Optional with capability signalling |
| 24-4 | 480KHz SCS support for DL | 1. 480KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (4, 3) and (7, 3) are supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Perband |  | Optional with capability signalling |
| 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | Per band |  | Optional with capability signalling |
| 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Per band | [Note: This FG is only supported in bands for shared spectrum operation] | Optional with capability signalling |
| 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24-4f | Enhanced PDCCH monitoring for 480KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys)=(4,2) 2.) Within each of the Ys = 2 slots (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot | 24-4 | Per band |  | Optional with capability signalling |
| 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=8), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y)= (7, 3) is supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] | 24-1 | Per band |  | Optional with capability signalling |
| 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1a, 24-5 | Per band |  | Optional with capability signalling |
| 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Per band | This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |
| 24-5f | Enhanced PDCCH monitoring for 960KHz in FR2-2 | 1. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys) 2.) Within each of the Ys = 2 (with Xs=4) or Ys = 4 (with Xs=8) slots, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS in the first 3 OFDM symbols of each slot or within the Ys = 1 (with Xs=4) slot, monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (7, 3) | 24-5 | Per band | Component 1 candidate values: one or more of {(4,1), (4,2), (8,4)} | Optional with capability signalling |
| 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] | 24-1a, 24-6 | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24-8 | 32 DL HARQ processes for FR 2-2 | Support 32 HARQ processes in DL for 120/480/960 kHz | 24-1 | FFS | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24-9 | 32 UL HARQ processes for FR 2-2 | Support 32 HARQ processes in UL for 120/480/960 kHz | 24-1 | FFS | A UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL in FR2-2 | Optional with capability signalling |
| 24-10 | Reduced beam switching time delay | Support of reduced beam switching time delay d = 56 symbols for 480 kHz SCS |  | per band | If this capability is not reported and the UE supports both FG 24-4 and 24-5, the default value of 112 symbols is assumed | Optional with capability signalling |

# Discussion on UE Features

## Wideband PRACH/PUCCH

For UE operating with unlicensed SA mode, it needs to also support wideband PUCCH and wideband PRACH together. The main reason is that the wideband PUCCH and wideband PRACH were specifically targeted to improve the coverage issues due to power spectral density in unlicensed bands. However, if the UE optionally do not support this feature, this would shrink the potentially coverage for unlicensed cell deployments. Therefore, all UEs that is able to operate uplink transmission should support wideband PUCCH and wideband PRACH. Otherwise, the main motivation to introduce the feature is lost.

Based on this we propose the following:

Proposal 1: FG 24-1b and 24-1c add/keep the note that “A UE that support FG24-2 must indicated this FG is supported”.

| 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Per band | [A UE that supports FG 24-2 must indicate this FG is supported] [Note: This FG is only supported in bands for shared spectrum operation] | Optional withcapability signalling |
| --- | --- | --- | --- | --- | --- | --- |
| 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Per band | [A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported] This FG is only supported in bands under PSD limitation in shared spectrum operation | Optional with capability signalling |

## Capability CSS for 480/960 kHz

For FG 24-4 and 24-5, feature component 7 has not been finalized. The following is a copy of the open issues for FG24-4 and 24-5.

| 24-4 | 480KHz SCS support for DL | 1. 480KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (4, 3) and (7, 3) are supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] |
| --- | --- | --- |
| 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=8), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y)= (7, 3) is supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD [7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.] |

The main issue for component 7 is that there is still on-going discussion the whether what is the default/mandatory monitoring capability related to CSS monitoring. Given that this has more to do with design aspect of CSS monitoring for 480 kHz and 960 kHz, our preference is to wait for further agreement to be made in the 8.2.2 PDCCH enhancement agenda for NR up to 71 GHz WI.

Proposal 2: Revise component 7 of FG24-4 and FG24-5 after further conclusion is made on CSS PDCCH monitoring in PDCCH enhancement agenda of NR up to 71 GHz WI.

## Per Carrier/BWP LBT

In prior RAN#1 meeting, the following was concluded:

Agreement For LBT for single carrier UL transmission, UE performs LBT over a BW that at least includes the active UL BWP bandwidth - The BW that at least includes the active UL BWP bandwidth is captured as “channel” in 37.213 Agreement For LBT for single carrier DL transmission to a UE, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE. - This does not rule out gNB implementation to perform LBT over a wider bandwidth - The BW that at least includes the active DL BWP bandwidth is captured as “channel” in 37.213 For LBT for single carrier DL transmission to multiple UEs, from each UE point of view, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE. - This does not rule out gNB implementation to perform LBT over a wider bandwidth that includes the active DL BWP of multiple UEs

In light of these agreements, it may be preferred to align the terminology and update the text for feature 24-6 and 24-7 as follows:

| Index | Feature group | Components | Prerequisite feature groups | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] channel bandwidth | 24-1a | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
| 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure [2. Support LBT performed per carrier/BWP bandwidth] channel bandwidth | 24-1a, 24-6 | per band | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |

Proposal 3: Update the text in 24-6 and 24-7 as indicated above.

# Conclusions

In this contribution, we discussed issues related to UE capability signaling for NR extension up to 71 GHz. The following is a summary of the proposals:

Proposal 1: FG 24-1b and 24-1c add/keep the note that “A UE that support FG24-2 must indicated this FG is supported”.

Proposal 2: Revise component 7 of FG24-4 and FG24-5 after further conclusion is made on CSS PDCCH monitoring in PDCCH enhancement agenda of NR up to 71 GHz WI.

Proposal 3: Update the text in 24-6 and 24-7 as indicated above.

# References

- R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues” Moderators (AT&T, NTT DOCOMO, INC.)