3GPP TSG RAN WG1 Meeting #109-E					 	R1-2204781

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	On UE features for Rel-17 RedCap

Agenda item:	8.16.6

Document for:	Discussion and Decision

# Introduction

In this contribution, we present our views on remaining details for defining UE capabilities for RedCap UEs.

During RAN1 #108-E meeting, the following decisions were made on UE features for Rel-17 RedCap [1].

Agreement - Following capabilities are added as components in FG 28-1 - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - b) Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included Agreement - Component in FG 28-1 is updated as follows - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Add a note in FG 28-1: It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A Agreement - Delete FG 28-2 - (As conclusion) It is up to RAN4 whether/how to report UE Rx structure for FR2

In this contribution, we share our views on the remaining details for UE features for Rel-17 RedCap.

# UE capabilities and supported features for RedCap

On FGs #28-1 and #28-3

The latest version of UE features relevant for Rel-17 RedCap is as in [2]. The relevant FGs (#28-x family) are quoted from [2] in Appendix A.

Further, during RAN #95-e meeting, the following was agreed regarding reporting types for FGs #28-1 and 28-3.

- FG 28-1 is reported per UE, and FG 28-3 is reported per band

Given the above, there remains two opens (FFS’s) regarding the components of FG #28-1:

- Following capabilities are added as components in FG 28-1 - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - b) Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included Currently listed components for FG #28-1: 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - FFS whether to add any other basic features for RedCap UE

On the potential addition of the detail regarding UE expectation on CD-SSB presence for paging monitoring in separate initial DL BWP, while not strictly necessary, it can be seen as a salient characteristic of Rel-17 RedCap UEs, at the same level as the already-agreed component “For separate initial DL BWP only used for RACH, SSB may or may not be included”. Thus, it would be reasonable to also capture this detail for consistent handling vis-à-vis separate initial DL BWP for RACH-related DL reception.

For the remaining outstanding aspects, we think that all the brackets can be removed, the contents inside the brackets confirmed, and the FFS on whether to add any other basic feature for RedCap UE removed.

Proposal 1:

- Confirm the following as a component for FG #28-1:

  - For separate initial DL BWP used for paging, CD-SSB is included

- Remove the brackets and confirm the current descriptions for remaining details for FG #28-1.

- Remove the following FFS component:

  - FFS whether to add any other basic features for RedCap UE.

Similarly, for FG #28-3, all the outstanding details can be confirmed by removing the current brackets.

Proposal 2:

- Remove the brackets and confirm the current descriptions for remaining details for FG #28-3.

On FGs #6-1 and #6-1a

It is necessary to update FG #6-1 for Rel-17 RedCap UEs.

In particular, for RedCap UEs, operation without CORESET #0 within the active DL BWP should be mandatory and only expectation on SSB in RRC-configured DL BWP may be considered as a modification to FG #6-1.

Alternatively, a new conditionally mandatory FG for RedCap UEs such that the RedCap UE expects SSB in RRC-configured DL BWP should be introduced with FG #28-1 as pre-requisite.

Proposal 3:

- Adapt FG # 6-1 for RedCap UEs such that operation without CORESET #0 within the active DL BWP is mandated for RedCap UEs.

  - To implement this, add a note to FG 6-1: “For a UE supporting FG 28-1, component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)””

- Alternatively, introduce a new conditionally mandatory FG for RedCap UEs such that a RedCap UE may expect an SSB in RRC-configured DL BWP, with FG #28-1 as pre-requisite.

On FG #6-1a, considering that the need for measurement gaps or other adjustments for support of FG #6-1a is currently under discussions in RAN4, it would be prudent to wait for further feedback from RAN4 before deciding on whether FG #6-1a may apply to RedCap UE as is or require any updates, including possibly a new separate FG for RedCap UEs.

Proposal 4:

- For FG #6-1a, RAN1 to wait for further inputs from RAN4 to determine whether any updates, including possibly defining a new separate FG for RedCap UEs, may be necessary.

# Conclusions

In this contribution, we presented our views on UE features for Rel-17 RedCap.

Based on the presented discussion, our views can be summarized via the following proposals.

On FGs #28-1 and #28-3

Proposal 1:

- Confirm the following as a component for FG #28-1:

  - For separate initial DL BWP used for paging, CD-SSB is included

- Remove the brackets and confirm the current descriptions for remaining details for FG #28-1.

- Remove the following FFS component:

  - FFS whether to add any other basic features for RedCap UE.

Proposal 2:

- Remove the brackets and confirm the current descriptions for remaining details for FG #28-3.

On FGs #6-1 and #6-1a

Proposal 3:

- Adapt FG # 6-1 for RedCap UEs such that operation without CORESET #0 within the active DL BWP is mandated for RedCap UEs.

  - To implement this, add a note to FG 6-1: “For a UE supporting FG 28-1, component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)””

- Alternatively, introduce a new conditionally mandatory FG for RedCap UEs such that a RedCap UE may expect an SSB in RRC-configured DL BWP, with FG #28-1 as pre-requisite.

Proposal 4:

- For FG #6-1a, RAN1 to wait for further inputs from RAN4 to determine whether any updates, including possibly defining a new separate FG for RedCap UEs, may be necessary.

# References

- 3GPP RAN1 Chairman’s notes, RAN1 #108-e.

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e,” Moderator (NTT DOCOMO, INC., AT&T), RAN1 #107-E.

# Appendix A (from R1-2112902 [2])

- NR_redcap

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included FFS whether to add any other basic features for RedCap UE |  | Yes |  | [Network assumes the UE is not a RedCap UE] | [Per UE] | [No] | [No] |  | [RedCap UEs do not support carrier aggregation or dual connectivity.] It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A | Optional with capability signaling RedCap UE must indicate this FG is supported |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | [UE is assumed to support FD-FDD in FDD bands] | [Per band] | [FDD only] | [FR1 only] |  |  | Optional with capability signaling |