3GPP TSG RAN WG1 Meeting #109-e			                       R1- 2204782

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Discussion on UE features for UE power saving

Agenda item:	8.16.7

Document for:	Discussion and Decision

Introduction

In this contribution, we express our views on remaining aspects of UE features regarding paging enhancements and TRS/CSI-RS occasions in idle/inactive mode.

# Discussion on UE features for UE power saving

Paging enhancements

The following was agreed regarding FG 29-1 Paging Enhancement. We do not see a strong need to add component 3 and we think this can be removed.

Agreement

•            FG 29-1 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1: Component 3 in FG 29-1 is not anything new with respect to FG 29-1 and can be removed. If anything needs to be kept in component 3 at all, it can be only for Type 2A CSS.

TRS/CSI-RS occasions

The following was agreed regarding FG 29-2 in RAN1 # 108e.

Agreement

- FG 29-2 is updated as follows

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB [2. Support receiving L1 indication for TRS availability] | FFS | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode | Per UE N/A | N/A | N/A | N/A | [Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7] | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For the FG 29-2, we have the following suggestions

- Update component description of FG 29-2 as follows:

  - We suggest to at least revise component description of FG 29-2 to make it more clear how the feature works. Supporting PEI based TRS availability indication would require the UE to support FG 29-1. Without support of FG 29-1, it implies that UE can only receive TRS availability from paging DCI. To this end, we suggest following revisions by explicitly capturing the candidate L1 signaling options for clarity. Support of FG 29-2 should not imply UE supports FG 29-1.

TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

- If the above revision to component description is not acceptable and component 2 is kept as in yellow, then at least Note column needs to also capture the following

Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7

Observation 1: Current component description of FG 29-2 does not clearly describe how the feature works, such as whether support of L1 indication mandates support of receiving PEI as well when TRS is configured.

Proposal 2: Support one of the following regarding FG 29-2:

- Update component description as follows and Note column can be empty (preferred)

TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

- Confirm the content in yellow in both component 2 description and Note column

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability | FFS | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode | Per UE N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusions

In summary, we have following observations and suggestions to update the feature lists table for FGs 29-1/2

Observation 1: Current component description of FG 29-2 does not clearly describe how the feature works, such as whether support of L1 indication mandates support of receiving PEI as well when TRS is configured.

Proposal 1: Component 3 in FG 29-1 is not anything new with respect to FG 29-1 and can be removed. If anything needs to be kept in component 3 at all, it can be only for Type 2A CSS.

Proposal 2: Support one of the following regarding FG 29-2:

- Update component description as follows and Note column can be empty (preferred)

TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1

- Confirm the content in yellow in both component 2 description and Note column

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability | FFS | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode | Per UE N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

References

[1]    R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, Moderators (AT&T, NTT DOCOMO, INC.)