3GPP TSG RAN WG1 #109-e																		    R1-2204783

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	Discussion on UE features for NR coverage enhancement

Agenda item:	8.16.8

Document for:	Discussion and Decision

# Introduction

At the RAN1#108-e meeting, the following agreements were made regarding UE features for NR coverage enhancement [1].

Agreement

- Merge FGs 30-2 and 30-2a into an FG with per band

Agreement

- Merge FGs 30-1 and 30-1a into an FG with per band

  - Value range K = {20, 24, 28, 32}

Agreement

- FG 30-3a is not separated to multiple FGs

Agreement

- FG 30-4 is updated as follows

| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] Per band | FFS N/A | No N/A | N/A | [This is applicable for QPSK or lower modulation orders] From RAN1 perspective, all modulation orders are applicable. RAN4 to decide whether this note is required not | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FFS Send an LS to ask RAN4 whether RAN4 specifies requirements for modulation order higher than QPSK

Agreement

- FG 30-4b is not separated to one for within a slot and the other for over slots

Agreement

- FG 30-5 is not split to one for PUCCH formats 1/3/4 and the other for PUCCH formats 0/2

Agreement

- Capture the following in the LS to RAN2/RAN4 on updated RAN1 UE features list for NR

  - Based on RAN4 feedback, FG 30-4 is defined without distinction of the modulation order. RAN1 has no intention to revert any existing RAN4 agreements on maximum duration. It is up to RAN4 to decide whether FG 30-4 is applicable only to QPSK and lower modulation orders

To RAN WG4

ACTION: RAN1 kindly asks RAN4 to take the above information into account for their future discussion.

- Remove the text in the note column in FG 30-4

Agreement

- FG 30-3 is not split into 2 separate FGs: 1st one for DG, 2nd one for CG

Agreement

- Type of FG 30-3 is per band

Agreement

- Type of FG 30-3a is per band

Agreement

- Add a note in FG 30-4c: “Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS”

Agreement

- FG 30-6 is supported as optional with capability signaling

Agreement

- FG 30-1 is updated as follows

| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. Maximum value of K (the number of repetitions) = 32 For DG PUSCH, Tthe number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17. For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. A row index of the TDRA list is indicated by a DCI. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. | 5-14, 5-16, or [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] Per band | No N/A | No N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |

Agreement

- FG 30-2 is updated as follows

| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14. 5-16, [5-17], [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] Per band | FFS N/A | No N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |

Agreement

- Type of FG 30-5 is per UE

In the contribution, we discuss UE features for NR coverage enhancement. The updated UE features for NR coverage enhancement after RAN1#108-e meeting was captured in [2] and are listed in the Appendix.

# Discussion on UE features for NR coverage enhancement

## UE features for PUSCH repetition type A enhancement

At the RAN1#108-e meeting, it was agreed to merge FGs 30-2 and 30-2a into an FG with per band and merge FGs 30-1 and 30-1a into an FG with per band with K = {20, 24, 28, 32} [1]. In addition, at the RAN1#106b-e meeting, it was agreed that maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32 [3]. In our view, for the UE feature groups for PUSCH repetition type A enhancement, all the K values should be supported for the case with counting based on available slots.

Based on the discussions above, Table 1 illustrates suggested update for UE feature group for PUSCH repetition type A enhancement.

Table 1. UE feature group for PUSCH repetition type A enhancement

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots., where K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} |

Proposal 1

- For UE feature group of PUSCH repetition type A enhancement,

  - Consider Table 1 for UE feature group of PUSCH repetition type A enhancement.

## UE features for TBoMS

At the RAN1#108-e meeting, it was agreed that FG 30-3 is not split into 2 separate FGs: 1st one for DG, 2nd one for CG, and Type of FG 30-3 and 30-3a is per band [1]. Note that the number of allocated slots for TBoMS is indicated via a new column added to the TDRA table. Further, only DG-PUSCH and Type-2 CG-PUSCH are supported for TBoMS.

Based on the discussions above, Table 2 illustrates suggested update for UE feature groups for TBoMS.

Table 2. UE feature groups for TBoMS

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and Type-2 CG in RRC connected mode. | [11-6] |
| 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD One of {5-16, 11-6, 30-1}, 30-3 |

Proposal 2

- For UE feature groups of TBoMS,

  - Consider Table 2 for UE feature groups of TBoMS.

## UE features for DMRS bundling for PUSCH and PUCCH

At the RAN1#107b-e meeting, the following agreement was made for DMRS bundling restart during a nominal TDW [1].

Agreement Update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows. - UE bundles PUSCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements - UE bundles PUCCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.

According to the agreement, it is evident that subject to UE capability, UE restarts DMRS bundling within a nominal TDW only for the dynamic events that violate power consistency and phase continuity. For semi-static events that not triggered by DCI or MAC-CE, UE is mandatory to restart the DMRS bundling within a normal TDW. Hence, the description on FG30-4g needs to be updated.

For DMRS bundling, UE is required to maintain power consistency and phase continuity during an actual TDW. In order to meet the requirement, UE may need to consider or implement different RF components for different bands or in FR1 and FR2, especially when considering that residual absolute frequency offset may have different characteristic in low band and mmWave band. Hence, it may be more appropriate to define UE feature groups for DMRS bundling per band.

Based on the discussions above, Table 3 illustrates suggested update for UE feature groups for DMRS bundling.

Table 3. UE feature groups for DMRS bundling

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1], or [30-2], one of {5-14, 5-16, 5-17, 11-6} |
| 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] |
| 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3], 30-3a |
| 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] |
| 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] |
| 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] |
| 30-4g | [Restart DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g., frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. | [30-4] 30-4a, 30-4b, 30-4c, or 30-4d |
| 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)], respectively, | [30-4a, 30-4b, 30-4c, or 30-4d] |

Proposal 3

- For UE feature groups of DMRS bundling for PUSCH and PUCCH

  - UE features for DMRS bundling are defined per band.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 3 for UE feature groups of DMRS bundling.

## UE features for dynamic repetition factor indication for PUCCH

At the RAN1#106b-e meeting, the following agreement was made for dynamic PUCCH repetition indication [3].

Agreement Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4 with a unified mechanism as agreed in RAN1#106e under agenda 8.8.2. Note: it does not impact the discussion of slot level or sub-slot level repetition

Based on this and considering that slot based PUCCH repetition is mainly targeted for coverage enhancement, Table 4 illustrates suggested update for UE feature groups for dynamic PUCCH repetition factor indication.

Table 4. UE feature groups for dynamic PUCCH repetition factor indication

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | [4-23 and/or 25-2] |

Proposal 4

- For UE feature group of dynamic PUCCH repetition factor indication

  - Consider Table 4 for UE feature group of dynamic PUCCH repetition factor indication.

## UE features for Msg3 PUSCH repetition

For Msg3 with repetition, mechanism on PUSCH repetition type A is employed, i.e., same time domain resource allocation in each slot and counting based on available slot. Further, number of repetitions for Msg3 initial and retransmission is indicated via MCS field in RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. For the UE feature group of Msg3 PUSCH repetition, it is more appropriate to define this per UE. Further, FDD/TDD differentiation is not necessary.

Based on the discussions above, Table 5 iillustrates suggested update for UE feature group for Msg3 PUSCH repetition.

Table 5. UE feature group for Msg3 PUSCH repetition

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  |

Proposal 5

- For UE feature group of Msg3 PUSCH repetition

  - UE feature for Msg3 PUSCH repetition is defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 5 for UE feature group of Msg3 PUSCH repetition.

# Conclusions

In this contribution, we discussed UE features for NR coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- For UE feature group of PUSCH repetition type A enhancement,

  - Consider Table 1 for UE feature group of PUSCH repetition type A enhancement.

Proposal 2

- For UE feature groups of TBoMS,

  - Consider Table 2 for UE feature groups of TBoMS.

Proposal 3

- For UE feature groups of DMRS bundling for PUSCH and PUCCH

  - UE features for DMRS bundling are defined per band.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 3 for UE feature groups of DMRS bundling.

Proposal 4

- For UE feature group of dynamic PUCCH repetition factor indication

  - Consider Table 4 for UE feature group of dynamic PUCCH repetition factor indication.

Proposal 5

- For UE feature group of Msg3 PUSCH repetition

  - UE feature for Msg3 PUSCH repetition is defined per UE.

  - FDD/TDD differentiation is not necessary.

  - Consider Table 5 for UE feature group of Msg3 PUSCH repetition.

# References

- Chairman’s notes, RAN1 #108 e-Meeting, February 2022

- R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues”, February 2022

- Chairman’s notes, RAN1 #106b e-Meeting, October 2021

- Chairman’s notes, RAN1 #107b e-Meeting, January 2022

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | 5-14, 5-16, or 5-17 | Yes | N/A | UE does not support more than 16 repetitions PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [30-4] | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] | [30-4a, 30-4b, 30-4c, or 30-4d] | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | [4-23 and/or 25-2] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | Per UE | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | [No] | [No] | [N/A ] |  | Optional with capability signalling |