3GPP TSG RAN WG1 #109-e	                                                   	R1-2204786

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	On two TAs for multi-DCI

Agenda item:	9.1.1.2

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss 2-TA enhancement for Rel-18 NR MIMO WI [1]

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

# Evaluation methodology

Measurement of RSTD (the relative timing difference between 2 TRPs) at a UE in the downlink is a simple way to show the need for multiple TAs in the uplink. This method allows to capture the timing difference due to propagation delay difference from the 2 TRPs to the UE (assuming perfect cell synchronization). Although TA is also used by a gNB to create switching time gap between UL and DL transmission at the TRP it is a secondary issue and may not be that relevant to the current WID. A table with basic multi-TRP SLS assumptions is shown below. In terms of KPIs, we believe the following is useful:

- CDF of RSTD

- CDF of cross-TRP interference for downlink reception (FR2)

- CDF of power difference of received signals in the downlink at a UE

| Parameters | Dense urban | Indoor hotspot |
| --- | --- | --- |
| Carrier frequency | 30GHz | 30GHz |
| Channel model | TR38.901 |  |
| TP antenna configuration | 2 ports (8,8,2,1,1) for 30GHz | 2 ports: (4,4,2,1,1) for 30GHz |
| UE antenna configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180 |
|  | scheduler/channel estimation/CSI etc. are not needed |  |

Proposal-1: Consider multi-TRP EVM in FR2 with at least 2 panels at the UE, there is no need to consider throughput evaluations (scheduler/CSI etc. not essential).

Proposal-2: In terms of KPI consider the following -  RSTD at the UE, cross-TRP interference for downlink reception, power difference of received signals in the downlink at a UE

# Specifications impact

In terms of TA difference between the two TRPs, it is natural to consider TA difference of less than a CP – RAN4 may be more suitable to finalize a value for max TA difference.

Currently a serving cell is associated with one uplink timing and multiple serving cells within the same TAG is associated with the same uplink timing. The key specifications impact is to associate a serving cell with 2 uplink timings (2 TA fields). In addition to this, a serving cell may transition from single TRP operation to multi-DCI multi-TRP operation and vice-versa so it is beneficial to be able to activate/de-active a TA field per serving cell.

Currently, a TAG includes multiple serving cells and a serving cell belong to one TAG – how to associate TAs with TAGs may be left to RAN2.

Proposal-3: Consider associating a serving cell with 2 TA fields and activation/de-activation of TA fields per serving cell. How to associate TAs with TAGs may be left to RAN2 and defining a max TA difference may be left to RAN4

In order to apply the TA values properly to uplink transmission, a UE needs to associate an uplink transmission to a TA value field. Note that DL and UL transmissions need not be associated with the same TRP (or TCI state) – so re-using CORESETPoolIndex may not be sufficient. In that sense there is also no need to limit such operation to multi-DCI mTRP only.

Proposal-4: Consider associating unified TCI States and UL-TCI States with a TRP-Id

Proposal-5: Consider specifications change that can be also utilized for single DCI mTRP uplink transmissions

In the case of intra-cell mDCI mTRP, each of the TRPs may benefit from estimating TA from PRACH transmission, so it is beneficial to associate a TRP-Id to a TA value included in a RACH-response (RAR)

Proposal-6: Consider associating a TRP-Id to a TA value included in a RACH response (RAR)

In the case of inter-cell mDCI mTRP, it is beneficial for the TRP associated with additional PCI to be able to receive PDCCH ordered PRACH transmissions from a UE in order to directly estimate initial TA. Further TA adjustments can be done from other uplink signals like SRS, PUSCH-DMRS etc. Note that in FR1, a TRP can simply “listen” into the PRACH transmission as UE transmissions are not directional in nature. However, this is not possible in FR2. One possibility is for the UE to perform TA pre-compensation based on downlink time-difference with refence to the serving cell and report this TA to the NW.

Proposal-7: Study methods for initial TA acquisition by the TRP associated with the non-serving cell PCI. Possible options include TA pre-compensation at the UE and reporting of TA to the NW.

# Conclusion

In this paper, we discuss our view for two TAs for multi-DCI and summarize our proposals below:

Proposal-1: Consider multi-TRP EVM in FR2 with at least 2 panels at the UE, there is no need to consider throughput evaluations (scheduler/CSI etc. not essential).

Proposal-2: In terms of KPI consider the following -  RSTD at the UE, cross-TRP interference for downlink reception, power difference of received signals in the downlink at a UE

Proposal-3: Consider associating a serving cell with 2 TA fields and activation/de-activation of TA fields per serving cell. How to associate TAs with TAGs may be left to RAN2 and defining a max TA difference may be left to RAN4

Proposal-4: Consider associating unified TCI States and UL-TCI States with a TRP-Id

Proposal-5: Consider specifications change that can be also utilized for single DCI mTRP uplink transmissions

Proposal-6: Consider associating a TRP-Id to a TA value included in a RACH response (RAR)

Proposal-7: Study methods for initial TA acquisition by the TRP associated with the non-serving cell PCI. Possible options include TA pre-compensation at the UE and reporting of TA to the NW.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, 3GPP TSG RAN #94e, Electronic Meeting, Dec. 6 - 17, 2021