3GPP TSG RAN WG1 #109-e	                                                              R1-2204788

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	DM-RS Enhancements for Rel-18 NR

Agenda item:	9.1.3.1

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss DM-RS capacity enhancement for Rel-18 NR MIMO WI [1].

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS - Study, and if justified, specify UL DMRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

# DM-RS Capacity Enhancement

In NR Rel-15, two different DM-RS types were designed namely Type-1 and Type-2 DM-RS which are shown in Figure 1.

Figure 1: NR DM-RS Type 1

For the single symbol case, Type 1 DMRS uses a comb-2 structure with 2 CDM-Groups and length-2 FD-OCC per pair of alternating REs in each CDM-Group as shown in Figure 1.  The length-2 OCC is given by the columns of the length-2 Hadamard matrix

For the case of 2 symbol DM-RS the same length 2 OCC is used across the two time-domain symbols to multiplex 2 additional ports in each CDM group such that a total of 8 ports are supported. The following table shows the mapping of time domain OCC and frequency domain OCC to the supported DM-RS ports.

Table 7.4.1.1.2-1: Parameters for PDSCH DM-RS configuration type 1.

| p | CDM group | Δ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 1 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 1 | +1 | -1 | +1 | +1 |
| 1004 | 0 | 0 | +1 | +1 | +1 | -1 |
| 1005 | 0 | 0 | +1 | -1 | +1 | -1 |
| 1006 | 1 | 1 | +1 | +1 | +1 | -1 |
| 1007 | 1 | 1 | +1 | -1 | +1 | -1 |

Type 2 DMRS uses a comb-3 structure with 3 CDM-Groups and length-2 FD-OCC per pair of adjacent REs in each CDM-Group as shown in Figure 2.

Figure 2:  NR DM-RS Type 2

The FD-OCC is given by the columns of the same matrix H. And similar time domain multiplexing as in the case of  Type 1 DM-RS is used as shown in the following table.

Table 7.4.1.1.2-2: Parameters for PDSCH DM-RS configuration type 2.

| p | CDM group | Δ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 2 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 2 | +1 | -1 | +1 | +1 |
| 1004 | 2 | 4 | +1 | +1 | +1 | +1 |
| 1005 | 2 | 4 | +1 | -1 | +1 | +1 |
| 1006 | 0 | 0 | +1 | +1 | +1 | -1 |
| 1007 | 0 | 0 | +1 | -1 | +1 | -1 |
| 1008 | 1 | 2 | +1 | +1 | +1 | -1 |
| 1009 | 1 | 2 | +1 | -1 | +1 | -1 |
| 1010 | 2 | 4 | +1 | +1 | +1 | -1 |
| 1011 | 2 | 4 | +1 | -1 | +1 | -1 |

We next discuss methods to increase the number of DM-RS ports for each DM-RS Type.

## Principles for DM-RS Capacity Enhancement

For single-TRP scenario, DM-RS enhancements in Rel-18 should focus on DM-RS overhead reduction for already supported maximum number of DM-RS ports. For example, 8 ports for Type I DM-RS should be supported with one-symbol DM-RS configuration instead of two-symbol configuration in Rel-17. A key design principle should be to maintain backwards compatibility with legacy DM-RS structure where pairing for MU-MIMO should be possible between new DM-RS ports and legacy DM-RS ports. Therefore, we have the following proposal for design principles of Rel-18 DM-RS

- The following are the key DM-RS design principles for Rel-18 NR

- Consider DM-RS design which is backward compatible with Rel-15 DM-RS structure, i.e., pairing of legacy DM-RS and new DM-RS ports should be possible

- For Type I DM-RS, additional CS may be considered

- For Type II DM-RS, OCC of larger length spanning 4 REs for single symbol DM-RS or 8 REs for 2 symbol DM-RS configuration may be considered

- For coherent JT, further study is needed on the required number of orthogonal DM-RS ports

## Type 1 DM-RS

### Additional FD-OCC

For Type 1 DM-RS, considering the case of single symbol configuration, the number of ports can be doubled to support 8 orthogonal DM-RS ports by introducing two additional orthogonal cover codes over the 6 REs in each CDM group of the DM-RS to additionally multiplex two more DM-RS antenna ports within each CDM group. The OCC can be a length-6 cover code which is given by the columns of the size 6 DFT matrix as follows

i.e.,

Considering the Rel-15 DM-RS ports, the first column of the matrix C is the currently supported FD-OCC for DM-RS ports 1000, 1002,1004,1006 and the fourth column of matrix C is the currently supported FD-OCC for DM-RS ports 1001,1003,1005,1007 respectively.

The addition of 4 more ports can be achieved by introducing two additional FD-OCCs, selected from the columns 2,3,5,6 of the matrix C. For example, columns 2 and 5 can be selected as the additional OCCs over the 6 REs belonging to each CDM group. Based on this choice of FD-OCC, the following new ports 1008-1015 can be added to Type 1 DM-RS for single symbol and two-symbol case, where the two-symbol case uses the same length 2 TD-OCC from matrix H. This can be captured in the following table

Table 1.1: Parameters for PDSCH DM-RS configuration type 1.

| p | CDM group | Δ |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | -1 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 1 | +1 | +1 | +1 | +1 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 1 | +1 | -1 | +1 | -1 | +1 | -1 | +1 | +1 |
| 1004 | 0 | 0 | +1 | +1 | +1 | +1 | +1 | +1 | +1 | -1 |
| 1005 | 0 | 0 | +1 | -1 | +1 | -1 | +1 | -1 | +1 | -1 |
| 1006 | 1 | 1 | +1 | +1 | +1 | +1 | +1 | +1 | +1 | -1 |
| 1007 | 1 | 1 | +1 | -1 | +1 | -1 | +1 | -1 | +1 | -1 |
| 1008 | 0 | 0 | +1 |  |  | -1 |  |  | +1 | +1 |
| 1009 | 0 | 0 | +1 |  |  | +1 |  |  | +1 | +1 |
| 1010 | 1 | 1 | +1 |  |  | -1 |  |  | +1 | +1 |
| 1011 | 1 | 1 | +1 |  |  | +1 |  |  | +1 | +1 |
| 1012 | 0 | 0 | +1 |  |  | -1 |  |  | +1 | -1 |
| 1013 | 0 | 0 | +1 |  |  | +1 |  |  | +1 | -1 |
| 1014 | 1 | 1 | +1 |  |  | -1 |  |  | +1 | -1 |
| 1015 | 1 | 1 | +1 |  |  | +1 |  |  | +1 | -1 |

The UE should assume the DM-RS sequence  is scaled by a factor  to conform with the transmission power specified in [TS 38.214] and mapped to resource elements k,lp,μ according to

where , , and Δ are given by Table 1.1.

- To increase the number of orthogonal DM-RS ports within each symbol, columns 2 and 5 of matrix C can be selected as the additional OCCs over the 6 REs belonging to each CDM group. New ports 1008-1015 can be added for Type 1 DMRS

## Type 2 DM-RS

For Type 2 DM-RS, considering the case of single symbol configuration, the number of ports can be doubled to support 12 orthogonal DM-RS ports.

- For Type-2 DM-RS, two additional orthogonal cover codes can be introduced over the 4 REs in each CDM group of the DM-RS to additionally multiplex two more DM-RS antenna ports within each CDM group.

The OCC can be a length-4 cover code which is given by the columns of the size 4 Hadamard matrix as follows

The first two columns of CH are currently supported for DM-RS ports 1000-1011 as shown in Table 7.4.1.1.2-2 from TS 38.211 v17.0.0 if we consider the mapping across all 4 REs in each CDM group from Figure 2. Using this approach, 6 additional ports (1012-1017) can be multiplexed in one symbol DM-RS and another 6 additional ports (1018-1023) can be multiplexed for two symbol DM-RS. This is shown in Table 1.2.

Table 1.2: Parameters for PDSCH DM-RS configuration type 2.

| p | CDM group | Δ |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 2 | +1 | +1 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 2 | +1 | -1 | +1 | -1 | +1 | +1 |
| 1004 | 2 | 4 | +1 | +1 | +1 | +1 | +1 | +1 |
| 1005 | 2 | 4 | +1 | -1 | +1 | -1 | +1 | +1 |
| 1006 | 0 | 0 | +1 | +1 | +1 | +1 | +1 | -1 |
| 1007 | 0 | 0 | +1 | -1 | +1 | -1 | +1 | -1 |
| 1008 | 1 | 2 | +1 | +1 | +1 | +1 | +1 | -1 |
| 1009 | 1 | 2 | +1 | -1 | +1 | -1 | +1 | -1 |
| 1010 | 2 | 4 | +1 | +1 | +1 | +1 | +1 | -1 |
| 1011 | 2 | 4 | +1 | -1 | +1 | -1 | +1 | -1 |
| 1012 | 0 | 0 | +1 | +1 | -1 | -1 | +1 | +1 |
| 1013 | 0 | 0 | +1 | -1 | -1 | +1 | +1 | +1 |
| 1014 | 1 | 2 | +1 | +1 | -1 | -1 | +1 | +1 |
| 1015 | 1 | 2 | +1 | -1 | -1 | +1 | +1 | +1 |
| 1016 | 2 | 4 | +1 | +1 | -1 | -1 | +1 | +1 |
| 1017 | 2 | 4 | +1 | -1 | -1 | +1 | +1 | +1 |
| 1018 | 0 | 0 | +1 | +1 | -1 | -1 | +1 | -1 |
| 1019 | 0 | 0 | +1 | -1 | -1 | +1 | +1 | -1 |
| 1020 | 1 | 2 | +1 | +1 | -1 | -1 | +1 | -1 |
| 1021 | 1 | 2 | +1 | -1 | -1 | +1 | +1 | -1 |
| 1022 | 2 | 4 | +1 | +1 | -1 | -1 | +1 | -1 |
| 1023 | 2 | 4 | +1 | -1 | -1 | +1 | +1 | -1 |

The UE should assume the DM-RS sequence  is scaled by a factor  to conform with the transmission power specified in [TS 38.214] and mapped to resource elements k,lp,μ according to

where , , and Δ are given by Table 1.2.

- For DM-RS Type-2, the OCC [1 1 -1 -1] can be used as the FD-OCC for new DM-RS ports 1012, 1014, 1016, 1018, 1020, 1022 and the OCC [1 -1 -1 1] can be used as the FD-OCC for new DM-RS ports 1013, 1015, 1017, 1019, 1021, 1023.

# DM-RS for 8Tx Uplink Transmissions

## DMRS enhancement for SU-MIMO

As described in previous sections, in Rel-15 uplink operation, 4-ports can be supported for Type-1 DMRS with single symbol, 6-ports can be supported for Type-2 DMRS with single symbol.

In Rel-18, up to 8Tx can be supported in uplink transmission. 4 or more layers will be supported for SU-MIMO. If more than 4 layers are supported, then the number of DMRS ports should be enhanced accordingly for SU-MIMO, e.g., 8-ports DMRS could be introduced for SU-MIMO operation.

In order to increase the number of DMRS ports for SU-MIMO, the solution to enhance DMRS capacity for uplink MU-MIMO as described in previous sections could be re-used.

- If more than 4 layers are supported for 8Tx uplink transmission, the scheme to enhance DMRS capacity for uplink MU-MIMO could be re-used to increase the number of DMRS ports for SU-MIMO.

If the number of DMRS ports is increased, one issue is the Antenna Ports field in DCI scheduling PUSCH should be enhanced. The existing Antenna Ports field in DCI scheduling PUSCH only support up to Rank-4 transmission.

The Antenna Ports field for sing symbols Type-1 DMRS (CP-OFDM) is shown in Figure 4 to Figure 7.

Figure 4 Antenna Ports field with single symbol Type-1 DMRS for CP-OFDM and Rank-1

Figure 5 Antenna Ports field with single symbol Type-1 DMRS for CP-OFDM and Rank-2

Figure 6 Antenna Ports field with single symbol Type-1 DMRS for CP-OFDM and Rank-3

Figure 7 Antenna Ports field with single symbol Type-1 DMRS for CP-OFDM and Rank-4

Therefore, if more than 4 layers are supported for 8Tx uplink transmission, the enhancement on Antenna Ports field should be further discussed in RAN1.

- If more than 4 layers are supported for 8Tx uplink transmission, RAN1 to discuss the Antenna Port indication for DMRS enhancement for SU-MIMO.

# Conclusion

In this paper, DM-RS design for Rel-18 NR was discussed and the following proposals were made

- The following are the key DM-RS design principles for Rel-18 NR

- Consider DM-RS design which is backward compatible with Rel-15 DM-RS structure, i.e., pairing of legacy DM-RS and new DM-RS ports should be possible

- For Type I DM-RS, additional CS may be considered

- For Type II DM-RS, OCC of larger length spanning 4 REs for single symbol DM-RS or 8 REs for 2 symbol DM-RS configuration may be considered

- For coherent JT, further study is needed on the required number of orthogonal DM-RS ports

- To increase the number of orthogonal DM-RS ports within each symbol, columns 2 and 5 of matrix C can be selected as the additional OCCs over the 6 REs belonging to each CDM group. New ports 1008-1015 can be added for Type 1 DMRS

- For Type-2 DM-RS, two additional orthogonal cover codes can be introduced over the 4 REs in each CDM group of the DM-RS to additionally multiplex two more DM-RS antenna ports within each CDM group.

- For DM-RS Type-2, the OCC [1 1 -1 -1] can be used as the FD-OCC for new DM-RS ports 1012, 1014, 1016, 1018, 1020, 1022 and the OCC [1 -1 -1 1] can be used as the FD-OCC for new DM-RS ports 1013, 1015, 1017, 1019, 1021, 1023.

- If more than 4 layers are supported for 8Tx uplink transmission, the scheme to enhance DMRS capacity for uplink MU-MIMO could be re-used to increase the number of DMRS ports for SU-MIMO

- If more than 4 layers are supported for 8Tx uplink transmission, RAN1 to discuss the Antenna Port indication for DMRS enhancement for SU-MIMO.

# References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, TSG RAN Meeting #94e, Samsung

- 3GPP TS 38.211 v17.0.0, NR Physical Channels and Modulation (Release 17)

- 3GPP TS 38.212 v17.0.0, NR Multiplexing and Channel Coding (Release 17)

- 3GPP TS 38.213 v17.0.0, NR Physical Layer Procedures for Control (Release 17)

- 3GPP TS 38.214 v17.0.0, NR Physical Layer Procedures for Data (Release 17)