3GPP TSG RAN WG1 #109-e	                                               R1-2204790

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	UL precoding indication for multi-panel transmission

Agenda item:	9.1.4.1

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss UL transmission enhancement to precoding indication for multi-panel transmission for Rel-18 NR MIMO WI [1]

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

# UL Precoding Indication for Multi-panel Transmission

In Rel-17 NR, for codebook (CB) based transmission, the UE determines its PUSCH transmission precoder(s) based on SRI(s), TPMI(s) and the transmission rank, where the SRIs can be given by the DCI field of SRS resource indication (SRI) and precoding information and number of layers (PINL), or given by RRC parameters such as srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2. For non-codebook (nCB) based transmission, the UE can determine its PUSCH precoder(s) and transmission rank based on the SRI(s) when multiple SRS resources are configured.

In Rel-17 NR, the DCI scheduling UL transmission can indicate the spatial filter and power control parameters in SRI(s). However, unified TCI framework has also been agreed in Rel-17 for single-TRP transmission, where the TCI states are mapped to reference signals and power control parameter sets. In Rel-18 multi-TRP operation, unified TCI framework should also be adopted. Thus, the DCI may not need to contain the SRI field(s) when scheduling CB-based PUSCH transmissions. For nCB-based PUSCH transmission, the transmission rank can be indicated through the DCI field of SRI(s).

Proposal-1. For simultaneous multi-panel PUSCH transmission, the precoding indication for CB-based transmission need not be based on SRI fields, while the precoding indication for nCB-based transmission can be based on SRI fields.

In Rel-17 NR, only single-DCI based PUSCH transmission and repetition is supported. To support simultaneous multi-TRP multi-panel PUSCH (SxPUSCH) transmission operations in Rel-18, as shown in Figure 1, different transmission schemes can be considered. For example, the mTRP transmissions can be scheduled by either a single DCI or multiple DCIs. In particular, single-DCI based scheme schedules PUSCH transmissions by one DCI that is either transmitted through one TRP or multiple TRPs. Multi-DCI based scheme schedules the PUSCH transmissions by multiple DCIs through multiple TRPs.

Figure 1. Simultaneous multi-TRP multi-panel UL transmission

As the Rel-18 MIMO WI indicated, the total number of layers is up to four across all panels and total number of codewords is up to two across all panels. There can be different several layer combinations, namely, 1 + 1, 1 + 2, 1 + 3, 2 + 1, 2 + 2, and 3 + 1, where the number at the left-hand-side and right-hand-side of ‘+’ represents the number of layers for the PUSCH towards TRP1 and TRP2 respectively. Thus, the maximum number of layer for one of the PUSCH transmission in SxPUSCH is up to 3. Then, we have the following proposal.

Proposal-2. For simultaneous multi-panel PUSCH transmission, consider the layer combinations 1 + 1, 1 + 2, 1 + 3, 2 + 1, 2 + 2, and 3 + 1.

Next, the Rel-17 precoding indication for single-DCI based multi-TRP PUSCH repetition is based on separate SRI fields and separate PINL fields for the transmission to TRP1 and TRP2. However, the two SRIs and two TPMIs can also be indicated jointly to reduce the SRI and PINL field overhead. Since there are six different layer combinations, the joint SRI indication and joint PINL indication can be an option to reduce the DCI field overhead comparing with separate SRI indication and separate PINL indication. Therefore, we have the following proposal.

Proposal-3. For simultaneous multi-panel PUSCH transmission, select one of the following options for SRI indication for CB-based and nCB-based transmission and PINL indication for CB-based transmission.

- Option-1: separate SRI indication, separate PINL indication.

- Option-2: joint SRI indication, joint PINL indication.

Furthermore, to indicate the precoders and transmission ranks for SxPUSCH transmission for all the above layer combinations can increase the DCI field overhead compared to Rel-17. To reduce the overhead, some restrictions can be considered in different scenarios. Thus, we have the following proposal.

Proposal-4. For simultaneous multi-panel PUSCH transmission, introduce restrictions to layer combinations.

## Precoding Indication for Single-DCI based Transmission

Since the PUSCH transmission to TRP1 and TRP2 can be either CB-based or nCB-based, there are four different combinations, CB + CB, nCB + nCB, CB + nCB, and nCB + CB.

In single-DCI based SxPUSCH transmission, the DCI should indicate SRIs and TPMIs and number of layer information for the PUSCH to both TRPs. In CB + CB case, two PINLs should be indicated. In nCB + nCB case, two SRIs should be indicated. In CB + nCB and nCB + CB cases, two SRIs and one PINL should be indicated. We prefer to consider the CB + CB and nCB + nCB cases as a starting point in single-DCI based simultaneous multi-panel PUSCH transmission.

Proposal-5. For single-DCI based simultaneous multi-panel PUSCH transmission, consider CB + CB based and nCB + nCB based transmissions.

## Precoding Indication for Multi-DCI based Transmission

In multi-DCI based SxPUSCH transmission, each DCI schedules a PUSCH transmission to a TRP. Since the two DCIs may not arrive to the UE at the same time, it is better that both DCI can include the whole precoding information for the scheduled transmission. Thus, the precoding indication for both PUSCH can follow the Rel-16 design.

Proposal-6. For multi-DCI based simultaneous multi-panel PUSCH transmission, reuse the Rel-16 design for precoding indication.

- Evaluation Methodology and Baseline

The following configuration can be used for LLS evaluations.

| Parameters | Potential values |
| --- | --- |
| Numerology | 30 kHz SCS, 50 MHz BW (4 GHz), 120 kHz SCS, 200 MHz BW (30 GHz) |
| Channel | TDL-C (300ns) in FR1, TDL-A (30ns) in FR2, 3 kmph |
| Baseline scheme | Rel-15/-16 PUSCH |
| TBS | 136 bits (0.1 QPSK), 14 symbol PUSCH |
| DMRS pattern | DM-RS configuration type 1 |
| Antenna config | 2Tx UE, 2Rx per TRP |
| Redundancy Version | [0 2 3 1] cycled |
| Schemes | TDM, SDM, FDM |
| Receiver assumption | selection diversity, soft-combining |
| Channel estimation | realistic |

# Conclusion

Proposal-1. For simultaneous multi-panel PUSCH transmission, the precoding indication for CB-based transmission need not be based on SRI fields, while the precoding indication for nCB-based transmission can be based on SRI fields.

Proposal-2. For simultaneous multi-panel PUSCH transmission, consider the layer combinations 1 + 1, 1 + 2, 1 + 3, 2 + 1, 2 + 2, and 3 + 1.

Proposal-3. For simultaneous multi-panel PUSCH transmission, select one of the following options for SRI indication for CB-based and nCB-based transmission and PINL indication for CB-based transmission.

- Option-1: separate SRI indication, separate PINL indication.

- Option-2: joint SRI indication, joint PINL indication.

Proposal-4. For simultaneous multi-panel PUSCH transmission, introduce restrictions to layer combinations.

Proposal-5. For single-DCI based simultaneous multi-panel PUSCH transmission, consider CB + CB based and nCB + nCB based transmissions.

Proposal-6. For multi-DCI based simultaneous multi-panel PUSCH transmission, reuse the Rel-16 design for precoding indication.

# References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021