3GPP TSG RAN WG1 #109-e	R1-2204791

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Discussion on enhancement for 8Tx UL transmission

Agenda item:	9.1.4.2

Document for:	Discussion and Decision

# Introduction

In RAN#94-e meeting the work item on MIMO evolution was agreed for Rel-18 [5]. The objective includes enhancements on 8Tx UL transmission.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we provide our views on the uplink transmission enhancements for Rel-18.

# Discussion on enhancement for 8Tx UL transmission

## 2.1 Codebook based transmission

### 2.1.1 UE coherence assumption

For codebook based PUSCH transmission, the UE coherence has been defined in Rel-15, including non-coherent, partial coherent, and full coherent. The full coherent UE can maintain relative phase among all the antennas, the partial coherent UE can maintain relative phase among a subset of the antennas, and the non-coherent UE can maintain relative phase among none of the antennas. The corresponding codebook subsets are also defined. For full coherent UE, it can be configured with full coherent codebook subset or partial coherent codebook subset or non-coherent codebook subset. For partial coherent UE, it can be configured with partial coherent codebook subset or non-coherent codebook subset. For non-coherent UE, it can be configured only with non-coherent codebook subset.

In Rel-15, the uplink transmission can support up to 4 ports. Figure 1 shows the precoders for Rank-1 with 4-port (CP-OFDM waveform). The precoding matrixes including non-coherent, partial coherent and full coherent TPMIs.

Figure 1 Rel-15 precoding matrix for Rank-1 with 4-ports (CP-OFDM)

It can be seen that in Rel-15 codebook, for partial coherent TPMIs, there are two ports between which the UE can maintain relative phase.

In Rel-18, in order to support 8Tx transmission in uplink, the UE coherence assumption should be further discussed, especially on the partial coherence.

There are two cases for partial coherent UE with 8-ports. The first case is that the number of ports among which the UE can maintain relative phase is 4. The second case is that the number of ports among which the UE can maintain relative phase is 2. Figure 2 illustrates the different structure for partial coherent UE with 8-ports.

Figure 2 Different structure for partial coherent UE with 8 Tx

Correspondingly, the precoding matrix for these two kinds of partial coherence will be different. For partial coherent UE with 4 co-phasing ports (as shown by part-a in Figure 2), the precoding matrix with Rank-1 could be:

12210101010,12210-1010-10,12201010101,122010-1010-1,…

For partial coherent UE with 2 co-phasing ports (as shown by part-b in Figure 2), the precoding matrix with Rank-1 could be:

12210100000, 122010-10000, 12200001010, 1220000010-1,…

Therefore, further discussion is needed on the partial coherence in Rel-18. We think full coherent UE could be a starting point for UL transmission with 8Tx since it is relatively simple.

Proposal 1:

- RAN1 to consider full coherence UE as starting point for UL transmission with 8Tx

### 2.1.2 Full power mode

For codebook based PUSCH transmission, full power operation was introduced in Rel-16. Three modes are defined, full power Mode 0, full power Mode 1 and full power Mode 2.

For full power Mode 0, the UE should be able to deliver full power over each PA. For full power Mode 1 and Mode 2, none of the UE PAs can deliver full power or only a subset of the UE PAs can deliver full power.

Table 1 shows a brief summary on the full power operation for different modes in Rel-16. It can be observed that full power Mode 1 and Mode 2 are more complicated compared with full power Mode 0.

Table 1 Summary of full power Modes in Rel-16

| Full power Mode | Summary |
| --- | --- |
| Full power Mode 0 | The power scaling factor is fixed to be 1. |
| Full power Mode 1 | The full coherent TPMI is added into the non-coherent and partial coherent codebook subset. The power scaling factor is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE. |
| Full power Mode 2 | The power scaling factor is 1 for those TPMIs that can deliver full power. For other TPMIs, the power scaling factor is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports indicated by SRI. The UE should report its capability on the TPMIs delivering full power. The SRS resources in the SRS resource set may have different number of ports in order to perform antenna virtualization. |

If the full power operation is not supported for 8Tx uplink transmission, the coverage and performance would be impacted. However, for full power Mode 1 and full power Mode 2, more discussion is required for 8Tx uplink transmission, such as the UE PA architecture, the TPMIs supporting full power operation, etc.

Considering that full power Mode 0 is the simplest mode, and the full power PA is more practical, we suggest that full power Mode 0 should be the starting point for Rel-18.

Proposal 2:

- RAN1 to consider full power Mode 0 as starting point for codebook-based UL transmission with 8Tx

### 2.1.3 Codebook construction

In NR Rel-15, the precoding matrixes are defined for 2-ports (Rank-1 and Rank-2) and 4-ports (Rank-1 to Rank-4). In order to enable 8Tx transmission in uplink, the codebook should be extended to 8-port and the codebook construction for 8Tx should be further discussed.

One possible solution to extend the codebook is to obtain the 8-port precoding matrix by the following equation.

W8Tx,R(X)=W1,2Tx,R(X1)⊗W2,4Tx,R(X2)	(Equation 1)

where W8Tx,R(X) is 8-port precoder with Rank-X, W1,2Tx,R(X1) is 2-port precoder with Rank-X1, W2,4Tx,R(X2) is 4-port precoder with Rank-X2, ⊗ represents Kronecker product. W1,2Tx,R(X1) and W2,4Tx,R(X2) could be the existing precoding matrix defined in Rel-15.

With Equation (1), the value of X is: X=X1×X2. Thus Equation (1) could be used to generate the codebook for rank value of {1, 2, 3, 4, 6, 8}.

For Rank value of {5, 7}, the codebook could be generated according to Equation (2).

W8Tx,R(X)=V1,2Tx,R(1)⊗W1,4Tx,R(X1)V2,2Tx,R(1)⊗W2,4Tx,R(X2)	(Equation 2)

where W8Tx,R(X) is 8-port precoder with Rank-X, W1,4Tx,R(X1) is 4-port precoder with Rank-X1, W2,4Tx,R(X2) is 4-port precoder with Rank-X2, ⊗ represents Kronecker product. Both V1,2Tx,R(1) and V2,2Tx,R(1) are 2-port precoder with Rank-1.

With Equation (2), the value of X is: X=X1+X2.

Therefore, with Equation (1) and (2), the codebook for all ranks could be generated. If up to 4 layers are supported in Rel-18, then Equation (1) is sufficient.

In addition, Rel-15 codebook is based on uniform antenna array. In Rel-18, whether different antenna structure could be supported, and the corresponding codebook could be further discussed.

Proposal 3:

- RAN1 to discuss the codebook construction for codebook-based UL transmission with 8Tx

### 2.1.4 Number of layers for 8Tx

In order to evaluate the PUSCH performance with 8Tx, system level simulation was performed. In the simulation, uplink SU-MIMO with 4Tx and 8Tx was utilized. The uplink precoding was wideband. SVD based precoding shown the performance upper bound. For PUSCH with 4Tx, different codebook subset configurations were also simulated.

Figure 3 shows the simulation output, including the case of maxRank=1 and maxRank=4. The simulation assumptions are provided in Appendix.

Figure 3 Uplink spectral efficiency with 4Tx and 8Tx

The simulation results are also summarized in Table 2 and Table 3 with different CDF percentile.

Table 2 Summary of UE spectral efficiency for maxRank=1

|  |  | 4Tx non-coherent | 4Tx partial coherent | 4Tx full coherent | 4Tx SVD | 8Tx SVD |
| --- | --- | --- | --- | --- | --- | --- |
| UE spectral efficiency | 5% of CDF | 0.13 | 0.17 | 0.22 | 0.23 | 0.25 |
|  | 50% of CDF | 0.22 | 0.28 | 0.36 | 0.37 | 0.41 |
|  | 95% of CDF | 0.4 | 0.49 | 0.6 | 0.63 | 0.67 |

Table 3 Summary of UE spectral efficiency for maxRank=4

|  |  | 4Tx non-coherent | 4Tx partial coherent | 4Tx full coherent | 4Tx SVD | 8Tx SVD |
| --- | --- | --- | --- | --- | --- | --- |
| UE spectral efficiency | 5% of CDF | 0.29 | 0.31 | 0.34 | 0.36 | 0.46 |
|  | 50% of CDF | 0.57 | 0.58 | 0.62 | 0.65 | 0.81 |
|  | 95% of CDF | 1.12 | 1.12 | 1.17 | 1.23 | 1.46 |

From Figure 3, Table 2 and Table 3, it can be observed that with more transmit antennas, additional gain can be provided.

However, we don’t see strong motivation for more than 4 layers since the performance requirements for DL 8 MIMO layers is not defined in RAN4. Therefore, we suggest taking 4 layers as starting point for uplink enhancement with 8Tx in Rel-18.

Proposal 4:

- RAN1 to consider 4 layers as starting point for codebook-based UL transmission with 8Tx

### 2.1.5 SRS configuration and TPMI indication

In Rel-16, for codebook-based transmission, the SRS configuration could be different depending on the full power operation, as shown in Table 4.

Table 4 Summary of SRS configuration for codebook-based UL in Rel-16

|  |  | SRS configuration |
| --- | --- | --- |
| Full power disabled |  | One SRS resource set. Up to two SRS resources with the same number of ports. |
| Full power enabled | Full power Mode 0 | One SRS resource set. Up to two SRS resources with the same number of ports. |
|  | Full power Mode 1 | One SRS resource set. Up to two SRS resources with the same number of ports. |
|  | Full power Mode 2 | One SRS resource set. Up to two or four SRS resources. The SRS resource may have different number of ports. |

In Rel-18, to support 8Tx uplink transmission, the SRS configuration should be discussed for codebook-based transmission, such as the number of SRS resource set, the number of SRS resources, and the number of ports per SRS resource, etc.

Proposal 5:

- RAN1 to discuss the SRS configuration for codebook-based UL transmission with 8Tx

In Rel-15/Rel-16, for codebook-based transmission, the TPMI is indicated by gNB via DCI field of “Precoding information and number of layers”. The field length depends on the number of ports, value of maxRank, and uplink codebook subset configuration.

In Rel-18, the codebook subset should also be used, including fullyAndPartialAndNonCoherent, partialAndNonCoherent, and nonCoherent. The TPMIs in each codebook subset should be extended to support 8-port transmission, while the DCI overhead should be reasonable. In addition, the codebook construction should be considered for the TPMI enhancement.

Proposal 6:

- RAN1 to discuss the TPMI indication enhancement for codebook-based UL transmission with 8Tx

## 2.2 Non-codebook based transmission

### 2.2.1 SRS configuration and SRI indication

In Rel-15, for non-codebook based transmission, one SRS resource set is configured with up to 4 SRS resources, and each SRS resource is single port. In the DCI scheduling PUSCH, the SRI filed could indicate the ports to be used for PUSCH transmission.

For non-codebook based transmission, the SRI field length depends on the number of SRS resources (NSRS) and the maximum number of MIMO layers (Lmax).

Figure 4 shows one example of the existing SRI field mapping for non-codebook based transmission with Lmax=4.

Figure 4 SRI field for non-codebook based transmission with Lmax=4

In Rel-17, in order to support PUSCH repetition in multi-TRP operation, two SRS resource sets could be configured for non-codebook based transmission. And correspondingly, two SRI fields could be conveyed over the DCI.

In Rel-18, in order to support 8Tx uplink transmission, the non-codebook based transmission should be enhanced to support 8Tx. The SRS configuration and SRI field should be enhanced.

For example, the UE could be configured with one SRS resource set, and up to 8 SRS resources can be included in the SRS resource set. And the SRI field should also be extended.

However, if one SRS resource set with up to 8 SRS resources is supported for 8Tx uplink transmission, the SRI field mapping will be a huge table.

Hence, another possible solution is to re-use the Rel-17 scheme to enable non-codebook based transmission with 8Tx. For example, two SRS resource sets could be configured, and two SRI fields could be included in the DCI.

Proposal 7:

- For non-codebook based transmission in Rel-18, RAN1 to discuss the SRS configuration, i.e., one SRS resource set vs. two SRS resource sets

Proposal 8:

- For non-codebook based transmission in Rel-18, RAN1 to discuss the SRI field enhancement, i.e., one SRI field vs. two SRI fields

# Conclusion

In conclusion, we have the following proposals on the uplink transmission enhancements with 8Tx in Rel-18.

Proposal 1:

- RAN1 to consider full coherence UE as starting point for UL transmission with 8Tx

Proposal 2:

- RAN1 to consider full power Mode 0 as starting point for codebook-based UL transmission with 8Tx

Proposal 3:

- RAN1 to discuss the codebook construction for codebook-based UL transmission with 8Tx

Proposal 4:

- RAN1 to consider 4 layers as starting point for codebook-based UL transmission with 8Tx

Proposal 5:

- RAN1 to discuss the SRS configuration for codebook-based UL transmission with 8Tx

Proposal 6:

- RAN1 to discuss the TPMI indication enhancement for codebook-based UL transmission with 8Tx

Proposal 7:

- For non-codebook based transmission in Rel-18, RAN1 to discuss the SRS configuration, i.e., one SRS resource set vs. two SRS resource sets

Proposal 8:

- For non-codebook based transmission in Rel-18, RAN1 to discuss the SRI field enhancement, i.e., one SRI field vs. two SRI fields

# References

- 3GPP TS 38.211 v17.1.0, Physical channels and modulation

- 3GPP TS 38.212 v17.0.0, Multiplexing and channel coding

- 3GPP TS 38.213 v17.0.0, Physical layer procedures for control

- 3GPP TS 38.214 v17.1.0, Physical layer procedures for data

- RP-213598, 3GPP TSG RAN Meeting #94-e, Dec 6 – Dec 17, 2021

# Appendix: System level simulation assumption

Table 5 System level simulation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | Umi |
| Layout | Hexagonal Grid with 2 tiers |
| ISD | 200 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 10 MHz |
| Subcarrier spacing | 15 kHz |
| Number of UEs | 210 |
| Traffic model | Full buffer |
| Scheduling | Proportional fair |
| # of Tx @ UE | 4Tx, 8Tx |
| Transmission scheme | SU-MIMO |
| Precoding | Wideband |