3GPP TSG RAN WG1 #109-e                                                              R1-2204796

e-Meeting, May 9th – 20th, 2022

Source:	Intel Corporation

Title:	Use Cases and Specification for Beam Management

Agenda item:	9.2.3.2

Document for:	Discussion and Decision

# Introduction

In this paper, we present initial views on the study of AI/ML applications to physical layer for beam management based on the objectives in the Rel-18 NR study item on AI/ML [1]:

Use cases to focus on: - Initial set of use cases includes: - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

The different sub-use-cases that can be considered for AI/ML aided beam management framework are discussed along with initial assessment of possible specification impact to enable these sub-use-cases. In our companion contribution [2], evaluation methodology and modelling assumptions have been discussed in detail along with initial evaluation results from some of the sub-use-cases.

# Beam Management Use Case Description

The beam management use case for AI/ML study can be broadly divided into two sub-cases i.e., spatial domain beam management and temporal domain beam management. Regardless for the specific use case, a common AI/ML model training and deployment framework can be considered.

Figure 1: AI/ML Model Training and Deployment

An example is shown in Figure 1, where a supervised machine learning model with offline training/validation is considered. The model is offline in the sense that dataset for training and testing are pre-generated, and the model is pre-trained in a non-real time manner. Once the model is trained, it can be deployed in a real-time environment to aid in specific beam management tasks based on the following sub-use cases:

- Spatial Domain Beam Management: Predict the optimal beam at UE or BS without exhaustive search with an aim to reduce measurement and reporting latency

  - Beam Prediction at the UE: For this use-case, consider a 2D planar array at UE with multiple analog Rx beams and fixed Tx beam at the gNB. The ML model is provided with channel or RSRP values for a subset of the beams and the best beam index or top K beam indices can be predicted from a subset of measurements thereby reducing the latency of beam acquisition or tracking

  - Beam prediction at the gNB: Similar to the previous use case but for Tx/Rx beam selection at the gNB for a fixed directional or omni beam at the UE. Only a subset of gNB are measured and the ML model predicts the beam index or top K beam indices from these subset of measurement leading to latency reduction for optimal beam selection

  - Joint UE-gNB beam pair link prediction: This use case would be similar to a joint P2+P3 procedure where the best beam at the gNB and the best beam at the UE are simultaneously identified by the ML model from a subset of measurements made on select beam pairs.

- Temporal Domain Beam Management: This use case predicts the best beam at gNB or UE for the next time instant given beams from a window of preceding time instants. Time series data can be collected from UEs moving across an environment and LSTM based models can be used to predict the best beam. Note that this use case does not assume trajectory prediction of the UE. The trajectory is part of the dataset generation process, and the ML model is tasked with predicting the time-series of best beam indices conditioned on the past beam indices.

- For AI/ML assisted beam management use-case, consider both spatial and temporal domain beam prediction sub-use cases within the purview of an offline learning framework.

Although more sophisticated online training methods including reinforcement learning exist in literature, the current study item should first concentrate on the offline training methodology with bigger emphasis on realistic evaluation assumptions and use-cases as highlighted in this section. Online models, if required, can be studied in future.

Baseline Schemes for Performance Comparison

For both sub-use cases, the baseline schemes should be based on exhaustive beam search and hierarchical beam search. While exhaustive beam search would provide the best performance, the goal of AI/ML based models could be to reduce the complexity of the prediction while providing performance identical to or close to exhaustive search. The complexity gains could be in the form or lesser number of measurements performed for the beam prediction leading to significant gains in latency. When considering hierarchical beam search procedures, the goal of the AI/ML algorithms should be to provide better performance i.e., more optimal beam selection with similar or lesser complexity in terms of the number of measurements and reporting.

# Specification Impact of Beam Management Use Cases

AI/ML model implementation in most cases may be dependent on implementation and specific models will be difficult to specify. However, from a standards perspective, the functionalities required to enable AI/ML models, irrespective of implementation details to be integrated into 5G-NR networks should be studied. Current networks offer a myriad of beam measurement and reporting functionalities which may be further augmented to enable AI/ML use cases.

Spatial domain beam prediction can be applied towards initial beam acquisition using SSB beams or beam tracking using narrower CSI-RS beams in conjunction with current beam reporting framework for faster and more accurate beam selection. The ML model may reside either at UE or gNB and the major specification impact will be triggering of beam measurement reports and reference signal transmissions.

- The ML model may reside either at UE or gNB

- One possible area of specification impact for AI/ML model integration may be for triggering of beam measurement reports and reference signal transmissions, as well new L1 reporting formats.

Some examples for AI/ML model integration into current 5G networks are provided below

## UE Spatial Domain Beam Prediction

Figure 2: ML Aided Beam Prediction at UE with Model residing at the gNB

For this case, the ML model may reside at the UE to enable ML Aided UE Rx beam tracking. It can work as follows:

- UE measures Rx beam quality for fixed gNB beam

- If Rx beam drops below threshold, UE triggers CSI-RS with repetition from gNB

- gNB transmits CSI-RS with repetition (BM)

- UE can measure subset of Rx beams to find the best Rx beam

The specification impact may be from UE triggered CSI-RS transmissions for beam measurements. Initial performance evaluation of such models has been provided in [2].

## gNB Spatial Domain Beam Prediction

Figure 3: ML Aided Beam Prediction at gNB with Model residing at the gNB

For this use-case, the ML model may reside at the network to enable UE and ML-aided gNB beam tracking using DL measurements. The process can function as follows:

- gNB triggers CSI-RS for CSI based on periodic/aperiodic beam report from UE if Tx beam drops below threshold

- gNB transmits M CSI-RS where M<< total number of CSI-RS beams

- UE measures L1-RSRP and reports to gNB

- gNB can use of M measurement to predict best Tx (Rx) beam

In this case, the UE does not need to know which CSI-RS beams are transmitted i.e., gNB can sample spatial domain based on its own implementation. The scheme relies on the L1 beam report from the UE for best beam prediction at the gNB. Potential gain is from reduced CSI-RS transmissions for measurement.

## Joint Beam Pair Link Prediction

Figure 4: ML Aided Joint Beam-Pair Link Prediction with Model residing at the gNB

This use case is similar to joint P2+P3 procedure and can work with either DL or UL measurements as shown in the figure. The following setups can be considered with the assumption that the AI/ML model resides at the gNB:

- Setup 1 – DL measurement and reporting

  - CSI-RS transmission from gNB on subset of beams

  - UE measures on sub-set of Rx beams and reports back to gNB

  - gNB predicts best beam pair link using AI/ML model

- Setup 2 – UL Measurement

  - SRS transmission using a subset of UE beams

  - gNB measures using a sub-set of Rx beams and predicts the best beam pair link

To enable this use case, new L1 reporting formats may need to be specified. For example, under setup 1, there may be a need for the gNB to signal to the UE that it should measure each Tx beam from gNB on a specific Rx beam and then the UE should be able to report the measurement corresponding to each of these beam pair links to the gNB using a new L1 reporting format. The major gains in this use case are envisioned to be from the latency gains due to reduction in the total number of measurements needed.

Similar specification impact can also be expected from temporal domain beam tracking. However, this use case heavily depends on the EVM for dataset generation and the specific algorithms used for AI/ML models. The details can be discussed after the EVM is finalized.

# Conclusion

In this paper, beam management use cases and related specification impact has been discussed. The main proposals from this paper are outlined here:

- For AI/ML assisted beam management use-case, consider both spatial and temporal domain beam prediction sub-use cases within the purview of an offline learning framework.

- The ML model may reside either at UE or gNB

- One possible area of specification impact for AI/ML model integration may be for triggering of beam measurement reports and reference signal transmissions, as well new L1 reporting formats.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, 3GPP TSG RAN#94e, December 6 – 17, 2021.

- R1-2204795, “Evaluation for Beam Management”, Intel Corporation, 3GPP RAN1#109e, May 9-20, 2022.