3GPP TSG RAN WG1 #109-e								R1-2204799

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	Discussions on the Evaluation Methodologies for NR Duplex Evolution

Agenda item:	9.3.1

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN Meeting #94 [1], a new SID related to evolution of NR duplex operation was approved. As part of the objectives of this study item (SI), the following objectives have been identified:

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this context, this contribution provides our views on the evaluation methodologies to adopt to study the NR duplex evolution, and in particular the following aspects will be discussed:

- Deployment scenarios and channel models to be considered in the evaluations.

- Basic assumptions to use to evaluate non-overlapping full duplex (NOFD) and dynamic/flexible TDD.

- Initial evaluations.

# Deployment Scenarios

In order to evaluate the system-level performance in the context of NOFD and dynamic/flexible TDD, the first step is to properly select a model, which realistically captures the scenario(s) for which these are intended.

In this matter, one of the common deployments often used in 3GPP is the indoor office scenario, which is illustrated in Fig.1.

Figure 1  - Indoor office deployment with 12 hotspots deployed within a 120 m x 50 m office.

Among other deployments, this scenario captures one of the typical deployments, which are more likely to experience a  predominant level of interference due to proximity across all nodes, and may allow to better highlight the pros and cons of possible interference mitigation scheme that could be considered to mitigate cross-link interferences (CLI) that may arise when dynamic/flexible TDD or NOFD  is enabled. Considering that both FR-1 and FR-2 are part of the study, this deployment could be additionally considered for both frequency ranges.

Another deployment that could be considered for both frequency ranges in the evaluation of duplexing is urban macro, which is another common deployment for which duplexing may be employed. In this deployment, while UEs may experience higher path losses due to the cell size, on the other end it is more likely that a UE may transmit at full power and generate higher interference to a nearby device.

Finally, specifically for FR-2, it may be useful to also consider a dense urban deployment, which is illustrated in Fig.2

Figure 2  - Dense urban deployment.

This deployment, similarly, as the indoor office deployment, is a typical deployment which may allow to better highlight pros and cons of possible interference mitigation scheme that could be considered to mitigate cross-link interferences (CLI) and interference that may arise when dynamic/flexible TDD is enabled.

Proposal 1:

- For the evaluations of both NOFD, and dynamic/flexible TDD,

- For FR-1, both indoor office and urban macro scenario could be considered:

- For FR-2, in addition to indoor office and urban macro scenario, the dense urban scenario could be considered.

# Channel Models

Another important aspect for system-level performance of both NOFD and dynamic/flexible TDD is regarding the channel models. As indicated in our companion contributions [2-3], in this context a proper modelling of cross-link interference is needed, since additionally interferences between gNB and UEs, also UE-to-UE and gNB-to-gNB interference must be considered. In this matter, several models are available, and have been employed in prior 3GPP studies which require CLI to be considered. While some of these models only apply to FR-1, other are tied with specific deployments which are not relevant here (e.g., channel models used for V2V). However, the channel models used in Rel.16 CLI/RIM study [4] seem to be proper for this study also in light of the deployments that could be considered. Table I and Table II summarize the channel models used in Rel.16 CLI/RIM study based on the frequency range and scenario considered.

Table I – Channel models used in Rel.16 CLI/RIM study for FR-1

| Urban Macro scenario | Macro(Aggressor) → Macro(Victim) - Macro-to-UE: UMa see TR 38.803 - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 - UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 [6] + penetration loss see TR 38.803 |
| --- | --- |
| Indoor scenario | Indoor (Aggressor) → Macro (Victim): - BS-to-BS: InH-office + penetration loss see TR 38.803 - BS-to-UE: InH-office + penetration loss see TR 38.803 - UE-to-UE: Outdoor UE – Outdoor UE + penetration loss see TR 38.803, TR 36.828 Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: InH-office see TR 38.803 - BS-to-UE: InH-office see TR 38.803 - UE-to-UE: InH-office see TR 38.803 |

Table II – Channel models used in Rel.16 CLI/RIM study for FR-2

| Urban Macro scenario | Macro(Aggressor) → Macro(Victim) - Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 [5] - Macro-to-UE(V): Uma + penetration loss see TR 38.803 [5] - UE-to-UE: UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss see TR 38.803 [5] |
| --- | --- |
| Indoor scenario | Indoor (Aggressor) → Macro (Victim): - BS-to-BS: InH-office see TR 38.803 - BS-to-UE: InH-office see TR 38.803 - UE-to-UE: InH-office see TR 38.803 Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: InH-office (h_UE = 3 m) + penetration loss see TR 38.803 [5] - BS-to-UE: InH-office (h_UE = 3 m) + penetration loss see TR 38.803 [5] - UE-to-UE: InH-office (h_BS = 1.5 m) + penetration loss see TR 38.803 [5] |
| Dense urban scenario | Micro (Aggressor) → Micro (Victim): - Micro-to-Micro: UMi (h_UE=10 m) - Micro-to-Micro UE: UMi + penetration loss - Micro (UE)-to-Micro (UE): UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss between UEs |

Proposal 2:

- Channel models used in Rel-16 CLI/RIM study could be used as a starting point.

# Basic Assumptions

In this section, while discussing basic assumptions/configurations and metrics that could be adopted at both link and system-level, different considerations will be made for NOFD and dynamic/flexible TDD.

## Consideration for Non-Overlapping Full Duplex

As described in our companion contribution [2], NOFD operation can help improve coverage/capacity for uplink transmission and reduce latency for both UL and DL transmission, by allocating the resource for PUSCH or PUCCH transmission in the uplink sub-band within the NOFD symbol, which is more critical in case of TDD UL/DL configuration with DL heavy pattern.

From a link-level perspective, to investigate the performance gain for coverage that can be realized by NOFD, simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI phase can be reused. More specifically, the following performance metrics can be considered as starting points for coverage analysis for NOFD [5]:

- MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2).

- MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain.

- MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains.

Further, for link-level simulations, assumptions for PUSCH and PUCCH as listed in [5] can be considered. To reduce the simulation workload, a subset of simulation assumptions can be selected for coverage analysis for NOFD operation. For instance, target data rate for PUSCH transmission under NOFD operation can be 100 kbps at 4 GHz carrier frequency in FR-1 and 5 Mbps at 28 GHz for FR-2. As for the specific simulation assumptions, these are summarized in the Table 1 and Table 2 in the Appendix I, and can be considered as starting points for NOFD performance evaluation for FR1 and FR2, respectively.

Note that for link-level simulation for NOFD coverage analysis, self-interference modelling needs to be considered. With practical self-interference modelling for NOFD operation, limited performance gain for coverage can be expected for coverage analysis, which may also depend on frequency location of DL/UL sub-band and PUSCH transmission. As discussed in our companion contribution [2], RAN1 needs to send an LS to RAN4 on self-interference modelling at gNB. Once self-interference modelling is available, further investigation is needed to analyze the coverage performance.

Proposal 3:

- Simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI can be considered as starting points for evaluation of coverage performance for NOFD operation.

- Consider Table 1 and Table 2 in the Appendix I for NOFD performance evaluation for FR1 and FR2, respectively.

- Self-interference modelling should be considered for link-level simulation for NOFD.

User plane (UP) latency performance achievable with NOFD operation may be evaluated based on numerical analyses in combination with link-level evaluations for a given traffic flow, e.g., PUSCH scheduling for UP latency for UL scheduling and PDSCH-to-HARQ-ACK flow for UP latency for DL scheduling, with parameterization of target reliability and data rate (packet size). The choice of operating SINR can be determined from SLS evaluations, e.g., to identify 5%-ile SINR, etc., while the link-level evaluations could be used to determine the appropriate TDRA assumptions for different physical channels involved.

Proposal 4:

- For target reliability and packet size assumptions, User Plane (UP) latency for UL and DL may be evaluated using numerical analyses while using results from link- and system-level evaluations to determine details of UL/DL scheduling flows for PUSCH and PDSCH scheduling.

From a system-level perspective, the assumptions, which include among other the layout, carrier frequency and antenna configurations, summarized in Appendix II within Table 3, 4, 5, 6 and 7, which were assumed in Rel.16 for CLI/RIM, could be used a baseline, while the user perceived throughput (UPT) could be used as one of the main KPIs. Furthermore, simulations could focus on non-full buffer case with FTP traffic model 3, while different traffic load cases could be considered (e.g., 20%, 40%, 60%,)

Proposal 5:

- For system-level evaluations on NOFD

  - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, as agreed during Rel-16 CLI/RIM, can be considered as starting points for system-level evaluations for NOFD operation.

    - Consider Table 3-7 in the Appendix II for NOFD evaluations in FR1 and FR2.

  - Non-full buffer with FTP traffic model 3 should be considered for traffic modelling.

  - DL/UL UPT should be used as the primary performance metric for SLS evaluations.

To study the performance of NOFD at system-level, both UPT for DL and UL should be evaluated. UL UPT depends on the performance gain provided by more UL resources vs the impact of self-interference within gNB and gNB-to-gNB CLI (DL-to-UL interference). gNB-to-gNB CLI can be classified as inter-subband CLI only, if NOFD configuration is aligned among neighbouring gNBs, and intra-subband CLI, if NOFD configuration is not aligned among neighbouring gNBs. DL UPT depends on the performance gain provided by shorter latency for HARQ-ACK feedback vs the degradation caused by reduced available DL resource and UE-to-UE CLI (UL-to-DL interference). UE-to-UE CLI includes intra-cell CLI, inter-subband CLI and inter-cell CLI inter-subband or intra-subband CLI, depending on whether NOFD configuration is aligned. As discussed in [2], the interference varies with filtering assumption at both gNB and UE side. All these interference types may need to be considered in the evaluation. Therefore, RAN1 needs to send an LS to RAN4 on modelling for these interference types.

The number of extra UL resources (correspondingly, reduced DL resources) and the impact of interference depend on the NOFD configuration, e.g., symbols/slots for NOFD and frequency resources for UL and DL sub-band respectively. Furthermore, the guard band between DL and UL subband also has impact on the overall performance. RAN1 needs to discuss one or more typical NOFD configuration for evaluation.

Proposal 6:

- RAN1 should consider appropriate NOFD configurations and modelling options for self- and cross-link interference for the evaluations for NOFD.

  - Send an LS to RAN4 during RAN1 #109-e to receive inputs from RAN4 on self- and cross-link interference for NOFD operation.

## Consideration for Dynamic/Flexible TDD

As mentioned above and described also in our companion contribution [3], for dynamic/flexible TDD system, CLI may arise due to different transmission directions among neighbouring gNBs in a same frequency resource, which may substantially degrade the system performance. For this purpose, as detailed in [3] some interference mitigation techniques may be needed, and in order to evaluate its effectiveness system-level evaluations may be required.

In this matter, the assumptions, which include among other the layout, carrier frequency and antenna configurations, summarized in Appendix II within Table 3, 4, 5, 6 and 7, which were assumed in Rel.16 for CLI/RIM, could be used a baseline, while the specific dynamic/flexible TDD configuration choices could be left up to companies. Similarly, as in prior sub-section, UPT could be used as the main KPI.

Proposal 7:

- For system-level evaluations on dynamic/flexible TDD

  - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, summarized in Table 3-7 in the Appendix II can be considered as starting points for system-level evaluations for dynamic/flexible TDD operation in FR1 and FR2.

  - Non-full buffer with FTP traffic model 3 should be used for traffic modelling.

  - DL/UL UPT should be used as the primary performance metric.

  - Specific dynamic/flexible TDD configuration choices could be reported by companies.

# Initial Evaluations

## Preliminary Coverage Analysis for NOFD Operation

To investigate the coverage performance for NOFD operation, a link-level simulation’s campaign has been conducted for PUSCH repetitions. The simulation assumptions used are those summarized in the Appendix I in Table 1 for FR1 and Table 2 for FR2, respectively. Furthermore, two scenarios, as illustrated in Figure 3, are considered:

- Scenario a): DDDSU for TDD configuration without NOFD operation,

- Scenario b): DNNNU for TDD configuration with NOFD operation, where N indicates as NOFD slot.

Figure 3 - TDD configurations with NOFD for PUSCH repetitions

Figure 4 and Figure 5 illustrate preliminary link-level simulation results for PUSCH repetition with NOFD operation in FR1 and FR2, respectively. From these figures, it can be observed that link-level performance for PUSCH can be improved by increasing the number of repetitions. More specifically, 2~3dB performance gain can be observed by doubling the repetition levels for the PUSCH transmission, depending on UE moving speed and targeted data rate.

Note that in the simulations, leakage from DL sub-band to UL sub-band for NOFD operation are not considered. As mentioned above, after RAN1 will be obtaining self-interference modelling from RAN4, further investigation would be needed to analyze the coverage performance for NOFD operation.

Figure 4 – Link-level simulation results for PUSCH repetition with NOFD operation in FR1

Figure 5 – Link-level simulation results for PUSCH repetition with NOFD operation in FR2

Observation 1

- Without self-interference modelling, 2~3dB link-level performance gain can be observed by doubling the repetition levels for PUSCH transmission in both FR1 and FR2.

## Preliminary SINR Analysis for Dynamic/Flexible TDD

In order to investigate the CLI impact on system-level perspective when dynamic/flexible TDD is used, some initial simulation campaign has been conducted, which focuses on geometry-only analysis. Four different scenarios are considered:

- Indoor office deployment in FR-1.

- Urban macro deployment in FR-1.

- Indoor office deployment in FR-2.

- Urban macro deployment in FR-2.

The simulation assumptions used are those summarized in the Appendix II in Table 3 for indoor office deployment in FR-1, Table 4 for urban macro deployment in FR1, in Table 5 for indoor office deployment in FR-2, Table 6 for urban macro deployment in FR2, respectively.

For each deployment, the following figures show both the CDF of the DL and UL SINR, and for each CDF it is also shown the case when a flexible/dynamic TDD is where through which half of the cells operate in UL and the other half in DL.

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 6 - CDF of DL and UL SINR in an indoor deployment for FR-1

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 7 - CDF of DL and UL SINR in an urban macro deployment for FR-1

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 8 - CDF of DL and UL SINR in an indoor deployment for FR-2

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 9 - CDF of DL and UL SINR in an urban macro deployment for FR-2

Figure 6-9 highlight when dynamic/flexible TDD operation is used, the UL performance degrades substantially, especially in FR-1, due to gNB-to-gNB cross-link interference, while the DL performance may only slightly increase due to the lower TX power used by the UEs. Thus, this motivates considerations of possible enhancements to address gNB-to-gNB CLI as described in our companion document [3].

Observation 2

- In absence of any coordination, due to gNB-to-gNB CLI the UL performance may substantially degrade in both FR-1 and FR-2 irrespective of the scenario when dynamic/flexible TDD operations is supported.

# Conclusions

In this contribution, we presented our views the evaluation methodologies to adopt to study the NR duplex evolution, and made the following observations and proposals:

Proposal 1:

- For the evaluations of both NOFD, and dynamic/flexible TDD,

- For FR-1, both indoor office and urban macro scenario could be considered:

- For FR-2, in addition to indoor office and urban macro scenario, the dense urban scenario could be considered.

Proposal 2:

- Channel models used in Rel-16 CLI/RIM study could be used as a starting point.

Proposal 3:

- Simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI can be considered as starting points for evaluation of coverage performance for NOFD operation.

- Consider Table 1 and Table 2 in the Appendix I for NOFD performance evaluation for FR1 and FR2, respectively.

- Self-interference modelling should be considered for link-level simulation for NOFD.

Proposal 4:

- For target reliability and packet size assumptions, User Plane (UP) latency for UL and DL may be evaluated using numerical analyses while using results from link- and system-level evaluations to determine details of UL/DL scheduling flows for PUSCH and PDSCH scheduling.

Proposal 5:

- For system-level evaluations on NOFD

  - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, as agreed during Rel-16 CLI/RIM, can be considered as starting points for system-level evaluations for NOFD operation.

    - Consider Table 3-7 in the Appendix II for NOFD evaluations in FR1 and FR2.

  - Non-full buffer with FTP traffic model 3 should be considered for traffic modelling.

  - DL/UL UPT should be used as the primary performance metric for SLS evaluations.

Proposal 6:

- RAN1 should consider appropriate NOFD configurations and modelling options for self- and cross-link interference for the evaluations for NOFD.

  - Send an LS to RAN4 during RAN1 #109-e to receive inputs from RAN4 on self- and cross-link interference for NOFD operation.

Proposal 7:

- For system-level evaluations on dynamic/flexible TDD

  - Simulation assumptions, including deployment scenarios, antenna configurations, and related assumptions, summarized in Table 3-7 in the Appendix II can be considered as starting points for system-level evaluations for dynamic/flexible TDD operation in FR1 and FR2.

  - Non-full buffer with FTP traffic model 3 should be used for traffic modelling.

  - DL/UL UPT should be used as the primary performance metric.

  - Specific dynamic/flexible TDD configuration choices could be reported by companies.

Observation 1

- Without self-interference modelling, 2~3dB link-level performance gain can be observed by doubling the repetition levels for PUSCH transmission in both FR1 and FR2.

Observation 2

- In absence of any coordination, due to gNB-to-gNB CLI the UL performance may substantially degrade in both FR-1 and FR-2 irrespective of the scenario when dynamic/flexible TDD operations is supported.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, December 2021.

- R1-2204800, “Discussions on subband non-overlapping full duplex”, Intel Corporation, May 2022.

- R1-2204801, “On potential enhancements to dynamic/flexible TDD in NR systems”, Intel Corporation, May 2022.

- TR 38.828, “Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”, 2019.

- TR 38.830, “Study on NR coverage enhancements”

# Appendix I

Table 1. Simulation assumption for PUSCH in FR1

| Parameters | Values |
| --- | --- |
| Physical channel | PUSCH |
| Carrier frequency | Rural: 4 GHz (TDD) – NLOS |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Bandwidth | 4 GHz – 100MHz |
| Waveform | DFT-s-OFDM |
| Subcarrier spacing for PUCCH | 30kHz for TDD |
| MCS and TBS | QPSK eMBB: Rural, TDD: MCS 3 and TBS 288 bits for 2 DMRS symbols MCS 4 and TBS 304 bits for 4 DMRS symbols |
| Resource allocation | eMBB: Rural: 14 symbols, 4 PRB |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 4Rx for 4GHz |
| Propagation channel | Rural: TDL-C 300ns |
| UE velocity | Rural: 3km/h for indoor, 120km/h for outdoor |
| Timing offset | 0 |
| Frequency offset | 0 |
| DMRS symbols | eMBB: 2 symbols for 3km/h, 4 symbols for 120km/h |
| Frequency hopping | Intra-slot FH |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | MMSE based channel estimation |
| Performance metrics | For eMBB, 10% iBLER |

Table 2. Simulation assumption for PUSCH in FR2

| Parameters | Values |
| --- | --- |
| Physical channel | PUSCH |
| Carrier frequency | 28 GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Bandwidth | 100 MHz |
| Waveform | DFT-s-OFDM |
| Subcarrier spacing for PUCCH | 120kHz |
| MCS and TBS | QPSK Urban/Indoor: MCS 6 and TBS 3824 for 2 DMRS symbols MCS 7 and TBS 3752 for 4 DMRS symbols |
| Resource allocation | eMBB: Urban: 14 symbols, 30 PRBs |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx |
| Propagation channel | Urban scenario: TDL-A 100ns |
| UE velocity | Urban: 3km/h for indoor, 30km/h for outdoor |
| Timing offset | 0 |
| Frequency offset | 0 |
| DMRS symbols | eMBB: 2 symbols for 3km/h, 4 symbols for 30km/h |
| Frequency hopping | Intra-slot FH |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | MMSE based channel estimation |
| Performance metrics | For eMBB, 10% iBLER |

# Appendix II

Table 3. Simulation assumption for indoor office deployment in FR1

| Parameters | Values |
| --- | --- |
| Layout |  |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 20 MHz |
| BS TX power | 24 dBm |
| UE TX power | 23 dBm |
| Path-loss model | - BS-to-BS: InH-office see TR 38.803 - BS-to-UE: InH-office see TR 38.803 - UE-to-UE: InH-office see TR 38.803 |
| BS antenna | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height: | 3 m |
| UE antenna | Omni |
| UE antenna height | 1.5 m |
| Antenna gain of UE | 0 dBi |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5 dB |
| UE receiver noise figure | 9 dB |

Table 4. Simulation assumption for urban macro deployment in FR1

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 20 MHz |
| Path-loss model | Macro-to-UE: UMa see TR 38.803 Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 UE-to-UE: Outdoor UE – Outdoor UE see TR 36.828 + penetration loss see TR 38.803 |
| BS Tx power | 49 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 5 dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | Omni |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |

Table 5. Simulation assumption for indoor office deployment in FR2

| Layout |  |
| --- | --- |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 80 MHz |
| BS TX power | 23 dBm |
| UE TX power | 22.4 dBm |
| Path-loss model | - BS-to-BS: InH-office see TR 38.803 - BS-to-UE: InH-office see TR 38.803 - UE-to-UE: InH-office see TR 38.803 |
| BS antenna | (Mg,Ng,M,N,P)=(1,1,4,8,2) (dH,dV)=(0.5,0.5)λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 2dB loss) |
| UE antenna configurations | (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.5)λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |

Table 6. Simulation assumption for urban macro deployment in FR2

| Network layout | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| --- | --- |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 80 MHz |
| BS Tx power | 43 dBm |
| UE Tx power | 22.4 dBm |
| Inter-site distance | 200 m |
| Path-loss model | Macro-to-Macro: UMa (h_UE = 25 m) see TR 38.803 Macro-to-UE(V): Uma + penetration loss see TR 38.803 UE-to-UE: UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss see TR 38.803 |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| UE antenna configurations | (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.5)λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |

Table 7. Simulation assumption for dense urban deployment in FR2

| Network layout | Fixed cluster circle within a macro cell. |
| --- | --- |
| # of micro BSs per macro cell | 3 |
| BS antenna height | 10 m |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 80 MHz |
| BS Tx power | 33 dBm |
| UE Tx power | 22.4 dBm |
| Inter-site distance | 200 m |
| Path-loss model | Micro-to-Micro: UMi (h_UE=10 m) Micro-to-Micro UE: UMi + penetration loss Micro (UE)-to-Micro (UE): UMi (h_BS=1.5 m ~ 22.5 m) + penetration loss between UEs |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,16,2) (dH,dV)=(0.5,0.5)λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| UE antenna configurations | (Mg,Ng,M,N,P)=(1,1,2,2,2) (dH,dV)=(0.5,0.5)λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |