3GPP TSG RAN WG1 #109-e																		    R1-2204801

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	On Potential Enhancements to Dynamic/Flexible TDD in NR systems

Agenda item:	9.3.3

Document for:	Discussion and Decision

# Introduction

In study on evolution of NR duplex operation, the following objectives have been identified [1]:

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we present our views on potential enhancements on dynamic/flexible TDD. Our views on evaluation methodologies, simulation assumption and subband non-overlapping full duplex are described in our companion contributions [3] and [4], respectively.

# Discussion on Cross-link Interference for Dynamic TDD

## Existing Solutions for Interference Mitigation

In NR, TDD UL/DL configuration can be semi-statically configured by gNB via tdd-UL-DLConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. In addition, dynamic TDD without semi-static TDD UL/DL configuration was introduced, where gNB may dynamically allocate UL and DL resources so as to match the instantaneous traffic conditions for UL and DL transmissions, respectively. This dynamic/flexible operation may help in maximizing resource utilization and improving the UE throughput.

However, for dynamic/flexible TDD system, cross-link interference (CLI) may arise due to different transmission directions among neighboring gNBs in a same frequency resource, which may substantially degrade the system performance. Figure 1 illustrates cross-link interference for dynamic/flexible TDD system. In the figure, two types of CLI can be observed under dynamic TDD operation: UE-to-UE interference and gNB-to-gNB interference. In particular, for UE-to-UE interference, CLI arises when UL transmission of an aggressor UE from neighboring gNB interferes with DL transmission of a serving gNB to a victim UE, while for gNB-to-gNB interference, CLI is generated when DL transmission of an aggressor gNB interferes with UL transmission of a UE targeted for a victim gNB.

Figure 1. Illustration of two types of cross link interference (CLI)

In Rel-14, studies have been conducted on how to mitigate cross-link interference under dynamic/flexible TDD operation. As summarized in [2], it was demonstrated that duplexing flexibility with cross-link interference mitigation shows better user throughput compared to static UL/DL operation or dynamic UL/DL operation without interference mitigation in indoor hotspot (4GHz and 30GHz) and urban macro scenarios (4GHz and 2GHz). The mitigation techniques include coordinated scheduling/beamforming, power control, link adaptation, and hybrid dynamic/static UL/DL resource assignment.

In Rel-16, in order to mitigate CLI and thereby improve system level performance in dynamic/flexible TDD system, CLI measurement and reporting from UE, including SRS-RSRP and CLI-RSSI were introduced. In this case, gNB may obtain certain statistic information regarding potential UE-to-UE interference between UEs of different cells. Further, network coordination mechanism was specified to allow gNB to exchange the information of intended DL/UL configuration.

## Inter-operator gNB-to-gNB CLI

As illustrated in the Figure 1, gNB-to-gNB CLI occurs when DL transmission of an aggressor gNB interferes with UL transmission of a UE targeted for a victim gNB. With certain level of coordination among gNBs, e.g., exchanging information of intended UL/DL configuration for TDD operation as introduced in Rel-16, the gNB-to-gNB CLI can be suppressed or mitigated to some extent, especially for synchronous network. In particular, with this knowledge on the UL/DL resource configuration of neighbouring gNBs, the scheduler may make appropriate decisions on the transmission and reception in DL/UL slots so as to avoid cross-link interference.

However, for inter-operator gNB-to-gNB CLI, the existing solution, which relies on network coordination among gNBs, may not work well for effective interference mitigation, due to the timing offset under asynchronous network. Note that the inter-gNB CLI may arise from the adjacent channel CLI due to out of band emission, or co-channel CLI, depending on the deployment scenario. While implementation-based solutions may work for dynamic/flexible TDD across cells belonging to the same operator, for inter-operator dynamic/flexible TDD operation, such implementation-based gNB-to-gNB CLI mitigation schemes may not be feasible.

Figure 2 illustrates one example of inter-operator gNB-to-gNB CLI with asynchronous network. From this figure, it can be observed that as the timing between gNBs is not aligned, gNB-to-gNB CLI may come from not only the same slot, but also the adjacent slots. This may make some interference mitigation solutions, e.g., advanced receiver-based approach, more challenging.

Figure 2. Inter-operator gNB-to-gNB CLI under asynchronous network

Observation 1

- For inter-operator dynamic TDD operation, gNB-to-gNB CLI may be more pronounced due to asynchronous network.

## Non-overlapping Full Duplex with TDD operation

As described in our companion contribution [4], in case of non-overlapping full duplex (NOFD) with TDD operation, the UE-to-UE and gNB-to-gNB CLI can be worse than TDD operation without NOFD. In such scenario, CLI may arise from the inter-subband emission for gNBs operating in the same or adjacent carriers, or from intra-subband interference. In the latter case, different gNBs may, in general, configure or indicate unaligned UL or DL subband in the same frequency region for NOFD operation. This would result in additional gNB-to-gNB CLI, which is different from the scenario where distinct TDD UL/DL slot configurations are configured among the gNBs.

Figure 3 illustrates one example of gNB-to-gNB CLI in case of NOFD with TDD operation. From this figure, it can be observed that DL and UL subband may overlap in the same frequency resource from different gNBs, which would lead to co-channel interference.

Figure 3. gNB-to-gNB CLI in case of NOFD with TDD operation

Observation 2

- Additional UE-to-UE and gNB-to-gNB CLI can be observed in case of NOFD with TDD operation.

# Discussions on Potential Enhancements on Dynamic TDD

## gNB-to-gNB CLI mitigation

In NR, only UE-to-UE interference measurement and reporting was specified, but gNB-to-gNB or TRP-to-TRP measurement was not introduced. As mentioned above, for inter-operator dynamic TDD operation where tight coordination is typically not feasible, gNB-to-gNB CLI may be more pronounced due to timing offset under asynchronous network. Similar issue may also exist for the case of intra-operator scenario if timing is not aligned between gNBs. This may motivate the introduction of standardized mechanisms to facilitate gNB-to-gNB CLI mitigation to enable dynamic/flexible TDD in commercial network.

Similar to what was defined for UE-to-UE CLI mitigation in Rel-16, CLI measurement and reporting among gNBs may be employed for gNB-to-gNB CLI handling. More specifically, short-term and long-term interference measurements can be considered as follows:

- Short-term interference measurement: this instantaneous L1 measurement and report can allow gNB scheduler to assign resource and quickly adapt to fast changing environment and channel/interference conditions. Typically, CSI/CQI like or RSRP/RSSI measurement can be used for measurement.

- Long-term interference measurement: the L3 measurements are filtered in order to reduce the impact of short-term variance, e.g., fast fading. In Rel-16, SRS-RSRP and CLI-RSSI measurement is obtained based on long term metrics for CLI mitigation.

To address the issue for gNB-to-gNB CLI due to timing offset under asynchronous network, certain timing-synchronization assistance information may be considered for exchange between gNBs that could further facilitate accurate synchronization via Over the Air Synchronization (OAS) in addition to existing methods relying on GNSS, Network Time Protocol (NTP), Precision Time Protocol (PTP, e.g., IEEE 1558). In this regard, gNB may determine relative timing offset in conjunction with TDD UL/DL configurations so as to suppress the cross-link interference and enable more efficient allocation of guard bands for inter-operator scenarios on different carriers.

Further, in case of NOFD with TDD operation, additional gNB-to-gNB CLI may occur due to the unaligned UL/DL subband configuration among different gNBs. To further supress the interference, extension of existing network coordination mechanisms as defined in Rel-16 may be considered. For instance, configuration of UL or DL subband for NOFD operation may be exchanged among gNBs, which may assist gNB to determine the transmission/reception of the channels/signals in NOFD symbols for effective interference mitigation.

Based on the discussions above, in our view, careful investigation is needed for the potential solutions for gNB-to-gNB CLI mitigation, with the consideration of potential specification impact and performance improvement compared to existing solutions as defined in NR, e.g., network coordination mechanisms in Rel-16.

Proposal 1

- RAN1 to further study the potential benefit and specification impact for gNB-to-gNB CLI mitigation.

## L1 CLI Measurement and Reporting at UE

In NR Rel-16, L3 CLI measurement and reporting at UE was specified, which mainly focused on long-term statistic of interference. To capture short-term interference characteristic, L1 CLI measurement and reporting at UE can be considered for potential enhancement on dynamic TDD operation. This may be more desirable, especially when considering beam oriented system, e.g., in FR2, where more dynamic channel and interference conditions are expected.

In Rel-14, several studies have been conducted to demonstrate the benefit of L1 CLI measurement and reporting at UE side [2]. For instance, as presented in our contribution, higher user packet throughput can be achieved by fast measurement and report and link adaptation in dynamic TDD system [5]. In our view, the dynamic CLI report can help handle the instantaneous interference conditions for intra-cell scenario, which allows gNB to make appropriate scheduling and link adaptation decision in a timely manner.

To support dynamic CLI measurement and report at UE, existing SRS framework as defined in NR can be extended for potential enhancement on dynamic TDD operation. More specifically, aperiodic SRS based measurement and reports, e.g., SRS-RSRP can be triggered by gNB in order to capture dynamic interference statistic, which can be exploited by the gNB to make proper scheduling and link adaptation decision. Figure 4 illustrates one example of L1 CLI measurement and reporting at UE.

Figure 4. L1 CLI measurement and reporting at UE

Proposal 2

- RAN1 to further study L1 CLI measurement and reporting at UE for dynamic TDD operation.

# Conclusions

In this contribution, we presented our views on potential enhancements on dynamic/flexible TDD. Further, we summarize the proposals as follows:

Observation 1

- For inter-operator dynamic TDD operation, gNB-to-gNB CLI may be more pronounced due to asynchronous network.

Observation 2

- Additional UE-to-UE and gNB-to-gNB CLI can be observed in case of NOFD with TDD operation.

Proposal 1

- RAN1 to further study the potential benefit and specification impact for gNB-to-gNB CLI mitigation.

Proposal 2

- RAN1 to further study L1 CLI measurement and reporting at UE for dynamic TDD operation.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, December 2021

- 3GPP TR38.802, “Study on New Radia Access Technology: Physical Layer Aspects,” Release 14, 2017

- R1-2204799, “On evaluations for NR duplex evolution”, Intel Corporation, May 2022

- R1-2204800, “Discussions on subband non-overlapping full duplex”, Intel Corporation, May 2022

- R1-1703446, “Evaluation results on dynamic TDD in dense urban scenario at 4GHz”, Intel Corporation, February 2017