3GPP TSG RAN WG1 #109-e			     R1-2204802

e-Meeting, May 9th – May 20th, 2022

Source:	Intel Corporation

Title:	On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum

Agenda item:	9.4.1.1

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94 [1], a new work item related to NR Sidelink (SL) evolution was approved. As part of the objectives of this working item (WI), that following aspects were included:

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, in the context of enhancements to enable NR SL to operate in the FR-1 unlicensed band, which we will refer to as NR-U SL in the remaining of this document, a few topics related to channel access mechanisms will be discussed, while view on other aspects are provided in our companion contribution [2]:

- Baseline for channel access types and mode of operations in NR-U SL

- Channel occupancy time (COT) sharing procedure

- Contention window adjustment mechanisms for NR-U SL

- Relationship between ProSe Per Packet Priorities (PPPPs) defined in SL and the channel access priority classes (CAPCs)

- Mechanisms to mitigate mutual blocking across UEs in NR-U SL

- Considerations regarding the ON/OFF and OFF/ON transient periods on the LBT procedure for NR-U SL

- Considerations on the sensing and selection procedure

- Considerations on short control signalling.

# Channel Access Modes and LBT Types

In order to comply with the global regulatory requirements in the FR-1 unlicensed band, and in particular with the more stringent rules dictated by the ETSI BRAN [3], in Rel.16 NR-U the listen before talk (LBT) procedure has been mandatorily adopted in order to sense a channel before a transmission may occur with the aim to either acquire a COT or resume transmission within a device’s COT or a COT shared by another device if the gap between the start of a new transmission burst and the end of the last transmission burst on that COT is sufficiently long. In Rel.16/17 NR-U, two modes of operation have been defined:

- Dynamic channel access mode: under this mode of operation, a device may perform LBT with the intention to acquire a COT at any time. For this mode of operation, four LBT types have been defined:

  - Type-1 (LBT Cat.4) – this LBT procedure is composed by a random backoff that is used to start/acquire a COT.

  - Type-2A (LBT Cat.2) – this LBT procedure does not have any random backoff and has a fixed sensing structure that is used either to transmit signals qualified as short control signaling (i.e., discovery reference signals with duty cycle less or equal than 1/20) or to resume/transmit within a shared COT when a gap larger than 25 us is present between the end of the last transmission burst on that COT and the start of the new burst.

  - Type-2B (LBT Cat.2 with smaller deferral sensing) - this LBT procedure also does not have any random backoff and has a fixed sensing structure and it is only used to resume/transmit within a shared COT when a gap smaller than 25 us, but larger than 16 us is present between the end of the last transmission burst on that COT and the start of the new burst.

  - Type-2C (LBT Cat.1) – this corresponds to no LBT procedure, and it is used to resume/transmit within a shared COT when a gap smaller than 16 us is present between the end of last transmission burst on that COT and the start of the new burst, and the new burst is no longer 584 us for dynamic channel access mode.

- Semi-static channel access mode: under this mode of operation, a device may only acquire a COT in specific instances of time defined by a fixed frame period (FFP) and is forbidden to transmit within the last portion of the COT (last 5 % of the COT or at least last 100 us) which must be left un-used so that to allow other devices to perform LBT and acquire the channel. Since a COT can be only acquired periodically, this mode of operation is not particularly suitable for deployments where there are incumbent technologies, which may more aggressively attempt to use the channel, and it is mean for deployments in controlled environments, where incumbent technologies are either absent or their deployment is well known. Furthermore, under this mode of operation a device may only need to perform a short sensing within a sensing slot duration of 9 us, unless longer sensing is required (e.g., in China 16 us is needed), to either acquire the COT or to resume/transmit within a shared COT when a sufficiently long gap occurs between the end of the last transmission burst on that COT and the start of the new burst.

Moving forward to Rel.18, NR-U SL could support both dynamic and semi-static channel access mode to support a more diversified and wider range of possible deployments, which may include deployments where incumbent technologies would be present as well as deployments where the environment is controlled, and absence of incumbent technologies could be guaranteed. Furthermore, all the channel access types defined in Rel.16 NR-U could be supported and used as a baseline in NR-U SL, while RAN1 should still further discuss when to use them based on the type of SL channels and signals transmitted and based on COT sharing conditions.

Proposal 1:

- SL operating is unlicensed spectrum supports both dynamic and semi-static channel access mode.

Proposal 2:

- The channel access types defined in Rel.16 NR-U (i.e., type 1, type 2A, 2B and 2C) are supported in SL operating in unlicensed spectrum. FFS: when to use them based on the type of SL channels and signals transmitted and based on COT sharing conditions.

# COT Sharing Procedure

In Rel. 16 SL, two modes of operation have been defined:

- Mode 1, where a gNB scheduled SL resource(s) to be used by a UE for SL transmission(s);

- Mode 2, where a UE determines (i.e., base station does not schedule) SL transmission resource(s) within a pool of SL resources configured by base station/network or within a pool of pre-configured SL resources.

Moving forward to Rel.18, based on the WID for NR-U SL [1] both modes of operation must be supported, but for mode 1 the Uu link is limited to the licensed spectrum. However, for this last case it is still unclear, whether the gNB may be still allowed to sense on it to determine whether a scheduled UE may have indeed acquired a COT. In this matter, it may be important for RAN1 as a first step to clarify further the intended deployment for mode 1, since multiple interpretations may exist:

- Mode-1-a: a gNB sends scheduling DCI on the licensed carrier, and can perform sensing on SL unlicensed carrier, but it is not allowed to transmit on the SL unlicensed carrier. This implies that a gNB can sense the SL channel but cannot occupy by its own COT.

- Mode-1-b: a gNB sends scheduling DCI on the licensed carrier and cannot either perform sensing or transmit on SL unlicensed carrier.

Proposal 3:

- RAN1 should agree on the exact interpretations of the intended deployment for mode-1:

- Mode-1-a: a gNB sends scheduling DCI on the licensed carrier, and can perform sensing on SL unlicensed carrier, but it is not allowed to transmit on the SL unlicensed carrier.

- Mode-1-b: a gNB sends scheduling DCI on the licensed carrier and cannot either perform sensing or transmit on SL unlicensed carrier.

In Rel.16 NR-U, in order to allow a better utilization of the spectrum, and mitigate LBT overhead, the concept of COT sharing has been introduced, where for instance:

- a gNB is allowed to share its own COT with its serving UEs, called DL-to-UL COT sharing.

- a UE is allowed to share its own COT with its associated gNB, called UL-to-DL COT sharing.

If in Rel.18 NR-U SL, regardless of whether mode 1-a or 1-b are assumed, then none of the COT sharing procedure considered in Rel.16 NR-U could be used, since the gNB may not be allowed to neither transmit nor acquire a COT within the unlicensed carrier. Furthermore, for both mode 1-b and mode 2, the gNB won’t know whether a UE has been able to acquire a COT or not since it may be unable to monitor the unlicensed carrier. In this case, if no enhancements are made, each UE may need to independently acquire their own COT, which may lead NR devices to compete with each other for the medium resulting in a high probability of blockage among them and highly increase the overall LBT overhead for the NR system. In order to mitigate these issues, it may be highly beneficial to allow a UE to share its own COT with other UEs, especially for the UEs for which the initiating UE intends to communicate. For this reason, RAN1 should study and define a COT sharing procedure suitable for both mode 1 and mode 2 to allow a UE to share its COT with other UE(s).

Proposal 4:

- RAN1 should study a unified procedure for both mode 1 and mode 2 to allow a UE to share its COT with other UE(s).

# Contention Window Size Adjustment

When the type 1 LBT procedure is used, the related backoff counter is drawn randomly within a specific contention window, whose size is adjusted based on the traffic and channel condition. In particular, during Rel. 13/14/15 and 16, a great effort was devoted in designing contention window size (CWS) adjustment mechanisms for both DL and UL (for ether scheduled or grant-free UL transmissions) to be on par with other incumbent technologies and allow fair co-existence among them. For either DL or UL, the CWS adjustment procedure is performed based on the HARQ-ACK feedback received within specific reference periods.

Given that in SL transmissions occur among UEs, and feedback information is shared without the use of the gNB, moving forward to Rel.18 NR-U SL, if type 1 LBT is used a CWS adjustment procedure would be also needed and while could be still based on the design principles used in LAA and NR-U, this should be designed based on the SL HARQ procedure or its enhancements for unlicensed spectrum.

Proposal 5:

- When operating an NR-U SL system in dynamic channel access mode, the contention windows size adjustment for channel access type 1 needs to be enhanced.

# Relationship Between PPPPs and CAPCs

As mentioned above, when operating NR-U SL in dynamic channel access mode type 1 LBT could be used. In this case, as detailed in Table I, the maximum COT that can be acquired with such LBT channel access type and some of the specifics of this type of LBT depends on a so-called channel access priority class (CAPC), which is configured by the network based on the type of traffic that a device may support as well as based on the QoS requirements that should be met.

Table I - Relationship between Channel Access Priority Class (CAPC) and MCOT.

As for the relationship between the CAPCs and the QoS control indicators (QCIs), this is summarized by Table II.

Table II - Mapping between CAPC and QCI.

| Channel Access Priority Class () | QCI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 69, 70 |
| 2 | 2, 7 |
| 3 | 4, 6, 8, 9 |
| 4 | - |

However, it is important to remark that for SL different QoS requirements have been defined, which are mapped to specific PC5 5G NR standardized QoS Identifiers (PQIs) and indicated in physical layer through the related PQI via the SL ProSe Per Packet Priorities (PPPPs) indication provided within the SL control indication (SCI).

Given that the information related to CAPC is necessary for unlicensed operation, but this will be referencing and will be configured based on different QoS, some mapping between CAPCs and PPPPs is necessary, unless both information would be carried in SCI, which would unnecessarily increase the SCI by 4 bits.

Proposal 6:

- RAN1 should discuss how to relate the ProSe Per Packet Priorities (PPPP) defined in SL and the Channel access priority classes (CAPC), since the latest must be supported for compliance to regulatory requirements. Furthermore, RAN1 should send an LS to RAN2 for the identified issue.

# Mechanisms to Mitigate Mutual Blocking

In Rel.16, SL has been designed so that to optimize UE’s multiplexing by allowing FDMed transmissions from different UEs over different sub-channels. If this design principles would like to be utilized in Rel.18 when operating SL in unlicensed operation some considerations are needed in terms of LBT procedure.  In fact, for SL operating in unlicensed band an LBT procedure may be needed at the UE to initiate a COT, and a UE is expected to perform energy measurements within specific instances of time over chunks of 20 MHz LBT bandwidth (BW), while a transmission may only occur within smaller sub-channels. However, if multiple UEs would like to operate in an FDMed manner, all UEs must perform LBT at the same time so that to prevent UEs from blocking each other, and sense other UE’s transmissions within the LBT BW.

In this matter, it must be important to note that in SL the UEs transmission time is aligned with the DL timing. However:

- a UE may incur into SL synchronization errors (e.g., GNSS sync error or gNB synchronization error), which may be in the order of up to 3 us.

- a gNB SL synchronization includes propagation delays that for macro cell deployments can be in the order of several us (e.g. 2 us or 4 us for gNB-UE distance of 600 m and 1200 m respectively).

- a UE typically has an ON/OFF and OFF/ON transient period in the order of 10 us, but these transient periods may vary based on UE’s capability.

- UEs’ transmissions are subject to different propagation delays.

Due to the aforementioned errors/uncertainties, UEs may end up blocking each other even if in principle the LBT procedure may be intended to be performed at the same time. For instance, the effect of the SL synchronization error is illustrated in Figure 1, which depicts the following two cases:

- Case 1: a UE2 may be blocked by UE1 transmission depending on the position of LBT type of UE2, the propagation delay between UE2 and UE1, ∆prop, and the location of the energy measurement interval (observation windows) within the LBT window of UE2. In this case propagation delay from UE1 to UE2 is in favor of UE2 and plays a positive role.

- Case 2: a UE1 may be blocked by UE2 on any follow up transmission depending on the LBT type, the propagation delay between UE2 and UE1, ∆prop, and the location of the energy measurement interval (observation windows) within the LBT window of UE1. In this case propagation delay from UE2 to UE1 is not in favor of UE1 and plays a negative role.

Furthermore, even if two or multiple UEs may attempt to align their starting transition time, and the synchronization errors may be negligible, blocking among them may still occur due to propagation delays as illustrated in Figure 2. In this specific example, while the LBT windows may be aligned across UEs, and allow both UE#1 and UE#2 to acquire a COT and transmit, the propagation delay may lead UE#2 to block UE#1 to transmit later on, since UE#2 transmission may overlap with the second LBT window of UE#1.

Figure 1 - Examples of the impact of SL synchronization error.

Figure 2 - Examples of impact of UE-UE propagation delay.

In this sense, RAN1 should discuss whether sub-channelization should be also supported in NR-U SL, and in case it decides to support this feature, it should study mechanisms to mitigate mutual blocking across frequency multiplexes transmissions.

Proposal 7:

- RAN1 should discuss whether to support sub-channelization and in case should study mechanisms to mitigate mutual blocking across frequency multiplexed transmissions.

On top of considering NR-U SL operating in FDMed mode, it is also important to investigate NR-U SL when this may operate in a TDMed mode. In this case, one scenario that should be highlighted is when two UEs may select the same set of resources from a resource pool or a set of resources which lead to the same starting time for their transmissions. In this case, by performing LBT in non-overlapping instance of time, the two UEs may not be able to hear each other, and while able to successfully assess that the channel is idle and transmit (by potentially even acquiring an overlapping COT), their transmission may collide with each other, as illustrated in Figure 3.

Figure 3- Illustration of the two TDMed UEs performing LBT at the same time.

In this case, RAN1 should also discuss and study mechanisms to mitigate interference across UEs.

Proposal 8:

- RAN1 should investigate mechanisms to mitigate interference across UEs transmitting in a TDMed manner, since different UEs could have overlapping pool of resources and can potentially start transmission at the same time.

# LBT Procedure

As mentioned above, a UE typically has an ON/OFF and OFF/ON transient period in the order of 10 us, and for operation in unlicensed spectrum the mask provided in Figure 4 is employed [5], where the OFF/ON transient period is incorporated in the CP extension applied prior to the actual transmission, while the ON/OFF transient period partially spills out of the end of such transmission.

End of OFF power

5µs

5µs

Transient period

Transient period

Start of OFF power

Start of ON power

requirement

Start of transmission

End of transmission

End of ON power

requirement

* The OFF power requirements do not

apply for DTX and measurement gaps

10µs

5µs

Figure 4- General ON/OFF time mask for shared spectrum channel access.

If two transmissions are back to back, and separated by a small gap, that still require LBT procedure, it may occur that the transient period may actually impact negatively the LBT if the transient period overlaps with the measurement window. While this issue was solved in Rel.16 NR-U via proper implementation, this could negatively impact NR-U SL especially in the case when a UE transmit PSFCH within a SL slot given that the gap between the two transmission bursts may only be a single OFDM symbol, which for 60 kHz SCS corresponds to roughly 16 us. So in this case, if the LBT is required at the UE before PSFCH, then the measurement may also capture in part the transmission the prior burst due to the transient time. In this regard, RAN1 should investigate further the impact of the ON/OFF and OFF/ON transient period to the LBT procedure

Proposal 9:

- RAN1 should investigate the impact of the ON/OFF and OFF/ON transient period to the LBT procedure performed by a UE when transmission of a PFSCH may require LBT and a SL slot may contain a PSFCH transmission.

# Sensing and Resource Selection Procedure

In Rel.16 SL, when operating in mode 2 some specific principles have been defined to allow a proper selection of the resources to be used by a UE. In particular a sensing procedure has been established so that to scan the medium within a given window to establish beforehand a set of candidate resources that are suitable to be used or can be used within a given pool. In fact, the UE performs RSRP measurements, and compares such measurements with a specific threshold, which dependents on the SL priority chosen so that to establish whether a specific resource if used will not create congestion or interfere with another SL UEs. In this sense, a summary schematic of the sensing and resource selection procedure is provided in Figure 5.

Figure 5 - Summary Schematic of the Rel.16 sensing and resource selection scheme.

Moving forward to Rel.18, as mentioned above when a SL system is operated in unlicensed spectrum, an LBT procedure is mandatorily needed to acquire a COT, or even within a COT, if COT sharing is allowed, based on the gap between SL transmission bursts within that COT. In this matter, given that LBT performs an instantaneous measurement of the medium and can be used as a method to determine whether a transmission or a resource could be used for a transmission, then the Rel.16 SL sensing and resource selection scheme could be modified to include the LBT procedure in it since the two perform in some sense redundant operation.

Proposal 10:

- RAN1 should discuss how to combine the LBT procedure with the SL sensing and resource selection procedure.

# Short Control Signalling

When discussing the channel access procedure for Rel.18 NR-U SL, it should be also discussed how to interpret the exemption provided by the ETSI BRAN [3] regarding the short control signalling exemption:

In this matter, the ETSI BRAN defines a short control signal as a signal/channel that is not transmitted more than 50 times within an observation time of 50 ms, and with a maximum duty cycle of 5 %. Furthermore, ETSI BRAN indicates that no LBT is required before the transmission of a short control signal.

In Rel.16 NR-U, the concept of short control signalling has been applied to the discovery reference signal (DRS) which carries the synchronization signal block (SSB). In fact, this is the only signal that is allowed to be transmitted by succeeding type 2A LBT when its duty cycle is less or equal than 1/20, instead of type 1 LBT as for other channel/signals.

Moving forward to Rel.18 NR-U SL, RAN1 should discuss whether same approach as Rel.16 NR-U should be used for the signals that qualify as short control signalling (i.e., use type 2A to transmit them) or the ETSI BRAN should be more literally followed, and in this case no LBT would be needed. Furthermore, RAN1 should further discuss which signals should be qualified as short control signals. In this matter, at least S-SSB and PSFCH could be considered to follow this exception as long as the minimum requirements are met.

Proposal 11:

- At least both S-SSB and PSFCH could be qualifies as a short control signalling. In this sense RAN1 should further discuss and converge on how to interpret the ETSI BRAN related text:

- Option 1: As long as these signals meet the minimum requirements to qualify as a short control signalling, they can be transmitted without LBT.

- Option 2: As long as these signals meet the minimum requirements to qualify as a short control signalling, a UE may transmit them using type 2A LBT.

Conclusions

In this contribution, we discussed several aspects related to channel access enhancements to enable NR SL to operate in the FR-1 unlicensed spectrum, and made the following proposals:

Proposal 1:

- SL operating is unlicensed spectrum supports both dynamic and semi-static channel access mode.

Proposal 2:

- The channel access types defined in Rel.16 NR-U (i.e., type 1, type 2A, 2B and 2C) are supported in SL operating in unlicensed spectrum. FFS: when to use them based on the type of SL channels and signals transmitted and based on COT sharing conditions.

Proposal 3:

- RAN1 should agree on the exact interpretations of the intended deployment for mode-1:

- Mode-1-a: a gNB sends scheduling DCI on the licensed carrier, and can perform sensing on SL unlicensed carrier, but it is not allowed to transmit on the SL unlicensed carrier.

- Mode-1-b: a gNB sends scheduling DCI on the licensed carrier and cannot either perform sensing or transmit on SL unlicensed carrier.

Proposal 4:

- RAN1 should study a unified procedure for both mode 1 and mode 2 to allow a UE to share its COT with other UE(s).

Proposal 5:

- When operating an NR-U SL system in dynamic channel access mode, the contention windows size adjustment for channel access type 1 needs to be enhanced.

Proposal 6:

- RAN1 should discuss how to relate the ProSe Per Packet Priorities (PPPP) defined in SL and the Channel access priority classes (CAPC), since the latest must be supported for compliance to regulatory requirements. Furthermore, RAN1 should send an LS to RAN2 for the identified issue.

Proposal 7:

- RAN1 should discuss whether to support sub-channelization and in case should study mechanisms to mitigate mutual blocking across frequency multiplexed transmissions.

Proposal 8:

- RAN1 should investigate mechanisms to mitigate interference across UEs transmitting in a TDMed manner, since different UEs could have overlapping pool of resources and can potentially start transmission at the same time.

Proposal 9:

- RAN1 should investigate the impact of the ON/OFF and OFF/ON transient period to the LBT procedure performed by a UE when transmission of a PFSCH may require LBT and a SL slot may contain a PSFCH transmission.

Proposal 10:

- RAN1 should discuss how to combine the LBT procedure with the SL sensing and resource selection procedure.

Proposal 11:

- At least both S-SSB and PSFCH could be qualifies as a short control signalling. In this sense RAN1 should further discuss and converge on how to interpret the ETSI BRAN related text:

- Option 1: As long as these signals meet the minimum requirements to qualify as a short control signalling, they can be transmitted without LBT.

- Option 2: As long as these signals meet the minimum requirements to qualify as a short control signalling, a UE may transmit them using type 2A LBT.

References

- RP-213678, “New WID on NR sidelink evolution”, OPPO, LGE, December 2021.

- R1-2204803, “On the Enhancements to the Physical Channel Design for SL Operating in Unlicensed Spectrum “, Intel Corporation, May 2022.

- ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, (2017-05).

- R4-1816666, “Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects”, Spokane, USA, November 2018.

- ETSI TS 38 101-1, “5G; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, ETSI, v16.9.0, October 2021.