pp3GPP TSG RAN WG1 #109-e			     R1-2204803

e-Meeting, May 9th – May 20th, 2022

Source:	Intel Corporation

Title:	On the Enhancements to the Physical Channel Design for SL Operating in Unlicensed Spectrum

Agenda item:	9.4.1.2

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94 [1], a new work item related to NR Sidelink (SL) evolution was approved. As part of the objectives of this working item (WI), that following aspects were included:

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, in the context of enhancements to enable NR SL to operate in the FR-1 unlicensed band, which we will refer to as NR-U SL in the remaining of this document, a few topics related to enhancements to the physical channel design will be discussed, while view on other aspects are provided in our companion contribution [2]:

- General consideration on the SL physical channel’s structure

- Automatic gain control (AGC) considerations

- Multi-slot transmissions

- Considerations on PSCCH Design

- Considerations on PSFCH Design

- Considerations on S-SSB Design

# SL Physical Channel Design

Operation in unlicensed bands requires the compliance with all regional regulatory requirements. Among others, the regulatory requirements mandated by the ETSI BRAN [3] are the most stringent, and the structure of the physical layer channels of the SL need to adapt to fulfil these requirements.

## Considerations on Interlaced Physical Structure

The ETSI BRAN [3] mandates a strict limitation on the maximum power spectrum density (PSD) of 10 dBM/MHz, but more importantly it mandates a minimum of 80 % occupied channel bandwidth (OCB), which implies that 80 % of the LBT bandwidth (BW), which is composed by chunks of 20 MHz BW, needs to be occupied:

In order to meet these requirements, in Rel.16 NR-U, an interlaced waveform was introduced and supported for both PUCCH and PUSCH transmissions. In particular, since these requirements are not mandate worldwide, the interlaced waveform is applied to PUCCH and PUSCH based on the higher layer parameter useInterlacePUCCH-PUSCH. In particular, when useInterlacePUCCH-PUSCH is configured, for 15 kHz and 30 kHz subcarrier spacing (SCS) the PRBs are interlaced to occupy the whole bandwidth as illustrated in Fig. 1, while for 60 kHz SCS, no interlace is supported.

Figure 1 - Example of interlaced waveform applied to a 20 MHz bandwidth with 30 kHz SCS (NPRB = 51),

Moving forward to Rel. 18 NR-U SL, given that this waveform is already defined in NR-U support of the interlaced waveform could be considered for regions where the OCB requirements must be met. In this sense, the interlaced waveform could be applied to at least PSCCH, and PSSCH.

Proposal 1:

- For regions which mandate the 80% OCB restriction, an interlaced resource block structure supporting at least for PSCCH and PSSCH could be considered.

TX/RX & RX/TX Switching Gaps

The Rel.16 SL physical structure defines one symbol (including cyclic prefix) as the TX/RX and RX/TX switching gaps (also called guard period) as illustrated in Figure 2.

Figure 2 – Illustration of switching times within a SL slot with and without PSFCH

The actual duration needed for TX/RX and RX/TX switching was determined by RAN4, and it should be at most 13 us in FR1 [4]. The allocated duration for TX/RX and RX/TX switching gap thus depends on SCS settings and is larger than the actual time needed. In fact, an OFDM symbol duration without cyclic prefix (CP) is 66.6 / 33.3 and 16.6 us for 15 /30 /60 kHz SCS, respectively.

Moving forward to Rel. 18 NR-U SL, given that a TX/RX and RX/TX switching gap may be always larger than 16 us, LBT may be always required, which is highly inefficient and always conditions the follow up transmission on the success of LBT. Considering, as mentioned above, that the actual switching time required is much less than the imposed TX/RX and RX/TX switching gap, RAN1 should thrive for a more efficient operation.

Proposal 2:

- RAN1 should thrive to reduce the TX/RX & RX/TX switching gaps within a SL slot.

In order to allow a more efficient use of the LBT procedure in NR-U SL, another additional aspect to consider is that due to the measurement windows numerologies, an LBT window may not always align with the symbol boundaries, while a UE should transmit at the earliest possible opportunity to acquire that channel. In this sense, in Rel.16 NR-U, a CP extension was mechanism was adopted to fill gaps and align the end of an LBT window with the start of a transmission or to make sure proper extension way applied to a transmission burst to that to artificially make sure that gaps among transmissions burst may lead to the least LBT overhead. While same design principle could be reused in NR-U SL, it is also important to remind that in Rel.16 NR-U, the value of CP extension depends on the timing advance so that to compensate for propagation delays. However, as mentioned above, for SL communication, the GNSS or network DL reception timing is used for the SL transmission.

Proposal 3:

- For SL communication in unlicensed band, the CP extension principles introduced in Rel.16 NR-U can be reused for NR-U SL communication without account for timing advance.

## Sidelink AGC Considerations

Based on Rel.16 SL design, all SL transmissions start at a predefined symbol positions within a slot. The first symbol of each SL transmission is a replica of the second symbol. Such physical structure was defined to support AGC settling at each slot to mitigate ADC quantization errors and clipping noise, and also following the RAN4 input on AGC convergence time [5]. In particular, for single CC using CP-OFDM waveform and at least 10 RB allocation at least the following time is required for AGC adaptation:

  - 35 us for 15 kHz SCS.

  - 35 us for 30 kHz SCS.

  - 18 us for 60 kHz SCS.

However, when operating in unlicensed spectrum performing channel access at fixed/predefined position in time is highly detrimental from a spectrum utilization point of view, since while a possible incumbent technology can access channel at arbitrary time and across slot boundaries, a UE may only have a single opportunity to perform LBT within a slot. However, on the other hand, as mentioned above, this allows a proper AGC handling.

With that said, different considerations could be drawn based on the deployment and whether or not an incumbent technology is absent. In particular, if the network can guarantee the absence of an incumbent technology, there would not be no particular impact on the spectrum utilization if all SL transmissions have a predefined starting position as in Rel.16 NR SL. However, when the absence of the incumbent technology cannot be guaranteed, the tradeoff between better spectrum utilization and better AGC handling should be further studied, also considering that having more flexible starting positions may lead to higher implementation complexity.

Proposal 4:

- At least in deployments where no incumbent technology is present, all SL transmissions should have a predefined starting position as in Rel.16 NR SL.

Proposal 5:

- Considering the AGC adaptation and spectrum utilization, RAN1 should study whether to support a more flexible starting position within a slot when an incumbent technology is present.

## Multi-slot transmissions

As the efficient use of a COT is a fundamental design target for NR-U SL, as in Rel. 16 NR-U multi-slot transmissions of multiple TBs should be also supported. In principle there are two main solution direction to enable this feature:

Option 1: One SCI information is signalling all resource in all slots used for transmission as in Figure 3a;

Option 2: Multiple SCI signal the resource for each individual slots or for a subset of slots as in Figure 3b and 3c.

From a system level perspective considering the limitation of the SL, the latter case is more attractive as it allows devices to demodulate the related shared channel even if the first SCI information is missed.

Figure 3: PSCCH resource signalling for the case that a) all resources are signaled in one SCI, b) each shared channel has its own SCI signalling, and c) each SCI is signalling a sub-set of all resources.

Proposal 6:

- To efficiently use a COT, multi-slot SL transmissions should be supported

## Considerations on PSCCH and PSFCH Design

As discussed in our companion contribution [2], one of the important aspects that RAN1 should discuss is on how to interpret the exemption provided by the ETSI BRAN [3] regarding the short control signalling exemption:

In this matter, the ETSI BRAN defines a short control signal as a signal/channel that is not transmitted more than 50 times within an observation time of 50 ms, and with a maximum duty cycle of 5 %. Furthermore, ETSI BRAN indicates that no LBT is required before the transmission of a short control signal.

In this case, if RAN1 decides to qualify PSFCH as a short control signalling, and interprets the ETSI BRAN text literally, then the HARQ feedback procedure defined in Rel.16 SL could be reused. Otherwise, in order to properly support SL in unlicensed spectrum, the HARQ-feedback procedure defined in Rel.16 SL should be enhanced as done in Rel.16 NR-U to mitigate the impact of potential LBT failures, which make the HARQ feedback transmission no longer deterministic.

Proposal 7:

- RAN1 should study whether HARQ feedback based on PSFCH is feasible for SL in unlicensed spectrum.

As discussed in our companion contribution [2], and along this contribution, there are several aspects that may impact and need to be considered for the design of PSCCH. For instance:

OCB requirements: this requirement may lead to utilize an interlaced physical layer, and also impact the SCI content.

Multi-slot design: This requires changes on the PSCCH to support this functionality.

COT sharing for NR-U SL: This requires changes in the related PSCCH signalling.

Sensing and resource selection procedure: if this is combined with the LBT procedure the benefits/rationale to have a 2-stage SCI may no longer exist.

HARQ-feedback: if HARQ feedback based on PSFCH is not feasible for SL in unlicensed spectrum, or if multi-slot design is supported, a new PSCCH format could be considered which may carry HARQ-feedback information

Proposal 8:

- RAN1 should study the design of PSCCH to support the new functionalities of NR-U SL.

## Considerations on S-SSB Design

In NR SL, the physical structure of S-SSB design is as shown in Figure 4, and it is relatively narrow band, motivated by complexity reduction of the SL synchronization search.

Figure 4: Physical structure of S-SSB (PSBCH, S-PSS, S-SSS)

As discussed along this disclosure, the ETSI BRAN [3] mandates a strict limitation in terms of OCB, which requires a device to occupy 80% of the LBT BW. However, as shown in Figure 4, the legacy S-SSB is not compliant with this requirement, and in this matter, the S-SSB for NR-U SL should be enhanced to span over a larger occupied BW. To increase the occupied bandwidth of S-SSB, RAN1 may consider one of the following options:

Option 1: S-SSB is compacted to a fewer number of symbols and/or multiplexed with other signals/channels to extend it in frequency domain over the whole LBT BW.

Option 2: Use of a comb-structure for S-SSB transmission to occupy 80% of the nominal BW, where the comb-structure could be at the PRB, sub-channel or RE level.

Option 3: Frequency repetition of S-SSB allocation.

Option 4: Design of a wideband S-SSB. This option requires more efforts and time for specification work to enable new S-SSB design for 10 MHz and 20 MHz bandwidth for various numerologies.

Proposal 9:

- RAN1 should study on how to meet the 80 % OCB requirement for the S-SSB in SL in unlicensed spectrum.

Conclusions

In this contribution, we have provided our views on the physical structure for SL operation in the unlicensed band. In summary, we have the following list of proposals and observations:

Proposal 1:

- For regions which mandate the 80% OCB restriction, an interlaced resource block structure supporting at least for PSCCH and PSSCH could be considered.

Proposal 2:

- RAN1 should thrive to reduce the TX/RX & RX/TX switching gaps within a SL slot.

Proposal 3:

- For SL communication in unlicensed band, the CP extension principles introduced in Rel.16 NR-U can be reused for NR-U SL communication without account for timing advance.

Proposal 4:

- At least in deployments where no incumbent technology is present, all SL transmissions should have a predefined starting position as in Rel.16 NR SL.

Proposal 5:

- Considering the AGC adaptation and spectrum utilization, RAN1 should study whether to support a more flexible starting position within a slot when an incumbent technology is present.

Proposal 6:

- To efficiently use a COT, multi-slot SL transmissions should be supported.

Proposal 7:

- RAN1 should study whether HARQ feedback based on PSFCH is feasible for SL in unlicensed spectrum.

Proposal 8:

- RAN1 should study the design of PSCCH to support the new functionalities of NR-U SL.

Proposal 9:

- RAN1 should study on how to meet the 80 % OCB requirement for the S-SSB in SL in unlicensed spectrum.

References

- RP-213678, “New WID on NR sidelink evolution”, OPPO, LGE, December 2021.

- R1-2204802, “On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Corporation, May 2022.

- ETSI EN 301 893, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, ETSI, V2.1.1, Mai 2017.

- R4-1816666, “Reply LS on NR V2X RF parameters for RAN1 physical layer design aspects”, Spokane, USA, November 2018.

- R4-1905241, “Reply LS on NR V2X UE RF parameters for NR V2X services”, 3GPP RAN4, April 2019.