3GPP TSG RAN WG1 #109-e			R1-2204806

e-Meeting, May 9th – May 20th, 2022

Source: 	Intel Corporation

Title: 	On SL positioning scenarios and requirements

Agenda item:	9.5.1.1

Document for: 	Discussion and Decision

# Introduction

The study item on expanded and improved NR positioning was approved in [1]. One of the study objectives is to identify specific target performance requirements and scenarios for sidelink positioning evaluations in Rel.-18:

Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR 38.845 and TS 22.261 and TS 22.104 - Use cases: V2X (TR 38.845), public safety (TR 38.845), commercial (TS 22.261), IIOT (TS 22.104) - Spectrum: ITS, licensed Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

In this contribution, we express our views on scenarios and performance requirements for NR sidelink positioning based on existing 3GPP work [2], inputs from industry forums [6], and references therein.

NR Sidelink Positioning

## Use Cases

The use cases for sidelink positioning were already discussed by 3GPP ecosystem in [2] - [5] and during preparation of the study item description document. The following use cases were explicitly captured in SID objectives [1]:

V2X (TR 38.845)

Public safety (TR 38.845)

Commercial (TS 22.261)

IIOT (TS 22.104)

In our view, the sidelink positioning framework once it is developed can be applicable to a broader set of use cases. The existing set of use cases is sufficient to trigger 3GPP study for analysis of sidelink positioning framework.

## Scenarios

Our views on scenarios were presented in [8]. In terms of network coverage scenarios, the following valid set of scenarios were included into the study item description document:

In-coverage scenario

Partial-coverage scenario

Out-of-coverage scenario

In terms of operation scenarios, in our view the following RAT dependent positioning options can be considered for all use cases. Without loss of generality, below we provide list of scenarios and illustrations for V2X use case:

- Scenario 1: PC5 (sidelink) based positioning (relative or absolute)

  - Radio-layer measurements over PC5 links are used to perform UE positioning either at UE or network side. This framework needs to be introduced.

- Scenario 2: Combination of Uu (downlink / uplink) and PC5 (sidelink) based positioning solutions

  - Radio-layer measurements over Uu and PC5 links are used to perform UE positioning either at UE or network side. This framework needs to be integrated and supplement existing NR positioning functionality.

- Scenario 3: Combination of NR RAT-dependent and RAT-independent solutions

  - Radio-layer measurements and RAT-independent information is utilized for positioning (e.g. GNSS, IMUs, sensors, etc.)

In terms of coordinate estimation, both UE assisted solutions (where coordinate is determined at network side) and UE based solutions (where coordinate is determined at UE side) should be supported.

In terms of study organization and sidelink positioning evaluations, we think the focus of the study should be given to Scenario #1 and #3.

  - The following scenarios in terms of network coverage are considered for sidelink positioning: in-coverage, partial coverage, out-of-coverage.

  - The following operation scenarios are considered for sidelink positioning study

    - Scenario 1: PC5 (sidelink) based positioning (relative or absolute)

    - Scenario 2: Combination of Uu (downlink / uplink) and PC5 (sidelink) based positioning solutions

    - Scenario 3: Combination of NR RAT-dependent and RAT-independent solutions.

## Target Performance Requirements

In [2], it is observed that the positioning requirements depend on the service the UE operates. The requirements typically applicable to relative and absolute positioning.

### V2X Use Cases

For V2X use cases, the requirements for the absolute position or relative position can be categorised into three sets as follows by incorporating the requirements from the sources mentioned above:

Set 1: 10 – 50 m with 68 – 95 % confidence level

Set 2: 1 – 3 m with 95 – 99 % confidence level

Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level

The sets provided above are applicable for absolute and relative positioning.

Requirements for additional performance metrics are also defined in a range depending on the positioning service level:

2 – 3 m (absolute) or 0.2 m (relative) vertical accuracy

95 – 99.9 % positioning service availability

10 ms – 1 s positioning service latency

Positioning service should be provided in indoor, outdoor, tunnel areas. The UE velocity up to 250 km/h needs to be supported for outdoor and tunnel areas, inside the network coverage and outside the network coverage. The requirements should be also fulfilled when the GNSS-based positioning is not available or not accurate enough.

In addition, basic ranging functionality is an important functionality and should also be considered for V2X use-cases to estimate distance between vehicles, etc.

### Public Safety

Public safety positioning requirements are defined in TS 22.261 [3] (please refer to positioning requirements for the "1st responders" use case in Table B.1-1):

1 m horizontal accuracy

2 m (absolute) or 0.3 m (relative) vertical accuracy

95 – 98 % positioning service availability

### Commercial (TS 22.261)

Requirements for commercial use case are presented in Table 1, from from TS 22.261.

Table 1. Performance requirements for Horizontal and Vertical positioning service levels (commercial use-cases) (from TS 22.261)

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

### Industrial IOT (TS 22.104)

Positioning requirements for I-IOT use cases are captured in the TS 22.104. In this context, positioning is especially important for warehousing and logistics processes, autonomous driving systems and fleet management and flexible adaptation in production. In the best case all relevant goods and products are continuously tracked from the moment they are received to the moment they are made available. The tracking process provides the relevant context information that is needed for real-time control and optimization of the material flow and subsequent production processes. Wireless positioning for human machine interfaces like AR/VR should also be possible.

Table 2. Positioning performance requirements – IIoT (from TS 22.104)

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

### Low Power High Accuracy Positioning

Low power high accuracy positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/‌synchronization and a difficult to predict factor to take additional losses through e.g., security, power management, microcontroller, and self-discharge of batteries into account.

Examples of target applications for low power high accuracy positioning are asset tracking in process automation, tracking of vehicles, and tool tracking.

Table A.7.2-1 gives an indication of the required operation time of the 5G enabled IoT device and duty cycle of the updated position information for different use cases.

Table 3. Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 10 m | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

Some of the tools towards support of LPHAP are offered by positioning support in RRC_INACTIVE states. In this context, further considerations may be needed to decide if LPHAP use-cases should be targeted for SL positioning.

In summary, analysing positioning requirements from existing 3GPP work and inputs from industry forums we have following proposals for the study item:

    - Study SL positioning methods targeting at least the following use-cases:

      - V2X

      - Public safety (first responders)

      - Commercial

      - Industrial IoT.

    - FFS: Targeting LPHAP use-cases for SL positioning.

    - Consider the requirements for SL positioning methods starting with the following use-cases as starting point:

      - V2X:

        - Set 1: 10 – 50 m with 68 – 95 % confidence level (for absolute and relative positioning)

        - Set 2: 1 – 3 m with 95 – 99 % confidence level (for absolute and relative positioning)

        - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level (for absolute and relative positioning)

        - Set 4: 2 – 3 m (absolute) or 0.2 m (relative) vertical accuracy

        - 95 – 99.9 % positioning service availability

        - 10 ms – 1 s positioning service latency

        - UE speed of up to 250 kmph, including service in-coverage/partial/out of coverage.

      - Public safety (first responders):

        - 1 m horizontal accuracy

        - 2 m (absolute) or 0.3 m (relative) vertical accuracy

        - 95 – 98 % positioning service availability

      - Commercial:

        - As in Table 1 (Table 7.3.2.2-1 in TS 22.261)

      - Industrial IoT:

        - As in Table 2 (Table 5.7-1 in TS 22.104).

Deployment scenarios for SL positioning evaluations

Both ITS and licensed bands should be considered for SL positioning evaluations. Depending on use-cases UE speeds from stationary to up to 250 kmph should be considered.

On bandwidth assumptions, for ITS band, the maximum bandwidth may be limited e.g., 10/20/30/etc. MHz depending on geographical region/country. Given the relevance of SL positioning to V2X use-cases for ranging and relative/absolute positioning, it may be instructive to also evaluate performance assuming, e.g., 20 MHz SL bandwidth. In practice, techniques like bandwidth aggregation may be utilized, amongst other methods (e.g., for RedCap UEs) to improve positioning performance.

Lastly, on simulation environments, UE/gNB Tx/Rx antenna configurations, and channel models as in TR 38.885 may be considered as a starting point, details of which may be discussed and decided as part of AI 9.5.1.2.

    - On deployment scenarios, RAN1 to consider the following:

      - Bands: ITS (n47; 5.9 GHz) and licensed (4 GHz, 30 GHz)

      - UE speed: 6 kmph and 250 kmph relative speeds

      - SL bandwidth: 20 MHz, 100 MHz

      - Simulation environment, UE Tx/Rx antenna assumptions, and channel models as in TR 37.885 may be considered as a starting point.

Conclusion

In this contribution, we provided our views on scenarios and requirements for NR sidelink positioning. In summary, we have following proposals:

  - The following scenarios in terms of network coverage are considered for sidelink positioning: in-coverage, partial coverage, out-of-coverage.

  - The following operation scenarios are considered for sidelink positioning study

    - Scenario 1: PC5 (sidelink) based positioning (relative or absolute)

    - Scenario 2: Combination of Uu (downlink / uplink) and PC5 (sidelink) based positioning solutions

    - Scenario 3: Combination of NR RAT-dependent and RAT-independent solutions.

    - Study SL positioning methods targeting at least the following use-cases:

      - V2X

      - Public safety (first responders)

      - Commercial

      - Industrial IoT.

    - FFS: Targeting LPHAP use-cases for SL positioning.

    - Consider the requirements for SL positioning methods starting with the following use-cases as starting point:

      - V2X:

        - Set 1: 10 – 50 m with 68 – 95 % confidence level (for absolute and relative positioning)

        - Set 2: 1 – 3 m with 95 – 99 % confidence level (for absolute and relative positioning)

        - Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level (for absolute and relative positioning)

        - Set 4: 2 – 3 m (absolute) or 0.2 m (relative) vertical accuracy

        - 95 – 99.9 % positioning service availability

        - 10 ms – 1 s positioning service latency

        - UE speed of up to 250 kmph, including service in-coverage/partial/out of coverage.

      - Public safety (first responders):

        - 1 m horizontal accuracy

        - 2 m (absolute) or 0.3 m (relative) vertical accuracy

        - 95 – 98 % positioning service availability

      - Commercial:

        - As in Table 1 (Table 7.3.2.2-1 in TS 22.261)

      - Industrial IoT:

        - As in Table 2 (Table 5.7-1 in TS 22.104).

    - On deployment scenarios, RAN1 to consider the following:

      - Bands: ITS (n47; 5.9 GHz) and licensed (4 GHz, 30 GHz)

      - UE speed: 6 kmph and 250 kmph relative speeds

      - SL bandwidth: 20 MHz, 100 MHz

      - Simulation environment, UE Tx/Rx antenna assumptions, and channel models as in TR 37.885 may be considered as a starting point.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, December 2021

- 3GPP TR 38.845: "Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases"

- 3GPP TS 22.261: "Service requirements for the 5G system"

- 3GPP TR 22.855: "Study on ranging-based services"

- 3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains"

- 3GPP RP-210040: "Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases," (source: 5GAA)

- 3GPP TR 37.885: "Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR"

- 3GPP RP-211306, “NR Positioning Deployment Scenarios for V2X and Public Safety,” Intel Corporation, 3GPP RAN #92-e.