3GPP TSG RAN WG1 Meeting #109-e					    	 R1-2204809

e-Meeting, May 9-20, 2022

Source:	Intel Corporation

Title:	On solutions to further reduce UE complexity

Agenda item:	9.6.1

Document for:	Discussion and Decision

# Introduction

At RAN plenary meeting #94-e, the study item (SI) for further NR RedCap UE complexity reduction was agreed with the following identified objectives [1]

To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx: - Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we present our views on different features that can be considered for complexity reduction. The potential complexity reduction ratio, the performance impacts and the specification impacts any analyzed for each feature.

# Complexity reduction analysis

The further reduced complexity UE (F-RedCap UE) is targeting peak data rate of 10Mbps. Assuming a maximum modulation order of 64QAM or 16QAM in the DL and UL, single MIMO layer, and under some typical assumptions of overhead (OH) factors for control channels of 8% and 14% for DL and UL respectively, the achievable peak DL and UL throughputs for 5MHz channel BWs are provided in Table 1. Based on the above calculation, a UE with 5MHz BW is capable to support target peak data rate with a margin.

Table 1: Peak DL/UL throughput in FR1 for 5MHz channel BW

| Channel BW (MHz) | Modulation Order | Peak DL throughput (Mbps) | Peak UL throughput (Mbps) |
| --- | --- | --- | --- |
| 5 | 4 | 13.4 | 14.3 |
| 5 | 6 | 20 | 21.5 |

The BW reduction for F-RedCap UE can be considered from RF and/or BB parts. The largest complexity reduction can be achieved when the BW of both RF and BB can be reduced to 5MHz (RBW-A). On the other hand, based on the analysis, the gain from reducing RF BW is relatively small, i.e., the complexity reduction mainly comes from BB part. Therefore, it can be considered to maintain 20MHz RF and reduce BW of BB only. In one option, all control and data processing at BB part can be limited to 5MHz BW (RBW-B). Another option is to only limit the data channel processing within 5MHz BW (RBW-C). As shown in Figure 1, the different combinations of options may be considered for DL and UL.

Figure 1: Options for BW reduction in RF and/or BB

As captured in the SID, instead of limiting the number of PRBs or REs for PDSCH/PUSCH transmission, another solution for complexity reduction is to directly limit the peak data rate, which also indicates a reduction on the maximum TBS for the F-RedCap UE. However, limiting peak data rate doesn’t necessarily mean the reduction of received REs in a BWP of up to 20MHz. In an example, UE may be allocated a PDSCH transmission in all PRBs and OFDM symbols in the BWP for lower coding rate, which provides better link performance. Therefore, the impact on total number of received PRBs or REs by restricting the peak data rate should be clarified.

In additional to the limitation of PRBs, REs or peak data rate, some other features for complexity reduction can be considered.

- Relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI, e.g., the required values of the timelines can be doubled

- Relaxed maximum modulation order. For example, 64QAM for DL and 16QAM for UL

- Reduced maximum number of HARQ processes

- Type-B HD-FDD operation

In the following, we provide the evaluation on complexity reduction for the different combinations of the above features in Table 2. The relative reduction compared to a simplest Rel-17 RedCap UE is also provided. The simplest Rel-17 RedCap UE is charactered as 20MHz, 1 MIMO layer, 1 Rx/Tx Chain, 64QAM for DL modulation and HD-FDD Type A.

Table 2: Cost reduction in FR1 for 5MHz channel BW

| FR1 TDD UE complexity reduction technique(s) | RF | BB | RF+BB | Reduction compared to simplest Rel-17 RedCap UE |
| --- | --- | --- | --- | --- |
| Simplest Rel-17 RedCap UE | 58.5% | 24.7% | 38.2% |  |
| 5 MHz RBW-A, 1L, 1Rx | 71% | 17.1% | 38.6% | -1.2% |
| 5 MHz RBW-B, 1L, 1Rx | 71% | 17.8% | 39.1% | -2.3% |
| 5 MHz RBW-C, 1L, 1Rx | 71% | 18.5% | 39.5% | -3.5% |
| 5 MHz RBW-A, 1L, 1Rx, Type A | 59.8% | 17.1% | 34.2% | 10.6% |
| 5 MHz RBW-B, 1L, 1Rx, Type A | 59.8% | 17.8% | 34.6% | 9.4% |
| 5 MHz RBW-C, 1L, 1Rx, Type A | 59.8% | 18.5% | 35% | 8.3% |
| 5 MHz RBW-A, 1L, 1Rx, DL 64QAM, Type A | 58.5% | 16.8% | 33.5% | 12.4% |
| 5 MHz RBW-B, 1L, 1Rx, DL 64QAM, Type A | 58.5% | 17.5% | 33.9% | 11.4% |
| 5 MHz RBW-C, 1L, 1Rx, DL 64QAM, Type A | 58.5% | 18.2% | 34.3 | 10.2% |
| 5 MHz RBW-A, 1L, 1Rx, DL 64QAM, Type A, Double N1/N2 | 58.5% | 14% | 31.8% | 16.8% |
| 5 MHz RBW-B, 1L, 1 Rx, DL 64QAM, Type A, Double N1/N2 | 58.5% | 14.7% | 32.2% | 15.7% |
| 5 MHz RBW-C, 1L, 1Rx, DL 64QAM, Type A, Double N1/N2 | 58.5% | 15.4% | 32.6% | 14.6% |
| 5 MHz RBW-A, 1L, 1Rx, DL 64QAM, UL 16QAM, Type A, Double N1/N2 | 56.6% | 13.8% | 30.9% | 19% |
| 5 MHz RBW-B, 1L, 1 Rx, DL 64QAM, UL 16QAM, Type A, Double N1/N2 | 56.6% | 14.4% | 31.3% | 18.1% |
| 5 MHz RBW-C, 1L, 1Rx, DL 64QAM, UL 16QAM, Type A, Double N1/N2 | 56.6% | 15.1% | 31.7% | 16.9% |
| 5 MHz RBW-A, 1L, 1Rx, DL 64QAM, Type B | 55.9% | 16.8% | 32.4% | 15.1% |
| 5 MHz RBW-B, 1L, 1Rx, DL 64QAM, Type B | 55.9% | 17.5% | 32.8% | 14.1% |
| 5 MHz RBW-C, 1L, 1Rx, DL 64QAM, Type B | 55.9% | 18.2% | 33.3% | 12.9% |
| 5 MHz RBW-A, 1L, 1Rx, DL 64QAM, Type B, Double N1/N2 | 55.9% | 14% | 30.8% | 19.5% |
| 5 MHz RBW-B, 1L, 1Rx, DL 64QAM, Type B, Double N1/N2 | 55.9% | 14.7% | 31.2% | 18.4% |
| 5 MHz RBW-C, 1L, 1Rx, DL 64QAM, Type B, Double N1/N2 | 55.9% | 15.4% | 31.6% | 17.3% |
| 5 MHz RBW-A, 1L, 1Rx, DL 64QAM, UL 16QAM, Type B, Double N1/N2 | 54.1% | 13.8% | 29.9% | 21.7% |
| 5 MHz RBW-B, 1L, 1Rx, DL 64QAM, UL 16QAM, Type B, Double N1/N2 | 54.1% | 14.4% | 30.3# | 20.7% |
| 5 MHz RBW-C, 1L, 1Rx, DL 64QAM, UL 16QAM, Type B, Double N1/N2 | 54.1% | 15.1% | 30.7% | 19.6% |

From Table 2, with same assumption as the simplest Rel-17 RedCap UE except BW reduction to 5MHz, it can provide a complexity reduction of 12.4%, 11.4%, 10.2% for RBW-A/B/C respectively. Further, additional complexity reduction can be achieved by limiting UL modulation to 16QAM, Relaxed UE processing timeline and HD-FDD operation type B. The gain is almost doubled if all the additional features are considered. From Table 2, it can also be observed that the difference of complexity reduction among RBW-A/B/C is relatively small.

Observation 1

- With further complexity reduction compared to a simplest Rel-17 RedCap UE,

  - the complexity reduction ranges from 12.4% - 21.7% for RBW-A

  - the complexity reduction ranges from 11.4% - 20.7% for RBW-B

  - the complexity reduction ranges from 10.2% - 19.6% for RBW-C

- The difference of complexity reduction among RBW-A/B/C is relatively small

# Bandwidth reduction to 5MHz

As discussed in Section 2, 3 options for bandwidth reduction, i.e., RBW-A/B/C can be considered. For RBW-A/B, PDSCH/PUSCH must be allocated within localized 25 PRBs. On the other hand, in RBW-C, up to 25 allocated PRBs for PDSCH/PUSCH could be localized or distributed in the BWP of up to 20MHz BW. The two options have the same level of complexity.

RBW-A/B enable relatively larger complexity reduction. On the other hand, the benefit of frequency diversity is limited due to the small BW, which results in degraded link level performance for control and data channel for both DL and UL.

For PDSCH, PUSCH and PUCCH transmission, in order to compensate the loss of frequency diversity, inter-BWP frequency hopping can be considered. However, the impact of BWP switching delay needs to be considered. Legacy BWP switching time can be one or multiple slots which results in long delay for the DL or UL transmission. Due to the long BWP switching time, inter-BWP switching within a slot may not be possible. Therefore, inter-BWP hopping may be only used together with PDSCH repetition, PUSCH repetition type A, and PUCCH repetitions. Since multiple hopped BWP can be configured with same transmission parameter except the frequency location, it may be possible that BWP switching time can be reduced. Specifically, for RBW-B, the BWP switching time may be reduced if the BWP hopping is performed within the 20MHz of the RF of the UE. In our view, this is RAN4 expertise and input from RAN4 is needed.

For PDCCH, the link level performance degradation is not only caused by loss of frequency diversity but also caused by smaller CCE aggregation level which can be supported by 5MHz BW. For example, up to AL=16 is currently supported in NR systems but currently supported CORESET configuration for 5MHz BW and 3 OFDM symbols only has 12 CCEs. To support same or even larger AL, e.g., even 32 may be required to compensate the loss of frequency diversity gain, options to increase the number of CCEs for a PDCCH candidate can be considered.

- Option 1: the maximum number of OFDM symbols of a CORESET may be increased to e.g., 6, which provides 24 CCEs in the CORESET

- Option 2: two or more CORESETs can be bundled to provide a PDCCH candidate with larger AL.

Observation 2: For RBW-A/B

- The link-level performance for all DL/UL channels may be impacted due to loss of frequency diversity gain.

- The impact of BWP switching delay needs to be studied if inter-BWP switching is to be employed to exploit the benefit of frequency diversity.

- It only has maximum 12 CCEs in a CORESET with 5MHz BW

Proposal 1: if RBW-A or RBW-B is supported

- Inter-BWP hopping may be considered to improve link level performance for PDSCH, PUSCH and PUCCH with repetitions.

- RAN4 input on the length of inter-BWP switching time for F-RedCap UE is needed.

Proposal 2: if RBW-A or RBW-B is supported

- Study potential solutions to increase the number of CCEs that can be used for PDCCH transmission.

  - Increase the maximum number of symbols of CORESET

  - CORSET bundling/repetition

For RBW-C, a first clarification is the DL or UL BWP can have a bandwidth of up to 20MHz, though the data transmission is limited to 25 PRBs. In this case, any kind of frequency hopping with the 20MHz of the DL/UL BWP doesn’t cause any increase of transition time compared to legacy UE. There is no limitation on PDCCH transmission compared with Rel-17 RedCap UE. It is expected that no enhancement on PDCCH is needed. For PUCCH, since the UE of RBW-C would be able to hop with the UL BWP of up to 20MHz with same transition time as Rel-17 RedCap UE, no enhancement to PUCCH is necessary. For PDSCH or PUCCH, if distributed resource allocation for RBW-C, maximal frequency diversity gain with DL or UL BWP is achievable. On the other hand, if the resource allocation has to be limited with localized 25 PRBs, intra-slot and inter-slot frequency hopping can be considered for PDSCH or PUSCH transmission within a BWP of up to 20MHz. Since it is not expected that distributed resource allocation leads to any increase on the complexity compared to localized resource allocation, a UE of RBW-C may be able to support both localized and distributed PDSCH/PUSCH transmission within the DL or UL BWP of up to 20MHz BW. It is then up to gNB to configure a proper frequency resource allocation type for the UE.

Proposal 3: for RBW-C operation

- A DL or UL BWP can have a bandwidth of up to 20MHz

- A UE can support both localized and distributed PDSCH/PUSCH transmission within the UL BWP

Based on the above analysis, RBW-A/B has the benefit of larger complexity reduction compared to RBW-C. On the other hand, RBW-C provides flexible resource allocation with limited specification impact. The above aspects should be considered in the down-selection of an option for F-RedCap UE. Further, as discussed in section 4, another candidate design for a F-RedCap UE is to limit the peak data rate. Such an option should also be considered in the down-selection.

Observation 3:

- RBW-A/B has the benefit of larger complexity reduction compared to RBW-C. On the other hand, RBW-C provides flexible resource allocation with limited specification impact.

Proposal 4: Down-selection of a design for F-RedCap UE among the options of RBW-A/B/C and limiting peak data rate.

# Peak data rate reduction

Except for the BW limitation discussed in section 3, another option for complexity reduction is to limit the peak data rate for F-RedCap UE. The limited peak data rate can be transformed into a limitation of the maximum TBS. In this option, the UE can operate in the full DL/UL BWP of up to 20MHz.

As specified in TS 38.306, for NR, approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

data rate (in Mbps)=10-6⋅j=1JvLayers(j)⋅Qm(j)⋅f(j)⋅Rmax⋅NPRBBW(j),μ⋅12Tsμ1-OH(j)      (1)

Further, based on TS 38.214, the maximum number of bits that can be transmitted in a slot can be derived based on the data rate calculated by the above formula. Within a cell group, a UE is not required to handle PDSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate	                                                      (2)

In the above two formulas, J=1 since a RedCap UE has only one carrier. With limited peak data rate, the LDPC decoding and HARQ buffer can be relaxed. To limit the peak data rate for a F-RedCap UE, the following options can be considered

- Option 1: One or more parameters in formula (1) can be limited to a lower value. For example, vLayers(j)⋅Qm(j)⋅f(j) is limited to no less than 4 in current specification. If the number of MIMO layers is limited to 1 and the modulation order is limited to 6 or even 4 (e.g., for UL), with a scaling factor less than 1, it is much likely the product of vLayers(j)⋅Qm(j)⋅f(j) will be smaller than 4.

- Option 2: The maximum TBS may be limited directly. With formula (2), the peak data rate calculated by formula (1) and the maximum TBS can be transformed each other.

- Option 3: There is another formula in TS 38.214 which is related to the maximum TBS in a slot. The UE is not expected to handle any transport blocks (TBs) in a 14 consecutive-symbol duration for normal CP (or 12 for extended CP) ending at the last symbol of the latest PDSCH transmission within an active BWP on a serving cell whenever

2max⁡(0,μ-μ').i∈SCi'Lixi.Fi>X4.1RLBRM.TBSLBRM                                            (3)

Based on formula (3), the maximum TBS in a slot can be limited by the limitation on TBSLBRM.

Proposal 5: Consider the limitation on the peak data rate together with options RBW-A/B/C for BW reduction.

With the above limitation on peak data rate or maximum TBS, it may not result in any limitation on the received REs in a slot if the lowest MCS is used. In such a case, it is expected that the complexity of post-FFT data buffering and receiving processing block, e.g., channel estimation cannot be reduced. On the other hand, limiting the total number of REs for PDSCH/PUSCH transmission in a slot can be considered for further complexity reduction. For example, the total number of REs may correspond to 25PRBs in 14 OFDM symbols or be even lower. Based on Table 1, it is possible to pose a stricter restriction on the total number of REs while still meeting the targeting peak data rate of 10Mbps. However, if the TBS is fixed, a lower total number of allocated REs impacts link performance.

Observation 4:

- Only limiting peak data rate or maximum TBS may result in no complexity reduction for post-FFT data buffering and receiving processing block.

Proposal 6: The total number of REs allocated for PDSCH or PUSCH transmission can be limited, with impact on the targeting peak data rate required by the SID.

# Relaxed UE processing time

Various processing times were defined in NR for PDSCH, PUSCH, CSI measurement/reporting, etc. For example, comparing for 15 kHz SCS, the UE minimum processing times based on PDSCH processing Capability 1 are at least six times more aggressive than their LTE counterparts at 4 ms. Therefore, for F-RedCap UE, it can be considered to relax the processing time for complexity reduction. A new set of values that are two times of the existing value for various processing time can be considered as a starting point.

On the other hand, if UE processing time is relaxed, the network has to perform conservative scheduling assuming a F-RedCap UE until gNB knows the type of a UE if blind detection is to be avoided. This motivates the early identification of F-RedCap UE.

Similarly, for A-CSI report, adjustments to CSI processing timelines are warranted. The “fast CSI feedback” requirements are neither necessary nor desirable for use cases targeted for RedCap UEs. This should include simplifications (reductions) to the number of CSI processes (also referred to as CSI processing units (CPUs), the number of ports in a CSI-RS resource, etc. The relaxations to processing times for CSI processing should consider the minimum gaps between A-CSI trigger in the DCI format and the CSI-RS resource, as well as from the CSI-RS to the PUSCH start. Further, such relaxations would be in-line with relaxed N1/N2 values. Otherwise, even with relaxed N1/N2 values, the UE will still have to process MIMO related processing, PDCCH processing, etc. without any relaxations compared to Rel-15 requirements.

Proposal 7: It is preferred to relax the UE processing time for PDSCH and/or PUSCH and/or CSI.

# Reduced maximum number of HARQ processes

NR supports up to 16 or 32 (optional) HARQ processes for PDSCH or PUSCH transmission. For a F-RedCap UE, considering repetitions may be considered to compensate the loss of limited number of PRBs or REs for the PDSCH or PUSCH transmission in a slot, the total number of necessary HARQ processes may be reduced. In this case, is expected that the HARQ buffer at UE side can be reduced though exact amount of HARQ buffer is not specified in NR. However, the impact to other functions should also be considered

Proposal 8: Study whether it is beneficial to reduce the maximum number of HARQ processes.

# Relaxed modulation order

The relaxed modulation order can reduce the complexity of the power amplifier, the transceiver and the DL or UL processing block. Further since a lower modulation order effectively reduce the maximum TBS, the complexity of LDPC decoding and HARQ buffering for DL can be reduced.

Based on the analysis, it could be considered to limit DL modulation to 64QAM and UL modulation to 16QAM. The specification impact from relaxed maximum modulation orders for RedCap UEs is small. For UL, if the modulation is limited to 16QAM, the MCS table of 64QAM may be still applied. Denote the modulation order of a MCS in the MCS table as Qm, Qm = 1, 2, 4, 6, the F-RedCap UE may use a modulation order min(Qm, 4).

Proposal 9: Consider limiting DL modulation to 64QAM and UL modulation to 16QAM.

# Half-Duplex FDD Type B

Half duplex operation is one way to reduce the cost or complexity of FDD UE, since the duplexer is not needed at all. Half duplex operation is already supported in NR Rel-15 with limited specification. In brief, the conflicting DL reception and UL transmission, i.e. without sufficient switching time is considered as an error case. A RedCap UE, due to the further reduced complexity, may end up with more restrictions on the half duplex operation. A gap amounting to exactly the Rel-15 switching time may be applied between a DL reception and a UL transmission at UE side, or it could be relaxed compared to Rel-15.

To further reduce the complexity (e.g., use a single LO), it is expected that a longer period for DL-to-UL switching and UL-to-DL switching may be used. For Type B HD-FDD, a slot or several symbols may need to be skipped before or after a DL reception or UL transmission. It may be up to gNB scheduling/configuration to guarantee that there is no conflict between DL reception or UL transmission at UE side. However, if PDSCH or PUSCH repetitions are considered to compensate the link performance due to the use of small bandwidth, low number of Tx/Rx antennas and other impacts of UE capability reduction, it may not be easy for gNB to always avoid the conflict.

Proposal 10: Half-Duplex FDD Type B can be considered for F-RedCap UE.

# Other specification impacts

In the existing NR system, the gNB may transmit CORESET #0 and SIB1 PDSCH in a BW larger than 5MHz. However, as discussion in section 3, if RBW-A/B is adopted, the CORESET #0 and SIB1 PDSCH need to be limited to 5MHz BW, which requires either separate resource for PDCCH and PDSCH for SIB1 for F-RedCap UE and legacy UEs or restriction of the PDCCH/PDSCH within 5MHz decodable by both F-RedCap UE and legacy UEs. Limiting PDCCH/PDSCH transmission bandwidth also for legacy UE degrades the performance of the legacy UEs, e.g., the longer delay for initial access. In case of RBW-C, legacy CORESET #0 can be directly reused because F-RedCap UE can receive DL channel within 20MHz, but potential impact on SIB1 PDSCH transmission depends on whether bandwidth limitation of 5MHz only applies to unicast PDSCH or both unicast and broadcast PDSCH, specially for SIB1. If bandwidth restriction only applies to unicast PDSCH, no special design for the transmission of SIB PDSCH is needed. However, since a SIB PDSCH may be scheduled in up to 20MHz BW and a full slot, it reduces the benefit of complexity reduction on post-FFT data buffer and receiver processing block . According to the analysis in Section 2, relatively large complexity reduction is not demonstrated for any option. In our view, it is slightly preferred to limit the BW of SIB1 PDSCH to 25PRBs as unicast PDSCH.

Proposal 11: Consider limiting the frequency allocation to no more than 25 PRBs for all broadcast/units PDSCH transmission.

Even if F-RedCap UE can receive SIB1 PDSCH without limitation of 5MHz, it is still desirable that gNB can identify the UE type as early as possible before RRC connection setup. For example, if UE type is not known to gNB, gNB has to perform conservative scheduling for RAR, Msg3 and/or Msg4/MsgB with limited PRBs or REs or with relaxed processing time. The support of early indication of F-RedCap UE can address some of the problems. For example, assuming F-RedCap UE can be identified by RACH Msg1/MsgA (dedicated initial UL BWP, dedicated RO or dedicated PRACH), gNB can perform a proper scheduling for RAR, Msg3 and/or Msg4/MsgB based on the identified UE type. If F-RedCap UE can only be identified by Msg3/MsgA, gNB can only perform a proper scheduling for Msg4/MsgB.

Proposal 12: Early identification of F-RedCap UE should be supported.

In the existing NR systems, a UE can receive two broadcast PDSCHs that are overlapped in time, or a broadcast PDSCH and a unicast PDSCH that are overlapped in time. For the low end F-RedCap UE, further discussion is needed on whether such requirement can be relaxed. If a scheduling restriction is not desired, a F-RedCap UE has to handle the PDSCHs with limited PRBs or REs as discussed in section 3 or 4.

Proposal 13: Consider limiting the number of broadcast and/or unicast PDSCHs in a slot, or process the broadcast and/or unicast PDSCHs with limited PRBs or REs supported by F-RedCap UE.

# Conclusions

In this contribution, we presented our views on potential complexity reduction techniques and the corresponding cost estimates for F-RedCap UEs. Based on the presented discussion, our views can be summarized via the following observations and proposals

Observation 1

- With further complexity reduction compared to a simplest Rel-17 RedCap UE,

  - the complexity reduction ranges from 12.4% - 21.7% for RBW-A

  - the complexity reduction ranges from 11.4% - 20.7% for RBW-B

  - the complexity reduction ranges from 10.2% - 19.6% for RBW-C

- The difference of complexity reduction among RBW-A/B/C is relatively small

Observation 2: For RBW-A/B

- The link-level performance for all DL/UL channels may be impacted due to loss of frequency diversity gain.

- The impact of BWP switching delay needs to be studied if inter-BWP switching is to be employed to exploit the benefit of frequency diversity.

- It only has maximum 12 CCEs in a CORESET with 5MHz BW

Observation 3:

- RBW-A/B has the benefit of larger complexity reduction compared to RBW-C. On the other hand, RBW-C provides flexible resource allocation with limited specification impact.

Observation 4:

- Only limiting peak data rate or maximum TBS may result in no complexity reduction for post-FFT data buffering and receiving processing block.

Proposal 1: if RBW-A or RBW-B is supported

- Inter-BWP hopping may be considered to improve link level performance for PDSCH, PUSCH and PUCCH with repetitions.

- RAN4 input on the length of inter-BWP switching time for F-RedCap UE is needed.

Proposal 2: if RBW-A or RBW-B is supported

- Study potential solutions to increase the number of CCEs that can be used for PDCCH transmission.

  - Increase the maximum number of symbols of CORESET

  - CORSET bundling/repetition

Proposal 3: for RBW-C operation

- A DL or UL BWP can have a bandwidth of up to 20MHz

- A UE can support both localized and distributed PDSCH/PUSCH transmission within the UL BWP

Proposal 4: Down-selection of a design for F-RedCap UE among the options of RBW-A/B/C and limiting peak data rate.

Proposal 5: Consider the limitation on the peak data rate together with options RBW-A/B/C for BW reduction.

Proposal 6: The total number of REs allocated for PDSCH or PUSCH transmission can be limited, with impact on the targeting peak data rate required by the SID.

Proposal 7: It is preferred to relax the UE processing time for PDSCH and/or PUSCH and/or CSI.

Proposal 8: Study whether it is beneficial to reduce the maximum number of HARQ processes.

Proposal 9: Consider limiting DL modulation to 64QAM and UL modulation to 16QAM.

Proposal 10: Half-Duplex FDD Type B can be considered for F-RedCap UE.

Proposal 11: Consider limiting the frequency allocation to no more than 25 PRBs for all broadcast/units PDSCH transmission.

Proposal 12: Early identification of F-RedCap UE should be supported.

Proposal 13: Consider limiting the number of broadcast and/or unicast PDSCHs in a slot, or process the broadcast and/or unicast PDSCHs with limited PRBs or REs supported by F-RedCap UE.

# References

- RP-213661, New SID on Study on further NR RedCap UE complexity reduction