3GPP TSG RAN WG1 Meeting #109-e	R1-2204811

e-Meeting, May 9th  – 20th, 2022

Source:	Intel Corporation

Title:	Discussion on Network Energy Saving Evaluations

Agenda item:	9.7.1

Document for:	Discussion

# Introduction

The following was agreed as objectives for the study on network energy savings [1]

- Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep states and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. - Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study. The following example scenarios are listed in no particular order. - Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells) - FR2 beam-based scenarios (note: this scenario can also model small cells) - Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) - EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments. Note 2: the study of energy savings specifically for IAB is not part of the scope. The study should coordinate with RAN4 as needed.

Energy consumption is a major contributor to the network OPEX. Increasing capacity while keeping network energy consumption low is a big challenge for 5G NR based network. Energy consumption model and KPI development are important parts of Rel-18 Network Energy Saving study, which allow us to evaluate different network energy saving techniques. In this contribution, we discuss our views on the objectives 1 and 2, i.e., base station energy consumption model and corresponding evaluation methodology and KPIs, respectively.

# Discussion on evaluation methodology

## BS Models

Evaluation methodology includes developing a suitable energy consumption model, KPIs for network energy saving study. Developing an energy consumption model for network should include not only when base station is actively transmitting or receiving, but also when base station is idle. From power modelling perspective, major components at gNB can be identified as

- Power amplifier,

- Analog front-end/RF,

- Digital baseband,

  - Note that digital baseband can be further broken down into smaller functional blocks, e.g. digital baseband front-end and digital baseband back-end.

- Controller and Network Backhaul,

- Power systems

Note that depending on implementation, different functional splits, such as among blocks comprising of analog front-end and digital base band processors, are possible and it is expected that energy consumption model remains generic enough to support various network architecture. A high-level diagram is provided below in Figure 1, where it is shown that it may be possible that core network (CN) supports traditional Macro-cells and also smaller form factor cells where control, digital, and radio units are collocated, as well as newer base station (BS) architectures where one or more of control, digital and radio units can be distributed over different nodes in a network.

Proposal 1: Energy consumption model should be flexible and generic to represent different potential NW architectures and functional splits at the BS.

Figure 1: An example of coexistence of different network architectures under a CN.

Based on various potential architecture implementations of the network functions, the energy consumption modelling should consider both active operation and absence of traffic, i.e., when BS is not communicating with the UEs and is idle. According to the guideline in SID, the model should be based on the relative power model developed in TR 38.840 and energy consumption value for different states of the BS is obtained relative to energy consumption value of a given state or a state which assumed to have unit energy consumption. When the BS is idle, several sleep modes can be considered to decrease network energy consumption.

Unlike the UE, where majority of the devices under power class 3 are small form factor, typically implemented in either single monolithic chip or few chip components, BS implementation can have much more widely ranging architecture choices. Therefore, we suggest considering at least two different BS categories for the evaluation and analysis of the network energy consumption.

BS category #1 is suggested to represent a general macro BS that may leverage various functional splits (at various levels). Since it can be composed of multiple functional blocks, various levels of sleep modes can be envisioned. BS category #2 is suggested to represent a smaller form factor BS where many of the system components are integrated together. It can have a simpler set of sleep modes that can be leveraged.

## Transmission modes when BS is idle

When the BS is not transmitting, energy consumption can be potentially reduced if BS enters different sleep modes where in each sleep mode, different hardware components of the BS can be turned off. The more functional components are turned off, the deeper the sleep mode can be.  During the time hardware transitions into or out of sleep mode, power is also consumed, and this can be referred to as the “overhead of sleep”. The number and extent of hardware blocks that can be put into sleep is proportional to the “depth of sleep”. It should be noted that in our view, gNB cannot be completely turned off. Network controller and backhaul still needs to be active so that the gNB wake up from sleep state to handle incoming traffic to users and receive user requests.

A macro BS may leverage functional splits at variety of levels, where different functional blocks may not necessarily be collocated. A set of sleep modes can be considered which could allow for different implementations, such as collocated and distributed placements of different functional components of the BS, which allow for flexible switch on/off of a selected set of components, such as at the distributed unit (DU) and/or remote unit (RU) level, without impacting regular NW control operation. Below in Table 1, we provide an example set of 4 sleep modes which can be realized for a such BS category which may offer different flavours of functional splits and grouping of hardware (HW) components.

Table 1:  Different sleep modes considering large scale Macro BS or BS with potentially flexible functional splits among different PHY blocks (a.k.a. BS category #1). It is assumed that controller and backhaul are open and functioning in all the sleep modes

| Sleep Modes (SM) | Characteristics |
| --- | --- |
| SM4: Deep sleep | All analog front-end and digital baseband components are turned off. |
| SM3: Light sleep 1 | Most of the PHY blocks are turned off. At least clock generator is still running. |
| SM2: Light sleep 2 | Many PHY blocks, comprising analog front-end and digital baseband are turned off. |
| SM1: Micro sleep | Some RF components are turned off. |

From Table 1, the light sleep modes 1 and 2 are intended to capture turning off HW components at different levels. We observe that at least for micro-sleep, it is possible that a first group of cell nodes (e.g., TRPs/RRHs/RUs) is in micro-sleep if they are not serving load, whereas a second group of cell nodes can be actively serving load. Hence, unlike UE power consumption model, here status of different cell nodes should be taken into account to obtain aggregate energy consumption of the network.

In addition to the above model, we provide example set of sleep modes for smaller form factor BSs where implementation may be less modular and/or distributed over geographical area. Example set of sleeps modes for the second class of BS is shown in Table 2.

Table 2:  Different sleep modes considering small cell BS (a.k.a. BS category #2). It is assumed that controller and backhaul are open and functioning in all the sleep modes

| Sleep Modes (SM) | Characteristics |
| --- | --- |
| SM2: Deep sleep | All analog front-end and digital baseband components are turned off. |
| SM1: Light sleep | Some RF and PHY components are turned off. |

Because the wide variety of BS implementation possible for networks, we think it is important to at least capture two distinct set of sleep modes and associated power profiles that can represent of at least two different BS categories. Obviously, there could be much more different type of BS implementation that may require slightly different definition of set of sleep modes. However, it would be impractical to define too many BS categories and its associated sleep modes. Therefore, our suggestion is to start with two BS categories and two sets of sleep modes associated with each BS category. RAN1 can discuss further about adding more categories to the evaluation study.

Proposal 2: Assume different BS categories for developing different sleep modes, considering their architectural differences. For evaluation purposes, adopt at least two different BS categories and associated power profiles.

Proposal 3: Adopt up to 4 sleep modes as potential BS states when BS is idle.

Figure 2: BS changing through SMs in order and becoming active directly.

Similar to UE power consumption model in TR 38.840, network energy consumption model should include transition times to enter and leave a given SM. In Figure 2, an example illustration of how BS can change between SMs is shown. If there is a need for an imminent communication, BS should be able to enter active state from any SM. This can be facilitated by controller portion of the BS always being on and monitoring the backhaul at all times.

Observation 1: It is not practical to assume complete turn off of all gNB architectural components. In particular, it is expected that at least network controller and backhaul always remain active to receive traffic and control from the core network.

For the UE power saving study, the energy consumed for each sleep mode was defined. This approach was ok for UE as UE has information about the sleep cycles and active cycles (e.g. PDCCH monitoring occasions) in advanced by gNB configuration. Therefore, it is possible for UE to pre-determine which sleep mode to enter at a given moment. However, for BS no such configurations are provided by the core network, and BS must react to the traffic and user requests much more dynamically. As such, having pre-determined information on which sleep modes can be entered is not typically available and BS should enter various sleep modes in stages.

Observation 2: It is difficult to pre-determine which sleep modes that BS can be entered and sleep modes at BS may need to be entered in stages.

In Table 3, we provide some example values for transition times and minimum sleep length for different SMs. The listed values are tentative values and further discussion in RAN1 would be needed to finalize for evaluation purposes. It should be noted that for SM that require transmission and/or minimum sleep length longer than 160 msec, the ability for BS to enter into such SM would be only possible if the BS does not need to send SSB periodically. Similarly for other SM, such SM can be only leveraged by the BS if the BS does not need to be more frequently active for essential operations such as trying to receive PRACH, send paging for the network, broadcasting system information, etc. Therefore, careful consideration is needed to finalize the transition times and minimum sleep lengths.

Table 3:  Transition periods associated with different sleep modes.

| Sleep Modes | BS Category #1 | SM1 | SM2 | SM3 | SM4 |
| --- | --- | --- | --- | --- | --- |
|  | BS Category #2 |  | SM1 | - | SM2 |
| Total Transition time (activation + deactivation) |  | 01 | [4] msec | [40] msec | [200] msec |
| Minimum Sleep Length |  | 0 | [4] msec | [40] msec | [200] msec |
| NOTE 1: In practice, transition to SM1 may require at least a symbol, e.g, 71.4 µs. However, for evaluation purposes, it is assumed negligible and set to 0. |  |  |  |  |  |

Several additions rules can be adopted for evaluation purposes when BS enters or leaves a SM, some examples are:

- BS may go to active state from any SM directly

- BS can enter a SM only if the time before the next (expected) transmission is greater than or equal to the sum of the transition (deactivation + activation) time and minimum sleep length for that SM

- The deactivation transition time is assumed to be equal to the activation transition time

The power during deactivation/activation transition to a SM or active state is equal to the average power consumed of the two states.

Proposal 4: Adopt the following rules for sleep mode operations

- BS may go to active state from any SM directly

- BS can enter a SM only if the time before the next (expected) transmission is greater than or equal to the sum of the transition (deactivation + activation) time and minimum sleep length for that SM

- The deactivation transition time is assumed to be equal to the activation transition time

The power during deactivation/activation transition to a SM or active state is equal to the average power consumed of the two states.

## Transmission modes when BS is communicating

In our view, we can divide processing blocks at BS for the DL and UL channels as follows and categorize for power consumption purposes. For example, it is expected that same processing block is used for transmitting PDCCH and PDSCH, and distinguishing them may not result in any meaningful power difference.

- DL

  - Coded transmission: PDCCH, PDSCH

  - Signal transmission: SSB, CSI-RS

    - Although SSB includes PBCH, for simplicity it is assumed that processing power of narrowband PBCH is on par with signal

- UL

  - Decoding: PUCCH (except PF0), PUSCH

  - Detection: SRS, PUCCH PF0, PRACH

## Energy consumption model

Next, we discuss energy consumption model based on relative values for BS category #1 and #2. An example of the relative power values for different sleep states and different active states are shown in Table 4 and 5 for BS category #1 and #2, respectively.

Table 4: Power consumption model with slot averaged values for BS category #1

| States/Modes and description | Relative Power Value |
| --- | --- |
| SM4 | [1] |
| SM3(1) | [10] |
| SM2(1) | [20] |
| SM1 | [40] |
| DL Sequence transmission only (e.g. SSB, CSI-RS, TRS) | [80] |
| DL coded transmission and other signals/channels (e.g. PDCCH(2), PDSCH) | [180] |
| UL Sequence detection/reception (e.g. PRACH, SRS, Short PUCCH PF 0) | [80] |
| UL Coded Transmission decoding (e.g. PUCCH, PUSCH) | [140] |
| NOTE 1: SM2 and SM3 distinction depends on CU/DU/RU functional splits at the base station NOTE 2: PDCCH implies there can be one or more PDCCH transmissions in different symbols of a slot. Few symbols of PDCCH in a slot and 1slot PDSCH may not result in significant difference in power consumption in a slot |  |

Table 5: Power consumption model with slot averaged values for BS category #2

| States/Modes and description | Relative Power Value |
| --- | --- |
| SM2 | [1] |
| SM1 | [30] |
| DL sequence transmission only (e.g. SSB, CSI-RS, TRS) | [80] |
| DL coded transmission and other signals/channels (e.g. PDCCH(1), PDSCH) | [180] |
| UL sequence detection/reception (e.g. PRACH, SRS, Short PUCCH PF 0) | [80] |
| UL coded transmission decoding (e.g. PUCCH, PUSCH) | [140] |
| NOTE 1: PDCCH implies there can be one or more PDCCH transmission in different symbols of a slot. Few symbols of PDCCH in a slot and 1slot PDSCH may not result in significant difference in power consumption in a slot |  |

Next, we provide estimate of energy consumed per transition while entering and leaving a SM. Based on observation 2, we proposed to define a simple formula to estimate the energy consumed during transition of various sleep/active states of the BS. Following the rules of operation when the BS is one of the SMs, the energy consumed during transition can be obtained as follows:

- Energy consumed during transition = (Power of Previous State + Power of Next State)/2 × Transition Time.

Some examples are provided below:

- If the BS transitions from SM1 to SM2, each consuming 40 and 20 units of power per slot, respectively, and transition time is 4 slots, the energy consumed during this transition is given by =  (40 + 20)*4/2 = 120.

- If the BS transitions from SM3 to active state SSB transmission, each consuming 10 and 80 units of power per slot, respectively, and transition time is 40 slots, the energy consumed during this transition is given by = (10 + 80)*40/2 = 1800

Proposal 5: Adopt the above energy consumption model in Tables 4, and 5.

- The values in brackets [] are tentative values, to be confirmed or revised in future RAN1 meetings.

Proposal 6: Adopt the following formula for energy consumption during SM transitions

- Energy consumed during transition = (Power of Previous State + Power of Next State)/2 × Transition Time

## Reference Configuration

Similar to UE power saving studies. We need to define reference configuration for evaluation purposes. We think similar configurations as studied during UE power saving could be a good starting point. Table 7 and Table 8 shows the proposed reference configuration for FR1 and FR2, respectively. Note that the BS antenna configuration values are taken from TR38.820.

Table 7: Reference configuration for FR1

| Parameter | Value |
| --- | --- |
| Duplex | TDD (DDDSU, DDDUU) |
| System (carrier) BW | 100 MHz |
| Number of Carrier | 1 |
| Number of TRP | 1 |
| Numerology | 30 kHz |
| Total transmit power per TRP | 47 dBm (Macro-cell), 24 dBm (Small Cell) |
| Carrier Frequency | 4GHz |
| BS Antenna Configuration | 64 TxRu, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) (dH, dV) = (0.5λ, 0.8λ) |
| NOTE 1: Power values are averaged over the operations within a slot. NOTE 2: Different and/or additional assumption parameters used for evaluations should be clearly stated. |  |

Table 8: Reference configuration for FR2

| Parameter | Value |
| --- | --- |
| Duplex | TDD |
| System (carrier) BW | 400 MHz |
| Number of Carrier | 1 |
| Number of TRP | 1 |
| Numerology | 120 kHz |
| Total transmit power per TRP | 40 dBm (Macro-cell), and EIRP should not exceed 73 dBm 23 dBm (Small Cell) and EIRP should not exceed 58 dBm |
| Carrier Frequency | 28 GHz |
| BS Antenna Configuration | 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) (dH, dV) = (0.5λ, 0.8λ) (dg,H, dg,V) = (4.0λ, 3.6λ) |
| NOTE 1: Power values are averaged over the operations within a slot. NOTE 2: Different and/or additional assumption parameters used for evaluations should be clearly stated. |  |

Proposal 7: Adopt the BS reference configurations in Tables 7 and 8.

- Parameters related to UE processing can follow reference configuration in TR 38.840 for UE power consumption evaluation, if needed.

## Scaling of energy consumption model

The energy consumption model presented in Table 4 and 5 is based on reference configuration. Following similar approaches as adopted in TR 38.840 to obtain power values for other configuration based on scaling, we suggest the following scaling relationship.

Table 9: Power scaling model

| Scaling Cases | Scaling for FR1 | Notes |
| --- | --- | --- |
| BW(DL) | Scaling of X MHz = [0.6] + [0.4]· X/100 Valid values for X = 5, 10, 20, 40, 80, 100 MHz | It is assumed that PA efficiency is same when transmission power is scaled with BW. FFS: how to factor PA efficiency and further updates to the scaling equation. |
| BW (UL) | None |  |
| CA (DL) | 2 CC = [1.3]* 1CC 4 CC = [2.6]* 1CC | FFS: whether to have separate scaling for different BS categories |
| CA (UL) | 2 CC = [1.3]* 1CC 4 CC = [2.6]* 1CC | FFS: whether to have separate scaling for different BS categories |
| Antenna Scaling (DL) | 32 TxRU = [0.7] * 64TxRU 16 TxRU = [0.5]* 64TxRU |  |
| Antenna Scaling (UL) | None |  |
| Short PUCCH only | [0.3] × Power consumed by UL Sequence detection/reception state |  |
| SM1 for N TRP, CCs (with different RF) | N × SM1 power | RF at each TRP maybe independently switched on/off. Similarly, for inter-band CA, CCs can be independently switched on/off |

For BS, it may need to transmit and receive multiple signals/channels for multiple users at the same time. Additionally, TDD slots can be further partitioned between DL and UL such that both DL and UL signal/channels exist in the same slot. Therefore, for simplicity of the evaluations, we additionally assume the following for power scaling relationship:

- The slot-averaged power for "PDCCH+PDSCH+PUCCH" is same as "PDCCH, PDSCH"

- The slot-averaged power for "PDSCH+PUCCH" or "PDCCH+PUCCH" is same as "PDCCH, PDSCH"

- The slot-averaged power for "PDSCH/PDCCH+SSB" is [0.85] * ([140] + [80])

Proposal 8:

- Adopt the above scaling model in Table 9.

- Adopt the following additional scaling rules:

  - The slot-averaged power for "PDCCH+PDSCH+PUCCH" is same as "PDCCH, PDSCH"

  - The slot-averaged power for "PDSCH+PUCCH" or "PDCCH+PUCCH" is same as "PDCCH, PDSCH"

  - The slot-averaged power for "PDSCH/PDCCH+SSB" is [0.85] * ([140] + [80])

One important element of BS energy consumption during active state is PA efficiency. For wide area BSs, the PA and the antenna system is one of the biggest components of power consumption. The operating power and mode of the PA and the antenna system can significantly impact the active power states. For example, when BS is transmitting in DL, different PA efficiency levels associated with different output transmit power can affect the total power consumption during active states.  However, we recognize that more discussion is needed on whether to model PA efficiency aspects for active states. Some example of further discussion points is whether or not absorb the PA efficiency as part of BW scaling or consider a separate scaling factor for DL active power states.

Another important aspect is consideration of changes of signal process flow/algorithms during active state of the BS. As discussed in our companion contribution [2], BS may consider utilizing different signal process flow/algorithms when transmitting and receiving signals and channels at the cost of some performance degradation. For example, use of different digital pre-distortion (DPD) levels or enabling/disabling of some filtering operations can impact the power consumption of the BS during active state. Such considerations of power consumption changes for active state is needed.

Proposal 9: Further discuss whether to consider an additional scaling parameter for DL active state power values to take into account various aspects of BS implementation such as PA efficiency and changes to signal processing flow and/or algorithms used during active state.

## KPIs, Traffic Models, Evaluation assumptions

To compute energy consumption of the NW and compare energy saving gain of different techniques, we think that average energy consumption per slot can be considered as target KPI, based on the above model. On the other hand, in order to observe impact to the overall system performance and UE’s experience when energy saving techniques are adopted, we think the following KPIs are useful to be reported as part of the study to compare different energy saving techniques with respect to a baseline (e.g., no sleep modes are assumed) or legacy operations.

- System level network energy consumption (new KPI)

- System capacity

- Scheduling delay

- Access delay (if applicable)

  - Access delay includes time required for UE to receive and transmit essential signals/channels, such as SSB, paging, PRACH, when performing initial access, reconfiguration with sync, cell selection/re-selection., etc.

- UPT

- UE power consumption

For the system level network energy consumption KPI, it would be ideal if companies can also provide information on energy consumption of different functionally split network components as well if such modelling and assumptions were made. At the very least some distinction may be needed to differentiate power consumed for core network upkeep operations versus power consumed by the BS equipment.

For system level evaluations, we suggest to use the following traffic models as starting point. Transmission modes can include both no DRX and CDRX configurations. For CDRX configurations, we suggest to use the configurations adopted in TR 38.840 as baseline.

- FTP3

  - 0.5Mbytes packet, 200ms mean IAT

- XR

  - VR 30, 45 Mbps, at 60fps, PDB 10ms

As per the guidelines, we think the evaluations should consider different loading scenarios, such as follows

- Zero load: Only always-ON transmissions are made, e.g., SSB, SI

  - RU < 5%

- Light load: 5%< RU < 25%

- Medium load: 25%< RU < 50%

Regarding UE dropping, we think non-uniform UE dropping can be considered in addition to uniform UE dropping to observe switching on/off some of the cell nodes, whenever possible. Deployment scenarios can be chosen so that network energy saving can be potentially realized by turning off one or more cell nodes. For this purpose, 2 layer HetNet Macro + Micro layout can be considered.

Proposal 10: Consider the following KPIs for evaluation

- Adopt mean energy consumption per slot as KPI to evaluate network energy consumption.

  - FFS: relative network energy consumption can be broken for evaluations down for different network components (e.g. core network, base station equipment, antenna systems, etc)

- System capacity, scheduling delay, UPT, UE power consumption

- FFS: KPI for Access delay

Proposal 11: Consider FTP3 and XR traffic models as starting point for evaluations.

Proposal 12: Include CDRX as part of the baseline configurations. CDRX configurations adopted in TR 38.840 can be used as starting point.

Proposal 13: Consider at least zero, light and medium loading scenario as part of the evaluations, where the loading criteria are defined as follows

- Zero load: Only always-ON transmissions are made, e.g., SSB, SI

  - RU < 5%

- Light load: 5%< RU < 25%

- Medium load: 25%< RU < 50%

Proposal 14: Consider deployment scenarios for study where potentially one or more cell nodes can be switched off. For example, 2 layer DU deployment scenario in TR 38.913 can be considered.

# Conclusions

In this contribution, we discussed issues related to evaluation methodology for network energy saving SI. The following is a summary of the proposals and observations:

Observation 1: It is not practical to assume complete turn off of all gNB architectural components. In particular, it is expected that at least network controller and backhaul always remain active to receive traffic and control from the core network.

Observation 2: It is difficult to pre-determine which sleep modes that BS can be entered and sleep modes at BS may need to be entered in stages.

Proposal 1: Energy consumption model should be flexible and generic to represent different potential NW architectures and functional splits at the BS.

Proposal 2: Assume different BS categories for developing different sleep modes, considering their architectural differences. For evaluation purposes, adopt at least two different BS categories and associated power profiles.

Proposal 3: Adopt up to 4 sleep modes as potential BS states when BS is idle.

Proposal 4: Adopt the following rules for sleep mode operations

- BS may go to active state from any SM directly

- BS can enter a SM only if the time before the next (expected) transmission is greater than or equal to the sum of the transition (deactivation + activation) time and minimum sleep length for that SM

- The deactivation transition time is assumed to be equal to the activation transition time

The power during deactivation/activation transition to a SM or active state is equal to the average power consumed of the two states.

Proposal 5: Adopt the above energy consumption model in Tables 4, and 5.

- The values in brackets [] are tentative values, to be confirmed or revised in future RAN1 meetings.

Proposal 6: Adopt the following formula for energy consumption during SM transitions

- Energy consumed during transition = (Power of Previous State + Power of Next State)/2 × Transition Time

Proposal 7: Adopt the BS reference configurations in Tables 7 and 8.

- Parameters related to UE processing can follow reference configuration in TR 38.840 for UE power consumption evaluation, if needed.

Proposal 8:

- Adopt the above scaling model in Table 9.

- Adopt the following additional scaling rules:

  - The slot-averaged power for "PDCCH+PDSCH+PUCCH" is same as "PDCCH, PDSCH"

  - The slot-averaged power for "PDSCH+PUCCH" or "PDCCH+PUCCH" is same as "PDCCH, PDSCH"

  - The slot-averaged power for "PDSCH/PDCCH+SSB" is [0.85] * ([140] + [80])

Proposal 9: Further discuss whether to consider an additional scaling parameter for DL active state power values to take into account various aspects of BS implementation such as PA efficiency and changes to signal processing flow and/or algorithms used during active state.

Proposal 10: Consider the following KPIs for evaluation

- Adopt mean energy consumption per slot as KPI to evaluate network energy consumption.

  - FFS: relative network energy consumption can be broken for evaluations down for different network components (e.g. core network, base station equipment, antenna systems, etc)

- System capacity, scheduling delay, UPT, UE power consumption

- FFS: KPI for Access delay

Proposal 11: Consider FTP3 and XR traffic models as starting point for evaluations.

Proposal 12: Include CDRX as part of the baseline configurations. CDRX configurations adopted in TR 38.840 can be used as starting point.

Proposal 13: Consider at least zero, light and medium loading scenario as part of the evaluations, where the loading criteria are defined as follows

- Zero load: Only always-ON transmissions are made, e.g., SSB, SI

  - RU < 5%

- Light load: 5%< RU < 25%

- Medium load: 25%< RU < 50%

Proposal 14: Consider deployment scenarios for study where potentially one or more cell nodes can be switched off. For example, 2 layer DU deployment scenario in TR 38.913 can be considered.

# References

- RP-213554, “New SI: Study on network energy savings for NR,” Huawei, RANP #93-e

- R1-2204812, “Discussion on Network Energy Saving Techniques,” Intel, RAN1 #109-e