3GPP TSG RAN WG1 #109-e																		    R1-2204813

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	Discussion on Side control information to enable NR network-controlled repeater

Agenda item:	9.8.1

Document for:	Discussion and Decision

# Introduction

In study on NR network-controlled repeaters (NCR), the following objectives have been identified [1]:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In the contribution, we present our views on necessity for side control information for NCR.

# Discussion

For NR coverage extension, a NCR with enhanced capability of receiving and processing side control information from the network on top of conventional RF repeater is to be studied in Rel-18. To process side control information, base band processing and control function is required. To differentiate from non-generative amplify-and-forward unit, we use MT to denote the mobile termination which is act as side control information processing unit and RU to denote simple amplify-and-forward radio unit.

Figure 1 Basic structure for network controlled repeater

The improvement of coverage by NCR depends on effectiveness of side control information. In the following, the discussion for beamforming, TDD UL/DL configuration, Timing and on/off information are provided.

## Side control information for beamforming

In Rel-17, RAN4 studied RF repeater, which simply amplifies and forwards whatever the repeater receives. Due to lack of beamforming control, RF repeater either receives and transmits in omni-directional manner or using a fixed directional beam.

With omni-directional reception and transmission, a repeater is more vulnerable to interference from other gNB/repeater or even UEs, and the repeater is also more likely to cause interference to other nodes or UEs. Moreover, Rx power of desirable signal would be much lower without Tx/Rx beamforming gain at repeater side. Consequently, the overall performance improvement would be limited, especially in FR2.

With fixed directional beam, considering almost stationary channel between a repeater and gNB, desirable effective SINR in donor path would be achieved in most cases (but still, beam blocking may happen). However, considering mobility of UE, fixed directional transmission and reception by repeater in access path is unlikely to work properly, which would be even worse than the case with omni-directional operation, if a UE happens to locate in coverage hole of the beam.

To improve coverage for NCR and reduce interference, dynamic beamforming at NCR side for access path needs to be considered. To support flexible beamforming at NCR side, beam sweeping/training as well as beam indication by side control information [2] needs to be studied. RAN1 should consider the overhead for beam sweeping/training and beam indication, e.g., update frequency (how frequent the beamforming information is to be updated) and granularity (symbol or slot-level beam indication), as well as the latency (gNB needs to make scheduling decision in advance and transmits beamforming indication to NCR, NCR decodes the side control information and then applies the indicated beam) to achieve a reasonable trade-off between overall performance improvement and complexity at NCR side.

Observation 1: Dynamic beamforming at NCR side for access path (between NCR and UE) can improve coverage and reduce interference, compared with omni-directional or fixed directional beamforming.

Proposal 1: Support beamforming information by side control information.

- RAN1 to study beam sweeping/training and beam indication mechanism taking signaling overhead, latency and complexity into account.

## Side control information for TDD UL/DL information

For NCR operating in TDD band, the NCR should be aware of TDD UL/DL configuration to determine whether to switch to DL chain or UL chain and avoid interference due to asynchronization.

In case of semi-static UL/DL resource assignment, which is typically applied in real deployment, TDD UL/DL configuration rarely changes. With semi-static TDD configuration, gNB may only configure few flexible symbols as gap for DL-to-UL switch, and gNB would not configure different UL/DL for different UEs in cell-specific flexible symbol. It seems sufficient that NCR obtains TDD UL/DL configuration simply by OAM or by receiving cell broadcast information, i.e., cell-specific TDD UL/DL configuration in SIB1. NCR switches to DL/UL chain for DL and UL symbols and NCR could stop forwarding and prepare for switch in flexible symbol. Therefore, there is no motivation to transmit TDD UL/DL information by side control information.

If NCR operators with dynamic TDD, it seems reasonable to support TDD UL/DL information by side control information, because additional information is needed to determine NCR behaviour in cell-specific flexible symbol.

- For some of UEs, gNB may semi-statically configure UL in cell-specific flexible symbol, while for other UEs, gNB may still configure flexible in the cell-specific flexible symbol. Considering the UL/DL configuration is UE specific, and NCR cannot read the UE-specific signaling, gNB needs to provide the union of UL/DL configurations of UEs served by the NCR, by side control information.

- gNB may dynamically determine the UL/DL resource in semi-static flexible symbol, and gNB informs UE of the dynamic UL/DL configuration by transmitting SFI to UEs. NCR may not monitor SFI PDCCHs for UE, thus gNB needs to indicate dynamic UL/DL configuration to NCR by side control information.

- gNB may dynamically determine the UL/DL resource in semi-static flexible symbol by dynamic DL or UL scheduling without transmission of SFI, e.g., gNB can schedule a PDSCH reception in a flexible symbol then UE knows the flexible symbol is changed to a DL symbol. NCR can not monitor UE-specific PDCCH, but NCR needs to know whether to forward DL or forward UL in the flexible symbol, thus gNB needs to indicate dynamic UL/DL configuration to NCR by side control information.

However, a key question to answer is, what is the motivation to support dynamic TDD for NCR. Due to CLI, the real deployment with dynamic TDD is quite limited, e.g., mainly for some isolated environment. However, the coverage for such scenario is not likely to be an issue, thus no need of NCR. Furthermore, if dynamic TDD operation by NCR is to be supported, the coexistence to other networks should also be studied, which is quite challenging with limited time for this SI.

Observation 2: For semi-static TDD, NCR can obtain TDD UL/DL configuration by OAM or by SIB1 from gNB.

Observation 3: The deployment with dynamic TDD typically does not require NCR for coverage improvement.

Proposal 2: Do not support side control information for TDD UL/DL configuration.

## Side control information for timing information

NCR has to be synchronized to the network to receive DL or transmit UL for NCR itself (by NCR MT) and for UEs (by NCR RU). Similar to normal UE, NCR needs to obtain exact symbol timing to align with gNB.  For the communication between NCR MT and gNB, NCR could obtain DL timing based on DL reference signal, e.g., SSB or TRS, and obtain UL timing based on legacy TA command. To determine the DL timing for DL forwarding to UE by NCR RU, the same DL timing obtained by NCR MT can be directly applied. Similarly, for UL timing for UL forwarding to gNB by NCR RU, the same TA of NCR MT is applied, and the timing to receive UL from UE is simply determined by the TA and group delay.

It is noted that, NCR operation is transparent to UE. Existing TA mechanism already works properly to ensure UL signals from UEs served by NCR (e.g., UE1 and UE2 in Figure 2) to arrive at NCR with aligned timing and then ensure same arriving time at gNB from the UEs served by NCR and UEs directly served by gNB (e.g., UE1, UE2 and UE3 in Figure 2) as well as NCR MT. Therefore, there is no need of additional timing information for NCR by side control information.

Figure 2 Aligned UL timing among UEs under repeater and UEs under gNB

Observation 4: Same DL timing/UL TA for NCR MT obtained by legacy way can be applied for DL/UL forwarding by NCR RU.

Proposal 3: Do not support side control information for timing information.

## Side control information for on/off information

In Rel-17, RF repeater is always on once it is powered on, regardless of presence of UEs under the repeater to serve. Apparently, always forwarding noise or interference (e.g., interference from other gNB or repeater/UEs from other cells in same or adjacent carriers) causes extra interference to other nodes. On/off operation for NCR is desirable for interference reduction as well as improved energy efficiency. Furthermore, on/off operation can also help gNB to differentiate UEs to be served by NCR or served by gNB. For example, gNB may transmit multiple reference signals while NCR only forwards a subset of the reference signals by on/off control. UEs relying on NCR may observe very low energy of the signals not forwarded by NCR while UEs in good coverage of gNB may observe strong energy of the signals, then gNB may identify UEs under NCR based on legacy measurement/report framework which is transparent to UE. The identification in turn helps gNB to make better decision to turn on/off NCR for interference reduction and energy saving, and it also helps gNB for proper resource management, e.g., to determine the resource for beam training.

Observation 5: On/off operation is beneficial for interference reduction, power saving and identification of UEs under NCR.

In general, on/off operation can be considered separately for NCR MT and NCR RU part. For NCR MT part, the only purpose is for power saving. There are already many power saving mechanisms in the Toolbox, e.g., DRX, it seems no strong motivation to introduce additional on/off operation for NCR MT. Having said that, unified solution for on/off operation by NCR MT and RU may be desirable. For NCR RU, on/off operation is performed according to on/off indication by side control information. For each forwarding direction (UL/DL), the chain consists of donor path and access path. Whether on/off operation is performed for both paths or separately for each path, e.g., if same RF is shared between NCR MT and NCR RU in donor path, NCR may separately turn off the forwarding chain for donor and access path, can be further studied.

Proposal 4: Support on/off information as side control information.

- RAN1 to study how to perform on/off operation for NCR MT and RU.

# Conclusions

In this contribution, we presented our views on necessity for side control information for network controlled repeater. Further, we summarize the observations and proposals as follows:

Observation 1: Dynamic beamforming at NCR side for access path (between NCR and UE) can improve coverage and reduce interference, compared with omni-directional or fixed directional beamforming.

Observation 2: For semi-static TDD, NCR can obtain TDD UL/DL configuration by OAM or by SIB1 from gNB.

Observation 3: The deployment with dynamic TDD typically does not require NCR for coverage improvement.

Observation 4: Same DL timing/UL TA for NCR MT obtained by legacy way can be applied for DL/UL forwarding by NCR RU.

Observation 5: On/off operation is beneficial for interference reduction, power saving and identification of UEs under NCR.

Proposal 1: Support beamforming information by side control information.

- RAN1 to study beam sweeping/training and beam indication mechanism taking signaling overhead, latency and complexity into account.

Proposal 2: Do not support side control information for TDD UL/DL configuration.

Proposal 3: Do not support side control information for timing information.

Proposal 4: Support on/off information as side control information.

- RAN1 to study how to perform on/off operation for NCR MT and RU.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, December 2021

- R1-2204814, “Discussions on L1/L2 signaling for side control information”, Intel, May 2022.