3GPP TSG RAN WG1 #109-e																		    R1-2204814

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	Discussions on L1/L2 signaling for side control information

Agenda item:	9.8.2

Document for:	Discussion and Decision

# Introduction

In study on NR network-controlled repeaters (NCR), the following objectives have been identified [1]:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In the contribution, we present our views on L1/L2 signaling (including its configuration) to carry the side control information.

# Basic procedure for repeater MT

To improve NR coverage, new types of network nodes have been considered, including Integrated Access and Backhaul (IAB) and RF repeater. RF repeater specified by RAN4 simply works with amplify-and-forward any signal the repeater receives. Since the RF repeater can work without any block for additional signal processing, the cost is much lower than IAB, but the efficiency is also much worse than IAB. In Rel-18, a network controlled repeater (NCR) with enhanced capability of receiving and processing side control information from the network on top of conventional RF repeater is expected to achieve a good trade-off between cost and performance. For NCR, to process side control information, base band processing and control function is required. To differentiate from non-generative amplify-and-forward unit, we use MT to denote the mobile termination which is act as side control information processing unit and RU to denote simple amplify-and-forward radio unit.

Figure 1 Basic structure for network controlled repeater

To enable side control information reception by a NCR, the NCR should first establish connection with gNB. NCR MT needs to synchronize to gNB, acquire system information, exchange NCR specific information to establish connection with gNB, either via gNB broadcast information and configuration or via OAM. After connection setup, configuration information for side control information is provided, and gNB transmits side control information to NCR MT. NCR MT receives information and adjusts parameters for NCR RU according to the information with some processing delay. In addition, NCR MT may also need to transmit UL, e.g., to provide HARQ-ACK feedback for the side control information with proper UL transmission timing.

Considering connection setup procedure is under RAN2’s work scope, we focus on RAN1’s aspects for side control information signaling and configuration in following sections.

# L1/L2 signalling design

## Configuration for L1/L2 signalling

To receive side control information by PDCCH or PDSCH, NCR firstly should identify resources to receive PDCCH or PDSCH. For PDCCH reception, CORESET, Search Space (SS) set and RNTI information are to be provided to NCR. Considering NCR MT only needs to receive configuration signalling and side control information without DL data for NCR MT, one or two SS set would be sufficient. For example, only Type-0 PDCCH CSS set is configured, and NCR MT can receive all PDCCHs in the CSS set, including PDCCH for side control information and PDCCH for other purpose (e.g., for SIB reception, if supported). If PDCCH for side control information would appear frequently, separate SS set for side control information and cell-specific broadcast information is desirable, to avoid blocking to cell-specific PDCCH. Due to relatively stationary channel between gNB and NCR, a small number of candidates with few aggregation levels could be sufficient. Therefore, the complexity of PDCCH monitoring is reduced. For PDSCH reception and PUSCH/PUCCH transmission, parameters for time/frequency resource allocation, MCS, HARQ information and QCL information can be semi-statically provided, similar as CG PUSCH or SPS PDSCH. Alternatively, dynamic scheduling information can be indicated by DCI. Again, the PDSCH/PUSCH/PUCCH is just for purpose of side control information, no data is expected to be generated by NCR, a subset of parameters for PDSCH/PUSCH/PUCCH would be sufficient, e.g., no need of multi-layer parameters.

Observation 1: For side control information reception, existing parameters for PDCCH and PDSCH configuration should be provided to NCR.

- A subset of the parameters would be sufficient for NCR.

- If feedback of side control information is supported, (a subset of) existing parameters of PUSCH/PUCCH is also needed.

In NR system, resources are always configurable, to adapt to channel and traffic variation, coordination among UEs as well as coordination among gNBs. Similar flexibility is desirable for side control information configuration. Side control information is transmitted in the same carrier with channels/signals for other UEs, therefore the flexibility of resource configuration for side control information does not only impact side control information performance but may also impact the operation of other UEs. For example, if PDCCH/PDSCH resource for NCR MT can only be configured in a fixed manner, DL scheduling for other UEs in the same serving cell would always be restricted by the PDCCH/PDSCH resource for the NCR MT. In that sense, adjusting and reconfiguring the resources per demand is more desirable.

Furthermore, flexible configuration for the set of parameters to be indicated by a side control information would be desirable. For example, the number of candidate beams for NCR RU may depend on UE distribution in NCR coverage. In one way, the set of beams for beam indication is always fixed as maximum number supported by NCR RU, and gNB indicates one of these beams by side control information. Alternatively, the set of beams can be flexibly configured or activated and gNB indicates one of the activated beams by side control information. Consequently, the overhead for side control information can be reduced, which in turn improves detection performance and resource efficiency.

Observation 2: Fixed configuration for side control information may not only degrade performance of side control information but also reduce flexibility and resource efficiency for other UEs.

On the other hand, comparing with configuration updated by gNB based on more timely information from the UE, configurations provided by OAM towards NCR after connection establishment may suffers some restriction. However, by receiving configurations via OAM, the operation at the NCR would be simpler, meanwhile the cost could be further reduced by supporting less functionalities at NCR.

Observation 3: Side control information configured by legacy gNB configuration procedure could provide better performance for both NCR and other UEs, while OAM-only based procedure may reduce the cost and complexity of NCR.

Proposal 1: RAN1 to study the configuration for side control information, with consideration of flexibility and complexity for NCR as well as the impact to overall system performance.

## Side control information by L1/L2 signalling

According to the objective for NCR SI, side control information may include at least one of beamforming information, Timing information, TDD UL/DL configuration, on/off information and power control information. The side control information can be L1 and/or L2 signalling, e.g., by PDCCH, or by MAC CE in a PDSCH. The decision of L1 or L2 signalling for side control information should consider the payload, reliability, sensitivity to latency and potential standard effort.

From payload’s perspective, PDCCH would be suitable for side control information with smaller payload around one hundred bits while PDSCH would be desirable for side control information with larger payload. legacy PDCCH can accommodate   tens of bits to hundreds of bits, e.g., 128 bits for DCI format 2_0. For PDSCH, the payload varies from tens of bits to tens of thousands of bits with 20MHz bandwidth (alternatively, the maximum payload of SIB PDSCH can also be a reference, i.e., 2976 bits). For some side control information type, e.g., long-term on/off information, payload range of legacy PDCCH would be sufficient, while for other side control information type, e.g., beamforming information, legacy PDCCH may not be sufficient if the time granularity for beamforming would be per symbol and there are multiple candidate beams for an NCR. Therefore, PDSCH is more suitable to carry such side control information. Furthermore, if multiple side control information for multiple NCRs under same gNB can be supported, PDSCH may also be a good choice, e.g., similar to multiple RARs for multiple UEs carried by the same PDSCH.

From latency’s perspective, side control information by PDCCH works in more timely manner than PDSCH, considering only few symbols processing time is needed to decode PDCCH while several slots would be required to decode MAC CE by PDSCH. For side control information which reflects semi-static decision by gNB, e.g., long-term on/off operation or semi-static TDD UL/DL configuration, MAC CE by PDSCH would meet the latency requirement. But for side control information which reflects dynamic scheduling by gNB, existing processing latency for MAC CE by PDSCH may be undesirable, which degrades scheduling flexibility. L1 signalling by PDSCH, which is similar to UCI-only PUSCH, can be considered to shorten the processing latency.

From reliability’s perspective, typical target BLER for PDCCH is 1% while target BLER for PDSCH is 10%. If the side control information is tolerable to latency, PDSCH with retransmission may still meet the requirement. If the side control information is sensitive to latency, similar BLER as PDCCH for PDSCH is required, which can be achieved if sufficiently low coding rate applies. Moreover, to avoid mis-alignment between gNB and NCR, HARQ-ACK feedback for PDCCH can also be considered, similar to PDCCH for SPS release/Scell dormancy/TCI update, etc.

Regarding the potential standard effort, to support L1 signalling for PDCCH or PDSCH, the bit map for each bit field should be defined. For L2 signalling, enhancement for new MAC CE should be specified, which is to be handled mainly by RAN2.

Proposal 2: RAN1 to study both L1 and L2 signalling for side control information with consideration of payload, latency, reliability and potential standard effort for various side control information types. L1 and L2 signalling includes,

- Side control information by L1 control information in PDCCH

- Side control information by L1 control information in PDSCH

- Side control information by MAC CE in PDSCH

# Conclusions

In this contribution, we presented our views on L1/L2 signaling (including its configuration) to carry the side control information. Further, we summarize the observations and proposals as follows:

Observation 1: For side control information reception, existing parameters for PDCCH and PDSCH configuration should be provided to NCR.

- A subset of the parameters would be sufficient for NCR.

- If feedback of side control information is supported, (a subset of) existing parameters of PUSCH/PUCCH is also needed.

Observation 2: Fixed configuration for side control information may not only degrade performance of side control information but also reduce flexibility and resource efficiency for other UEs.

Observation 3: Side control information configured by legacy gNB configuration procedure could provide better performance for both NCR and other UEs, while OAM-only based procedure may reduce the cost and complexity of NCR.

Proposal 1: RAN1 to study the configuration for side control information, with consideration of flexibility and complexity for NCR as well as the impact to overall system performance.

Proposal 2: RAN1 to study both L1 and L2 signalling for side control information with consideration of payload, latency, reliability and potential standard effort for various side control information types. L1 and L2 signalling includes,

- Side control information by L1 control information in PDCCH

- Side control information by L1 control information in PDSCH

- Side control information by MAC CE in PDSCH

# References

- RP-213700, “New SI: Study on NR Network controlled repeaters”, ZTE, December 2021

- R1-2204813, “Discussion on Side control information to enable NR network-controlled repeater”, Intel, May 2022.