3GPP TSG RAN WG1 #109-e																		    R1-2204816

e-Meeting, May 9th – 20th, 2022

Source: 	Intel Corporation

Title:	Discussions on multi-cell scheduling with a single DCI

Agenda item:	9.10.1

Document for:	Discussion and Decision

# Introduction

In NR multi-carrier enhancements, the following objectives have been identified [1]:

- Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

  - Identify the maximum number of cells that can be scheduled simultaneously

  - Consider both intra-band and inter-band CA operation

  - Consider both FR1 and FR2

  - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In the contribution, we present our views on multi-cell scheduling with a single DCI, with primary focus on the DCI content, HARQ-ACK feedback and beam management for multi-cell scheduling. Our views on multi-carrier UL Tx switching scheme are described in our companion contribution [2].

# General aspects for multi-cell scheduling

## Maximum number of cells for multi-cell scheduling

For multi-cell scheduling, the DCI format size is expected to be much smaller than the total size of N separate DCIs for single-cell scheduling. In addition, CRC overhead can be reduced accordingly. This is more pronounced when relatively number of cells can be scheduled simultaneously via multi-cell scheduling, and various DCI fields can be shared and commonly applied for all different cells.

To investigate the benefit and identify the maximum number of cells for multi-cell scheduling, PDCCH block probability is utilized as a performance metric. Figure 1 and Figure 2 illustrates the PDCCH blocking probability comparison between single-cell and multi-cell scheduling at 700MHz and 4GHz, respectively. In the simulations, it is assumed that all UEs are scheduled with transmissions on N cells (N = 1, 2 and 4). Further, two scenarios are considered for performance evaluation as follows:

- Single-cell scheduling: N DCI formats are allocated to a scheduled UE (N = 1, 2 and 4).

- Multi-cell scheduling: a single DCI is allocated to the UE for N cells.

Note that it is assumed the total number of CCEs in a CORESET is 32. When multiple UEs are scheduled, the AL for the multiple UEs are determined based on the required SINR values for various DCI payload sizes and ALs and the geometry distribution as shown in the Table 1, Table 2 and Figure 8 in Appendix. If the total number of CCEs exceeds 32, the PDCCH of a UE is dropped randomly until the number of occupied CCEs does not exceed 32. Each dropped PDCCH is counted as one blocking event.

From the figures, it can be observed that for multi-cell scheduling at both 700MHz and 4GHz carrier frequencies, PDCCH blocking probability can be reduced significantly compared to single-cell scheduling. In particular, substantial performance gain can be achieved when relatively small payload size is assumed for multi-cell scheduling.

Observation 1

- Based on the required SINR values and geometry distributions obtained by LLS and SLS, respectively, for multi-cell scheduling at both 700MHz and 4GHz carrier frequencies, PDCCH blocking probability can be reduced significantly compared to single-cell scheduling.

Figure 1. PDCCH blocking probability for single-cell and multi-cell scheduling at 700MHz

Figure 2. PDCCH blocking probability for single-cell and multi-cell scheduling at 4GHz

Based on the simulation results, it is evident that performance gain can be observed when relatively large number of cells can be scheduled via multi-cell scheduling and when relatively small DCI payload size can be defined for multi-cell scheduling. However, when a variety of DCI fields are commonly applied for the PDSCH and PUSCH transmission in different cells, this would pose certain restriction on scheduling flexibility and may lead to performance degradation.

More specifically, for multi-cell scheduling, if a common FDRA field is applied for DCI overhead reduction, i.e., same frequency resource is allocated for DL or UL transmission in different cells, performance loss may be expected given the fact that typically channel conditions in two cells, even in case of intra-band scenario, may be largely different. Hence, considering the tradeoff between scheduling flexibility, PDCCH blocking probability and system performance, maximum number of cells for multi-cell scheduling can be 8.

Proposal 1

- Maximum number of cells for multi-cell scheduling can be 8.

## Use cases

In the objectives of NR multi-carrier enhancements WID, it is clarified that both intra-band and inter-band CA operation, and both FR1 and FR2 are considered for multi-cell PDSCH/PUSCH scheduling with a single DCI [1]. To fully exploit the benefit of multi-cell scheduling, joint scheduling of cells in intra-band, inter-band CA scenario and in FR1 and FR2 can be supported, which can help further reduce the PDCCH blocking probability and improve the data throughput.

However, it may not be reasonable to consider a joint scheduling of cells in a mixed operation of licensed and unlicensed bands. In particular, gNB and UE may need to perform LBT before the DL and UL transmission on the shared spectrum, respectively. This may complicate the scheduler implementation as the decision on scheduling DL and UL data transmission may be coupled with channel access mechanism even for the licensed spectrum. To simplify the design and implementation, in our view, joint scheduling of cells in licensed and unlicensed bands is not supported for multi-cell scheduling.

Proposal 2

- Joint scheduling of cells in intra-band, inter-band CA scenario and in FR1 and FR2 for multi-cell scheduling is supported.

- Joint scheduling of cells in licensed and unlicensed bands for multi-cell scheduling is not supported.

# DCI content for multi-cell scheduling

## General design consideration

In Rel-17 NR extension to 71GHz WI, multi-PDSCH and multi-PUSCH scheduling was defined, where a DCI is used to schedule multiple PDSCHs/PUSCHs with different TBs within a cell. For this mechanism, PDSCH and PUSCH transmission in adjacent slots typically experience similar channel condition, which is feasible for DCI payload size reduction. In particular, some DCI fields including frequency domain resource allocation can be commonly applied for the scheduled PDSCHs and PUSCHs.

As mentioned above, for multi-cell scheduling, FDRA field may not be commonly applied for different cells, given that channel conditions for different carriers can be vastly different. Further, considering that different cells may be configured with different channel bandwidths and subcarrier spacings, certain common fields in the DCI may not be commonly applied for all the scheduled PDSCHs or PUSCHs.

When the number of cells for multi-cell scheduling is relatively large, it may be beneficial to group a number of cells into a group. More specifically, some DCI fields including resource allocation in frequency may not be shared between different groups. This may be particularly desirable when considering the case that a DCI is used to jointly schedule multiple PDSCHs/PUSCHs in both FR1 and FR2. Figure 3 illustrates an example of CC grouping for multi-cell scheduling.

In the following sections, DCI fields may be commonly applied for a group of cells unless otherwise stated.

Figure 3. CC Grouping for multi-cell scheduling

Proposal 3

- A number of cells can be grouped for multi-cell scheduling, where some DCI fields may not be shared between different groups.

When designing the DCI content for multi-cell scheduling, certain DCI fields, e.g., MCS, VRB-to-PRB mapping, priority indicator, TPC command for PUSCH/PUCCH transmission, etc, may be commonly applied for all the scheduled PDSCHs and PUSCHs in different cells, which can help in saving DCI payload size. For other DCI fields, separate DCI fields or bitmap may need to be included in the DCI, e.g., RV, NDI, etc. Further, for MIMO related parameters, including SRS request, DMRS sequence initialization, ZP CSI-RS trigger, etc., a common RRC configuration may be configured for the scheduling cell for multi-cell scheduling, and the indicated values are commonly applied for the transmission of PDSCHs and PUSCHs in the scheduled cells.

The detailed designs for DCI fields for multi-cell scheduling are presented in the following sections.

## Carrier and BWP indication

In NR, a carrier indicator and BWP indicator are separately included in the DCI format 1_1/1_2 and 0_1/0_2, where UE can derive a single cell and BWP for PDSCH and PUSCH transmission, respectively. For multi-cell scheduling with a single DCI, existing mechanism on the carrier and BWP indication cannot be directly applied.

To address this issue, one straightforward solution is to configure a table for carrier and BWP indication, where a joint carrier and BWP indicator can be included in the DCI to determine a set of carriers and corresponding BWPs from the configured table for multi-cell scheduling. Further, one row of the configured carrier and BWP table may include a single carrier and BWP, which may enable the support of dynamic switching between single-cell and multi-cell scheduling.

Proposal 4

- For multi-cell scheduling, a joint carrier and BWP indication field is included in the DCI to determine a set of carriers and BWPs from a configured table.

- Dynamic switching between single-cell and multi-cell scheduling is supported.

## TDRA

As defined for multi-PSDCH/PUSCH scheduling, a row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots, by configuring {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH (or PUSCH) in the row of TDRA table, respectively.

To maximize scheduling flexibility, especially when considering different TDD UL/DL configurations and traffic pattern for different cells, similar design principle can be applied for multi-cell scheduling. More specifically, in a row of TDRA table, separate resource allocation in time can be configured for all the configured cells for multi-cell scheduling. This may include separate {SLIV, mapping type, scheduling offset K0 (or K2)} for each PDSCH or PUSCH, or can be determined based on a row index of configured TDRA table of the corresponding cell/BWP.

Further, based on the determined cell and BWP index from the DCI, UE can derive the corresponding time domain resource allocation for each scheduled PDSCH or PUSCH. Figure 4 illustrates an example of TDRA for multi-cell scheduling. In the example, separate SLIV and K0 are allocated for each scheduled PDSCHs.

Figure 4. TDRA for multi-cell scheduling

As the main motivation of multi-cell scheduling is to reduce PDCCH blocking probability and improve the data throughput, it is expected that multi-cell scheduling is primarily targeted for cell center UEs with relatively good channel conditions. Towards this direction, repetition may not be supported if more than one PDSCHs or PUSCHs are scheduled for multi-cell scheduling.

Proposal 5

- For multi-cell scheduling, a row of the TDRA table can configure separate resource allocation in time for all the configured cells.

- Repetition is not supported if more than one PDSCHs or PUSCHs are scheduled for multi-cell scheduling.

## FDRA

In NR, downlink resource allocation schemes (type 0 and type 1) and uplink resource allocation schemes (type 0, type 1, and type 2) are supported. In addition, different resource allocation types may be configured or dynamically indicated for different cells depending on scheduler decision. For multi-cell scheduling, resource allocation type may be configured or dynamically indicated in the DCI and commonly applied for the scheduled PDSCHs/PUSCHs.

In order to reduce DCI payload size and fully exploit the benefit of reduction on PDCCH blocking probability for multi-cell scheduling, it is more appropriate to include a single FDRA field, which can be commonly applied for all the scheduled PDSCHs/PUSCHs. However, given that different BWPs in the scheduled cells may have different bandwidths, FDRA size in the DCI needs to be determined so as to ensure the alignment between gNB and UE.

Following this design principle, a reference cell may need to be assumed to determine the FDRA size in the DCI for multi-cell scheduling. For other BWPs in the scheduled cells that have different bandwidths, truncation or zero padding can be further applied to determine the frequency resource allocation for the PDSCHs/PUSCHs.

Proposal 6

- For multi-cell scheduling, a resource allocation type is configured or dynamically indicated in the DCI, which is commonly applied for the scheduled PDSCHs/PUSCHs.

- A reference cell is defined to determine the FDRA size in the DCI.

## MCS, RV and NDI

For multi-cell scheduling, it is more desirable to support the scheduling of two codeword transmission for PDSCHs, with the motivation of achieving high data rate. From system operation perspective, the feature defined for single-cell scheduling, including the two codeword transmission for PDSCH, should be also supported for multi-cell scheduling.

For MCS, RV, NDI indication, similar mechanism as defined for multi-PSDCH/PUSCH scheduling in Rel-17 NR extension to 71GHz WI can be reused for multi-cell scheduling. In particular, when 1 TB is configured for PDSCH transmission, one MCS field is included in the DCI, which is commonly applied for the scheduled PDSCHs/PUSCHs. Similarly, when 2 TBs are configured and enabled for multi-cell scheduling, two MCS fields are included in the DCI and commonly applied for the 1st and 2nd TB of the scheduled PDSCHs, respectively.

In addition, a RV and NDI bitmap can be defined in the DCI, where each bit in the bitmap can be used to indicate the RV and NDI for each scheduled PDSCH/PUSCH, respectively. In case of multi-cell PDSCH scheduling, when 2 TBs are configured and enabled for multi-cell scheduling, the bitmap includes the RV and NDI indication for the 1st and 2nd TB for each scheduled PDSCH. Further, when a single PDSCH/PUSCH is scheduled in the DCI, 2-bit RV can be used to indicate the RV for the scheduled PDSCH or PUSCH.

Proposal 7

- 2 TBs are supported for multi-cell PDSCH scheduling.

- For multi-cell PDSCH/PUSCH scheduling,

  - MCS is commonly applied for the scheduled PDSCHs (1st and 2nd TB), and PUSCHs, respectively.

  - RV and NDI bitmap is defined, where each bit in the bitmap is used to indicate the RV and NDI for each scheduled PDSCH (1st and 2nd TB) and PUSCH, respectively.

## HARQ process number

For the multi-PDSCH/PUSCH scheduling in a cell as defined in Rel-16/17, indicated HARQ process number is applied for the first valid PDSCH/PUSCH. In addition, HARQ process number is incremented by 1 for subsequent valid PDSCHs/PUSCHs. This mechanism, however, may not be directly applied for multi-cell scheduling, where different cells may have independent management on HARQ process numbers for PDSCH/PUSCH transmissions.

To simplify the operation for multi-cell scheduling, same HARQ process number can be commonly applied for all the scheduled PDSCHs/PUSCHs. In case when 2 TBs are configured and enabled for multi-cell scheduling, two HARQ process number fields are included in the DCI and commonly applied for the 1st and 2nd TB of the scheduled PDSCHs, respectively.

Proposal 8

- HARQ process number is commonly applied for the scheduled PDSCHs (1st and 2nd TB), and PUSCHs, respectively.

# DCI size and BD/CCE budget for multi-cell scheduling

For cross-carrier scheduling in NR, search space sets and associated DCI formats, e.g., DCI format 0_1/1_1 can be configured for each scheduled cell separately. The DCI fields in the DCI format for a scheduled cell can be adjusted to indicate the transmission parameters of the scheduled cell. Accordingly, the PDCCH monitoring capability at UE is defined per scheduled cell including the DCI size budget and maximum number of BDs/CCEs. Specifically, the DCI size budget includes up to 4 DCI sizes in total with up to 3 DCI size for unicast scheduling.

For multi-cell scheduling, as discussed in Section 3, it is a tradeoff between the overhead in a DCI format for multi-cell scheduling and the flexibility for the PDSCH/PUSCH scheduling. The DCI format for multi-cell scheduling may be configured with a single set of parameters of the DCI fields and can be used to schedule all the scheduled cells. On the other hand, multiple sets of parameters of the DCI fields may be configured to provide more flexible scheduling. For example, as shown in Figure 5, each set of parameters of the DCI field may target same or different scheduled cells. Correspondingly, different SS sets can be configured. Additionally, when multi-cell scheduling is configured on the scheduling cell, single-cell scheduling may still be supported at least for a cell that cannot scheduled by the DCI format for multi-cell scheduling.

Figure 5: Multiple configurations of the DCI format for multi-cell scheduling

With multi-cell scheduling, one issue is which cell or cells the size of a DCI format for multi-cell scheduling should be counted to. Such a rule will impact the DCI size budget checking. Specifically, the scheduling cell can be the PCell in a typical case. If the DCI size for multi-cell scheduling is counted to the PCell, it may result in exceeding the limit of (4, 3). In fact, since the total number of PDCCHs would be reduced for the sake of multi-cell scheduling, it is expected that total number of DCI sizes across all cells can be reduced. Therefore, the overall complexity of DCI sizes may not be a limitation factor for UE. Similarly, the maximum BD/CCE handling needs special consideration for DCI format for multi-cell scheduling。

Proposal 9

- Single or multiple configurations for the DCI fields of the DCI format for multi-cell scheduling can be configured on the scheduling cell.

- Single-cell scheduling may still be supported at least for a cell that cannot be scheduled by the DCI format for multi-cell scheduling.

- Special handling on the DCI format for multi-cell scheduling is necessary for the DCI size budget and maximum number of BD/CCEs.

# HARQ-ACK feedback for multi-cell scheduling

## PUCCH resource determination

For HARQ-ACK feedback in response to the PDSCHs in different carriers scheduled by multi-cell scheduling DCI, similar to multi-PDSCH scheduling as defined in NR extension to 71GHz WI, single PUCCH for all PDSCHs scheduled by a multi-cell scheduling DCI would be sufficient, which can also reduce DCI payload for HARQ-ACK feedback, i.e., single PRI, K1 and DAI per DCI.

To determine single PUCCH slot for all PDSCHs, a reference PDSCH should be defined to apply K1 value, if multiple PDSCHs scheduled by a multi-cell scheduling DCI are located in different slots. Similar to multi-PDSCH scheduling as defined in Rel-17, a PDSCH with last ending symbol can be used as reference PDSCH to ensure sufficient preparation time for PUCCH. Figure 6 illustrates an example of reference PDSCH for multi-cell scheduling. In the example, PDSCH #1 is reference PDSCH, thus PUCCH is in the same slot as PDSCH #1 with K1 = 0. Note that a PDCCH schedules PDSCH #0 and PDSCH #1 in different carries in different slots.

Figure 6. Reference PDSCH for K1

Proposal 10

- Single PRI and K1 per DCI are included per DCI.

- HARQ-ACK information corresponding to PDSCHs in one codebook scheduled by the DCI is multiplexed in a single PUCCH in a slot that is determined based on K1, where K1 is the slot offset between the last PDSCH (with the last ending symbol) and the PUCCH.

## Type 1 HARQ-ACK codebook

In Rel-15, Type 1 HARQ-ACK codebook is determined based on K1 set and TDRA for each configured serving cell. In Rel-17, Type 1 HARQ-ACK codebook is enhanced for multi-PDSCH scheduling by extension of K1 set and the slot offset between last PDSCH and other PDSCHs in the same row of TDRA table (e.g., slot offset = K0 for last PDSCH – K0 for other PDSCHs).

For multi-cell scheduling, similar mechanism can be applied with enhancement for slot offset determination, i.e., Type-1 HARQ-ACK codebook is generated according to extended K1 based on K1 for reference PDSCH and slot offset between reference PDSCH and PDSCH in different CCs. Further, if different SCS can be supported for multiple cells, the slot offset between reference PDSCH and other PDSCHs needs to consider transformation between SCS for different PDSCHs.

Proposal 11

- Type-1 HARQ-ACK codebook is generated according to extended K1 based on K1 for reference PDSCH and slot offset between reference PDSCH and PDSCH in different CCs.

  - Further study on how to derive slot offset between reference PDSCH and PDSCH in different CCs.

## Type 2 HARQ-ACK codebook

For type-2 HARQ-ACK codebook, due to different number of PDSCHs schedulable by a single PDCCH, separate sub-codebooks can be considered, similar to multi-PDSCH scheduling in Rel-17. For example, PDCCHs scheduling single PDSCH belong to 1st sub-codebook with 1 or 2 bits for each PDCCH, while PDCCHs scheduling multiple PDSCHs belong to 2nd sub-codebook with N or 2N bits for each PDCCH, where N is maximum number of PDSCHs scheduled by a PDCCH.

For CBG-based reception, RAN1 needs further discussion for the necessity of CBG configuration for a cell if at least one cell within same PUCCH group is configured with multi-cell scheduling. Considering multi-cell scheduling can be applied to cells with lower SCS e.g., in FR1 and is mainly targeted for large volume of traffic, which is also a typical scenario for CBG, further study is needed on the enhancement of type-2 codebook supporting both CBG operation and multi-cell scheduling on different cells.

Furthermore, enhancements to C-DAI should be considered for multi-cell scheduling. Currently, C-DAI denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pairs first in increasing order of the PDSCH reception starting time for a PDCCH MO for a serving cell, and then, in ascending order of serving cell index, and lastly in ascending order of PDCCH monitoring occasion index. For multi-cell scheduling, it is unclear which serving cell index should be used to determine C-DAI order, because there are multiple PDSCHs on multiple CCs scheduled by a single PDCCH. Therefore, a reference PDSCH should be defined, e.g., the reference PDSCH for K1 value determination can be reused for serving cell index determination.

Figure 7 illustrates an example for reference PDSCH for DAI order. In the example, in a PDCCH MO, PDCCH1 schedules PDSCH #0 and PDSCH #1 on CC0 and CC3 respectively, and PDCCH2 schedules PDSCH #2 and PDSCH #3 on CC1 and CC2 respectively. To determine the DAI order, serving cell index for reference PDSCH #1 (CC#3) and serving cell index for reference PDSCH #3 (CC#2) is used, thus smaller DAI value is indicated for PDCCH2 (DAI=0) and larger DAI value is indicated for PDCCH (DAI=1).

Figure 7. Reference PDSCH for serving cell index to determine DAI order

Proposal 12

- Type-2 HARQ-ACK codebook considers at least two sub-codebooks for single PDSCH and multi-cell PDSCH scheduling.

  - Further study the case with CBG transmission

  - Further study reference PDSCH for serving cell index to determine DAI order

# Beam management for multi-cell scheduling

In Rel-15/16, for MIMO transmission in NR, TCI states can be separately configured, activated, and indicated for different DL or UL channels/signals. Specifically, for the PDSCH transmission scheduled by a DCI, applicable TCI state for a PDSCH depends on the delay between the end of the DCI and the start of the PDSCH. If the delay is no less than timeDurationForQCL, the indicated TCI state by the DCI can be used in PDSCH transmission, otherwise, a default TCI state applies. In NR extension to 71GHz WI, it is supported to separately determine the default TCI state or the DCI-indicated TCI state for each of the multiple PDSCHs scheduled by a DCI. On the other hand, a UE can assume the set of active TCI sates that are activated by a MAC CE will not change with the slots carrying the multiple PDSCHs scheduled by the DCI.

For multi-cell scheduling, as discussion Section 3.3, the PDSCHs on the multiple scheduled cells may not be scheduled with the same timing. Therefore, the delay between the end of DCI and the start of a scheduled PDSCH can be different for different scheduled cells. Given that timeDurationForQCL is dependent on SCS of the scheduled cell, the threshold can be different for different scheduled cells. Further, an additional number of PDCCH symbols is added to timeDurationForQCL only for cross-carrier scheduling if the PDCCH has smaller SCS than the scheduled PDSCH.

Based on the above analysis, the different PDSCHs scheduled by a DCI may use the default TCI state or DCI-indicated TCI state, which is similar to multi-PDSCH transmission in NR extension to 71GHz WI. Further, as to the activated TCI states by MAC CE, UE may also expect the activated TCI states are not changed in the span from first PDSCH to the last PDSCH that are scheduled by the same DCI.

In NR Rel-17, a unified TCI framework which could be applicable to all/multiple DL and UL channels/signals was introduced, which should also be considered in the multi-carrier enhancement WI. However, further study may be needed on the behaviour of applying the unified TCI state for multi-cell scheduling.

Proposal 13

- Both Rel-15/16 TCI framework and Rel-17 unified TCI framework are supported for multi-cell scheduling.

- With Rel-15/16 TCI framework, different PDSCHs scheduled by a DCI may use default TCI state or DCI-indicated TCI state depending on the delay between DCI and scheduled PDSCH.

- With Rel-15/16 TCI framework, UE may also expect the activated TCI states are not changed in the span from first PDSCH to last PDSCH that are scheduled by the same DCI.

# Conclusions

In this contribution, we presented our views on multi-cell scheduling with a single DCI, with primary focus on the DCI content, HARQ-ACK feedback and beam management for multi-cell scheduling. Further, we summarize the proposals as follows:

Observation 1

- Based on the required SINR values and geometry distributions obtained by LLS and SLS, respectively, for multi-cell scheduling at both 700MHz and 4GHz carrier frequencies, PDCCH blocking probability can be reduced significantly compared to single-cell scheduling.

Proposal 1

- Maximum number of cells for multi-cell scheduling can be 8.

Proposal 2

- Joint scheduling of cells in intra-band, inter-band CA scenario and in FR1 and FR2 for multi-cell scheduling is supported.

- Joint scheduling of cells in licensed and unlicensed bands for multi-cell scheduling is not supported.

Proposal 3

- A number of cells can be grouped for multi-cell scheduling, where some DCI fields may not be shared between different groups.

Proposal 4

- For multi-cell scheduling, a joint carrier and BWP indication field is included in the DCI to determine a set of carriers and BWPs from a configured table.

- Dynamic switching between single-cell and multi-cell scheduling is supported.

Proposal 5

- For multi-cell scheduling, a row of the TDRA table can configure separate resource allocation in time for all the configured cells.

- Repetition is not supported if more than one PDSCHs or PUSCHs are scheduled for multi-cell scheduling.

Proposal 6

- For multi-cell scheduling, a resource allocation type is configured or dynamically indicated in the DCI, which is commonly applied for the scheduled PDSCHs/PUSCHs.

- A reference cell is defined to determine the FDRA size in the DCI.

Proposal 7

- 2 TBs are supported for multi-cell PDSCH scheduling.

- For multi-cell PDSCH/PUSCH scheduling,

  - MCS is commonly applied for the scheduled PDSCHs (1st and 2nd TB), and PUSCHs, respectively.

  - RV and NDI bitmap is defined, where each bit in the bitmap is used to indicate the RV and NDI for each scheduled PDSCH (1st and 2nd TB) and PUSCH, respectively.

Proposal 8

- HARQ process number is commonly applied for the scheduled PDSCHs (1st and 2nd TB), and PUSCHs, respectively.

Proposal 9

- Single or multiple configurations for the DCI fields of the DCI format for multi-cell scheduling can be configured on the scheduling cell.

- Single-cell scheduling may still be supported at least for a cell that cannot be scheduled by the DCI format for multi-cell scheduling.

- Special handling on the DCI format for multi-cell scheduling is necessary for the DCI size budget and maximum number of BD/CCEs.

Proposal 10

- Single PRI and K1 per DCI are included per DCI.

- HARQ-ACK information corresponding to PDSCHs in one codebook scheduled by the DCI is multiplexed in a single PUCCH in a slot that is determined based on K1, where K1 is the slot offset between the last PDSCH (with the last ending symbol) and the PUCCH.

Proposal 11

- Type-1 HARQ-ACK codebook is generated according to extended K1 based on K1 for reference PDSCH and slot offset between reference PDSCH and PDSCH in different CCs.

  - Further study on how to derive slot offset between reference PDSCH and PDSCH in different CCs.

Proposal 12

- Type-2 HARQ-ACK codebook considers at least two sub-codebooks for single PDSCH and multi-cell PDSCH scheduling.

  - Further study the case with CBG transmission

  - Further study reference PDSCH for serving cell index to determine DAI order

Proposal 13

- Both Rel-15/16 TCI framework and Rel-17 unified TCI framework are supported for multi-cell scheduling.

- With Rel-15/16 TCI framework, different PDSCHs scheduled by a DCI may use default TCI state or DCI-indicated TCI state depending on the delay between DCI and scheduled PDSCH.

- With Rel-15/16 TCI framework, UE may also expect the activated TCI states are not changed in the span from first PDSCH to last PDSCH that are scheduled by the same DCI.

# References

- RP-213577, “New WID on Multi-carrier enhancements”, NTT DOCOMO, December 2021

- R1-2204817, “Discussions on multi-carrier UL Tx switching scheme”, Intel Corporation, May 2022

# Appendix

### Geometry SINR distribution

Figure 8. Geometry SINR distribution at 700MHz and 4GHz

### Required SINR for various DCI payload sizes and Als

Table 1. Required SINR for 1% BLER at 700MHz

|  | Required SINR (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | AL 16 | AL 8 | AL 4 | AL 2 | AL 1 |
| 60 bits | -5.97 | -3.94 | -1.12 | 2.67 | 10.5 |
| 72 bits | -5.66 | -3.26 | -0.42 | 3.63 | 12.9 |
| 96 bits | -4.86 | -2.44 | 0.5 | 5.43 | - |
| 104 bits | -4.7 | -2.16 | 0.98 | 5.83 | - |

Table 2. Required SINR for 1% BLER at 4GHz

|  | Required SINR (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | AL 16 | AL 8 | AL 4 | AL 2 | AL 1 |
| 60 bits | -9.3 | -7.16 | -4.78 | -1.51 | 4.18 |
| 72 bits | -8.84 | -6.72 | -4.29 | -0.75 | 6 |
| 96 bits | -8.17 | -6.03 | -3.34 | 0.72 | - |
| 104 bits | -8 | -5.8 | -3.13 | 1 | - |