3GPP TSG-RAN WG1 Meeting #109 electronic    	             R1-2204820

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.11.3

Source:	III

Title:	Views on XR specific RAN enhancement in QoS

Document for:	Discussion and Decision

Introduction

During the RAN#95e online discussion [1], the following objectives were approved:

Objectives on XR-awareness in RAN (RAN2):

-	Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

-	Study how the above information aids XR-specific traffic handling.

Objectives on XR-specific Power Saving (RAN1, RAN2):

-	Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

-  C-DRX enhancement.

-  PDCCH monitoring enhancement.

Objectives on XR-specific capacity improvements (RAN1, RAN2):

-	Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

-  SPS and CG enhancements;

-  Dynamic scheduling/grant enhancements.

Topics relative to QoS issues was raised as mentioned above. In this paper, we will address some characteristics of QoS that can be helpful to RAN for XR support in this meeting.

Discussion

During previous meetings, Application Data Unit (ADU) are mentioned. XR traffic from the application layer will be processed into several ADUs. Each ADU can consist multiple IP packets for XR traffic, i.e. I-frame/I-slice(s) or P-frame/P-slice(s). Each IP packet will become a Packet Data Unit (PDU) and every PDU Session has its QoS rule, including the requirement of Packet Delay Budget (PDB) and Packet Error Rate (PER). Normally XR traffic is transparent to the 3GPP network and gNB will treat each PDU independently. However, to increase capacity, gNB can drop PDUs belong to the same ADU after any of them have violate the PDB constrain based on the evaluation of XR traffic as mentioned in [2]. According to the objective to the revised SID above, in order to handle the dropping of the PDU properly, gNB should have information from upper layer to know which PDU(s) belong to an ADU.

Meanwhile, a delay aware scheduler is also discussed in [2]. For a delay aware scheduling model, gNB will take the remaining delivery time of a frame/slice into consideration to increase its overall capacity performance. If the remaining delivery time of a frame/slice is limited, PDU(s) of that frame/slice will be considered to be prioritized even though these PDU(s) are in the low priority at the beginning.

Hence, the priority of a PDU must be adjusted dynamically if a delay aware scheduling mechanism is enabled. Consequently, the existing prioritization mechanism won’t provide adequate PDU session treatment to the gNB for XR application. Because the gNB should schedule these PDU(s) according to the remaining delivery time as its priority level(s), especially for time sensitive XR PDU(s), delay budget information relevant to a PDU must be considered by the schedule of the gNB. For example, the correct delay budget information of PDU(s) could be derived from the time stamp of the input XR ADU(s).

Observation 1: How to derive the delay budget information and/or provide the time stamp information of a PDU toward RAN should be studied.

Furthermore, a gNB can manage its radio resource more efficiently and flexibly by scheduling a PDU according to the delay budget and/or time stamp information of the PDU. Issues should be considered for Rel-18 XR scheduling are the following:

- In order to increase its capacity to provide XR services for more users, the gNB can drop PDUs belonging to an ADU failing to be delivered within its PDB requirement as mentioned above.

- When considering the prioritization of a PDU, both the QoS of the PDU and the QoS of the ADU, to which the PDU belong to, should be taken into account. In order to not violate the overall QoS constrain of an ADU, all PDU of the same ADU must be scheduled consistently. In addition, a gNB may somehow need prioritize particular PDUs belonging to an ADU to fulfil the PDB and PER for PDU sessions.

- When a NACK is received, a re-transmission for that PDU is triggered. If there is no enough time for proper CG/SPS resource(s) allocation to fulfill the delay budget of retransmitted and other non-transmitted PDU(s) belonging to the same ADU, a gNB may allocate DG resource(s) for transmission to avoid unnecessary PDU(s) dropping. In this case, the gNB can prioritize the transmission occasion(s) if needs and dynamically re-adjust other already scheduled transmissions accordingly. Another solution to reduce unnecessary PDU(s) dropping is that gNB can burst its throughput temporary, for example either by utilizing multiple component carriers or performing BWP switching or by adjusting its L1 parameters.

Proposal 1: When considering the prioritization of a PDU, both the QoS of the PDU and the QoS of the ADU, to which the PDU belong to, should be taken into account.

Proposal 2: The gNB can prioritize the transmission occasion(s) if needs and dynamically re-adjust other already scheduled transmissions accordingly.

Conclusions

We discuss QoS issues for XR traffic and the related proposals and observations from above discussions are below:

Observation 1: How to derive the delay budget information and/or provide the time stamp information of a PDU toward RAN should be studied.

Proposal 1: When considering the prioritization of a PDU, both the QoS of the PDU and the QoS of the ADU, to which the PDU belong to, should be taken into account.

Proposal 2: The gNB can prioritize the transmission occasion(s) if needs and dynamically re-adjust other already scheduled transmissions accordingly.

# References

[1] RP-220285, Revised SID: Study on XR Enhancements for NR, Nokia.

[2] TR 38.838 V17.0.0