3GPP TSG RAN WG1 #109	R1-2204823

e-Meeting, 9 – 20 May, 2022

Agenda item:		9.9.1

Source:	Nokia, Nokia Shanghai Bell

Title:	DSS – NR PDCCH overlapping with LTE CRS

WI code:	NR_DSS_enh

Document for:		Discussion and Decision

# Introduction

RAN#94 in December 2021 approved a new WI: Enhancements of NR Dynamic spectrum sharing (DSS) [1], with the following objectives:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

In this contribution the 1st objective on NR PDCCH reception in symbols with LTE CRS REs is discussed.

# Evaluation of NR PDCCH in symbols with LTE CRS REs

## Scenarios of interest

Extension of the LTE CRS rate matching DSS pattern operation for overlapping CRS and NR PDCCH sets certain boundaries on the possible deployment scenarios. First and foremost, the SCS for the NR operation should be 15 kHz for the LTE CRS and any NR signal to coexist in the same symbols and PRBs. Furthermore, we should assume that the LTE PCFICH and LTE PDCCH for scheduling needs to occupy the symbol#0 and the NR PDCCH won’t be present there. This means that LTE CRS with 2 CRS antenna ports doesn’t lead to any CRS collisions with the NR PDCCH, and the focus could be on 4 CRS antenna port cases, for the cases where the NR PDCCH collides with the LTE CRS.

|  |  |  | 9 REs/PRB/symbol for PDCCH (red) 3 REs/PRB/symbol for PDCCH DMRS (purple) |
| --- | --- | --- | --- |
| 2-port LTE CRS pattern | 4-port LTE CRS pattern | 1-symbol and 2-symbol NR PDCCH CORESET |  |

Figure 1: Illustration of LTE CRS and NR CORESET

|  | 1-symbol CORESET |  | 2-symbol CORESET |
| --- | --- | --- | --- |
|  | SS start in symbol #1 | SS start in symbol #2 | SS start in symbol #1 |
| 2-port CRS | No collision | No collision | No collision |
| 4-port CRS | Collision case | No collision | Collision case |

Figure 2: 2-port and 4-port LTE CRS overlaid on 1-symbol and 2-symbol NR CORESET

- With 2-port LTE CRS, there is no CRS-to-PDCCH collision regardless of the number of symbols in the CORESET (as long as symbol#0 is not used for NR PDCCH)

- With 4-port LTE CRS with SS start at symbol #1, on the 1st PDCCH symbol 1/9 PDCCH REs and 1/3 DMRS REs collide with LTE CRS

- With 4-port LTE CRS with SS start at symbol #2, there is no CRS-to-PDCCH collision

As the DSS reference case should be seen as the 4-port LTE CRS with 1-symbol NR PDCCH in symbol #2, the case of interest is where the NR PDCCH can be extended also to symbol #1. Thus

- 15 kHz SCS

- 4-port LTE CRS

- LTE PCFICH/PDCCH occupies symbol #0

- 2-symbol CORESET with SS start in symbol #1

Proposal 1: Consider 4-port LTE CRS with 2-symbol CORESET and SS start symbol#1 as the scenarios of interest

## Simulation assumptions and scenarios

| Time and frequency alignment | Full time and frequency alignment between LTE and NR |
| --- | --- |
| LTE CRS | 4-port |
| LTE Carrier BW | 20 MHz |
| Carrier frequency | 2 GHz |
| LTE PDCCH region size | 1 symbol (symbol #0) |
| NR subcarrier spacing | 15 kHz |
| CORESET BW | 96 PRBs |
| CORESET length | 2 symbols |
| Search space location | Start symbol: symbol #1 (2nd symbol of the slot) |
| REG bundle size | 6 |
| REG-to-CCE mapping | Distributed |
| PDCCH Aggregation Level | 1,2,4,8,16 |
| CRS power boosting P(CRS/PDCCH) | 0, -3, +3 dB |
| Channel model | TDL-C-300 ns |
| UE speed | 3/30/120 km/h |
| Interleaving blocks | 2 |
| PDCCH payload | 60 bits + 24-bit CRC |
| Tx diversity | Precoder cycling |

The following cases are simulated

- Reference: No LTE CRS present

- 4-port LTE CRS and 2-symbol CORESET with Search Space start at symbols #1 as in the figure 3 below

  - NR PDCCH and PDCCH DMRS REs overlapping with LTE CRS REs are punctured in the receiver

    - The punctured DMRS REs are not used in channel estimation

    - The punctured PDCCH REs are not used in decoding

  - NR PDCCH and PDCCH DMRS REs overlapping with LTE CRS REs are superpositioned and used in receiver processing

    - The CRS-interfered DMRS REs are used in channel estimation

    - The CRS-interfered PDCCH REs are used in decoding

    - No CRS interference supporession techniques are used.

Figure 3: 4-port CRS and 2-symbol PDCCH CORESET starting at symbol #1 (again)

## Simulation results

Six simulation result pairs are presented in the following tables, recordingthe PDCCH SNR loss relative to a transmission where LTE CRS is not present. On each table

- On the left, with NR PDCCH and DMRS REs overlapping the LTE CRS REs punctured at the receiver, and

- On the right the PDCCH receiver experiences LTE CRS interference where overlaps occur, but all REs are used in the receive processing

- Table 1: Target PDCCH BLER: 1%, P(CRS/PDCCH) = 0 dB

- Table 2: Target PDCCH BLER: 1%, P(CRS/PDCCH) = -3 dB

- Table 3: Target PDCCH BLER: 1%, P(CRS/PDCCH) = +3 dB

- Table 4: Target PDCCH BLER: 10%, P(CRS/PDCCH) = 0 dB

- Table 5: Target PDCCH BLER: 10%, P(CRS/PDCCH) = -3 dB

- Table 6: Target PDCCH BLER: 10%, P(CRS/PDCCH) = +3 dB

Table 1: NR PDCCH SNR loss due LTE CRS. 
2os NR PDCCH, P(CRS/PDCCH) = 0 dB, target PDCCH BLER 1%

|  | PDCCH puncturing |  |  | PDCCH super positioning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 3km/h | 30km/h | 120km/h | 3km/h | 30km/h | 120km/h |
| AL1 | 2.78 | 3.56 | 3.50 | 320 | 103 | 104 |
| AL2 | 2.72 | 2.25 | 2.72 | 5.84 | 3.78 | 4.16 |
| AL4 | 1.55 | 1.5 | 1.48 | 1.09 | 0.84 | 0.81 |
| AL8 | 1.03 | 1.08 | 1.02 | 0.27 | 0.30 | 0.27 |
| AL16 | 0.90 | 0.92 | 0.94 | 0.12 | 0.11 | 0.09 |

Table 2: NR PDCCH SNR loss due to LTE CRS. 
2os NR PDCCH, P(CRS/PDCCH) = -3 dB, target PDCCH BLER 1%

|  | PDCCH puncturing |  |  | PDCCH super positioning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 3km/h | 30km/h | 120km/h | 3km/h | 30km/h | 120km/h |
| AL1 | 2.78 | 3.56 | 3.50 | 65 | 35 | 54 |
| AL2 | 1.81 | 1.75 | 1.97 | 1.91 | 1.34 | 1.47 |
| AL4 | 1.31 | 1.22 | 1.31 | 0.5 | 0.44 | 0.34 |
| AL8 | 0.94 | 1.02 | 0.95 | 0.14 | 0.13 | 0.13 |
| AL16 | 0.85 | 0.85 | 0.88 | 0.07 | 0.06 | 0.06 |

Table 3: NR PDCCH SNR loss due to LTE CRS. 
2os NR PDCCH, P(CRS/PDCCH) = +3 dB, target PDCCH BLER 1%

|  | PDCCH puncturing |  |  | PDCCH super positioning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 3km/h | 30km/h | 120km/h | 3km/h | 30km/h | 120km/h |
| AL1 | 2.78 | 3.56 | 3.56 | NAN | ~460 | ~370 |
| AL2 | 8.66 | 4.34 | 6.72 | 61 | 54 | 50 |
| AL4 | 2.03 | 1.97 | 2.06 | 2.38 | 2.06 | 1.97 |
| AL8 | 1.28 | 1.33 | 1.25 | 0.63 | 0.63 | 0.58 |
| AL16 | 1.01 | 1.02 | 1.05 | 0.29 | 0.27 | 0.28 |

Table 4: NR PDCCH SNR loss due LTE CRS. 
2os NR PDCCH, P(CRS/PDCCH) = 0 dB, target PDCCH BLER 10%

|  | PDCCH puncturing |  |  | PDCCH super positioning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 3km/h | 30km/h | 120km/h | 3km/h | 30km/h | 120km/h |
| AL1 | 2.60 | 2.65 | 2.63 | 61.3 | 15.6 | 15.4 |
| AL2 | 1.59 | 1.62 | 1.67 | 1.27 | 1.07 | 1.09 |
| AL4 | 1.24 | 1.22 | 1.27 | 0.39 | 0.38 | 0.41 |
| AL8 | 1.01 | 0.99 | 1.00 | 0.18 | 0.16 | 0.16 |
| AL16 | 0.92 | 0.91 | 0.89 | 0.09 | 0.08 | 0.08 |

Table 5: NR PDCCH SNR loss due to LTE CRS. 
2os NR PDCCH, P(CRS/PDCCH) = -3 dB, target PDCCH BLER 10%

|  | PDCCH puncturing |  |  | PDCCH super positioning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 3km/h | 30km/h | 120km/h | 3km/h | 30km/h | 120km/h |
| AL1 | 2.60 | 2.65 | 2.63 | 3.43 | 2.33 | 2.25 |
| AL2 | 1.44 | 1.45 | 1.48 | 0.59 | 0.50 | 0.52 |
| AL4 | 1.13 | 1.13 | 1.16 | 0.20 | 0.18 | 0.20 |
| AL8 | 0.96 | 0.94 | 0.94 | 0.08 | 0.08 | 0.08 |
| AL16 | 0.88 | 0.88 | 0.86 | 0.04 | 0.04 | 0.05 |

Table 6: NR PDCCH SNR loss due to LTE CRS. 
2os NR PDCCH, P(CRS/PDCCH) = +3 dB, target PDCCH BLER 10%

|  | PDCCH puncturing |  |  | PDCCH super positioning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 3km/h | 30km/h | 120km/h | 3km/h | 30km/h | 120km/h |
| AL1 | 2.60 | 2.64 | 2.63 | NAN | ~150 | ~120 |
| AL2 | 1.98 | 1.99 | 2.03 | 3.22 | 2.54 | 2.61 |
| AL4 | 1.46 | 1.45 | 1.49 | 0.82 | 0.78 | 0.85 |
| AL8 | 1.12 | 1.11 | 1.12 | 0.35 | 0.34 | 0.34 |
| AL16 | 0.98 | 0.98 | 0.96 | 0.17 | 0.19 | 0.18 |

Observation 1:

- Superpositioning the LTE CRS and NR PDCCH/DMRS works fairly well for higher aggregation levels

  - With 0 dB power difference, AL4/8/16 shows limited loss

  - With -3dB LTE CRS AL2 works as well

  - AL1 cannot be used with superpositioning with PDCCH BLER target of 1%

- Puncturing the LTE CRS REs out of the NR PDCCH/DMRS outperforms superpositioning on low ALs

  - With AL4 and higher, when looking at all the simulated cases, puncturing achieves similar or worse performance than superpositioning

## Proposal for NR PDCCH in symbols with LTE CRS REs

Based on simulation results and the observations made based on them in section 2, the following proposal is made:

Proposal 2: Allow both super positioning LTE CRS on NR PDCCH & PDCCH DMRS, and puncturing LTE CRS REs out of NR PDCCH & PDCCH DMRS

# NR PDCCH in symbols with LTE CRS REs

## Puncturing and super positioning impacts

When the configuration of a search space with CORESET and PDCCH candidates that have REs overlapping with LTE CRS is allowed, the question to answer is what the UE behaviour should be. As discussed in section 2 of this document, in some cases superpositioning is beneficial, while in other cases puncturing of the LTE CRS REs out of the NR PDCCH is beneficial. When one or the other performs better depends on the aggregation level of the PDCCH candidate as well as the relative power difference of the LTE CRS and NR PDCCH.

The gNB should be able to inform the UE when the NR PDCCH and LTE CRS are superpositioned, and when overlapping NR PDCCH REs are punctured. As observed with the results in section 2, it should be possible to provide the puncturing vs. superpositioning information in the search space configuration, either as a property of an individual PDCCH candidate, or related to the aggregation level of the candidates

Proposal 3: The gNB can configure the UE to expect either superpositioned or punctured NR PDCCH REs, when they overlap with LTE CRS REs.

- When the PDCCH and CRS are superpositioned, it is up to the UE whether or not to use the overlapping PDCCH REs in the demodulation and decoding process

- When the PDCCH is puncured, the UE cannot use the overlapping PDCCH REs

Proposal 4: The superpositioning/puncturing configuration is provided per PDCCH candidate or per PDCCH AL

## Superpositioning in Idle-mode and RRC_Inactive

In Idle-mode the UE is not provided with the LTE CRS rate matching pattern, so the UE would not know if LTE CRS is superpositioned with NR PDCCH. Hence the idle-mode superpositioning is up to the network, but the UE would not be use any CRS interference suppression techniques that are based on a-priori knowledge that the CRS is present. It would be beneficial for the UE to be able to know of the superpositioning and the CRS patter in idle mode too.

One possibility would be for the UE to remember the LTE CRS rate matching pattern provided to it in RRC_Connected as long as it stays in the same cell and take it into account with NR PDCCH decoding process.

Observation 2: The gNB can superposition NR PDCCH and LTE CRS for idle-mode PDCCH, including the legacy UEs, without any specification change

Observation 3: If the UE implements any CRS interference suppression techniques, it would benefit from knowing if the NR PDCCH and the LTE CRS are superpositioned, and what the relative power of the CRS also in the idle mode.

Proposal 5: Consider using the CRS rate matching pattern also for RRC idle-mode and RRC_Inactive UEs, either by providing the configuration in system information (or during RRC release), or by retaining the configuration received while in RRC_Connected.

## Superpositioning impact on LTE UEs

When the NR PDCCH is superpositioned on the LTE CRS in symbols #1 and #2, it impacts half the CRS REs for  antenna ports 2 and 3 in a sub-frame, but does not impact ports 0 and 1. This would have a negative impact on 4-port UE’s CSI estimation and channel estimation on the sub-frames and PRBs where the collision takes place. The actual interference experienced by these CRS REs depend on the relative power difference between the PDCCH and CRS. In addition the interference scenario would only be prevalent in subframes which are at least partially used for NR, i.e., not all LTE CRS RE’s antenna port 2 and 3 may be interfered if LTE and NR employ FDM and impact is further reduced considering DSS will employ at least TDM schem at a subframe level for NR and LTE. LTE schedulers typically compensate for UE CSI reporting inaccuracies and this interference scenario could be handled via these methods.

RRM measurements are based on port #0 or port #0 and port#1 and are not impacted.

Figure 4: 4-port CRS and 2-symbol PDCCH CORESET starting at symbol #1 (yet again)

## Existing specification restrictions

Today, both TS38.211 and TS38.213 forbid a configuration where the LTE CRS overlaps with a PDCCH, when precoderGranularity equals allContiguousRBs.

[TS38.211 subclause 7.3.2.2 Control-resource set (CORESET)]

For a CORESET configured by the ControlResourceSet IE: […] - for both interleaved and non-interleaved mapping, the UE may assume - the same precoding being used within a REG bundle if the higher-layer parameter precoderGranularity equals sameAsREG-bundle; - the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET, and that no resource elements in the CORESET overlap with an SSB or LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround, if the higher-layer parameter precoderGranularity equals allContiguousRBs.

[TS38.213 clause 10 UE procedure for receiving control information]

When precoderGranularity = allContiguousRBs, a UE does not expect - to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency - any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround or with any RE of a SS/PBCH block.

In addition, also for the case where precoderGranularity does not equal to allContiguousRBs, PDCCH assignment determination clause of 38.213 says that when a PDCCH candidate overlaps with LTE CRS REs, then that monitoring that candidate is not required.

[TS38.213 clause 10.1 UE procedure for determining physical downlink control channel assignment]

For monitoring of a PDCCH candidate in a slot […] - If at least one RE of a PDCCH candidate on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, the UE is not required to monitor the PDCCH candidate.

The above-referred restriction should be lifted, and the specification should allow for configuring a Search Space so that the associated CORESET and the PDCCH candidates may collide with LTE CRS resources, and that the UE is expected to monitor a PDCCH candidate with REs overlapping with LTE CRS, if it supports the REl-18 functionality.

Proposal 6: Support configuring a Search Space so that the associated CORESET and the PDCCH candidates are allowed to collide with LTE CRS resources

Proposal 7: The UE is expected to monitor a PDCCH candidate even if it has REs overlapping with LTE CRS REs.

# Conclusion

In this document, the following observations and proposals are made:

Proposal 1: Consider 4-port LTE CRS with 2-symbol CORESET and SS start symbol#1 as the scenarios of interest

Observation 1:

- Superpositioning the LTE CRS and NR PDCCH/DMRS works fairly well for higher aggregation levels

  - With 0 dB power difference, AL4/8/16 shows limited loss

  - With -3dB LTE CRS AL2 works as well

  - AL1 cannot be used with superpositioning with PDCCH BLER target of 1%

- Puncturing the LTE CRS REs out of the NR PDCCH/DMRS outperforms superpositioning on low ALs

  - With AL4 and higher, when looking at all the simulated cases, puncturing achieves similar or worse performance than superpositioning

Proposal 2: Allow both super positioning LTE CRS on NR PDCCH & PDCCH DMRS, and puncturing LTE CRS REs out of NR PDCCH & PDCCH DMRS

Proposal 3: The gNB can configure the UE to expect either superpositioned or punctured NR PDCCH REs, when they overlap with LTE CRS REs.

- When the PDCCH and CRS are superpositioned, it is up to the UE whether or not to use the overlapping PDCCH REs in the demodulation and decoding process

- When the PDCCH is puncured, the UE cannot use the overlapping PDCCH REs

Proposal 4: The superpositioning/puncturing configuration is provided per PDCCH candidate or per PDCCH AL

Observation 2: The gNB can superposition NR PDCCH and LTE CRS for idle-mode PDCCH, including the legacy UEs, without any specification change

Observation 3: If the UE implements any CRS interference suppression techniques, it would benefit from knowing if the NR PDCCH and the LTE CRS are superpositioned, and what the relative power of the CRS also in the idle mode.

Proposal 5: Consider using the CRS rate matching pattern also for RRC idle-mode and RRC_Inactive UEs, either by providing the configuration in system information (or during RRC release), or by retaining the configuration received while in RRC_Connected.

Proposal 6: Support configuring a Search Space so that the associated CORESET and the PDCCH candidates are allowed to collide with LTE CRS resources

Proposal 7: The UE is expected to monitor a PDCCH candidate even if it has REs overlapping with LTE CRS REs.

# References

- RP-213575,	New WI: Enhancement of NR Dynamic spectrum sharing (DSS)