3GPP TSG RAN WG1 #109e		R1-2204826

e-meeting, May 9th – 20th, 2022

Source:	Lenovo

Title: 	Remaining issues on channel access for NR from 52.6 GHz to 71GHz

Agenda Item:   8.2.4

Document for:	Discussion

# 1	Discussion

## 1.1 Beam update for consecutive PUSCH transmissions

A UE may need to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, and the CG-PUSCH transmissions and dynamically scheduled PUSCH transmissions may be indicated by different UL Tx beams. For example, as the Figure 1 shows, a UE is scheduled to transmit a PUSCH transmission by an UL grant following a CG-PUSCH transmission. In this case, beam 0 is indicated to the UE by higher layer signaling for CG-PUSCH transmission and then, beam 1 is indicated to the UE by UL grant for dynamically scheduled PUSCH transmission. There may be no gap enough for switching UL Tx beam from beam 0 to beam 1, especially higher SCS values are considered, and the UE may also need to perform LBT using a wider sensing beam ‘covering’ beam 0 and beam 1. Therefore, before transmitting the set of consecutive PUSCH transmissions, it’s better for the UE to determine one UL Tx beam to transmit all the consecutive UL transmissions. In this example, the UE can use beam 1 to transmit the two PUSCH transmissions because beam 1 is indicated later and may be more suitable for the current channel condition, after completing LBT with a sensing beam ‘covering’ beam 1. Above all, we propose that the UE may use the latest indicated UL Tx beam to transmit a set of consecutive UL transmissions including dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions.

Proposal 1: For NR unlicensed bands between 52.6 GHz and 71 GHz, if a UE is going to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, the UE may select the latest indicated UL Tx beam for the consecutive UL transmissions. LBT procedures apply to the UL Tx beam as usual.

Figure 1: Illustration of beam update for consecutive CG PUSCH and DG PUSCH

# 2	Conclusion

Proposal 1: For NR unlicensed bands between 52.6 GHz and 71 GHz, if a UE is going to transmit a set of consecutive PUSCH transmissions including both dynamically scheduled PUSCH transmissions and CG-PUSCH transmissions, the UE may select the latest indicated UL Tx beam for the consecutive UL transmissions. LBT procedures apply to the UL Tx beam as usual.