3GPP TSG RAN WG1 #109-e	                         		       R1-2204828

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.7.2

Source:	InterDigital, Inc.

Title:	Remaining issues on DCI-based power saving adaptation

Document for:	Discussion

# Introduction

One issue that has not been decided yet for DCI-based power saving adaptation during DRX Active Time is how retransmissions are handled [1]. To this end, several options have been discussed. In this contribution our views on this issue are expressed.

# Discussion

## PDCCH skipping

In case of PDCCH skipping, when there is a pending or potential HARQ retransmission, performing PDCCH skipping may result in data loss if the retransmission cannot be received properly. If the skipping indication is received in the initial grant, then the UE should monitor the PDCCH for a possible retransmission grant. Since the retransmission grant is expected while the retransmission timer is running, the UE should monitor the PDCCH at least in the interval from the beginning of the DL retransmission timer until the last symbol of the retransmission. Similar consideration holds for UL transmission as well.

Other alternative solutions to handle retransmissions are possible. For example, one such solution that was proposed is for the UE to perform skipping only after an ACK feedback (for DL) or PUSCH transmission (for UL). However, these alternatives are expected to reduce power saving gain since the UE would need to monitor PDCCH for a longer duration compared to the solution expressed above.

Based on this discussion, the following is proposed:

Proposal 1:

Case 1: Upon detecting a scheduling DCI format 1-1/1-2 indicating PDCCH skipping (i.e., Beh 1A), the UE performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL is running.

Case 2: Upon detecting a scheduling DCI format 0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), the UE performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerUL is running.

## SSSG switching

In case of SSSG switching, when two SSSGs are configured, a special mechanism is not needed to handle retransmissions because both SSSGs are expected to include at least one SS. It has been proposed that a third SSSG can be configured as a null SSSG to emulate PDCCH skipping. If a retransmission handling solution to accommodate this scenario is adopted, and if this solution is used for all possible configurations (e.g., with two SSSG), unnecessary reduction in power saving would occur. On the other hand, a solution just for the scenario with three SSSGs with one of them being null seems to be unnecessary over-optimization. Therefore, we think this case can be handled by the gNB. Even with a third SSSG, one SS (e.g., with sparser monitoring periodicity) can be configured to handle retransmissions.

# Summary

In this contribution, retransmission handling has been analyzed. The following is proposed:

Proposal 1:

Case 1: Upon detecting a scheduling DCI format 1-1/1-2 indicating PDCCH skipping (i.e., Beh 1A), the UE performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL is running.

Case 2: Upon detecting a scheduling DCI format 0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), the UE performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerUL is running.

# References

Chairman’s Notes, 3GPP RAN1 #108-e