3GPP TSG RAN WG1 #109-e		                             R1-2204829
e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.6.1

Source:	InterDigital, Inc.

Title:	Potential techniques for further RedCap UE complexity reduction

Document for:	Discussion and Decision

# Introduction

One motivation behind Rel-17 Reduced Capability (RedCap) feature was to support industrial sensors, video surveillance, and wearables. To further expand RedCap use cases, additional complexity reduction enhancements have been considered. To this end, a new Study Item was agreed in RAN #94e [1]; the objectives of the SID are as follows:

To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx: - Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

This contribution discusses our initial views on additional complexity reduction techniques for RedCap UEs.

# Discussion

## Cost saving aspects for reduced bandwidth options

In Rel-17, maximum RedCap UE bandwidth has been reduced to 20 MHz in FR1 and 100 MHz in FR2. According to the SID objectives quoted above, further reduction of maximum UE BW to 5 MHz in FR1 must be studied. Following the same approach used in [2], three options can be considered to support reduced BW:

- Option 1: Reduced BW in both RF and baseband

- Option 2: Reduced BW in baseband for data and control channels

- Option 3: Reduced BW in baseband for data channel only; control channel uses the existing BW

Using the reference cost model agreed for Rel-17 RedCap [3] and the methodology as in [2], Table 1 provides an estimate of cost savings achieved by the three options when maximum BW is reduced from 20 MHz to 5 MHz. Note that these numbers are estimates and actual savings would be determined by the specific implementation; they may nevertheless provide some insight as to potential savings.

Table 1 Cost saving of BW reduction options (5 MHz) over Rel-17 RedCap UE

| Reduced UE bandwidth (5 MHz instead of 20 MHz) | Reference | Option1 | Option2 | Option3 |
| --- | --- | --- | --- | --- |
| ## RF: Power amplifier | ## 25.0% | ## 20.0% | ## 25.0% | ## 25.0% |
| ## RF: Filters | ## 10.0% | ## 10.0% | ## 10.0% | ## 10.0% |
| ## RF: Transceiver (including LNAs, mixer, and local oscillator) | ## 45.0% | ## 35.0% | ## 45.0% | ## 45.0% |
| ## RF: Duplexer / Switch | ## 20.0% | ## 20.0% | ## 20.0% | ## 20.0% |
| # RF: Total | # 100.0% | # 85.0% | # 100.0% | # 100.0% |
| ## BB: ADC / DAC | ## 10.0% | ## 2.5% | ## 10.0% | ## 10.0% |
| ## BB: FFT/IFFT | ## 4.0% | ## 1% | ## 1% | ## 4.0% |
| ## BB: Post-FFT data buffering | ## 10.0% | ## 2.5% | ## 2.5% | ## 10% |
| ## BB: Receiver processing block | ## 24.0% | ## 6% | ## 6% | ## 12% |
| ## BB: LDPC decoding | ## 10.0% | ## 2.5% | ## 2.5% | ## 2.5% |
| ## BB: HARQ buffer | ## 14.0% | ## 3.5% | ## 3.5% | ## 3.5% |
| ## BB: DL control processing & decoder | ## 5.0% | ## 3% | ## 3% | ## 5% |
| ## BB: Synchronization / cell search block | ## 9.0% | ## 9.0% | ## 9.0% | ## 9.0% |
| ## BB: UL processing block | ## 5.0% | ## 2.0% | ## 2.0% | ## 2.0% |
| ## BB: MIMO specific processing blocks | ## 9.0% | ## 9.0% | ## 9.0% | ## 9.0% |
| # BB: Total | # 100.0% | # 41% | # 48.5% | # 67% |
| # RF+BB: Total (with RF:BB cost split 40:60) | # 100.0% | # 58.6% | # 69.1% | # 80.2% |
| Cost savings | 100.0% | 41.4% | 30.9% | 19.8% |

From Table 1, we can see that meaningful cost saving could be achieved from reducing BW in RF and/or baseband for all channels (i.e., Option 1 and 2). Considering that the Rel-18 RedCap targets new use cases (e.g., smart grid), the cost saving should be considered as a high priority. Therefore, at least BW reduction Option 1 or Option 2 should be considered.

Proposal-1: Consider BW reduction of RF and/or baseband for all channels in Rel-18 RedCap.

### Standards impact analysis based on BW reduction options

### Cell search and system information acquisition

In NR, the PSS and SSS consist of 127 subcarriers, resulting in 1.905 and 3.810 MHz of BW for 15 kHz and 30 kHz SCS, respectively. Therefore, 5 MHz UE BW would be sufficient to cover the synchronization signals.

The PBCH consists of 240 subcarriers, resulting in 3.6 and 7.2 MHz of BW for 15 kHz and 30 kHz SCS, respectively. So, 5 MHz UE BW would not be sufficient to cover the PBCH for 30 kHz SCS. We can see from Table 2 that many FR1 bands support 30 kHz SCS with a few of them supporting only 30 kHz SCS [4].

To address this issue, several solutions can be considered. One approach is that the UE can simply discard the subcarriers beyond the maximum UE BW and compensate for the loss, e.g., by time accumulation. This approach is evaluated in our companion Tdoc [5]. An alternative solution with higher impact is to design a new channel for RedCap UEs to replace the legacy PBCH. A third approach would be to use only 15 kHz SCS for SSBs; this approach, however, would create significant inflexibility for legacy operations and so is not desirable.

Table 2 NR operating bands in FR1

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 15 kHz | Case A | 5279 – <1> – 5419 |
| n2 | 15 kHz | Case A | 4829 – <1> – 4969 |
| n3 | 15 kHz | Case A | 4517 – <1> – 4693 |
| n5 | 15 kHz | Case A | 2177 – <1> – 2230 |
|  | 30 kHz | Case B | 2183 – <1> – 2224 |
| n7 | 15 kHz | Case A | 6554 – <1> – 6718 |
| n8 | 15 kHz | Case A | 2318 – <1> – 2395 |
| n12 | 15 kHz | Case A | 1828 – <1> – 1858 |
| n13 | 15 kHz | Case A | 1871 – <1> – 1885 |
| n14 | 15 kHz | Case A | 1901 – <1> – 1915 |
| n18 | 15 kHz | Case A | 2156 – <1> – 2182 |
| n20 | 15 kHz | Case A | 1982 – <1> – 2047 |
| n24 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n25 | 15 kHz | Case A | 4829 – <1> – 4981 |
| n26 | 15 kHz | Case A | 2153 – <1> – 2230 |
| n28 | 15 kHz | Case A | 1901 – <1> – 2002 |
| n29 | 15 kHz | Case A | 1798 – <1> – 1813 |
| n30 | 15 kHz | Case A | 5879 – <1> – 5893 |
| n34 | 15 kHz | Case A | NOTE 5 |
|  | 30 kHz | Case C | 5036 – <1> – 5050 |
| n38 | 15 kHz | Case A | NOTE 2 |
|  | 30 kHz | Case C | 6437 – <1> – 6538 |
| n39 | 15 kHz | Case A | NOTE 6 |
|  | 30 kHz | Case C | 4712 – <1> – 4789 |
| n40 | 30 kHz | Case C | 5762 – <1> – 5989 |
| n41 | 15 kHz | Case A | 6246 – <3> – 6717 |
|  | 30 kHz | Case C | 6252 – <3> – 6714 |
| n463 | 30 kHz | Case C | 8993 – <1> – 9530 |
| n48 | 30 kHz | Case C | 7884 – <1> – 7982 |
| n50 | 30 kHz | Case C | 3590 – <1> – 3781 |
| n51 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n53 | 15 kHz | Case A | 6215 – <1> – 6232 |
| n65 | 15 kHz | Case A | 5279 – <1> – 5494 |
| n66 | 15 kHz | Case A | 5279 – <1> – 5494 |
|  | 30 kHz | Case B | 5285 – <1> – 5488 |
| n67 | 15 kHz | Case A | 1850 – <1> – 1888 |
| n70 | 15 kHz | Case A | 4993 – <1> – 5044 |
| n71 | 15 kHz | Case A | 1547 – <1> – 1624 |
| n74 | 15 kHz | Case A | 3692 – <1> – 3790 |
| n75 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n76 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n77 | 30 kHz | Case C | 7711 – <1> – 8329 |
| n78 | 30 kHz | Case C | 7711 – <1> – 8051 |
| n79 | 30 kHz | Case C | 8480 – <16> – 88807 |
|  |  |  | 8475 – <1> – 88848 |
| n85 | 15 kHz | Case A | 1826 – <1> – 1858 |
| n90 | 15 kHz | Case A | 6246 – <1> – 6717 |
|  | 30 kHz | Case C | 6252 – <1> – 6714 |
| n91 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n92 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n93 | 15 kHz | Case A | 3572 – <1> – 3574 |
| n94 | 15 kHz | Case A | 3584 – <1> – 3787 |
| n964 | 30 kHz | Case C | 9531 – <1> – 10363 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [8]. NOTE 2: The applicable SS raster entries are GSCN = {6432, 6443, 6457, 6468, 6479, 6493, 6507, 6518, 6532, 6543}. NOTE 3: The following GSCN are allowed for operation in band n46: GSCN = {8996, 9010, 9024, 9038, 9051, 9065, 9079, 9093, 9107, 9121, 9218, 9232, 9246, 9260, 9274, 9288, 9301, 9315, 9329, 9343, 9357, 9371, 9385, 9402, 9416, 9430, 9444, 9458, 9472, 9485, 9499, 9513}. NOTE 4: The following GSCN are allowed for operation in band n96: GSCN = {9548, 9562, 9576, 9590, 9603, 9617,9631, 9645, 9659, 9673, 9687, 9701, 9714, 9728, 9742, 9756, 9770, 9784, 9798, 9812, 9826, 9840, 9853, 9867, 9881, 9895, 9909, 9923, 9937, 9951, 9964, 9978, 9992, 10006, 10020, 10034, 10048, 10062, 10076, 10090, 10103, 10117, 10131, 10145, 10159, 10173, 10187, 10201, 10214, 10228, 10242, 10256, 10270, 10284, 10298, 10312, 10325, 10339, 10353}. NOTE 5: The applicable SS raster entries are GSCN = {5032, 5043, 5054} NOTE 6: The applicable SS raster entries are GSCN = {4707, 4715, 4718, 4729, 4732, 4743, 4747, 4754, 4761, 4768, 4772, 4782, 4786, 4793} NOTE 7: The SS raster entries apply for channel bandwidths larger than or equal to 40 MHz NOTE 8: The SS raster entries apply for channel bandwidths smaller than 40 MHz |  |  |  |

Observation-1: PBCH bandwidth is larger than RedCap UE bandwidth with 30 kHz SCS.

Proposal-2: Study PBCH performance impact due to smaller UE bandwidth for 30kHz SCS.

### Type0-PDCCH CSS

UE determines from MIB Type0-PDCCC CSS and the configuration of CORESET#0 [6]. CORESET#0 may contain 24, 48, or 96 RBs over 1 to 3 OFDM symbols with the exact numbers depending on the specific case signalled in the MIB. Of these cases, 24 RBs with 15 kHz and 12 RBs with 30 kHz can fit into 5 MHz. For the remaining cases, the BW of CORESET#0 exceeds 5MHz and a solution is needed.

One potential approach is that only CORESET#0 configurations that can fit into the RedCap UE BW are used by the gNB; this approach, however, is not desirable since it will impact legacy UE operation. Another solution is for the RedCap UE to discard the subcarriers beyond the maximum UE BW which would result in coverage loss. A potential solution with greater impact is to design a new channel to replace the functionality of Type-0 PDCCH and CORESET#0.

Once SIB1 is received, UE acquires the scheduling information of remaining SIs. If all SI transmission is constrained to be within 5 MHz, this could impact legacy UE operation. One solution would be to introduce a new mechanism for RedCap SI transmission.

Proposal-3: Study new mechanisms for RedCap SI transmission.

### Random access

Random access procedure consists of preamble transmission on PRACH, msg2/3/4 transmission and related control signaling. The preamble bandwidth is within RedCap BW, same PRACH preambles can be used. However, in NR, message 2 and message 4 are transmitted within CORESET#0 which could be larger than 5 MHz. In addition, due to the association between SSBs and RACH occasions, PRACH may cover a BW larger than 5 MHz.  Finally, early identification of RedCap UEs may be needed to handle some of these problems, e.g., by allocating exclusive PRACH resources to RedCap UEs.

Proposal-4: Study potential impact on RACH msg2/4 reception for Rel-18 RedCap UE.

### Paging

For paging, coverage loss may occur due to reduced BW, especially for paging PDCCH and paging early indication channel.

Proposal-5: Study coverage impact for paging channel due to reduced BW.

### Connected Mode:

In general, the biggest performance impact from reduced BW seems to be potential coverage and/or SNR loss for control channels since higher AL cannot be supported with 5 MHz. For example, with 30 kHz SCS, the maximum supportable AL is 4 and with 15 kHz SCS the maximum supportable AL is 8. If ALs larger than these are used, then some of the subcarriers would need to be discarded by the UE. The performance of PDCCH is evaluated in our companion Tdoc [5]. Furthermore, PDCCH blocking probability is expected to increase due to reduced number of PDCCH candidates.

In addition, some loss in PDSCH and PUSCH performance may occur due to reduced frequency diversity, but this is loss should be manageable since required data rates are also lower for RedCap UEs.

Proposal-6: Study coverage impact for PDCCH, PDSCH, and PUSCH due to reduced BW.

## Relaxed Processing Time and Reduced Data Rate

### Relaxed processing time

The PDSCH processing time, PUSCH preparation time, and CSI computation time for NR are provided in [7]. If the processing time is relaxed, cost and complexity of baseband processing (e.g., receive processing block, UL processing block, LDPC decoder) can be reduced. During the Rel-17 RedCap SI, companies provided cost reduction estimates of 5% to 10% with doubling of N1 and N2 [3].

The most significant impact of relaxed processing time would be increased latency. For RedCap use cases with relaxed latency requirements, (e.g., <500 ms for video surveillance), relaxed processing time should not impact latency. However, for use cases with relatively strict latency requirements (e.g., wireless sensors with latency requirement of 5-10 ms), UE processing time is crucial in achieving the latency budget. During the Rel-17 RedCap SI, several companies provided latency analysis. For example, according to the analysis presented in [8], latency requirement of 5-10 ms can still be met if N1/N2 is doubled from current values.

The specification impact of relaxed processing is expected to be limited; the main impact would be to introduce the new processing times in 38.214.

Observation-2: Relaxed processing time could achieve additional 5-10% cost saving gain with very limited specification impact.

Proposal-7: Study relaxed processing time for Rel-18 RedCap UE.

### Reduced data rate

Peak data rate can be reduced by restricting TBS, number of PRBs, and modulation orders. The impact of reduced peak data rate is expected to be limited. DCI, MCS tables and CQI tables may be optimized to reflect the reduction in data rate.

Proposal-8: Study potential cost saving and standards impact from reduced data rate for Rel-18 RedCap UE.

- Summary

In this contribution, potential techniques to further reduce RedCap UE complexity has been analyzed. The following observations and proposals have been provided.

Observation-1: PBCH bandwidth is larger than RedCap UE bandwidth with 30 kHz SCS.

Observation-2: Relaxed processing time could achieve additional 5-10% cost saving gain with very limited specification impact.

Proposal-1: Consider BW reduction of RF and/or baseband for all channels in Rel-18 RedCap.

Proposal-2: Study PBCH performance impact due to smaller UE bandwidth for 30kHz SCS.

Proposal-3: Study new mechanisms for RedCap SI transmission.

Proposal-4: Study potential impact on RACH msg2/4 reception for Rel-18 RedCap UE.

Proposal-5: Study coverage impact for paging channel due to reduced BW.

Proposal-6: Study coverage impact for PDCCH, PDSCH, and PUSCH due to reduced BW.

Proposal-7: Study relaxed processing time for Rel-18 RedCap UE.

Proposal-8: Study potential cost saving and standards impact from reduced data rate for Rel-18 RedCap UE.

- References

- [1] RP-213661, “New SID on Study on further NR RedCap UE complexity reduction,” Ericsson

[2] TR 36.888, “Study on provision of low-cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”

[3] TR 38.875, “Study on support of reduced capability NR devices”

- [4] TS 38.101, “User Equipment (UE) radio transmission and reception”

- [5] R1-2204830, “Simulation needs for further RedCap UE complexity reduction,” InterDigital, Inc.

- [6] TS 38.213, “Physical layer procedures for control”

- [7] TS 38.214, “Physical layer procedures for data”

- [8] R1-2007668, “Complexity reduction for Reduced Capability NR devices,” vivo, Guangdong Genius