3GPP TSG RAN WG1 #109-e	R1-2204835

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.3

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Potential solutions for SL positioning

Document for:	Discussion & Decision

# Introduction

One of the study item objectives related to SL positioning listed in RP-213588 [1] is:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This contribution covers first considerations for possible solutions motivated by the scenarios, requirements and evaluations in our companion contributions R1-2204833 [2] and R1-2204834 [3].

# Bandwidth requirement needed to meet the identified accuracy requirements

The achievable accuracy for timing methods has been evaluated in the NR positioning study items during Rel-16 and Rel-17 ( [4], [5]). Similar observations can be also carried on the bandwidth needed to achieve the accuracy requirements for sidelink applications. In addition, the very high availability requirements demand, especially in challenging multipath scenarios, higher bandwidth allocated for the positioning reference signal used. For positioning applications, when measuring the ToA of the first arriving path especially the multipath components arriving with a low delay have a high impact to the accuracy. To characterize this impact, we used instead of the normal K-factor definition, which considers all multipath components including components with a high delay a modified K-factor definition, called “KF early”.

The “KF-early” calculates the ratio of the LOS path to the multipath components arriving within a low delay relative to the LOS path. For the simulations we used the models as defined by TR38.901 [6] and calculated the “KF-early” from the channel impulse response generated by the model. Hence, this evaluation methodology can be performed using the 3GPP models without modification. Optionally, instead of running long simulations, modified parameter settings can be used to generate more data points for the area of interest. As a first example we considered the multipath components arriving with a delay less than 50ns for the evaluation and generated ToA error CDFs for different KF-early ranges. The methodology was:

- For each drop we extracted the “KF-early” from the channel coefficients generated by models in line with  [6]

- We sorted the ToA-Error results according this “KF-early”

Figure 1: TOA accuracy for different bandwidth for different LOS conditions.

Figure 1 shows first evaluation results of the impact of the bandwidth and the LOS conditions on ToA accuracy. It should be noted that the channel models in [6] are not optimized for positioning purpose. The probability of “early cluster” (multipath components arriving with low delay) depends on the number of clusters and the delay spread. Higher delay spread reduces the probability of an early cluster. This is not in line with real-world experience. For example, effects like ground reflections are independent from the delay spread. The delay of the ground reflection depends mainly on the UE and gNB height (or height of the UEs in case of sidelink) and is in all cases less than 9ns for a UE height of 1.5m, as 2*1.5m = 3m is the additional multipath if the TRP is located directly above the UE which corresponds to a time delay of approximately 9ns. Using models like InF-LOS in 50% of the cases no multipath components arrive with a delay of less than 10ns and/or may be very weak. Only if “ground reflection” defined by [6] is taken into account the CIR characteristics match the real-world measurement results better. These aspects are discussed in further details in [2] and [3]  within the scenarios and evaluations AIs.

Observation 1: 	Ground reflection and other multipath components arriving with a low delay are not covered may channel model parameter sets as agreed for InF-LOS, for example. Hence, effects relevant for high positioning accuracy are not covered by the models.

Observation 2: 	TR38.901 includes a model for ground reflection. Using this feature the evaluations match measurements from real-world scenarios to identify solutions for the high accuracy and high availability requirements.

Observation 3: 	Considering only nearly ideal LOS conditions (KF-Early higher than 10dB) a high accuracy (error < 1ns, 90% percentile) can be achieved already with a bandwidth of 50MHz

Observation 4: 	For more critical LOS scenarios a higher bandwidth is required to achieve a target accuracy of 30cm (app. 1ns)

- Based on these observations, for the sidelink reference signal bandwidth allocation in NR bands over the PC5 interface, the UE shall be allowed to use the all PRBs in a configured carrier bandwidth allowing configurations with {10,20,30,40}MHz.

Proposal 1: 	Support the sidelink transmission in the whole carrier bandwidths up to 40 MHz which are defined in TS 38.101-1

- We propose to study the support of the sidelink positioning RS transmission for bandwidths beyond 40 MHz by considering the following option:

- Option 1: allow for a sidelink positioning reference signal a bandwidth up to the bandwidth configured for Uu signals.

- Option 2: allow bandwidth extension by applying CA including the positioning feasibility considerations to support aggregation inter-band or intra-band carriers.

- Option 3: Support “coherent frequency hopping” for sidelink positioning reference signals

We assume that high accuracy distance measurements are mainly required for UEs with low (e.g. <100m) distance between the UEs. The implementation of option 3 may require a UE supporting the full carrier bandwidth. If the UE supports the full bandwidth Option 1 is straightforward from implementation perspective. Hence, we propose to focus on option 1 and 2.

Proposal 2: 	RAN1 to study the impact of the following options to improve the sidelink accuracy in Rel-18:

- Option 1: for sidelink positioning in inter and intra-band operation: allow a maximum transmission and reception for UE capable devices according to the bandwidth allowed for Uu: {50, 60, [70], 80, 90, 100} MHz.

- Option 2: allow bandwidth extension by applying CA for SL

# Reference signal for sidelink positioning

## Type of positioning reference signal

- SRS-Pos and DL-PRS have been designed to provide high flexibility and are optimized to achieve the maximum positioning performance in diverse scenarios. There is no motivation for designing a new RS for SL positioning in terms of accuracy, latency or coverage. RS configuration and interference aspects can build on SRS-Pos or DL-PRS.

Observation 5:	DL-PRS and SRS-Pos are optimized for positioning purposes and require significantly less standardization overhead than a new design of a sidelink RS.

For down-selecting between DL-PRS and SRS-Pos as a baseline for SL positioning we think that SRS-Pos is the more appropriate candidate for the following reasons:

- The specification allows that UEs can already measure SRS-RSRP [8], hence support of SRS-Pos requires only extending the SRS measurements and reports

- SRS provides more flexibility for resource allocation because of ZC properties. This allows full orthogonal sequences and provides a good separation of signals arriving with high level difference. Hence, more UEs may share the same resources with no or minor interference between the signals.

Proposal 3: 	The sidelink reference signals for positioning should be based on SRS-Pos.

## Sidelink positioning reference signal parameter considerations

A gNB typically uses antennas with higher gain than a UE. Hence, for a given UE transmit power the coverage radius feasible using the PC5 interface may be lower than for the Uu interface. Especially for areas with no gNB deployment a high coverage radius at least for measurements of the positioning reference signal may be worthwhile. This may be essential for out-of-coverage scenarios where a UE wants to measure the distance to several UEs. The staggering introduced for SRS-pos is a technology to significantly increase the coverage radius for a given UE transmit power.

Proposal 4: 	Support staggered SRS for the sidelink positioning reference signal.

Using several OFDM symbols with staggering allows the decoding of the SRS-pos with very low SINR 
(< -20dB, for example). For low (< 100m, for example) distances between UEs this facilitates very low power operation modes for sidelink positioning reference signals. This is similar to UWB application where the power spectral density of the received signal is typically below the noise floor. In this case the interference generated by such signals may be low and may allow that the sidelink positioning reference signal uses resources not exclusively reserved for sidelink operation. Hence, it may be possible to allow in an area configured by the network the transmission of low power sidelink reference signals for positioning use. These low power signals will generate negligible interference to neighboring areas and may be a straight forward way to allow a sidelink positioning signal with a bandwidth higher than the bandwidth allocated for sidelink communication applications, which typically operate at much higher SINR level.

Proposal 5: 	Study the use of low power positioning reference signals with a bandwidth higher than the bandwidth allocated for sidelink (communication) signals

## Configurations for low latency distance change (relative speed) measurement

A possible method for relative speed measurement is the measurement of the distance at different times and the relative speed is calculated as the distance change divided by the time between the measurements. If the measurement accuracy of the distance measurement is limited, the device must travel a distance significantly higher than the measurement accuracy to reduce the impact to the speed measurement. Hence a long time interval between the two measurements is required to achieve a desired accuracy. This may result in a high latency for relative speed measurements.

Using the carrier phase allows the detection of small distance changes with high accuracy, whereas absolute distance may be derived from ToA measurements based RTT measurements (further details see [9]). If a distance change can be detected with high accuracy a low latency relative speed measurement can be implemented.

As described in [9] the phase difference is measured by fully synchronized devices. For UL-TDOA based solutions the LMUs of different TRPs use fully synchronized (phase and frequency) reference clocks. For DL-TDOA/OTDOA the DL-PRS signals may be transmitted phase-synchronized or the phase offset is known and reported to the UE (for UE based positioning) or taken into account by the positioning algorithm for UE assisted/network based position.

For sidelink operation this is not applicable. The UEs might not all be fully synchronized. Only a frequency offset estimation may be performed. Hence, the measured phase change is the sum of the phase change resulting from the remaining frequency offset and the phase change resulting from the movement (distance change).

To distinguish the two contributions to the measured phase-change a “double difference” method using reference signal pairs can be used. An example for possible allocation of reference signals is depicted in Figure 2. In each direction a “double burst” (e.g. two SRS-pos or a SRS-pos with several OFDM symbols) is transmitted. Depending on the parameter selection for dt1 and dt2 and assuming no speed change between t1 (time when the first burst of the forward link leaves the transmit device) and t3 (time when signal for return link leaves the device) the relative speed can be estimated with a desired accuracy. For the parameter selection the remaining frequency offset between the devices (typically much higher than the effective Doppler frequency) must be taken into account.

Proposal 6: 	Support the allocation of “double burst” (e.g. pairs of SRS-pos) for relative speed measurements

Figure 2: Double burst allocation example for phase based distance (change) measurement

# Support of PRUs and combination with other RAT dependent positioning measurements

## PRU SL measurements

- The positioning improvements introduced by enabling sidelink measurements between a target device and the PRUs are straightforward. For example, an RSRP measurement at the PRU and UE or by accurately estimating the PRU-UE distance through timing-based ranging.

Proposal 7: 	Support the transmission and reception of positioning sidelink RSs with PRUs.

Figure 3 – Example of PRUs configured to receive or transmit positioning sidelink RS

## Combination of SL positioning measurements with other RAT dependent positioning measurements

Combining SL positioning measurements with established UL and DL positioning methods can provide significant gains for three types of scenarios:

- Number of anchors is not sufficient: A UE can measure the distance to another UE with known position (e.g.: PRU). A UE-type PRU may act as additional anchor for UL or DL based positioning algorithms.

- For 3D positioning and non-ideal deployments (e.g.: The anchors have similar heights) the positioning algorithm may generate several solutions. Using additional information from sidelink may help to remove this ambiguity

- In NLOS and LOS conditions with low measurement accuracy any additional measurement may improve the accuracy

In an example scenario where only measurements between 2 UEs and 3 gNB are available, there are 4 possible combinations of the locations of the 2 UEs. In case distance information between the 2 UEs is available, those ambiguities can be resolved (see Figure 4).

Observation 6: 	Association of SL ranging with TDOA and RTT measurements is beneficial for the overall positioning performance

Proposal 8: 	Integrate the reporting of sidelink measurements into the reporting framework for DL-TDOA, UL-TDOA and multi-RTT based positioning

Figure 4- Schematic sketch of SL assisted TDOA with 3 gNBs and 1 range between 2 UEs

# Conclusions

In this contribution, the first considerations for possible solutions are covered and we made the following observations and proposals:

Observation 1: 	Ground reflection and other multipath components arriving with a low delay are not covered may channel model parameter sets as agreed for InF-LOS, for example. Hence, effects relevant for high positioning accuracy are not covered by the models.

Observation 2: 	TR38.901 includes a model for ground reflection. Using this feature the evaluations match measurements from real-world scenarios to identify solutions for the high accuracy and high availability requirements.

Observation 3: 	Considering only nearly ideal LOS conditions (KF-Early higher than 10dB) a high accuracy (error < 1ns, 90% percentile) can be achieved already with a bandwidth of 50MHz

Observation 4: 	For more critical LOS scenarios a higher bandwidth is required to achieve a target accuracy of 30cm (app. 1ns)

Observation 5:	DL-PRS and SRS-Pos are optimized for positioning purposes and require significantly less standardization overhead than a new design of a sidelink RS.

Observation 6: 	Association of SL ranging with TDOA and RTT measurements is beneficial for the overall positioning performance

Proposal 1: 	Support the sidelink transmission in the whole carrier bandwidths up to 40 MHz which are defined in TS 38.101-1

Proposal 2: 	RAN1 to study the impact of the following options to improve the sidelink accuracy in Rel-18:

- Option 1: for sidelink positioning in inter and intra-band operation: allow a maximum transmission and reception for UE capable devices according to the bandwidth allowed for Uu: {50, 60, [70], 80, 90, 100} MHz.

- Option 2: allow bandwidth extension by applying CA for SL

Proposal 3: 	The sidelink reference signals for positioning should be based on SRS-Pos.

Proposal 4: 	Support staggered SRS for the sidelink positioning reference signal.

Proposal 5: 	Study the use of low power positioning reference signals with a bandwidth higher than the bandwidth allocated for sidelink (communication) signals

Proposal 6: 	Support the allocation of “double burst” (e.g. pairs of SRS-pos) for relative speed measurements

Proposal 7: 	Support the transmission and reception of positioning sidelink RSs with PRUs.

Proposal 8: 	Integrate the reporting of sidelink measurements into the reporting framework for DL-TDOA, UL-TDOA and multi-RTT based positioning

# References

| [1] | 3GPP RP-213588, „Revised SID on expanded and improved NR positioning (Release 18),“ 3GPP, December 6-17, 2021. |
| --- | --- |
| [2] | 3GPP R1-2204833, „SL positioning scenarios and requirements,“ Fraunhofer, eMeeting, May 9-20, 2022. |
| [3] | 3GPP R1-2204834, „SL positioning evaluation methodology,“ Fraunhofer, eMeeting, May 9-20, 2022. |
| [4] | 3GPP TR 38.855, „Technical Specification Group Access Network: Study on NR positioning support,“ 3GPP, 2019-03. |
| [5] | 3GPP TR 38.857, „Technical Specification Group Radio Access Network; Study on NR Positioning Enhancements,“ 3GPP, Sophia Antipolis, 2021-03. |
| [6] | 3GPP TR38.901, „Technical Specification Group Radio Access Network; Study on channel models for frequencies from 0.5 to 100 GHz,“ 3GPP, 2022-03. |
| [7] | 3GPP TS38.101-1, „Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone,“ 3GG, 2022-03. |
| [8] | 3GPP TS 38.215, „Technical Specification Group Radio Access Network; NR; Physical layer measurements,“ 3GPP, 2022-03. |
| [9] | 3GPP R1-2204836, „NR carrier phase measurements for positioning,“ Fraunhofer, eMeeting, May 9-29, 2022. |