3GPP TSG RAN WG1 #109-e	R1-2204836

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	NR carrier phase measurements for positioning

Document for:	Discussion & Decision

# Introduction

In the SID of RP-213588, the following objectives define the scope in the study item of the accuracy improvements when applying carrier phase measurements:

- … - Improved accuracy, integrity, and power efficiency: - … - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary - …

In this contribution we discuss on different carrier phase measurement options and resulting reporting and applicability requirements.

# Carrier phase measurement options

## Example scenario for carrier phase measurement application

Figure 1: Example scenario for positioning with phase measurement

Figure 1 shows an example scenario. In the example we assume a moving UE in a multipath environment. For simplicity reasons we depict 3 paths only

- Path 1: LOS signal: This signal has the shortest propagation path to the base station

- Path 2: Ground reflection: For the example we assume a specular reflection. The additional path length depends on the UE height, base station height and the distance

- Path 3: Reflection of from a wall

An example for the resulting channel impulse response (magnitude) is given in Figure 2. The channel impulse response is a complex valued signal. The complex valued signal can be depicted as two components (I and Q) component or as “I/Q diagram”, wherein the x-axis represents the I component (real part) and the y-axis the Q component (imaginary part).

Figure 3 shows an example for the I/Q diagram of the 3 paths of the CIR, where the paths are separated and the effects resulting from the UE movement indicated in Figure 1.

Figure 2: CIR example for scenario given in Figure 1

Figure 3: I/Q diagram of complex valued CIR (3 taps only)

For the example we assume the phase is measured relative to a reference common for the transmit and receive part (“ideal synchronization”). In this case the phase change depends directly on the distance change resulting from the movement.

In the example the path length for path 1 (LOS) and 2 (ground reflection) is increase due to movement. For path 3 (distance to the wall is reduced) the path length is reduced, resulting in an opposite sign of the phase change.

Observation 1: 	The phase change resulting from movement may be different for each multipath component

Figure 3 assumes ideal synchronization (= both devices use a common reference). A frequency offset between the devices causes an additional (common) phase offset depicted in Figure 4. The impact of this additional phase offset depends on the positioning method. For UL-TDOA the base stations may be synchronized to a common reference and the phase difference of arrival (PDOA) is used as measurement, the common phase offset is removed by calculating the phase difference between two signals transmitted at the same time.

Figure 4: Impact of frequency offsets

## Evaluations

In line with the scenario described in Figure 1 we built a simulation setup using the following setup

- Channel model as defined by TS38.901, InF-LOS.

- Ground reflection features enabled

- Position of the cluster representing the wall defined according the wall position and the relationship between TRP-, UE- and wall position. A specular reflection was assumed for the model.

- For demonstration purpose we run a simulation with 200 positions. Due to the short simulation a statistical analysis is not feasible. We replaced the randomly generated LSP by fixed values to allow several simulation runs with different signal parameter settings.

- The UE is was moved in steps of 1mm. Assuming a speed of 1m/s this is equivalent to a measurement every 1ms. This high resolution was used to demonstrate the impact of small position changes to the phase. Assuming a carrier frequency of 3.75GHz the wavelength is 8cm and a distance change of 1mm is equivalent to a phase change of up to 4.5degree.

- We used 2 TRPs at different positions (and angles) relative to the UE movement track ( different relative speed)

- To better demonstrate the different phase behavior of the multipath components we used different bandwidth

  - 400MHz: We used a configuration defined for FR2 (120kHz SCS) and a FFT length of 4096

  - 100MHz: SCS=30kHz, FFT length 4096

  - 50MHz: SCS=15kHz, FFT length 4096

The first set of plots compares the estimated channel impulse response (magnitude) for different bandwidth at different positions (begin and end of a short track of 20cm)

| 400MHz: | 50MHz: |
| --- | --- |
| 100MHz: | The distance to the wall was app. 4m. The resulting additional path-length for this reflection is app. 2*4m = 8m (app. 27ns additional delay relative to LOS delay). For this example a weak ground reflection was applied, but the tap related to the ground reflection can be observed (in the 400MHz plot). For lower bandwidth the correlation lobes overlaps. |

The correlation function (estimated CIR) can be depicted as “I/Q-Diagram”. The real part of the complex valued correlation function is used for the X-axis, the imaginary part for the Y-axis.

In the following set of plots we demonstrate the impact to the phase for a movement of 1mm. As expected, the phase changes by app. 4.5 degree. It is observed that the phase change of the LOS component and the multipath component has – according to the different relative speed between UE and TRP and UE and reflector - different signs.

| 400MHz: | 50MHz: |
| --- | --- |
| 100MHz: | For the high bandwidth the lobes related to the LOS path and wall reflection are well separated. For 50MHz the correlation lobes overlap, but the correlation peak still represents the expected phase change according to the distance change LOS UE. For lower delays between the LOS component and the first (strong) reflection it may become more difficult to separate the peaks. But the characteristics of the correlation lobe may help to identify such scenarios and may be able to derive further information, at least reliability indicator. |

The phase extracted from the correlation peak are depicted in the next figures. The figure shows the phase changes for the movement along the short 20cm track. Due to the different angle the relative speed between the UE and TRPs is different. The positions of the TRPs and UE are depicted in Figure 5.

UE positions

TRP1

TRP2

Figure 5- TRP and UE position in the evaluation scenario.

| 400MHz | 50MHz |
| --- | --- |
| 100MHz | The phase change depends on the AoA/AoD (effective relative speed). If multipath components with different AoA/AoD arrive the components should be separated. If this is not feasible a “composite phase” may be measured resulting a deviation from the ideal phase slope. The impact to the accuracy depends on the delay and strength of the multipath components arriving with low delay relative to the component of interest (typically the LOS component) |

The measurement of the absolute phase requires a precise synchronization and calibration. The use of the phase difference avoids this issued. In this case a synchronization of the TRPs is sufficient. Phase changes resulting from frequency offsets are cancel out, if the different TRP transmit at the same time or receive the signal at (nearly) the same time (same OFDM symbol is used for each UE TRP link). In this case the phase difference can be calculated. In the following we compare the measured phase difference with the expected phase difference resulting from the difference of the true distance (expected phase difference). For the short track length of 20cm and the given TRP and UE positions the difference between the path-length UETRP1 and UETRP2 at the begin of the track and end of the track is 10.84cm (equivalent to a phase change of 488 degree). Due to the 360degree periodicity of the phase the ambiguity must be solved by other technologies. The phase measurement in combination with other technologies are an efficient method to track small position changes (“phase based smoothing”), even if the absolute accuracy is limited.

| 400MHz: | 50MHz |
| --- | --- |
| 100MHz: | For the given example it is observed that the bandwidth has a low impact to the measured phase difference. |

## Phase measurement options

We distinguish three types of carrier phase measurements

- The carrier phase is derived from the carrier frequency recovery.

- The phase is derived from the correlation in the frequency domain. This allows to determine the phase offset for different frequencies (e.g.: subcarriers or BWP)

- The phase is derived from the correlation function in the “delay domain” (= IFFT of the measured (complex valued) frequency response)

Deriving the carrier phase from the frequency recovery provides the average phase for all multipath components. Measurement of the phase of subcarriers or (narrow) BWPs may require a high SINR and may suffer from “frequency selective fading” caused by multipath propagation. As highlighted in chapter 2.1 the phase change may be different for each multipath component. Using the delay domain representation of the correlation function, the phase can be determined for each path.

Proposal 1: 	The carrier phase shall be determinable for different components of the channel impulse response and shall be measured in the delay domain

Proposal 2: 	Support reporting the phase measurements from the delay domain for the first and additional paths.

Proposal 3:     	Consider one or more of the following options for the delay domain phase measurement: 
- Option 1: The first path and N-path reporting is extended by a phase value for each reported path
- Option 2: A magnitude the complex valued correlation value associated to the path is reported. 
- Option 3: The full complex valued CIR for a given length is reported.

- Option 4: Relevant parts of the complex valued CIR are reported.

For option 3 we assume the measurement unit measures or predicts a ToA for the LOS path. The measured CIR is aligned to this time and the correlation function. The same principle applies to option 4. But for channels with high delay spread it may be sufficient to report the parts around relevant paths only. The relevant paths may be selected by the measurement device or the LMF requests the relevant parts.

# Phase based positioning enhancement

## Phase based smoothing for tracking of moving devices

Changes of the distance by a fraction of the wavelength can be detected using the carrier phase. In combination with periodic transmission of reference signals this allows a high accuracy tracking of devices due to the high resolution provided by the phase measurements.

A phase based smoothing of the estimated trajectory of the moving device requires a synchronization of the base stations only and can be implemented using DL or UL reference signal. Especially for small deployments (e.g.: InF scenarios) a sufficient synchronization of the base stations may be feasible. The implementation of a phase based smoothing for UL-TDOA has no RAN1 standard impact. For DL based methods like DL-TDOA the network may signal to the UE that the DL-PRS signals are transmitted phase coherent. For UL based methods the TRP may report the phase to the LMF together with information related to the coherency of the measurements.

Proposal 4: 	Support phase based smoothing for DL-TDoA and UL-TDoA methods in Rel-18.

Consider the signaling of the coherency transmission status to the UE (UE based positioning) or to the LMF to notify on the applicability of phase based smoothing.

## Detection of distance changes and relative speed measurement

In case of continuous position tracking of devices the relative speed can be derived from the position change. In case of ranging between two devices (two UE or measurement of distance between UE and one base station) the phase difference can be used for distance change. A possible solution taking into account non-ideal synchronization (e.g.: remaining frequency offsets between devices) is described in R1-2204835 (sidelink related TDOC). The described double-difference phase method can be also applied to DL and UL based method to detect distance changes with low latency.

Proposal 5: 	Allow the double difference phase reporting for phase based positioning enhancements

## High accuracy positioning using the carrier phase

In principle the carrier phase measurements can be considered as high accuracy ToA measurement with an ambiguity of N*wavelength. Especially for scenarios with good (dominant LOS path) LOS conditions this can be a method for high accuracy ToA measurements, especially for reference signals with limited bandwidth. Similar to ToA based method a calibration is required to compensate phase offsets resulting from implementation.

Proposal 6: 	Study the impact and feasibility of carrier phase calibration for phase based high accuracy positioning

# Conclusions

Based on the discussions, we made the following observations and proposals:

Observation 1: 	The phase change resulting from movement may be different for each multipath component

Proposal 1: 	The carrier phase shall be determinable for different components of the channel impulse response and shall be measured in the delay domain

Proposal 2: 	Support reporting the phase measurements from the delay domain for the first and additional paths.

Proposal 3:     	Consider one or more of the following options for the delay domain phase measurement: 
- Option 1: The first path and N-path reporting is extended by a phase value for each reported path
- Option 2: A magnitude the complex valued correlation value associated to the path is reported. 
- Option 3: The full complex valued CIR for a given length is reported.

- Option 4: Relevant parts of the complex valued CIR are reported.

Proposal 4: 	Support phase based smoothing for DL-TDoA and UL-TDoA methods in Rel-18.

Consider the signaling of the coherency transmission status to the UE (UE based positioning) or to the LMF to notify on the applicability of phase based smoothing.

Proposal 5: 	Allow the double difference phase reporting for phase based positioning enhancements

Proposal 6: 	Study the impact and feasibility of carrier phase calibration for phase based high accuracy positioning