3GPP TSG RAN WG1 #109-e	R1- 2204837

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.4.1

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion & Decision

# Introduction

In this contribution different aspects related to AI/ML positioning use-cases and evaluation according to objectives of the SID in RP-213599 [1] will be discussed.

# Positioning uses cases identification and evaluation

Three possible accuracy enhancements using ML based technologies can be foreseen for NR:

- Use case 1: ML measurement accuracy and reliability enhancements: 
The ToA/AoA measurement accuracy depends highly on the channel condition. ML based technologies can either enhance the accuracy or can provide channel classification information for reliability estimation, for example.

- Use case 2: ML based positioning algorithms:
Positioning methods making use of environmental information can be realized using ML based concepts.

- Use case 3: ML assisted algorithms for NR positioning methods: 
For 3D positioning algorithms at minimum 4 (TDOA) or 3 (RTT) measurements are required to determine the 3D position. If more measurements are available the measurement quality should be taken into account. The delay or direction measurements can provide a reliability indicator or information on the reception signal sufficient for classification of the measurement quality. Furthermore, some positioning algorithms may provide several potential solutions. ML assisted methods can be used to identify and resolve ambiguities.

The signaling ML assisted algorithms can be mostly supported by Rel-16 and Rel-17 specification and hence enabled by implementation. Optional some additional reporting may be worthwhile. Nevertheless we focus in this contribution on the discussion of the first identified two use cases.

## Use case 1: ML measurement accuracy and reliability enhancements

Part of the objectives in [1] includes:

… - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] …

During the Rel 17 WI, the impact of multipath signals arriving within the width of the “main lobe” was discussed as part of challenging scenario degrading the positioning performance despite the presence of a weak LOS path. Since high accuracy with high availability is required attention on scenarios typical for real-world deployments is essential. Evaluations based on the TR38.901 channel models might cover such scenarios with low probability of occurrence only. For positioning especially, the characteristics of the first arriving signals (LOS, ground reflections, objects close to the UE or TRP = “early cluster”) have a high impact to the estimation accuracy and require super resolution technologies able to estimate the time-of-arrival of the first path with high accuracy. For the evaluation of the possible enhancements attention shall be put on these conditions. Figure 1 shows the early clusters reception obtained from ray-tracing near the LOS path while similar observations where provided in [2] with simulated and real-world measurements.

LOS 
tap

LOS 
tap

Taps causing no or minor impairment to correlation peak related to LOS („late Cluster“)

Taps causing no or minor impairment to correlation peak related to LOS („late Cluster“)

Taps with low delay („early Cluster“)

Taps with low delay („early Cluster“)

Figure 1 -  InF-LOS CIR generated with ray-tracing

Figure 2 show the resulting correlation function (using a SRS with 272RB and a cyclic correlation) for the given CIR example. We are mainly interested in the characteristics of the peak related to the LOS component. Hence the figure split the correlation function into the part resulting from the LOS component (blue, dashed), the multipath components (orange) and the composite (yellow). It is observed, that the peak of the composite signal is shifted by app. 25cm. The example covers a “good LOS” scenario with a K-factor = 6dB and assuming ideal UE antenna. Further challenging scenarios as for example described in [2] shall be considered.

Figure 2 - Comparison of “wideband CIR” and correlation function (for bandwidth 100MHz)

### ML model pipeline for use case 1

Part of the objectives in [1] includes:

… - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating Dataset(s) for training, validation, testing, and inference …

The ML model learns the spatial correlations of the multipath components with the first direct path of arrival (FDPoA) to identify the TOA of the FDPoA. During a training phase, the model learns to map the CIR to a corresponding ToA. For measured CIRs (e.g.: the correlation function) with unknown ToA the model predicts the ToA or assists other methods.

Different options are considered for the deployment of the concept:

- The measurement unit includes already the model. The model can be installed by the vendor or deployed by the network. Existing reporting protocols may be used or extended. Especially for TDOA based methods it may be worthwhile to inform the positioning algorithm about the used method or the positioning algorithm requests a common method from all measurement units.

- The model is implemented in the network (e.g.: part of the LMF). In this case sufficient input data shall be provided to the model. The measured correlation may be preprocessed and reported to the model.

Preprocessing/preparation of input data. In our evaluation we preprocessed the CIR in two variants:

- Magnitude: The first variant uses the magnitude of the CIR to have 1D input sequence for the neural network, not exploiting the phase information of the signal.

- IQ: The second variant uses IQ information of the CIR by creating a 2D vector with the stacked I and Q parts. To accelerate the processing of the neural network, the input vectors are normalized with a constant factor.

Training phase. For the training, a representative dataset has to be provided, representing CIR with similar propagation conditions and channel (i.e. bandwidth, …) of the target environment. Furthermore, the training data shall cover a wide range of possible CIRs. The training data can be either captured by measurement or generated in simulation. If generated in simulation, the true ToA is known which simplifies the generation of high accuracy training data. A careful selection of the channel parameter is required to ensure that also critical scenarios are well covered by the training.

Estimation phase. After training the neural network, the model can be deployed on the target device to estimate TOA of the FDPoA for unseen CIRs. Like in the training phase, the CIRs have to be preprocessed before providing the signals to the neural network.

### Evaluation

For the evaluation we apply a SRS-pos with reference signal different bandwidth to an extended “link level simulator” (LLS). The LLS is based on the system level channel models defined by TR38.901 and extensions of it (wider parameter sets, ground reflections enabled, early cluster,…). Pathloss models and power control features are supported as well. We found that the performance highly depends on the characteristics of the early cluster. Hence, the evaluations characterizes the ToA measurement accuracy performance versus the early clusters strength. We used a criterion similar as the K-factor (KF), but we considered only multipath components arriving with a low delay for the calculation. The “KFearly 10ns” considers multipath components arriving within a time window of 10ns relative to the LOS delay. The scenarios where a LOS component is still available, but impaired by multipath components with low delay is typically called “obstructed LOS” (OLOS). For the statistical evaluation we sorted the data in three sets.

- OLOS-Light		KF-early = [0,20] dB

- OLOS-Moderate 	KF-Early = [-10,10] dB

- OLOS-Heavy 		KF-Early = [-100,-10] dB

The performance of the ML based variants was compared with a satisfactory classical approach which estimates the rising edge of the FDPoA. From the results in Figure 3 we make the following observations:

Observation 1: In OLOS conditions influenced by the presence of early clusters, the ToA

performance for:

- ML based measurements outperforms the classical approaches and assure high accuracy at high availability

- Classical approaches may fail to achieve the Rel-17 accuracy targets even in Light OLOS condition

Observation 2: Information available in the measured IQ CIR (complex valued CIR) are valuable for the performance of ML approaches especially in heavy OLOS scenarios.

Proposal 1: Support the positioning use cases for ML measurement accuracy and reliability enhancements in Rel-18.

Figure 3 - ToA performance with ML-Magnitude, ML-IQ and a classical approach in an early cluster (a) light , (b) moderate, (c) heavy scenarios

(a)

(b)

(c)

## Use case 2:AI/ML Improved measurement accuracy in heavy NLOS conditions

### Use case description and motivation

Rel-16 introduced different UL and DL positioning methods to support timing and angular based solutions. In Rel-17, accuracy enhancements focused on the LOS and NLOS classification and additional path reporting in addition to introducing RSRPP measurements. Nevertheless, NR positioning solutions are dependent on the LOS conditions while NR positioning solutions in NLOS were left for discussions in Rel-18 since approaches are mostly AI/ML related or enabled.

We consider the example scenario in Figure 3 with the target to position UE2 in heavy NLOS conditions. In the scenario, we have multiple UEs at different positions resulting in different channel conditions w.r.t. the TRPs: UE1 having sufficient number of LOS links and achieves desired accuracy with NR-positioning methods. UE2 performance is highly deteriorated due from partly blocked TRPs on the most parts of the track. UE4 is in a good LOS and DOP condition and can serve as a temporal PRU (or temporal anchor). UE3 is governed by a low DOP gain however the information in the highlighted NLOS-area are not valid for the area of UE3.

In this scenario, we highlight reflecting objects at fixed positions as Virtual-TRPs wherein other blockers or unknown reflecting clusters are assumed to present. The Virtual-TRPs are expected to have certain known behavior on the received signal at different location w.r.t transmitter and receiver position and antenna configuration. In a related aspect, surrounding UEs with different positioning capabilities or in better channel conditions provides an art of temporal anchor or unit useful to identify relevant multipath components. The measurements can provide further information for the localization of devices in heavy NLOS areas. Furthermore the measurements can be used as  “on the side” training information and/or information to check the validity of the model.

Figure 5 - Example for NLOS positioning

### Evaluation

Figure 6 shows a real world evaluation of an indoor scenario. Parts of the areas may be well covered TDOA or RTT based positioning technologies. In areas where the number of TRPs received under LOS conditions is not sufficient (“NLOS areas”) or areas with low accuracy according the DOP criterion (typically in the corners of an area) these methods may fail. For ML based concepts a gain is especially expected for “critical areas”, whereas the “traditional methods” provide sufficient accuracy for LOS areas. Hence, the evaluation of the gain achieved by ML based concepts shall focus on “critical areas”. To reduce the effort for training it may be worthwhile to support ML based technologies only for areas where other technologies fail or provide a low accuracy only.

As an example, shown in Figure 6(a), we selected for the scenario three areas (A1, A2 and A3) where we expect reduced performance for multi-RTT or TDoA based position concept. A ML model is trained only using data from these areas. For the evaluation, we tested the performance of the trained model on the entire area including the areas used for the training and the unknown areas not used for training, where we assume that these areas may be covered by other positioning methods. The results of combining the ML data with data from other technologies is not depicted.

For the visualization we use two scales.

- Figure 6(b): Depicts the error and shall demonstrate in which area the ML-based approach is able to provide position estimated

- Figure 6(c): For the accuracy evaluation in the areas covered by the training we set errors exceeding 1.2m to the value 1.2m better see the behavior inside the area covered by the training.

For the areas covered by the training data a high accuracy can be achieved. At the edge of the training area the performance degrades. As expected, the performance in areas not covered by the training the performance of ML based positioning is reduced, or the position is considered as invalid. However, in some transition areas between the training sections (i.e. A1 and A3 and partly also between A2 and A1/A3 the model can interpolate in between.

| (a) | (b) (without clipping) |
| --- | --- |
| (c) (large errors clipped to 1.2m) |  |

Figure 6 - ML based positioning in a real world scenario: (a) scenario TRPs in green, blockers/reflectors in Red and highlighted training areas A1, A2 and A3,  (b) and (c) Performance in the whole positioning area

Observation 3:  For Positioning ML approaches trained with environment information a high accuracy (< 0.3m in the example) is achieved, if the evaluation areas was covered by the training data.

Observation 4:	At the edge of the training area the performance may already degrade

Observation 5:	Even in areas between the training areas ML based approaches are able to provide an position estimate.

Proposal 2: 	Evaluate the performance of ML based positioning technologies for at least two areas:
- Areas was covered by the training data
- Area is in between areas covered by the training.

Proposal 3: 	Training may be performed for selected areas only (“training spots”). Study the performance degradation for areas outside the training spots versus distance between training spots.

# Conclusion

Observation 1: In OLOS conditions influenced by the presence of early clusters, the ToA  performance for:

- ML based measurements outperforms the classical approaches and assure high accuracy at high availability

- Classical approaches may fail to achieve the Rel-17 accuracy targets even in Light OLOS condition

Observation 2: Information available in the measured IQ CIR (complex valued CIR) are valuable for the performance of ML approaches especially in heavy OLOS scenarios.

Observation 3:  For Positioning ML approaches trained with environment information a high accuracy (< 0.3m in the example) is achieved, if the evaluation areas was covered by the training data.

Observation 4:	At the edge of the training area the performance may already degrade

Observation 5:	Even in areas between the training areas ML based approaches are able to provide an position estimate.

Proposal 1: 	Support the positioning use cases for ML measurement accuracy and reliability enhancements in Rel-18.

Proposal 2: 	Evaluate the performance of ML based positioning technologies for at least two areas:
- Areas was covered by the training data
- Area is in between areas covered by the training.

Proposal 3: 	Training may be performed for selected areas only (“training spots”). Study the performance degradation for areas outside the training spots versus distance between training spots.

# References

- 3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, Electronic Meeting, December 6th – 17th, 2021.

- 3GPP R1-2109866, “Potential positioning enhancements for multipath/NLOS mitigation” Fraunhofer IIS Fraunhofer HHI

DOP: Dilution of precision. This method characterizes the quality of the anchor position for positioning algorihms.