3GPP TSG RAN WG1 #109-e	 R1-2204838

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.4.2

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	On potential specification impact of AI/ML for positioning

Document for:	Discussion & Decision

# Introduction

Part of the objective of the SID in RP-213599 on study of the 3GPP framework for AI/ML for air-interface with regards to potential specification impact consists of:

… - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we provide our views on potential specification impact for the positioning use cases. According to the evaluation and observations in [2] two use case are proposed, namely:

- ML measurement accuracy and reliability enhancements:

- ML based positioning algorithms

For these use cases we provide our views on potential specification impact.

# Potential specification impact of ML Positioning uses cases

## Use case 1: ML measurement accuracy and reliability enhancements

To support the use of ML for measurement accuracy enhancements identified in [2]. the ML model has to be trained and deployed at the NW side or the UE or TRP. Depending on the relevant architecture and position methods (UE-based, UE-assisted, network based) the requirements to the related interfaces must be studied. Assuming that a neural network (“model”) is trained using measurements performed on RSs, the related reporting shall cover the data relevant for ML/AI enhanced positioning. We consider especially two cases:

- UE-based positioning: In this case the model may be deployed to the UE. The network may provide data for model maintenance or even assistance data for training (fine-tuning of the models or re-training).

- Network (LMF) based positioning: In this case the UE (UE-assisted methods) and/or the LMU must provide sufficient data to the model.

Proposal 1:  Support signaling and reporting enhancements on LPP / NRPPa to enable ML measurement approaches for accuracy improvements

Proposal 2:  Study the reporting enhancements to enable ML measurement accuracy including IQ reporting and selection criteria for the additional path reporting.

## Use case 2: ML based positioning algorithms

ML based positioning methods may require environment dependent training of the model. The main challenge is the management of the training and live cycle management of model. Two scenarios are distinguished:

- UE based positioning: 
The model (a CNN, for example) may be implementation specific.

- Network based positioning: 
The model may be managed by the network operator. For ML based positioning algorithms additional measurements or extended reports may enhance the positioning accuracy. Also for UL-TDOA, UL-AoA and multi-RTT technologies where the measurements are performed by the RAN. This might require management of the neural network in a given positioning area.

The collection of measurements and labels for training the ML model can be done at the LMF. The UEs and TRPs may provide measurement and additional information to enable training of the model. To support ML, the LMF may need to collect additional measurements that collected using the positioning methods currently specified. To support AI/ML in NLOS and challenging LOS (“OLOS” or bad DOP conditions) areas following aspects needs to be further studied:

- Additional reporting for environment information in the training phase

  - Identification of ML-Assisted area, including conditions to request UEs to report measurements in the training phase and updating the training model. In one example, UE with AD to detect V-TRP relevant for positioning in NLOS environments

  - Study improvements by introducing calibration and association spots as guideline reporting of UE-A and calibration for UE-B. In some scenarios, the calibration spot can provide information on the LOS attenuation providing assistance on the threshold level a UE or TRP need to apply to correctly acquire the LOS.

  - Additional signaling needed for making use of Virtual-TRPs

- Temporal PRUs or anchors to maintain the AI/ML model or to enhance accuracy by supporting sidelink measurements, for example.

Proposal 3: Study in Rel-18 the following aspects to support AI/ML in challenging positioning environments:

  - Additional reporting for environment information in processing and training phase

  - Study improvements by introducing calibration and association spots as guideline reporting of UE-A and calibration for UE-B.

  - Additional signaling needed for making use of Virtual-TRPs

  - Temporal PRUs/anchors to enhance accuracy and maintain the AI/ML model

Proposal 4:  For UE-based AI/ML models may be subject of implementation. Identify requirements for the network for the maintenance of such models.

# Conclusion

We made the following observations and proposals in this contribution:

Proposal 1:  Support signaling and reporting enhancements on LPP / NRPPa to enable ML measurement approaches for accuracy improvements

Proposal 2:  Study the reporting enhancements to enable ML measurement accuracy including IQ reporting and selection criteria for the additional path reporting.

Proposal 3: Study in Rel-18 the following aspects to support AI/ML in challenging positioning environments:

  - Additional reporting for environment information in processing and training phase

  - Study improvements by introducing calibration and association spots as guideline reporting of UE-A and calibration for UE-B.

  - Additional signaling needed for making use of Virtual-TRPs

  - Temporal PRUs/anchors to enhance accuracy and maintain the AI/ML model

Proposal 4:  For UE-based AI/ML models may be subject of implementation. Identify requirements for the network for the maintenance of such models.

# References

- 3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, Electronic Meeting, December 6th – 17th, 2021.

- 3GPP R1-2204837, “Evaluation on AI/ML for positioning accuracy enhancements” Fraunhofer IIS Fraunhofer HHI