3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204839

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.1

Source:	NVIDIA

Title:	On general aspects of AI and ML framework for NR air interface

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

One objective of the SI is to investigate AI/ML framework for NR air interface:

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

In this contribution, we discuss the general aspects of AI/ML framework for NR air interface.

# 2	Background

AI applications—running on vast data centers in the cloud—have already transformed many industries. From retail to infotainment, AI has augmented human cognition and decision making, automated routine tasks, and unearthed unique insights from data. AI enables enterprises to detect fraud, improve customer relationships, optimize the supply chain, and deliver innovative products and services in an increasingly competitive marketplace.

Running AI applications at the edge brings the benefits of localization by processing data closer to where it is generated, captured, and used. This enables enterprises to react and adapt based on local conditions and requirements. Functionally, the edge cloud inherits most principles, mechanisms, and tools from the data center cloud. In addition, its proximity to end users brings new benefits such as lower latency for time-sensitive applications, transport and backhaul efficiencies for data-intensive applications, and data compliance for regulatory reasons.

Since AI is a highly compute-intensive process, having the right infrastructure optimized for the unique demands of AI workloads at the edge is essential. Data center clouds typically use lots of GPU-accelerated server infrastructure to ensure their high efficiency and effectiveness. Bringing this supercomputing capability to the edge requires integrating GPU-accelerated servers into the edge, together with specialized provisioning, management, and monitoring.

5G is poised to overcome the challenges of creating a seamless connection between data center AI and edge AI. Running AI applications on 5G will bring the true vision of AI applications anytime, anywhere to reality. It will bring extended coverage, mobility support, improved reliability, and enhanced security. This will enable, for example, more powerful deep learning training and inference—making AI better able to guide traffic flows, route autonomous vehicles, make factory robots more efficient at picking and packing goods, and more.

Observation 1: Running AI applications on 5G will bring the true vision of AI applications anytime, anywhere to reality.

On the other hand, 5G systems are becoming increasingly complex for supporting various deployment scenarios and applications. They generate humongous data. The use of rapidly advanced AI techniques, such as ML and data analytics, is critical to managing the complexity, identifying patterns in data, optimizing network design, enhancing system performance, and reducing operating costs. The current use of AI techniques in 5G systems primarily relies on proprietary implementations and solutions. To reap the full benefits from AI techniques in 5G and beyond, we must optimize the integration of AI into the system design and realization, including the air interface.

Observation 2: Integrating AI techniques in 5G systems, including NR air interface, is crucial to further enhancing 5G performance.

# 3	Defining stages of AI/ML related algorithms

AI/ML related algorithms vary in scale and complexity, but their general defining stages are similar. Figure 1 provides an illustration of the general defining stages of AI/ML related algorithms.

Figure 1: Defining stages of AI/ML related algorithms.

The functions of the defining stages of AI/ML related algorithms are described below.

- Data extraction is a stage where the relevant data for the AI/ML algorithm is extracted.

- Data validation is a stage where the decision on whether to proceed with (re)training the model is made. For example, at this stage, we validate if there are data schema skews which are considered anomalies in the input data. Examples of schema skews include receiving unexpected features, not receiving all the expected features, or receiving features with unexpected values. As another example, at this stage, we may check data values skews. If there are significant changes in the statistical properties of data, a retraining of the model is necessary to capture the changes.

- Data preparation is a stage where data cleaning, data transformation, and feature engineering are performed and where the data is split into training, validation, and test sets.

- Model training is a stage where various AI/ML modes are trained with different algorithms and hyperparameter tuning. The output of this stage is a trained model.

- Model evaluation is a stage where the quality of the trained model is evaluated using the test data set. The output of this stage is a set of metrics that characterize the quality of the model.

- Model validation is a stage where whether the model is adequate for deployment is validated. Examples include checking whether the model predictive performance is better than a certain baseline, validating the model performance is consistent on various slices of the data, testing mode for deployment such as infrastructure compatibility, among others.

- Model serving is a stage where the validated model is deployed to a target environment to serve predictions.

- Performance monitoring is a stage where the model predictive performance is monitored to potentially trigger a retraining of the model.

- Trigger is a stage where the retraining of the model is triggered. Depending on the use cases, the trigger can be on demand, on a schedule, on availability of new training data, on model performance degradation, on significant changes in the data distributions, among others.

Proposal 1: Characterize the defining stages of AI/ML related algorithms as Data extraction, Data validation, Data preparation, Model training, Model evaluation, Model validation, Model serving, Performance monitoring, and Trigger.

Proposal 2: The defining stages of AI/ML related algorithms, i.e., Data extraction, Data validation, Data preparation, Model training, Model evaluation, Model validation, Model serving, Performance monitoring, and Trigger, can serve as the basis for the characterization of the lifecycle management of AI/ML models.

# 4	Levels of collaboration between UE and gNB

The following collaboration framework was identified earlier despite it is not a consensus.

- 0a) No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes.

- 0b) No collaboration framework with modified Air-Interface catering to efficient implementation-based AI/ML algorithms.

- 1) Inter-node assistance to improve the respective nodes’ AI/ML algorithms. This would apply to UEs getting assistance from gNBs (for training, adaptation, etc.) and vice-versa. This level does not require model exchange between network nodes.

- 2) Joint ML operation between UEs and gNBs. This level requires AI/ML model instruction or exchange between network nodes.

The framework above is a good starting point for categorizing different levels of collaboration between UE and gNB. However, the description is not entirely clear. For example, the difference between Category 0b) and Category 1) is ambiguous.

In general, the discussion on the levels of collaboration between UE and gNB would become more concrete when the study on the selected use cases progresses. After that, the categorization would become more straightforward.

Proposal 3: Categorization of the levels of collaboration between UE and gNB can be identified as the study on the selected use cases progresses.

# 5	Datasets for training, validation, testing, and inference

Data is the new oil in today’s age of AI. When it comes to telecom, despite the mobile systems generate humongous data, their general accessibility is limited for various reasons. Nonetheless, it would be beneficial to identify existing sets of real data and/or build up new sets of real data, as part of the 3GPP Rel-18 AI/ML study for NR air interface. Such efforts would pay off as it is anticipated that AI/ML will become increasingly more integrated into the 3GPP family of technologies from 5G Advanced to 6G.

Observation 3: Identifying existing sets of real data and/or building up new sets of real data will be of high value as it is anticipated that AI/ML will become increasingly more integrated into the 3GPP family of technologies from 5G Advanced to 6G.

Proposal 4: Identifying existing sets of real data should be part of the 3GPP Rel-18 AI/ML study for NR air interface.

Proposal 5: Companies are encouraged to contribute real data to the 3GPP Rel-18 AI/ML study for NR air interface to help start to build up sets of real data in 3GPP.

The 3GPP Rel-18 AI/ML study for NR air interface may need large, carefully labelled datasets to train AI/ML models. More diverse training data generally makes for more accurate AI models. The problem is gathering and labelling datasets that may contain a few thousand to tens of millions of elements is time consuming and often prohibitively expensive.

In other words, despite the importance of real data for AI/ML development cannot be emphasized enough, it is acknowledged that building up solid sets of real data would likely be a long-term effort which would take time to mature. Therefore, it is equally important that we use synthetic data in the 3GPP Rel-18 AI/ML study for NR air interface, as synthetic data is less expensive while being effective when generated properly.

Synthetic data reflects real-world data, mathematically or statistically. Developers of deep neural networks increasingly use synthetic data to train their models. Indeed, a 2019 survey of the field [2] calls use of synthetic data “one of the most promising general techniques on the rise in modern deep learning, especially computer vision” that relies on unstructured data like images and video. In particular, because synthetic datasets are automatically labelled and can deliberately include rare but crucial corner cases, it is sometimes better than real-world data.

3GPP has well established simulation methodology, which can be used to generate synthetic data.

Observation 4: Synthetic data reflects real-world data. It is less expensive compared to real-world data while being effective for AI/ML development when generated properly.

Observation 5: Developers of AI/ML models increasingly use synthetic data to train their models.

Observation 6: 3GPP has established simulation methodology, which can be used to generate synthetic data.

Proposal 6: Use of synthetic data should be the baseline for the 3GPP Rel-18 AI/ML study for NR air interface.

During the study of the 3GPP Rel-18 AI/ML study for NR air interface, additional simulation methodology for generating synthetic data, such as digital twins, can be explored. A digital twin is a virtual representation — a true-to-reality simulation of physics and materials — of a real-world physical asset or system, which is continuously updated. Digital twins can help generate synthetic data that are closer to real-world data, compared to the traditional 3GPP statistical simulation methodology.

Observation 7: Digital twins can help generate synthetic data that are closer to real-world data, compared to the traditional 3GPP statistical simulation methodology.

Proposal 7: Additional simulation methodology for generating synthetic data, such as digital twins, can be explored as part of the 3GPP Rel-18 AI/ML study for NR air interface.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: Running AI applications on 5G will bring the true vision of AI applications anytime, anywhere to reality.

Observation 2: Integrating AI techniques in 5G systems, including NR air interface, is crucial to further enhancing 5G performance.

Observation 3: Identifying existing sets of real data and/or building up new sets of real data will be of high value as it is anticipated that AI/ML will become increasingly more integrated into the 3GPP family of technologies from 5G Advanced to 6G.

Observation 4: Synthetic data reflects real-world data. It is less expensive compared to real-world data while being effective for AI/ML development when generated properly.

Observation 5: Developers of AI/ML models increasingly use synthetic data to train their models.

Observation 6: 3GPP has established simulation methodology, which can be used to generate synthetic data.

Observation 7: Digital twins can help generate synthetic data that are closer to real-world data, compared to the traditional 3GPP statistical simulation methodology.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Characterize the defining stages of AI/ML related algorithms as Data extraction, Data validation, Data preparation, Model training, Model evaluation, Model validation, Model serving, Performance monitoring, and Trigger.

Proposal 2: The defining stages of AI/ML related algorithms, i.e., Data extraction, Data validation, Data preparation, Model training, Model evaluation, Model validation, Model serving, Performance monitoring, and Trigger, can serve as the basis for the characterization of the lifecycle management of AI/ML models.

Proposal 3: Categorization of the levels of collaboration between UE and gNB can be identified as the study on the selected use cases progresses.

Proposal 4: Identifying existing sets of real data should be part of the 3GPP Rel-18 AI/ML study for NR air interface.

Proposal 5: Companies are encouraged to contribute real data to the 3GPP Rel-18 AI/ML study for NR air interface to help start to build up sets of real data in 3GPP.

Proposal 6: Use of synthetic data should be the baseline for the 3GPP Rel-18 AI/ML study for NR air interface.

Proposal 7: Additional simulation methodology for generating synthetic data, such as digital twins, can be explored as part of the 3GPP Rel-18 AI/ML study for NR air interface.

# Reference

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #94-e, December 2021.

S. I. Nikolenko, “Synthetic Data for Deep Learning,” September 2021. Available at https://arxiv.org/abs/1909.11512.