3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204841

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.2.2

Source:	NVIDIA

Title:	On other aspects of AI and ML for CSI feedback enhancement

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to finalize representative sub use cases and assess potential specification impact

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss the other aspects of AI/ML for CSI feedback enhancement.

# 2	Representative sub-use cases

AI/ML techniques, such as autoencoders, can be used in CSI compression and feedback. The basic principle of an autoencoder is to take an input, perform non-linear compression by an encoder to a lower-dimensional latent representation, and decompress the latent representation by a decoder to a defined target.

In the context of CSI compression and feedback, the UE’s estimated CSI is taken as the input to the encoder. The output of the UE’s encoder (i.e., the latent representation of the channel in a quantized form) is transmitted over the air interface to the gNB. The gNB uses a matching decoder to reconstruct the CSI. Hence, the autoencoder model is divided between the UE and gNB, creating an inter-node dependency for model training and model management.

Figure 1 provides an illustration of autoencoder based CSI feedback.

Figure 1: Illustration of CSI autoencoder.

Observation 1: Autoencoder based CSI feedback is a promising AI/ML technique for CSI feedback enhancement.

Proposal 1: Autoencoder based CSI feedback enhancement should be selected as one representative sub use case.

# 3	Potential specification impact

For autoencoder based CSI feedback, one important question is how to train an autoencoder model for deployment. There are different training options that have various levels of collaboration between UE and gNB and can lead to different levels of specification impact:

- Option 1: Network performs autoencoder training

- Option 2: UE performs autoencoder training

- Option 3: Joint training between network and UE via federated learning

- Option 4: Joint training between network and UE via split learning

These different options are good starting points to investigate various levels of collaboration between UE and gNB, which is a key objective of the study item.

Proposal 2: Study different training methods for autoencoder based CSI feedback including at least the following options:

- Option 1: Network performs autoencoder training

- Option 2: UE performs autoencoder training

- Option 3: Joint training between network and UE via federated learning

- Option 4: Joint training between network and UE via split learning

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: Autoencoder based CSI feedback is a promising AI/ML technique for CSI feedback enhancement.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Autoencoder based CSI feedback enhancement should be selected as one representative sub use case.

Proposal 2: Study different training methods for autoencoder based CSI feedback including at least the following options:

- Option 1: Network performs autoencoder training

- Option 2: UE performs autoencoder training

- Option 3: Joint training between network and UE via federated learning

- Option 4: Joint training between network and UE via split learning

# Reference

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #94-e, December 2021.