3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204842

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.1

Source:	NVIDIA

Title:	On evaluation assumptions of AI and ML for beam management

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to evaluate performance benefits of AI/ML based algorithms:

- Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

  - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

  - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

  - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

  - Consider adequate model training strategy, collaboration levels and associated implications

  - Consider agreed-upon base AI model(s) for calibration

  - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

- KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

  - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

  - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, we discuss the evaluation assumptions for beam management.

# 2	Methodology

NR can operate at a wide range of frequencies, ranging from sub-6 GHz to millimeter wave frequencies. To support operation over such a wide range of carrier frequencies, NR has been designed to utilize beam-based operation, where gNB and UE may use transmit and receive beamforming for all channels and signals.

Beam management procedures include beam determination, beam measurement and reporting, beam indication and switching, and beam recovery. In beam determination, the gNB and UE find suitable transmit and receive beam directions for communication. In beam measurement and reporting, the UE measures the link quality of multiple beam pairs and reports the measurement results to the gNB. In beam indication and switching, the gNB can indicate to the UE which beam to use for the communication, including switching to a beam different from the current beam. In beam recovery, the UE can attempt to re-establish a transmit-receive beam pair with the gNB from beam failure by using a L1 procedure without declaring radio link failure.

In general, beam management procedures consist of three steps:

- Procedure 1 (P-1): SSB-based beam sweeping

- Procedure 2 (P-2): CSI-RS based transmit-end beam refinement

- Procedure 3 (P-3): CSI-RS based receive-end beam refinement

AI/ML based algorithms may find applications in all the three steps of beam management. It is worthwhile discussing which of the three steps should be the focal point for evaluation.

Observation 1: AI/ML based algorithms may find applications in different beam management procedures.

Proposal 1: Discuss which of the three steps (P1, P2, and P3) should be the focal point for evaluating AI/ML based algorithms for beam management.

3GPP has established simulation methodology (see, e.g., TR 38.901 [2]), which can be used to generate synthetic data for the study of AI/ML based algorithms for beam management.

Proposal 2: Use the simulation assumptions in TR 38.901 as a starting point for the evaluation of AI/ML based algorithms for beam management.

Beam management is particularly relevant for NR operation in millimeter wave frequencies. The typical millimeter wave deployment is in urban scenarios where there is high-capacity demand. Therefore, the evaluation could focus on UMi-street canyon and UMa scenarios,

Proposal 3: The evaluation scenarios for beam management include UMi-street canyon and UMa scenarios.

Proposal 4: Focus on millimeter wave frequencies for the evaluation of AI/ML based algorithms for beam management.

However, the simulation layout for UMi-street canyon and UMa scenarios described in TR 38.901 is much simplified compared to real-world urban scenarios. Additional simulation methodology for generating synthetic data, such as digital twins, can be explored. A digital twin is a virtual representation — a true-to-reality simulation of physics and materials — of a real-world physical asset or system, which is continuously updated. Digital twins can help generate synthetic data that are closer to real-world data, compared to the traditional 3GPP statistical simulation methodology.

Proposal 5: Additional simulation methodology for generating synthetic data, such as digital twins, should be explored for the study of AI/ML based algorithms for beam management.

While synthetic data can be the baseline for evaluating AI/ML based algorithms for beam management, it would be beneficial to identify existing sets of real data and/or build up new sets of real data, as part of the 3GPP Rel-18 AI/ML study for NR air interface. Such efforts would pay off as it is anticipated that AI/ML will become increasingly more integrated into the 3GPP family of technologies from 5G Advanced to 6G.

Proposal 6: Identifying existing sets of real data should be part of the evaluation work for AI/ML based beam management.

Proposal 7: Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for beam management.

As the Rel-18 study on AI/ML for NR air interface is the first one in 3GPP that explores the benefits of augmenting air interface with features enabling improved support of AI/ML based algorithms, it is important to calibrate evaluation results from different companies in order to facilitate drawing observations and making conclusions. To this end, it is beneficial to agree on baseline AI model(s) for calibration.

Proposal 8: Baseline AI model(s) should be identified for the purpose of calibration in the study of AI/ML based algorithms for beam management.

# 3	KPIs

The study item description lists many dimensions for KPIs, including performance, inference latency, computational complexity, overhead, power consumption, memory storage, hardware requirements, and generalization capability.

Though it is beneficial to have a full characterization of the performance of AI/ML based algorithms for NR air interface, it is important to focus on a few most important KPIs in the initial phase to understand the gains of AI/ML based algorithms.

From beam management perspective, overhead reduction, latency reduction, and accuracy improvement are highlighted in the study item description.

- Overhead may include reference signal overhead and measurement reporting overhead. With beam prediction, less frequent reference signal transmission and/or less measurement reporting may be needed.

- Latency may include reference signal transmission time, UE measurement time, UE measurement reporting time, among others. With beam prediction, the latency may be reduced.

- Beam management based on UE measurement and reporting may not be accurate due to UE measurement error and the feedback delay in UE measurement reporting. Beam prediction may improve the accuracy.

Also, many of the KPIs such as inference latency depend on the used computing platform (such as the GPU model). Therefore, it is important to report the KPIs together with the used computing platform (such as the GPU model).

Proposal 9: Define a KPI which measures the reference signal overhead for beam management with and without AI/ML based algorithms.

Proposal 10: Define a KPI which measures the measurement reporting overhead for beam management with and without AI/ML based algorithms.

Proposal 11: Define a KPI which measures the latency for beam management with and without AI/ML based algorithms.

Proposal 12: Define a KPI which measures beam management accuracy with and without AI/ML based algorithms.

Proposal 13: Provide information on the used computing platform (such as the GPU model) when reporting KPIs for AI/ML based beam management.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: AI/ML based algorithms may find applications in different beam management procedures.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Discuss which of the three steps (P1, P2, and P3) should be the focal point for evaluating AI/ML based algorithms for beam management.

Proposal 2: Use the simulation assumptions in TR 38.901 as a starting point for the evaluation of AI/ML based algorithms for beam management.

Proposal 3: The evaluation scenarios for beam management include UMi-street canyon and UMa scenarios.

Proposal 4: Focus on millimeter wave frequencies for the evaluation of AI/ML based algorithms for beam management.

Proposal 5: Additional simulation methodology for generating synthetic data, such as digital twins, should be explored for the study of AI/ML based algorithms for beam management.

Proposal 6: Identifying existing sets of real data should be part of the evaluation work for AI/ML based beam management.

Proposal 7: Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for beam management.

Proposal 8: Baseline AI model(s) should be identified for the purpose of calibration in the study of AI/ML based algorithms for beam management.

Proposal 9: Define a KPI which measures the reference signal overhead for beam management with and without AI/ML based algorithms.

Proposal 10: Define a KPI which measures the measurement reporting overhead for beam management with and without AI/ML based algorithms.

Proposal 11: Define a KPI which measures the latency for beam management with and without AI/ML based algorithms.

Proposal 12: Define a KPI which measures beam management accuracy with and without AI/ML based algorithms.

Proposal 13: Provide information on the used computing platform (such as the GPU model) when reporting KPIs for AI/ML based beam management.

# Reference

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #94-e, December 2021.

TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz.”