3GPP TSG-RAN WG1 Meeting #109-e	R1- 2204843

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.3.2

Source:	NVIDIA

Title:	On other aspects of AI and ML for beam management

Document for:	Discussion

# 1	Introduction

The approval of the Rel-18 work package marks the start of 5G Advanced evolution. The package includes a study item on AI/ML for NR air interface, and the work item description can be found in [1].

The initial use cases focused in this study include:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration, the study aims to finalize representative sub use cases and assess potential specification impact:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss the other aspects of AI/ML for beam management.

# 2	Representative sub-use cases

5G NR can operate at a wide range of frequencies, ranging from sub-6 GHz to millimeter wave frequencies. To support operation over such a wide range of carrier frequencies, NR has been designed to utilize beam-based operation, where gNB and UE may use transmit and receive beamforming for all channels and signals.

Initial transmit and receive beam finding is used in initial access and connected UEs on, for example, a non-standalone NR carrier in MR-DC. The gNB transmits multiple SSBs that are beamformed in different directions. Up to a maximum of 64 such SSBs can be transmitted in a burst of 5 ms within a period of 20 ms. The UE detects an SSB and stores the information related to the corresponding receive beam. After detecting the SSB and decoding the MIB, the UE knows which SSB it has detected.

In initial access, the UE may use the same receive beam to receive SIB1 as it used to receive the detected SSB. SIB1 conveys the association information which links the index of a SSB to a corresponding set of PRACH preambles in time and frequency. In a non-standalone NR carrier in MR-DC, the UE does not need to read SIB1 and instead may get the information from dedicated RRC signaling. The UE would perform PRACH transmission in the PRACH resource associated with its detected SSB.

The gNB can detect the PRACH using a beam direction same as the beam direction it used to transmit the SSB. When the gNB detects a PRACH from the UE, it knows which SSB the UE detected and can determine its transmit beam for communicating with the UE. The transmit and receive beams established in initial transmit and receive beam finding are used to complete random access procedure and the subsequent procedures before dedicated RRC configuration is set up.

In general, beam management procedures consist of three steps:

- Procedure 1 (P-1): SSB-based beam sweeping

- Procedure 2 (P-2): CSI-RS based transmit-end beam refinement

- Procedure 3 (P-3): CSI-RS based receive-end beam refinement

AI/ML techniques can be used to predict beam in time and/or spatial domain, which can reduce the overhead and latency associated with all the three steps of beam management.

Observation 1: AI/ML techniques can be used to predict beam in time and spatial domain, which can reduce the overhead and latency associated with the P1/P2/P3 beam management procedures.

Proposal 1: Beam prediction should be selected as one representative sub use case.

# 3	Potential specification impact

Although it is likely that most of the AI/ML algorithms for beam prediction can be up to implementation, there are several aspects deserving studies in 3GPP. These include:

- Signalling support for the training and execution of AI/ML models for beam prediction

- Data required by the AI/ML models for beam prediction (e.g., data reported by UE to gNB, assistance data from gNB to UE)

- Outputs generated by the AI/ML models for beam prediction to be delivered from gNB to UE or from UE to gNB

Proposal 2: Study the signalling support for the training and execution of AI/ML models for beam prediction.

Proposal 3: Study the data required by AI/ML models for beam prediction (e.g., data reported by UE to gNB, assistance data from gNB to UE).

Proposal 4: Study how to deliver outputs generated by AI/ML models for beam prediction from gNB to UE and from UE to gNB.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: AI/ML techniques can be used to predict beam in time and spatial domain, which can reduce the overhead and latency associated with the P1/P2/P3 beam management procedures.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Beam prediction should be selected as one representative sub use case.

Proposal 2: Study the signalling support for the training and execution of AI/ML models for beam prediction.

Proposal 3: Study the data required by AI/ML models for beam prediction (e.g., data reported by UE to gNB, assistance data from gNB to UE).

Proposal 4: Study how to deliver outputs generated by AI/ML models for beam prediction from gNB to UE and from UE to gNB.

# Reference

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP TSG RAN #94-e, December 2021.