3GPP TSG RAN WG1 #109-e		                                   					       												                 R1-2204851

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.4

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR NTN

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [109-e-R17-UE-features-NR-NTN-01] during RAN1 #109-e. According to the Chairman’s Notes:

[109-e-R17-UE-features-NR-NTN-01] Email discussion on UE features for NR-NTN – Ralf (AT&T) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

The following was discussed and/or agreed during RAN1 #109-e within the scope of [109-e-R17-UE-features-NR-NTN-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in [1].

# Summary of Contributions Submitted to RAN1 #109-e

The following is the moderator’s summary of contributions submitted to RAN1 #109-e in this agenda item.

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE calculates common TA according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. |
| ZTE [3] | Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] | RAN2 raised concerns on both of FGs 26-1 and FGs 26-8 which have “N/A” in the column of “Need for the gNB to know if the feature is supported” while indicate in the column of “Mandatory/Optional” as “optional with capability signalling”. From RAN2 perspective, if there is no need for gNB to know whether a feature is supported or not, no capability signalling should be defined. RAN2 would like to know whether such capabilities are really “optional with capability signalling”. FGs 26-1 must be supported for UEs supporting NR communication via satellite, as described in the column of “Mandatory/Optional”. However, it is not mandated for UEs not supporting satellite communication but supporting other NTN scenarios, e.g. HAPS, ATG. In one word, UE can be optional with capability signalling and gNB needs to know if the feature is supported. As for FGs 26-8, gNB would indicate the polarization for UE with circular polarization to take the advantage of polarization information to save power. For UE with linear polarization, reading the polarization signalling may be unnecessary and UE can also work well in NTN even without the polarization signalling. However, in NTN, various UE types could coexist, e.g. UE with linear polarization, UE with LHCP, UE with RHCP, or with a combination of different polarization types. Thus, gNB could always indicate the polarization and UE can optionally support the feature without capability signalling. This should be captured into UE feature list, e.g. according to the text proposal we added in red in following list. 26. NR_NTN_solutions; 26-1; Uplink Time and Frequency pre-compensation and timing relationship enhancements; - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE calculates common TA according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information; NoYes; No; Release 17 NR UE cannot communicate via satellite; per band; No; No; No; An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]; Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported.. According to above, we have following proposal which is also captured in our draft reply LS [3]. Proposal 2: RAN1 should send a reply LS to RAN2, indicating: - FGs 26-1 should be optional with capability signalling and gNB needs to know if the feature is supported. - FGs 26-8 should be optional without capability signalling. |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] |  |
| Apple [8] | The feature 26-1 was updated or expanded to uplink time and frequency pre-compensation and timing relationship enhancements. This feature is mandatory in supporting NR communication via satellite. In the latest version of this feature, the field of “Need for the gNB to know if the feature is supported” is No. However, we think gNB needs to know if a UE supports this feature to serve it via satellite. Hence, we have the following proposal. Proposal 1: It is necessary for the gNB to know if the feature 26-1 is supported. |
| NTT DOCOMO, INC. [9] | One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG, or at the very least carry a note that they must be indicated as supported by UEs supporting NR communication via satellite. |
| Ericsson [12] | Align wording by consistent use of (e.g.) "support of". Editorial change (one comma added) 26. NR_NTN_solutions; 26-1; Uplink Time and Frequency pre-compensation and timing relationship enhancements; - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of pre-compensation of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of receiving cell-specific K_offset/K_mac in system information; No; No; Release 17 NR UE cannot communicate via satellite; per band; No; No; No; An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]; Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported.. |

| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optionalwith capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. |
| ZTE [3] | Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] |  |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] |  |
| Apple [8] |  |
| NTT DOCOMO, INC. [9] | One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. |
| Ericsson [12] | Editorial change (one space added) 26. NR_NTN_solutions; 26-4; UE reporting of information related to TA pre-compensation; 1. Support UE reporting of information related to TA pre-compensation; 26-1; Yes; No; UE does not support reporting of information related to TA pre-compensation for NR communication via satellite; Per band; No; No; No; Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]; Optional with capability signalling. |

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, the situation is a bit different compared to other “NTN-specific” FGs. As discussed in [1], the support of 32 HARQ processes can be beneficial for other scenarios for a UE supporting this feature, e.g. used for FR1 licensed terrestrial bands and FR2-1. For example, for the support of FR1 + FR2 CA, if the PUCCH is transmitted on FR1 with small SCS, then it can be expected that a small number of slots on FR1 would correspond to large number of slots on FR2, which will result in the need of larger number of HARQ processes on FR2, otherwise it will degrade the system performance for FR2. Another example scenario that would be beneficial from the support of 32 HARQ processes is multi-TRP case, in which case the need of number of HARQ processes would be increased. As a matter of fact, the possibility to extend the 32 HARQ processes has been discussed before as summarized in [2]. Consensus cannot be reached due to different flavors of HARQ process ID indication. However, in NTN, the explicit HARQ process indication has already been agreed. Therefore, there seems no obstacles to extend this FG in other scenarios. Overall, we see a clear benefit of extending this FG to other scenarios and suggest to remove the note in the last column. |
| ZTE [3] | Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] |  |
| Xiaomi [5] |  |
| Samsung [6] | NTN-TN mobility is one of the objectives of Rel-18 NR NTN enhancements. Therefore, 3GPP will start investigating systems with both NTN and TN components in Rel-18. In order to avoid any issues when discussing features from Rel-18 onward, we suggest we clarify that FG 26-5 covers operation with NTN carriers only. While this clarification has no impact for Rel-17, it will be useful in the future if there are scenarios where one carrier is for NTN and one for TN. For component description add “when a UE is configured with NTN carriers only” as follows. 26. NR_NTN_solutions; 26-5; Increasing the number of HARQ processes; The maximal supported HARQ process number is X for UL and Y for DL, when a UE is configured with NTN carriers only.; Yes; No; Increased number of HARQ processes is not supported for NR communication via satellite; Per band; No; No; No; Candidate component values for (X,Y): {(16,32),(32,16),(32,32)}; Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. |
| OPPO [7] |  |
| Apple [8] |  |
| NTT DOCOMO, INC. [9] | One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. |
| Ericsson [12] | Editorial change (deleted "supported]" 26. NR_NTN_solutions; 26-5; Increasing the number of HARQ processes; 1. The maximal supported HARQ process number is X for UL and Y for DL; Yes; No; Increased number of HARQ processes is not supported for NR communication via satellite; Per band; No; No; No; Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]; Optional with capability signalling. |

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. Remove FFS in 5-th column. |
| ZTE [3] | W.r.t FG 26-6, the prerequisite column can be updated from “FFS” to “4-10”, i.e., “Dynamic HARQ-ACK codebook”, since type-2 HARQ codebook should be first supported and then the enhancement on type-2 HARQ codebook can be performed. Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] |  |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] | The features 26-6, 26-6a, 26-6b are respectively defined for Type-1, Type-2, Type-3 HARQ codebook enhancement when there are feedback disabled HARQ processes. The pre-requisite features are open for these three features. In our view, UE needs to support the PDSCH reception associated with HARQ processes with feedback disabling, before it can construct the enhanced HARQ-ACK codebook construction. The feature of downlink HARQ disabling is defined in RAN2, which could serve as pre-requisite of features 26-6, 26-6a and 26-6b. Proposal 2: The pre-requisite of features 26-6, 26-6a and 26-6b are the RAN2 feature of downlink HARQ disabling. |
| Apple [8] |  |
| NTT DOCOMO, INC. [9] | At the last meeting, ‘UE supports HARQ disabling’ was removed from these FGs since RAN2 has the corresponding FG. Then pre-requisite of these FGs is FFS to discuss whether the RAN2 FG should be added in these FGs as pre-requisite. In our understanding, HARQ-ACK CB enhancements introduced in Rel-17 NTN WI are only related to feedback-disabling mechanism. To add the RAN2 FG as pre-requisite would be straightforward without any problem. The following is proposed based on this discussion. Proposal 2: - harq-FeedbackDisabled is added as pre-requisite of FGs 26-6/26-6a/26-6b. One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. In R2-2230551, FG for disabling HARQ feedback for downlink transmission is captured as below. Hence, prerequisite feature groups for 26-6, 26-6a and 26-6b should be x-2 where x is TBD. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG, or at the very least carry a note that they must be indicated as supported by UEs supporting NR communication via satellite. |
| Ericsson [12] |  |

| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | - Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. Remove FFS in 5-th column. |
| ZTE [3] | W.r.t FG 26-6a and 26-6b, the prerequisite column can be updated from “FFS” to “4-11”, i.e., “Semi-static HARQ-ACK codebook”, since type-1 and type-3 HARQ codebook should be first supported and then the enhancement on them can be performed. Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] |  |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] |  |
| Apple [8] | The features 26-6, 26-6a, 26-6b are respectively defined for Type-1, Type-2, Type-3 HARQ codebook enhancement when there are feedback disabled HARQ processes. The pre-requisite features are open for these three features. In our view, UE needs to support the PDSCH reception associated with HARQ processes with feedback disabling, before it can construct the enhanced HARQ-ACK codebook construction. The feature of downlink HARQ disabling is defined in RAN2, which could serve as pre-requisite of features 26-6, 26-6a and 26-6b. Proposal 2: The pre-requisite of features 26-6, 26-6a and 26-6b are the RAN2 feature of downlink HARQ disabling. |
| NTT DOCOMO, INC. [9] | At the last meeting, ‘UE supports HARQ disabling’ was removed from these FGs since RAN2 has the corresponding FG. Then pre-requisite of these FGs is FFS to discuss whether the RAN2 FG should be added in these FGs as pre-requisite. In our understanding, HARQ-ACK CB enhancements introduced in Rel-17 NTN WI are only related to feedback-disabling mechanism. To add the RAN2 FG as pre-requisite would be straightforward without any problem. The following is proposed based on this discussion. Proposal 2: - harq-FeedbackDisabled is added as pre-requisite of FGs 26-6/26-6a/26-6b. One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. In R2-2230551, FG for disabling HARQ feedback for downlink transmission is captured as below. Hence, prerequisite feature groups for 26-6, 26-6a and 26-6b should be x-2 where x is TBD. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG, or at the very least carry a note that they must be indicated as supported by UEs supporting NR communication via satellite. |
| Ericsson [12] |  |

| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. Remove FFS in 5-th column. |
| ZTE [3] | W.r.t FG 26-6a and 26-6b, the prerequisite column can be updated from “FFS” to “4-11”, i.e., “Semi-static HARQ-ACK codebook”, since type-1 and type-3 HARQ codebook should be first supported and then the enhancement on them can be performed. Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] |  |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] |  |
| Apple [8] | The features 26-6, 26-6a, 26-6b are respectively defined for Type-1, Type-2, Type-3 HARQ codebook enhancement when there are feedback disabled HARQ processes. The pre-requisite features are open for these three features. In our view, UE needs to support the PDSCH reception associated with HARQ processes with feedback disabling, before it can construct the enhanced HARQ-ACK codebook construction. The feature of downlink HARQ disabling is defined in RAN2, which could serve as pre-requisite of features 26-6, 26-6a and 26-6b. Proposal 2: The pre-requisite of features 26-6, 26-6a and 26-6b are the RAN2 feature of downlink HARQ disabling. |
| NTT DOCOMO, INC. [9] | At the last meeting, ‘UE supports HARQ disabling’ was removed from these FGs since RAN2 has the corresponding FG. Then pre-requisite of these FGs is FFS to discuss whether the RAN2 FG should be added in these FGs as pre-requisite. In our understanding, HARQ-ACK CB enhancements introduced in Rel-17 NTN WI are only related to feedback-disabling mechanism. To add the RAN2 FG as pre-requisite would be straightforward without any problem. The following is proposed based on this discussion. Proposal 2: - harq-FeedbackDisabled is added as pre-requisite of FGs 26-6/26-6a/26-6b. One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. In R2-2230551, FG for disabling HARQ feedback for downlink transmission is captured as below. Hence, prerequisite feature groups for 26-6, 26-6a and 26-6b should be x-2 where x is TBD. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. |
| Ericsson [12] |  |

| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [For UE supports NR communication via satellite, UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. Remove “[For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported]”. |
| ZTE [3] | Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] | RAN2 raised concerns on both of FGs 26-1 and FGs 26-8 which have “N/A” in the column of “Need for the gNB to know if the feature is supported” while indicate in the column of “Mandatory/Optional” as “optional with capability signalling”. From RAN2 perspective, if there is no need for gNB to know whether a feature is supported or not, no capability signalling should be defined. RAN2 would like to know whether such capabilities are really “optional with capability signalling”. FGs 26-1 must be supported for UEs supporting NR communication via satellite, as described in the column of “Mandatory/Optional”. However, it is not mandated for UEs not supporting satellite communication but supporting other NTN scenarios, e.g. HAPS, ATG. In one word, UE can be optional with capability signalling and gNB needs to know if the feature is supported. As for FGs 26-8, gNB would indicate the polarization for UE with circular polarization to take the advantage of polarization information to save power. For UE with linear polarization, reading the polarization signalling may be unnecessary and UE can also work well in NTN even without the polarization signalling. However, in NTN, various UE types could coexist, e.g. UE with linear polarization, UE with LHCP, UE with RHCP, or with a combination of different polarization types. Thus, gNB could always indicate the polarization and UE can optionally support the feature without capability signalling. This should be captured into UE feature list, e.g. according to the text proposal we added in red in following list. 26. NR_NTN_solutions; 26-8; Support of polarization signalling in NR NTN; - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration; No; No; UE is not able to take the advantage of polarization information to save power; Per band; No; No; No; [For UE supports NR communication via satellite, UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]; Optional with without capability signalling. According to above, we have following proposal which is also captured in our draft reply LS [3]. Proposal 2: RAN1 should send a reply LS to RAN2, indicating: - FGs 26-1 should be optional with capability signalling and gNB needs to know if the feature is supported. - FGs 26-8 should be optional without capability signalling. In RAN1 #108-e meeting, UE feature on support of polarization signalling in NR NTN is listed. If UE is equipped with linear polarization, reading such polarization signalling seems unnecessary as UE cannot be able to take the advantage of polarization information to save power. On the other hand, for UE supporting this feature, i.e. UE is capable of reading such polarization signalling, it’s not clear whether UE is required to support circular polarization or not and there could be different understandings: - UE is required to support circular polarization if UE is capable of reading the polarization signalling; - UE is not required to support circular polarization if UE is capable of reading the polarization signalling. In our understanding, UE should not be required to support circular polarization even if UE is capable of reading the polarization signalling, which should be clarified and captured in the feature description, e.g. according to the text proposal we added in red in below table. 26. NR_NTN_solutions; 26-8; Support of polarization signalling in NR NTN; - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration; No; No; UE is not able to take the advantage of polarization information to save power.; Per band; No; No; No; [For UE supports NR communication via satellite, UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] Note: UE supporting this feature is not required to support circular polarization.; Optional with capability signalling. |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] |  |
| Apple [8] |  |
| NTT DOCOMO, INC. [9] | One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. |
| Ericsson [12] |  |

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE Determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column. |
| ZTE [3] | Moreover, for the note column of FG26-1, 26-4, 26-5, 26-6, 26-6a, 26-6b, 26-8, 26-9, the note [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] should be removed since the listed FGs are defined per band, thus the note of each FG is implicitly associated with the band in which the corresponding FG is supported. |
| Vivo [4] |  |
| Xiaomi [5] |  |
| Samsung [6] |  |
| OPPO [7] |  |
| Apple [8] |  |
| NTT DOCOMO, INC. [9] | One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9. Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent. Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18. Based on the above, the following is proposed. Proposal 1: - The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell. - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell. |
| LG Electronics [10] | In the note column, there may be copy-and-paste error. It should be replaced by [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. However, the note can be removed as it can be left for UE to indicate this FG per band. |
| Nokia/Nokia Shanghai Bell [11] | - The following highlighted note can be found on these FGs: - [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] - However, it is not clear how such a limitation would be applicable in practice. - Are the UEs expected to report support for the feature conditionally on the type of cell? This would not follow the UE capability framework for NR. - Another interpretation is that this would be intended at limiting the gNB implementation to prevent usage of some features that UEs have already reported to support. It is questionable what would be benefits of such approach, especially considering the said FGs are reported per band already. - Remove the note above in all these FGs. These are basic features for UEs supporting NR over NTN.” Hence, they should be combined into a single FG, or at the very least carry a note that they must be indicated as supported by UEs supporting NR communication via satellite. |
| Ericsson [12] | Align wording by consistent use of (e.g.) "support of". 26. NR_NTN_solutions; 26-9; UE-specific K_offset; - Support reception of UE-specific K_offset via MAC-CE Support determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific K_offset; 26-1, 26-4; Yes; No; UE-specific K_offset reception is not supported for NR communication via satellite; Per band; No; No; No; [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]; Optional with capability signalling. |

Others

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | In addition, the following agreement on the K1 extension was made in RAN1 #104-e: Agreement: For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI. The above feature has not been captured in the NTN UE capability. A Rel-17 UE should be able to support this feature as an optional UE capability. 26. NR_NTN_solutions; 26-X; K1 range extension; 1. Support of extended K1 value range of (0..31) for unpaired spectrum.; Yes; No; Per band; No; No; No; Optional with capability signalling. |
| ZTE [3] |  |
| Vivo [4] |  |
| Xiaomi [5] | In RAN1 104e meeting, an agreement on the K1 extension was made as follows: Agreement: For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI. The motivation to have extension of K1 is to support the contiguous DL slots in specific TDD deployment scenario. UE should be able to support this feature when operating with some specific TDD configuration. Thus this should be added as an optional UE feature. Proposal 1: Add the extension of K1 value as the UE feature specifically to TDD cases. 26. NR_NTN_solutions; 26-x; K1 extension; extend the value range of K1 from (0..15) to (0..31) for unpaired spectrum; No; No; HARQ function cannot be supported on specific band; Per band; Yes; No; optional. |
| Samsung [6] |  |
| OPPO [7] | In RAN1 #104e meeting, the following agreement was achieved: Agreement For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.. The support of extending K1 range from (0..15) to (0..31) should be a UE capability. Therefore, a new FG should be introduced to define this feature. The following table gives an example. 26. NR_NTN_solutions; 26-10; K1 extension; 1. Extending K1 range from (0..15) to (0..31); 26-1. Proposal: introducing a new FG to define the capability of support extending K1 range from (0..15) to (0..31). |
| Apple [8] |  |
| NTT DOCOMO, INC. [9] |  |
| LG Electronics [10] |  |
| Nokia/Nokia Shanghai Bell [11] |  |
| Ericsson [12] |  |

# Discussion/Approval Items during RAN1 #109-e — First Checkpoint

After review of contributions submitted to RAN1 #109-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #109-e.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 26-1

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: Proposals to change the sixth column are discussed in in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of UE calculates common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are is not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of UE pre-compensationes of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receivinges cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| NTT DOCOMO | Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Since the report is per-band, it is already possible for the UE to limit the support of the feature to NTN. However, it has been acknowledged already that ATG cells are a special case, and it is not straightforward, and not very productive either, to make a strong distinction in that case. It is also technically challenging to condition the report of the feature to cell types, as the UE reports its capabilities independently of the cell it is currently connected to. Finally, from procedural point of view, we do not define RAN1 capabilities depending on the available RAN4 requirements by the time of defining the capabilities. Hence, we support removing the note in this and other impacted FGs. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Samsung | Support changes in column 4. |
| Ericsson | Support the changes. In addition, we suggest the following editorial changes in column 4: - Item 2: "Support of common TA calculation…" - Item 7: "…, transmission of aperiodic SRS, activation of TA command,…" |
| Lockheed Martin | Fine, except there is a typo in 2. Should be “ Support of UE common TA calculation…” or “Support of common TA calculation…” |
| Huawei, HiSilicon | Support. Same view as Nokia that the FG has already been defined as per band. A UE only needs to report what it supports on the particular band. RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 2: FG 26-4

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optionalwith capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| NTT DOCOMO | Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support. See comment to 26-1. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Ericsson | We support the changes. In addition we suggest the following editorial change to column 14: "Optional with capability signalling" |
| Lockheed Martin | Support |
| Huawei, HiSilicon | Support. |

# Issue 3: FG 26-5

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL, when a UE is configured with NTN carriers only. |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | We think this FG need not be restricted to NTN only since it can be beneficial for other TN scenarios, e.g., NR-U. Meanwhile, it has already been defined as the per-band feature to satisfy the needs for UE's implementation. |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] In addition, for this particular FG, there was explicit discussion in RAN#108e whether such feature can be made a general feature beyond NTN use case and the decision was not to do so. Deleting the note above would be inconsistent with previous decision. |
| Apple | Support the change in column 4. |
| NTT DOCOMO | Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ With this note, we are not sure the update in column 4 is necessary. However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support removal of the note. See comment to 26-1. However, the added sentence in column 4 requires further discussion, as the meaning of NTN carrier itself is not clear. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Samsung | It is true that since this feature is defined as per-band, for now there should be no problem. But in the future when NTN-TN carrier aggregation is supported this will lead to UE overdesigning its HARQ buffer, when it wants to support a combination of carriers in different bands. That is why we suggested add “when a UE is configured with NTN carriers only” to the component description. For other features, if we don’t have such a restriction, it may not cause any issue. But for this feature, which deals with baseband HARQ buffer design, not having such a restriction will limit the choices of the UE designer to two undesirable options: either overdesign the UE HARQ buffer or under-report the UE capability. |
| Ericsson | Support |
| Lockheed Martin | Support |
| Huawei, HiSilicon | Support. @vivo The discussion in RAN1#108-e was about whether a unified feature group of 32 HARQ processes can be defined and the conclusion is the same functionalities are discussed separately in NTN and above 52.6GHz. There is no conclusion that whether this FG cannot be extended to TN or other cases in both WIs. Conclusion: The Rel. 17 features that increase the number of HARQ processes are separate FGs for FR2-2 and NR NTN On the potential use cases of this feature, we provided two examples in RAN1#108-e: (1) FR1 + FR2 CA, if the PUCCH is transmitted on FR1 with small SCS, then it can be expected that a small number of slots on FR1 would correspond to large number of slots on FR2, which will result in the need of larger number of HARQ processes on FR2, otherwise it will degrade the system performance for FR2. (2) Another example scenario that would be beneficial from the support of 32 HARQ processes is multi-TRP case, in which case the need of number of HARQ processes would be increased. As a matter of fact, the possibility to extend the 32 HARQ processes has been discussed before. Consensus cannot be reached due to different flavors of HARQ process ID indication. However, in NTN, the explicit HARQ process indication has already been agreed. Therefore, there seems no obstacles to extend this FG in other scenarios. On the newly added sentence “when a UE is configured with NTN carriers only.” , it is not clear to us why it is needed. A UE always only reports what it can support on the given band. If the UE already has the capability but cannot be used on the other band/other use cases, the UE capability is not under-utilized which should always be avoided. We don’t think there is a restriction to UE designs on HARQ buffer. If there is a wish not to support 32 HARQ processes for a TN band, there is full freedom not to do so in the dimension phase. |

# Issue 4: FG 26-6

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | Support the change in column 5. |
| NTT DOCOMO | Support the change in column 5. Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support. See comment to 26-1. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Samsung | Support column 5 suggestion. |
| Lockheed Martin | Support |
| Huawei, HiSilicon | Support |

# Issue 5: FG 26-6a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | - Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | Support the change in column 5. |
| NTT DOCOMO | Support the change in column 5. Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support. See comment to 26-1. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Samsung | Support column 5 suggestion. |
| Lockheed Martin | Support |
| Huawei, HiSlicon | Support |

# Issue 6: FG 26-6b

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | Support the change in column 5. |
| NTT DOCOMO | Support the change in column 5. Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support. See comment to 26-1. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Samsung | Support column 5 suggestion. |
| Lockheed Martin | Support |
| Huawei, HiSlicon | Support |

# Issue 7: FG 26-8

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: Proposals to change the sixth and 14th column are discussed in in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [For UE supports NR communication via satellite, UE must indicate this FG is supported] Note: UE supporting this feature is not required to support circular polarization. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| NTT DOCOMO | Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support. See comment to 26-1. Please note that this feature should be revised to “Optional without capability signaling” to match the fact it has been agreed that gNB doesn’t need to know about its support. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Lockheed Martin | Support |
| Huawei, HiSlicon | Support |

# Issue 8: FG 26-9

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Support of reception of UE-specific K_offset via MAC-CE - Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| LG Electronics | Support |
| vivo | We do not think the following note should be deleted. Delete the note would mean the FG is generalize to TN as well, however, there is no good reason to do that [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| NTT DOCOMO | Same view with vivo. In addition, ATG is not supported in RAN4 Rel-17. The note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ However, we can accept to remove it if majority think the FG is really necessary for TN as well as NTN. |
| Nokia, NSB | Support. See comment to 26-1. |
| SoftBank | Support the note modified by DOCOMO above for clarification, but we also can accept to remove it if majority companies think the note is unnecessary. |
| Samsung | Support changes in column 4. |
| Ericsson | Support |
| Lockheed Martin | Support |
| Huawei, HiSlicon | Support |

# Issue 9: New FGs

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following new FG

| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum |  | Yes | No | K1 range extension is not supported | Per band | No | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Fine |
| LG Electronics | Fine to add this new FG. The prerequisite for this FG can be 26-1. |
| vivo | Agree with LG that the prerequisite for this FG should be 26-1. The following note is also needed for this new FG as it was introduced only for NTN use case and there was no discussion/conclusion on its applicability to TN. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | Fine. |
| NTT DOCOMO | OK and same comment as above for note. However, we can accept not to add the note if majority think the FG is really necessary for TN as well as NTN. |
| SoftBank | Same as our comment of the note. |
| Samsung | Fine with the new FG. |
| Ericsson | Support |
| Lockheed Martin | Support |
| Huawei, HiSilicon | Support |

# Discussion/Approval Items during RAN1 #109-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 3 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 26-1

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: Proposals to change the sixth column are discussed in in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of UE calculates common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are is not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of UE pre-compensationes of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receivinges cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | Support |
| Apple | We think the note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. Support the changes in column 4 only. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | The first component already delimits the scope of the FG to scenarios where there is a serving cell satellite. In that respect the note is not critical. Vivo’s version is an improvement, but it still has the issue that “NTN operation” and “TN operation” are undefined terms in specifications (as far as we know). |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 2: FG 26-4

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think this note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | Support. Since this FG has FG 26-1 as prerequisite, any restrictions on applicability to NTN only are inherited from FG 26-1 as well. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 3: FG 26-5

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think this note is needed. This feature is only agreed in NTN. It is not generally applicable to TN (except for FR2-2). |
| Samsung | The note is needed. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] The impact of removal of this note on FR2-2 is not clear at all, and we should at least make sure that this does not absolutely affect FR2-2. Regarding the comments made by Huawei in the first round about extending this FG to other scenarios: We are not necessarily against this, but the fact that the type of this FG is per band is seriously problematic. The only reason why we accepted per band for this FG was because everyone thought that there was no CA involved with this. Given that Huawei explicitly mentions CA scenarios for a use case, it makes it extremely dangerous to extend this FG. We are definitely open to discuss possible extension as long as the type does not stay per band. Since that is not happening, we cannot agree on extending this FG. We understand that the added sentence we suggested, i.e. “when a UE is configured with NTN carriers only.” would be problematic for extending this FG. But extending it would not be feasible in our view at this point as we mentioned above. Given this situation, the added sentence just provides clarity and does not add anything new to a condition we assumed when this FG was agreed. Hence, we do not see any reason why the added sentence cannot be added. Not having the added sentence and seeing the proposal for generalization extremely concerns us, and it makes the added sentence even more important to us. We are open to alternate wording to alleviate any concern that any company might have. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | Here the note is more relevant given that it is not dependent on other FGs. Again vivo’s version is an improvement, but it still has the issue that “NTN operation” and “TN operation” are undefined terms in specifications (as far as we know). |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the potential impact to FR2-2, we don’t see any issue. In principle, the UE capability will be defined without any ambiguity. For a per band UE capability, the UE only needs to report what it can support on a given band. If the concern is due to duplicated UE capability definition on 32 HARQ processes, we could add a note “This FG does not apply to FR2-2”. On the comment from Samsung, we would like to provide some of our thinking. If a UE reports the support of 32 HARQ processes on a particular band, say B1, it has to dimension its HARQ soft buffer for this band. In case of CA, if the UE supports B1 + B2 and support 32 HARQ processes on B1 but not B2, then the UE needs to dimension its HARQ soft buffer for B1 (32) + B2 (16). If I understand the comment from Samsung correctly, the concern is more related to reporting flexibility in case of CA. If this FG is defined per FS or per FSBC, the HARQ soft buffer can be utilized with more efficiently. We are open to discuss this possibility if the majority agree with this direction. |

# Issue 4: FG 26-6

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think this note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | See comments above regarding the note. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 5: FG 26-6a

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | - Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think this note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | See comments above regarding the note. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 6: FG 26-6b

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think this note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | See comments above regarding the note. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 7: FG 26-8

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: Proposals to change the sixth and 14th column are discussed in in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [For UE supports NR communication via satellite, UE must indicate this FG is supported] Note: UE supporting this feature is not required to support circular polarization. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think the last note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | See comments above regarding the note on NTN cell. This FG still has the problem that has been indicated by RAN2 in an LS to this meeting, as the FG has an inconsistent definition. It says gNB doesn’t need to know about the support of the feature but still it is “optional with capability signaling”. We do not see how the capability signaling would be justified in this case, given that indeed the gNB doesn’t need to know if the UE can receive the corresponding signaling, given that there is no specific action the gNB can do taking this information into account. We don’t support the note on circular polarization either, this doesn’t seem to belong in such FG. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 8: FG 26-9

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Support of reception of UE-specific K_offset via MAC-CE - Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| MediaTek | The note should be kept [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| Apple | We think this note is needed. This feature is only agreed in NTN. It is not applicable to TN. |
| Samsung | The note should be kept. Fine with changes in column 4. |
| NTT DOCOMO | OK. If the note is back, the note should be updated as ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to keep the note. If the note will be kept, it should be modified as Docomo mentioned above. |
| vivo | As commented in the 1st round, the note should be kept. To Address the concern of linking the UE capability to cell type, we can revise the note that the following Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | The note should be kept. |
| ZTE | Support |
| Nokia, NSB | See comments above regarding the note. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Issue 9: New FGs

Proposed Agreement: Introduce the following new FG

| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum | 26-1 | Yes | No | K1 range extension is not supported | Per band | No | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – please leave table empty unless there are major concerns or formal objections]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We think the similar note as above may also need to be added. |
| Samsung | Same view as Apple. A note should be added [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| NTT DOCOMO | OK. If the note is added, the note should be ‘Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.’ |
| SoftBank | Okay but we prefer to have the note and it should be modified as Docomo mentioned above. |
| vivo | The same note should be added Note: This UE feature group is applicable only for NR NTN operation, and is not applicable for TN operation. |
| Xiaomi | Our understanding is that the motivation to support this feature is to enable HARQ operation in specific TDD configuration. No sure 26-1 is needed as the prerequisite for this FG |
| Ericsson | We do not think 26-1 should be a prerequisite for 26-10 since the motivation to introduce K1 range extension has mainly been HAPS and ATG, for which 26-1 may not be needed. |
| ZTE | Fine. |
| Nokia, NSB | We agree with Ericsson and Xiaomi that this FG is not related to NTN but to HAPS and ATG, and hence FG 26-1 should not be a pre-requisite. |
| LG Electronics | Support |
| Huawei, HiSilicon | Support On the note, the FG has already been defined as per band and RAN4 will define separate bands for TN and NTN. There is no implication that this FG has been generalized to TN without the note. |

# Discussion/Approval Items during RAN1 #109-e — Third Checkpoint

Based on the comments/questions/suggestions received by the second checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 4 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Regarding 26-8, it is still confusing to us if support 26-8 implies that UE should support the same antenna polarization.as the satellite, e.g circular polarization, since the Consequence if the feature is not supported by the UE column says “UE is not able to take the advantage of polarization information to save power”. However, our understanding is that this feature is only about receiving the NW indication, and if UE can take the advantage by the indication is dependent on UE antenna structure which is not mandated by this FG. To solve this issue, our original proposal was to add “Note: UE supporting this feature is not required to support circular polarization.”, but Nokia had concern. The alternative solution could be to change Consequence if the feature is not supported by the UE column from “UE is not able to take the advantage of polarization information to save power” to “UE does not support polarization indication” . |

# Issue 1: FG 26-10

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum | [26-1] | Yes | No | K1 range extension is not supported | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | The prerequisite 26-1 should be kept, otherwise, the feature is extended to regular TN operation as well, however, there is no agreement to allow that. |
| Apple | Agree to remove the pre-requisite. The note could ensure this feature is applicable only to NTN. |
| LG | If the note is kept, we are fine to remove the pre-requisite. |
| Xiaomi | Fine with the FG, Vivo’s concern may be resolved by the note. |
| Huawei, HiSilicon | Support. |
| Samsung | Fine to remove the prerequisite, as long as the note is kept. |
| Ericsson | We support removing 26-1 as a prerequisite. The motivation to introduce K1 range extension has mainly been HAPS and ATG, which can be supported without support of 26-1. |
| ZTE | Support |

# Discussion/Approval Items during RAN1 #109-e — Fourth Checkpoint

Void

# Conclusion

The following was agreed during RAN1 #109-e in this agenda item.

## Final Agreements by the First Check Point

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: Proposals to change the sixth column are discussed in in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of UE calculates common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are is not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of UE pre-compensationes of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receivinges cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | FFS harq-FeedbackDisabled | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: Proposals to change the sixth and 14th column are discussed in in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [For UE supports NR communication via satellite, UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Support of reception of UE-specific K_offset via MAC-CE - Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following new FG

| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum | [26-1] | Yes | No | K1 range extension is not supported | Per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Final Agreements by the Final Check Point

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | 1. Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. 2. Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) 3. For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 4. Support of pre-compensation of the calculated TA in its uplink transmissions 5. Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT 6. Support of frequency pre-compensation to counter shift the Doppler experienced on the service link 7. Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated 8. Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure 9. Support of UE receiving cell-specific K_offset/K_mac in system information |  | Yes | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optionalwith capability signalling |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | 1. Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | harq-FeedbackDisabled | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | 1. Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | harq-FeedbackDisabled | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | 1. Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | harq-FeedbackDisabled | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | 1. Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear 2. Support polarization signalling for target serving cell in handover command message 3. Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power UE does not support polarization indication | Per band | No | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional without capability signalling |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | 1. Support of reception of UE-specific K_offset via MAC-CE 2. Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-10 | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum | 26-1 | Yes | No | K1 range extension is not supported | Per band | No | No | No | Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5 and HAPS operation bands in Clause 5.2 of TS 38.104 | Optional with capability signalling |

## Final Agreements by GTW

Agreement:

- In the FGs below,  the note “[Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported]” is replaced with a new note “Note: This UE feature group is applicable only for bands in Table 5.2.2-1 in TS 38.101-5”

  - FFS whether FG 26-5 is also supported in bands not in Table 5.2.2-1 in TS 38.101-5

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - Support of UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - Support of common TA calculation according to the parameters provided by the network (UE considers common TA as 0 if the parameters are not provided) - For TA update in RRC_CONNECTED state, support of combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - Support of pre-compensation of the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window by UE-gNB RTT - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Support of determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Support of determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - Support of UE receiving cell-specific K_offset/K_mac in system information |  | Yes | No | Release 17 NR UE cannot communicate via satellite | per band | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling For UE supports NR communication via satellite, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information related to TA pre-compensation | 1. Support UE reporting of information related to TA pre-compensation | 26-1 | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | Per band | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optionalwith capability signalling |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | 1. The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | Per band | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ codebook enhancement | - Support of type-2 HARQ codebook enhancements when there are feedback-disabled HARQ processes | harq-FeedbackDisabled | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6a | Type-1 HARQ codebook enhancement | - Support of Type-1 HARQ codebook enhancements when there are feedback-disabled HARQ processes | harq-FeedbackDisabled | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-6b | Type-3 HARQ codebook enhancement | - Support of Type-3 HARQ codebook enhancements when there are feedback-disabled HARQ processes | harq-FeedbackDisabled | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-8 | Support of polarization signalling in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration |  | No | No | UE is not able to take the advantage of polarization information to save power | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional without capability signalling |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Support of reception of UE-specific K_offset via MAC-CE - Support of determining the timing of PUSCH, PUCCH, CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |
| 26. NR_NTN_solutions | 26-10 | K1 range extension | - 1. Support of extended K1 value range of (0..31) for unpaired spectrum | [26-1] | Yes | No | K1 range extension is not supported | Per band | No | No | No | [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] | Optional with capability signalling |

# References

- R1-2202929, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2203090, Rel-17 UE features for NR NTN, Huawei/HiSilicon

- R1-2203233, Discussion on UE feature for NR-NTN, ZTE

- R1-2203532, Remaining issues on UE features for NR NTN, vivo

- R1-2203782, Discussion on UE features for NR-NTN, xiaomi

- R1-2203879, UE features for NR NTN, Samsung

- R1-2204008, Discussion on UE features for NTN-NR, OPPO

- R1-2204221, Views on Rel-17 NR NTN UE features, Apple

- R1-2204359, Discussion on Rel.17 UE features for NR NTN, NTT DOCOMO, INC.

- R1-2204520, Discussion on Rel-17 UE feature for NR NTN, LG Electronics

- R1-2204589, On UE features for NR NTN, Nokia/Nokia Shanghai Bell

- R1-2204661, On UE features for NR NTN, Ericsson