3GPP TSG RAN WG1 #109-e		                                   					       												                 R1-2204852

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.5

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR positioning enhancements

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [109-e-R17-UE-features-ePos-01] during RAN1 #109-e. According to the Chairman’s Notes:

[109-e-R17-UE-features-ePos-01] Email discussion UE on features for NR positioning enhancements – Ralf (AT&T) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

The following was discussed and/or agreed during RAN1 #109-e within the scope of [109-e-R17-UE-features-ePos-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in [1].

# Summary of Contributions Submitted to RAN1 #109-e

The following is the moderator’s summary of contributions submitted to RAN1 #109-e in this agenda item.

| 27. NR_pos_enh | 27-1-1 | UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning | 1. Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the UE Rx timing errors for the measurements | per band | n/a | n/a | n/a | Component 1 candidate values: {UE-assisted DL TDOA, Multi-RTT positioning, UE-assisted DL TDOA and Multi-RTT positioning} Component 2 candidate values: {1, 2, 3, 4, 6, 8} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported If the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the UE Rx timing errors for this measurement Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-1-1: UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning - A consistent wording should be used as follows: - “UE assisted DL TDOA” can be changed to “UE-assisted DL TDOA” |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG for SRS resource for positioning, which is supported and reported by UE for UL TDOA | 13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the UE Tx timing error for the SRS resource for positioning | per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA Note: If the UE does not include TxTEG-ID associated with a SRS resource for positioning, no assumption can be made on the UE Tx timing error for this SRS resource for positioning. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported and no assumption can be made on the UE Tx timing error for the SRS resource for positioning | per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported If the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the UE Tx timing errors for this SRS resource for positioning Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4 and 13-8 | No |  | UE RxTx for Multi-RTT is not supported and no assumption can be made on the UE RxTx timing error for the measurement | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 36, 48, 64} Need for location server to know if the feature is supported If the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the UE RxTx timing errors for this measurement Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 255 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP | 27-1-1 | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously | 27-1-4 | No |  | No assumption can be made regarding multiple Rx TEGs measuring the same DL PRS resource simultaneously | Per band | n/a | n/a | n/a | The candidate values are {1,2,34,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-1-4a; Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously; The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously; 27-1-4; No; No assumption can be made regarding multiple Rx TEGs measuring the same DL PRS resource simultaneously; Per band; n/a; n/a; n/a; The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-2-1 | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP | 13-5 | No |  | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD is not supported | per band | n/a | n/a | n/a | Component 2 candidate values: 1, 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) [Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-2-1; DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD; 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP; 13-5; No; DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD is not supported; per band; n/a; n/a; n/a; Component 2 candidate values: 1, 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2); Optional with capability signaling. |
| CATT [3] | For the FG, we think there is a need to keep the note of “Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource”. Based on the above discussions, our proposal on FG27-2-1 as follows, Proposal 2: Adopt the following modifications marked as red colour to FG 27-2-1 based on the agreement in RAN1#108-e: 27. NR_pos_enh; 27-2-1; DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD; 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP; 13-5; No; per band; n/a; n/a; n/a; Component 2 candidate values: 1, 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource; Optional with capability signalling. |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] | In our view, the [Note:] shall be kept in the FG. FG 13-5 is prerequisite for FG 27-2-1. But reporting PRS-RSRPP of the first path is independent from the reporting PRS-RSRP. The added note is good clarification. Proposal 1: Keep the note in FG 27-2-1 |
| NTT DOCOMO, INC. [9] | - FG 27-2-1: DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD - Support the current FG 27-2-1 (i.e. the brackets on the note can be removed) |
| Nokia/Nokia Shanghai Bell [10] | - The yellow highlighted note is confusing, and in principle something to resolved in the main session, not in UE features session. Propose to remove it. |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | Yes | n/a | The candidate values are {16, 24} Need for location server to know if the feature is supported The maximum number of reported DL PRS RSRP in the capability signaling should be no less than the maximum number of reported DL PRS RSRPP of the first path in the capability signaling | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-2-2; DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning; Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps.; 13-5; No; UE report of more than 8 DL PRS-RSRP is not supported.; Per UE; No; Yes FR2 only; No; The candidate values are {16, 24} Need for location server to know if the feature is supported The maximum number of reported DL PRS RSRP in the capability signaling should be no less than the maximum number of reported DL PRS RSRPP of the first path in the capability signaling; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-3-1 | M-sample measurements | The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set | 13-1 | No |  | If the UE does not provide the capability, the UE is assumed to support M=4 only | per band | n/a | n/a | n/a | The candidate values are {1 [FFS others]} Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-3-1; M-sample measurements; The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set; 13-1; No; If the UE does not provide the capability, the UE is assumed to support M=4 only; per band; n/a; n/a; n/a; The candidate values are {1 } Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | Regarding the candidate values, M=2 has been agreed for the AGC case based on the following agreement in R4-2206979 in the RAN 4 #102-e meeting. Agreements: - Define low latency requirements the reduced number of samples: - Nsample= 1 if the condition under which AGC is not required is met - Nsample= 2 if the condition under which AGC is not required is NOT met. So, we propose - For FG 27-3-1, add 2 as one of candidate values based on RAN4 conclusion. |
| ZTE [5] | In RAN4#102-e meeting, the following agreements were achieved where Nsample can be either 1 or 2 depending on whether the condition is met or not. - Agreements in RAN4#102-e: - Additional samples for AGC for PRS measurements are not required in case the following conditions are met: - PRS bandwidth is within the active BWP and - Difference between the serving cell SS-RSRP and neighbor cell/TRP PRS-RSRP is within [6] dB - Agreements in RAN4#102-e: - Define low latency requirements the reduced number of samples: - Nsample = 1 if the condition under which AGC is not required is met - Nsample = 2 if the condition under which AGC is not required is NOT met. Hence, RAN1 defined FG 27-3-1 should be updated accordingly. Proposal 1: For FG 27-3-1, M=2 should be added. |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-3-1: M-sample measurements - “[FFS others]” can be removed since only the value 1 is enough and is aligned with the motivation of this feature. - A consistent wording should be used as follows: - “UE based positioning” can be changed to “UE-based positioning” |
| Nokia/Nokia Shanghai Bell [10] | - No need for extra component values. |
| Ericsson [11] | The agreed feature 27-3-1 indicates whether the UE supports measuring M=1 samples. However, in the UE feature table, there is some text regarding candidate values and FFS for other values. Given that this feature only indicates support for M=1, we propose to remove the text related to candidate values as follows: 27. NR_pos_enh; 27-3-1; M-sample measurements; The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set; 13-1; No; If the UE does not provide the capability, the UE is assumed to support M=4 only; per band; n/a; n/a; n/a; Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning; Optional with capability signaling. |
| Qualcomm Incorporated [12] | Do not include additional values beyond 1 27. NR_pos_enh; 27-3-1; M-sample measurements; The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set; 13-1; No; If the UE does not provide the capability, the UE is assumed to support M=4 only; per band; n/a; n/a; n/a; The candidate values are {1 } Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning; Optional with capability signaling. |

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS | 13-1 | Yes |  | DL PRS measurement outside MG and in a PRS processing window is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE, details left to RAN2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | FG 27-3-2 should be separated into the FG reported to gNB and the FG reported to the LMF. For the split FG reported to LMF, FG 27-3-3 should be included as components. A component without explicit signaling for the support of PPW configuration and activation should be added. Type-1A, Type-1B, Type-2 should be further separated from the FG so that each Type can be associated with its individual capabilities, including priority options, buffering capabilities, etc. 27. NR_pos_enh; 27-3-2a; DL PRS measurement outside MG and in a type-1A PRS processing window - gNB; 1. Supported of PRS process outside the measurement gap in a type-1A PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS 3. Support of pre-configuration of PRS processing window in RRC and activation/deactivation by a DL MAC CE 4. The detection time for the collision between a low priority PRS and a high priority DL signals/channels; Yes; DL PRS measurement outside MG and in a type-1A PRS processing window is not supported; per band; n/a; n/a; n/a; Component 2 candidate values: {option1, option2, option3} Component 4 candidate values: {0.125, 0.25, 0.5, 1, 1.5, 2, 2.5, 3} ms Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP; Optional with capability signaling. 27. NR_pos_enh; 27-3-2b; DL PRS measurement outside MG and in a type-1B PRS processing window - gNB; 1. Supported of PRS process outside the measurement gap in a type-1B PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS 3. Support of pre-configuration of PRS processing window in RRC and activation/deactivation by a DL MAC CE 4. The detection time for the collision between a low priority PRS and a high priority DL signals/channels; Yes; DL PRS measurement outside MG and in a type-1B PRS processing window is not supported; per band; n/a; n/a; n/a; Component 2 candidate values: {option1, option2, option3} Component 4 candidate values: {0.125, 0.25, 0.5, 1, 1.5, 2, 2.5, 3} ms Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP; Optional with capability signaling. 27. NR_pos_enh; 27-3-2c; DL PRS measurement outside MG and in a type-2 PRS processing window - gNB; 1. Supported of PRS process outside the measurement gap in a type-2 PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS 3. Support of pre-configuration of PRS processing window in RRC and activation/deactivation by a DL MAC CE 4. The detection time for the collision between a low priority PRS and a high priority DL signals/channels; Yes; DL PRS measurement outside MG and in a type-2 PRS processing window is not supported; per band; n/a; n/a; n/a; Component 2 candidate values: {option1, option2, option3} Component 4 candidate values: {0.125, 0.25, 0.5, 1, 1.5, 2, 2.5, 3} ms Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP; Optional with capability signaling. 27. NR_pos_enh; 27-3-4a; DL PRS measurement outside MG and in a type-1A PRS processing window - location sever; 1. Supported of PRS process outside the measurement gap in a type-1A PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS 3. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of DL PRS resources that UE can process in a slot under it; 13-1; No; DL PRS measurement outside MG and in a type-1A PRS processing window is not supported; per band; n/a; n/a; n/a; Component 2 candidate values: {option1, option2, option3} Component 3 candidate values: {Type 1, Type 2} Component 4 candidate values: a) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms b) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms Component 5 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | - Agreement For capability 2 as per working assumption made in RAN1#106-e For FR1, only the DL signals/channels from a certain CC inside the PRS processing window, which overlap with DL PRS symbols in time, are dropped if the DL PRS is determined to be higher priority For FR2, only the DL signals/channels from a certain band inside the PRS processing window, which overlap with DL PRS symbols in time, are dropped if the DL PRS is determined to be higher priority. - Based on the above agreement in RAN1#108-e meeting, the note for Type 2 needs to be updated to per CC capability for FR1 and per band capability for FR2, that is, only the DL signalings/channels from a certain carrier in the PRS symbols inside the PRS processing window are dropped if the DL PRS is determined to be a higher priority for Type 2 in FR1, and only the DL signalings/channels from a certain band in the PRS symbols inside the PRS processing window are dropped if the DL PRS is configured to be a higher priority for Type 2 in FR2. So, we propose - The note for Type 2 in FG 27-3-2 can be modified as the following. - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window, - The DL signals/channels from a certain CC, which overlap with DL PRS symbols in time are affected in FR1, - The DL signals/channels from a certain band, which overlap with DL PRS symbols in time are affected in FR2 |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] | For component 2, we don't think this is a great way that the value of T to be defined as N+5, when it is actually used, the processing time is T-N. In actual, the processing time is also {4,5,6,8}, so, why don't we just define the processing time directly? For us, we prefer reusing component 3 in FG 13-1 and discuss the candidate value in FG 27-3-3. In addition, in the RAN4#102-e meeting, the following agreement has been achieved that Tlast was only needed to consider the MGL if all of the PRS resources to be measured are available in the same MG occasion so that the latency can be significantly reduced Agreements in RAN4#102-e: Define Tlast as T+MGL when all of the PRS resources to be measured are available in the same MG occasion during Tavailabe. TS 38.133 h50 is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, = +MGL. Otherwise, = +. In our view, we also can refer to the RAN4 agreement to define a similar Tlast for PRS processing outside MG to reduce latency. And then, there is no need to split PPW into two windows. For example, define Tlast as T+PPWL when UE is expected to measure up to the N ms PRS within a PRS processing window. So, we prefer reusing component 3 in FG 13-1 and adding a component for indicating the UE mode for PRS processing. - The FG 27-3-3 can be modified as the following marked by blue. - Suggest modifying the candidate value of T as specific values - Reusing the description of FG 13-1 in component 2 - Adding a component for indicating the UE mode for PRS processing - Mode 1: A UE is expected to measure all the PRS within the PRS processing window - Mode 2: A UE is expected to measure only up to the N ms PRS within a PRS processing window 27. NR_pos_enh; 27-3-3; DL PRS Processing Capability outside MG - buffering capability; 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE.] 3. Max number of DL PRS resources that UE can process in a slot under it 4. DL PRS processing capability a) mode 1 –all the PRS can be processed in a PPW cycle b) mode 2 –up to PRS can be processed in a PPW cycle; 27-3-2; No; DL PRS measurement outside MG and in a PRS processing window is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: { 4, 5, 6, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band. |
| ZTE [5] | In Rel-16, during the measurement window, the N ms of PRS symbols may be located in anywhere within the duration of T ms as shown in following figures. Figure 1a Figure 1b Figure 1c As defined in the formula of TS 38.133 for the measurement period, the component Tlast is the measurement duration for the last PRS sample, including the sampling time and processing time. The Tlast is to consider the cases that PRS resources from different sets are not concentrated on the same MG instance or PRS resources appear in the end of the processing window (e.g. Figure 1c shown above). This component leads to additional latency for the sampling and processing of the last PRS sample. As we are trying to reduce the latency as much as possible, it’s not acceptable to take additional time after the end of the PRS processing window. In order to reduce the latency for DL PRS measurement in the PRS processing window outside MG, the location information report should be ready right after the end of the PRS processing window. That is, UE has to finish all the DL PRS receiving and computation in the PRS processing window to make full use of its hardware resources. Otherwise, there is no reason to drop other signals including PDSCH, PDCCH, CSI-RS etc. during the window in the case of PRS with higher priority for processing Type 1A and 1B. As discussed in RAN1#107e meeting, we propose the following UE PRS processing capability in PRS processing window outside MG. PRS processing capability is shown in the Figure 2 below, a PRS processing window is divided into a PRS buffering window and a PRS computation window. UE is only expected to receive the DL PRS in the PRS buffering window. Then, based on the buffered DL PRS, UE can compute/process the DL PRS in the PRS computation window to get ready for a location information report by the end of PRS processing window. According to this understanding, UE has to report its capability with combinations of {N, T} under the following interpretations, where T is equal to N+X, - A PRS processing window (with duration L) is divided into a PRS buffering window with duration L-(T-N) and a PRS computation window with duration T-N. The PRS computation window starts right after the end of the PRS buffering window. - UE shall take T-N ms of time to process up to N ms of symbols containing PRS resources received by UE in the PRS buffering window - UE is not expected to be configured a PRS processing window with duration smaller than T-N. Figure 2 Type 1 PRS processing capability In summary, we have the following proposal: Proposal 2: Define the component 2 of FG 27-3-3 as: Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol. Furthermore, for gapless PRS measurement, both FG 27-3-2 and 27-3-3 should be reported. Otherwise, the feature is not complete. Proposal 3: FG 27-3-2 and 27-3-3 should be reported together. |
| China Telecom [6] |  |
| Samsung [7] | In R16 UE capability, the component 3 in 13-1 Common DL PRS Processing Capability is described as follows: - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms. In our view, for low latency positioning, the PRS processing time need to be smaller than the values in R16. Thus for the set of values of T in component 2, we suggest to include some smaller value in T, e.g., {1,2,4}ms. And we find it’s not necessary to build fixed dependence on N for T, e.g., the N+5 etc, as it is sufficient that UE can choose to report the suitable value from the value sets. Proposal 1: Support smaller numbers for T in the existing UE PRS processing capability (N, T). - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms 27-3-3; DL PRS Processing Capability outside MG - buffering capability; …… [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] ……; 27-3-2; DL PRS measurement outside MG and in a PRS processing window is not supported; Per band; …… [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms] ……. |
| OPPO [8] | In our view, component 2 in [] shall not be included here. The formulation of component 2 with “in the first part of PRS processing window…” is not aligned with previous RAN1 discussion and agreement. The feature of PRS processing window is to support processing PRS resource outside MG with a given processing priority. Defining “in the first part” would change the UE behavior and also specification, which is not preferred. Proposal 3: FG 27-3-3 does not include the component 2. |
| NTT DOCOMO, INC. [9] | - FG 27-3-3: DL PRS Processing Capability outside MG - buffering capability - Support the current FG 27-3-3 (i.e. the brackets on the component 2 can be removed) |
| Nokia/Nokia Shanghai Bell [10] | - Original formulation of component 2 was more appropriated, and it had the benefit of being aligned with the Rel-16 PRS processing capability. |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] | - For Type-1A/1B agree with the current description and values of Component 2 - For Type-2, reuse the description and values of the legacy MG-based PRS processing - We support current description and values of component 2 at least for Type 1A/B. - We are OK to accept the legacy description and values for component 2 for Type 2. 27. NR_pos_enh; 27-3-3; DL PRS Processing Capability outside MG - buffering capability; 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Description of component 2 for Type 1A/1B: - Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol Description of component 2 for Type 2: - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot; 27-3-2; No; DL PRS measurement outside MG and in a PRS processing window is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {Type 1, Type 2} Candidate 2 component values: For Type 1A/1B: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms For Type 2: component values: - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band; Optional with capability signaling. |

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity | one of 13-5,13-6, or 13-11 | No |  | LOS/NLOS Indicator for UE-assisted positioning is not supported | Per UE | n/a | n/a | n/a | [Component 1 candidate values: {hard value, soft value[, both]}] Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity; one of 13-5,13-6, or 13-11; No; LOS/NLOS Indicator for UE-assisted positioning is not supported; Per UE; No; No; No; [Component 1 candidate values: {hard value, soft value, both}] Component 2 candidate values: {trpSpecific, resourceSpecific, both} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | For LoS/NLoS indicators, it has been agreed in RAN1 #107 meeting that both soft values and hard values are supported for LoS/NLoS indicator reporting. Agreement - Support the following two options of values for LoS/NLoS indicator reporting from UE/TRP: - Soft values: [0, 0.1, …, 0.9, 1] (in steps of 0.1) - Hard values: [0, 1] - The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS. It has been agreed to support reporting LoS/NLoS indicator type to LMF, but the candidate value for component 1 needs further discussion. As far as we are concerned, if a UE supports LOS/NLOS indicator of soft value type, it will of course support the hard value 0 and 1. That is, no matter a UE is supportive of hard value only or soft value, it can firstly identify whether a measurement is LoS or NLoS, then UE supportive of soft value type can further report the soft value to indicate the probability. Therefore, for component 1 candidate values, only supporting soft value LOS/NLOS indicator is not needed. Agreement: - For UE-based positioning, support the following options for LoS/NLoS indicators within positioning assistance data: - Option 1 (Working assumption): LMF associates UE-based LoS/NloS indicators with each DL PRS resource for each TRP - Option 2: LMF associates UE-based LoS/NloS indicators with each TRP - Note: For option 1, one LoS/NloS indicator is associated with one DL-PRS resource Agreement Confirm the working assumption on UE-based LoS/NloS indicators option 1 with the following revision: - Option 1: LMF associates UE-based LoS/NloS indicators with each DL PRS resource for each TRP, provided the LMF can give different values for Los/NLos indicators of different DL PRS resource of one TRP.. For component 2 candidate values, in RAN1 #106-bis and #107 e-meeting, we are okay to support one of two options for progress, but we also think “both” is reasonable since it is only for UE capability that indicates which granularity indicator can be supported for a UE and doesn’t mean the two levels indicator will be reported. - For UE’s capability to support reporting LoS/NLoS indicator (FG27-4-1), - support component 1 candidate value {hard value, both hard value and soft value} - support component 2 candidate value {trpSpecific, resourceSpecific} |
| ZTE [5] |  |
| China Telecom [6] | In RAN1#107e, the following agreement has been reached. Agreement - Support the following two options of values for LoS/NLoS indicator reporting from UE/TRP: - Soft values: [0, 0.1, …, 0.9, 1] (in steps of 0.1) - Hard values: [0, 1] - The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS. In RAN1 #108-e meeting, the majority agreed to wait for the discussion in AI 8.5.5 to finally decide whether the {both} should be added to the component 1 and 2 candidate value . However, no more agreement has been reached on this item in the last meeting [3]. For component 1, we think if the UE support the soft value of LoS/NLoS indicator, then the hard value can must be supported, since UE can only report one type of indicator at a time, so the candidate value {both} seems reduntant. But for making further progress on this item, according to the majority view during the discussion in both AI 8.5.5 and AI 8.16.5, we can live with that the {both} added as a candidate value for the LOS/NLOS indicator type, but at least the note that “UE can not report both hard and soft values for the same association in the same time stamp” should be added. As for the component 2, we also agree to add the candidate value {both} to the LOS/NLOS indicator granularity. We don’t see the difference of the indicator reporting between the multi-RTT and DL-TDOA, we think the signaling or the two method can be the same and the “Note: a single value is reported when both multi-RTT and DL-TDOA are supported” can be be reserved here. Proposal 1: For FG 27-4-1, - Support to add {both} as a candidate value for component 1 only if the following note be added: “Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp”. - Support to add {both} as a candidate value for component2. The FG 27-4-1 can be modified as follows, 27. NR_pos_enh; 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity; one of 13-5,13-6, or 13-11; No; LOS/NLOS Indicator for UE-assisted positioning is not supported; Per UE; n/a; n/a; n/a; [Component 1 candidate values: {softValue, hardValue, both}] Note: Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported; Optional with capability signaling. |
| Samsung [7] |  |
| OPPO [8] | Regarding the candidate value for component 1, including ‘hard value’ and ‘soft value’ is sufficient. There is no use case to include candidate value ‘both’. Because candidate value ‘both’ would mean that the UE supports both softvalue and hardvalue. That is not practical. If a UE is capable of supporting softvalue, it is not technically feasible for the UE to report supporting hardvalue. Furthermore, if a UE reporting support ‘both’, indicating a value of 0 or 1 would cause ambiguity. The UE would not know if the indicated value 0 or 1 is from a softvalue or hardvalue set. Regarding the LOS/NLOS indicator granularity of component 2, we also prefer not to include the candidate value “both”. There is no technical motivation to let one UE support both granularity and then switch them during NR positioning operation. Proposal 5: On FG 27-4-1: - The candidate values for component 1 are: {soft value, hard value}. - The candidate value of component 2 does not include ‘both’ |
| NTT DOCOMO, INC. [9] | - FG 27-4-1: LOS/NLOS Indicator for UE-assisted positioning - Regarding component 1 candidate values, considering the common understanding of the AI 8.5.5 Note: “Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp.”, “both” can be reported. - Regarding component 2 candidate values, we prefer to keep “both”. - “[Note: a single value is reported when both multi-RTT and DL-TDOA are supported]” can be removed. - “FFS: signalling per method” part can be removed. |
| Nokia/Nokia Shanghai Bell [10] | - No need to signal candidate values, as UE should be able to support reports with either hard or soft values. |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] | - Support including hard, soft and both values for component 1 - Support including “both” as value in component 2 - Keep the Note - Support signaling per method 27. NR_pos_enh; 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity; one of 13-5,13-6, or 13-11; No; LOS/NLOS Indicator for UE-assisted positioning is not supported; Per UE; n/a; n/a; n/a; Component 1 candidate values: {hard value, soft value, both} Component 2 candidate values: {trpSpecific, resourceSpecific, both} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Signalling per method Need for location server to know if the feature is supported; Optional with capability signaling. |

| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot |  | No |  | DL PRS processing in RRC inactive state is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] | The following agreement was reached in RAN4: Agreements: - Support of the reduced number of samples in RRC_INACTIVE state is UE capability - The UE capability is under discussion in RAN1, and wait for RAN1 agreement. - PRS measurement requirements with reduced number of samples in RRC_INACTIVE state are applicable only for UE which supports PRS measurements with reduced number of samples. - The requirements with reduced sample number are applicable when UE is requested by LMF to perform measurement with reduced sample number. - FFS: PRS measurement requirements with reduced number of samples in RRC_INACTIVE are defined under the same side conditions as agreed for RRC CONNECTED state.. - Based on the above agreement, we suggest adding a new component in the FG 27-6: “Support of single sample measurements in RRC Inactive state: The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set”. 27. NR_pos_enh; 27-6; DL PRS processing capabilities in RRC inactive state; 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot 4. Support of single -sample measurements in RRC Inactive state: The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set; No; DL PRS processing in RRC inactive state is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: The sample number M=1 does not account for the potential AGC sample; Optional with capability signaling. |

| 27. NR_pos_enh | 27-7 | Multiple measurement instances which can be included in a single measurement report | Support of mutiple measurement instances which can be included in a single measurement report |  | No |  | Multiple measurement instances which can be included in a single measurement report are not supported | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-7; Multiple measurement instances which can be included in a single measurement report; Support of multiple measurement instances which can be included in a single measurement report; No; Multiple measurement instances which can be included in a single measurement report are not supported; Per UE; No; No; No; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning | 13-1 | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-8; Support of PRS TEG association information for UE-based DL-TDOA; Support of reception of association between PRS and TRP Tx TEG for UE-based positioning; 13-1; No; Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band | n/a | n/a FR2 only | n/a | Component 2 candidate values: {1,2,4,6} Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request for PRS measurements | 1. Support of using UL MAC CE to request measurement gap for PRS measurements: The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG 2. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID | 27-11 | Yes |  | Using UL MAC CE to indicate measurement gap for PRS measurements to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-10; Support of UL MAC CE based MG activation request for PRS measurements - gNB; 1. Support of using UL MAC CE to request measurement gap activation/deactivation for PRS measurements: The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG 2. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID; 27-11; Yes; Using UL MAC CE to indicate measurement gap for PRS measurements to the gNB is not supported.; Per UE; No; No; No; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-10] | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-10a; Low latency MG activation request for PRS measurements - location sever; support of low latency MG activation request for PRS measurements; No; Low latency MG activation request for PRS measurements is not supported; Per UE; No; No; No; Need for location server to know if the feature is supported; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | We prefer to keep the [27-10] as a prerequisite feature group considering FG 27-10 includes component 2 “Support of preconfiguration of MGs in RRC signaling for PRS measurements”. We believe in supporting of preconfiguration of MGs is a prerequisite for feature group 27-10a. - For FG27-10a, keep the [27-10] as a prerequisite feature group. |
| ZTE [5] | For FG 27-10a, we think both 27-10 and 27-11 is needed as the prerequisite. First, 27-11 implies UE supports pre-configuration of MGs, and UE supporting 27-10a needs to support pre-configuration of MGs. Second, support of 27-10a must support 27-10 as well. If UE doesn’t support 27-10 reporting to gNB, it is not needed to report 27-10a. Proposal 4: For FG 27-10a, both 27-10 and 27-11 should be prerequisite. |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-10a: Low latency MG activation request for PRS measurements - The prerequisite FG 27-10 can be removed. |
| Nokia/Nokia Shanghai Bell [10] | - Confirm 27-10 as pre-requisite. |
| Ericsson [11] | FG 27-10a informs the LMF that the UE supports the use of low latency activation of MG configurations. Based on the information, the LMF can request the gNB to reconfigure the MG in UE. the UE should not have to report this capability if it is not capable to receive fast DL MAC CE reconfigurations. Therefore we think that 27-11 should be a prerequisite, instead of 27-10, which signals the UL MAC CE capability. The following change is proposed: 27. NR_pos_enh; 27-10a; Low latency MG activation request for PRS measurements; support of low latency MG activation request for PRS measurements; [27-11]. |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation request for PRS measurements | 1. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID 2. Support of using DL MAC CE to activate the MG for PRS measurements: The DL MAC CE for MG activation indicates the ID associated with the preconfigured MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG for PRS measurements is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-11; Support of DL MAC CE based MG activation request for PRS measurements; 1. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID 2. Support of using DL MAC CE to activate/deactivation the MG for PRS measurements: The DL MAC CE for MG activation indicates the ID associated with the preconfigured MG; Yes; Using DL MAC CE to activate the preconfigured MG for PRS measurements is not supported; Per UE; No; No; No; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity |  | No |  | LOS/NLOS indicator for UE-based positioning assistance data is not supported | Per UE | No | No | No | [Component 1 candidate values: {softValue, hardValue, both}] Component 2 candidate values: {resourceSpecific, trpSpecific[, both]} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-12; LOS/NLOS indicator for UE-based positioning assistance data; Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity; No; LOS/NLOS indicator for UE-based positioning assistance data is not supported; Per UE; No; No; No; Component 1 candidate values: {softValue, hardValue, both} Component 2 candidate values: {resourceSpecific, trpSpecific, both} Need for location server to know if the feature is supported.; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] | For FG 27-12, similar to FG27-4-1, we think the component 1 candidate value should be {hard value, both hard value and soft value}, and component 2 candidate value should be {trpSpecific, resourceSpecific}. - For UE’s capability to support LOS/NLOS indicator for UE-based positioning assistance data (FG27-12), support the following: - support component 1 candidate value {hard value, both hard value and soft value}; - support component 2 candidate value {trpSpecific, resourceSpecific}. |
| ZTE [5] |  |
| China Telecom [6] | Since the FG 27-12 and 27-4-1 have the same motivation, the candidate value for FG 27-12 should be the same as that of FG 27-4-1. Proposal 2： The candidate value of FG 27-12 should keep the same as that of FG 27-4-1. The FG 27-12 can be modified as follows, 27. NR_pos_enh; 27-12; LOS/NLOS indicator for UE-based positioning assistance data; Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity; No; LOS/NLOS indicator for UE-based positioning assistance data is not supported; Per UE; No; No; No; [Component 1 candidate values: {hard value, soft value[, both]}] Note: Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} Need for location server to know if the feature is supported.; 27. NR_pos_enh. |
| Samsung [7] | According to the RAN1 #107 agreement, the LOS/NLOS indicators can be either soft values or hard values. Agreement - Support the following two options of values for LoS/NLoS indicator reporting from UE/TRP: - Soft values: [0, 0.1, …, 0.9, 1] (in steps of 0.1) - Hard values: [0, 1] - The values correspond to the likelihood of LoS, with a value of 1 corresponding to LoS and a value of 0 corresponding to NLoS. A UE could support one of these options (either soft reporting or hard reporting). However, we did not find much motivation for supporting both (soft and hard reporting). For an advanced UE, if it’s capable of having soft values, it can choose to report hard or soft value, while the hard value is the only choice for a less advanced UE. But if “both” is indicated, which means UE gives the right to gNB to decide which type to be reported. While gNB or LMF did not have to choose since it’s the fact that soft value is more useful than hard value. Thus, we support including either hard or soft values for component 1. For component 2, “both” is useful because the two types are not exclusive. Thus, “both” is supported. Proposal 2：Support the following statements regarding LOS/NLOS indicators for feature 27-4-1: - Component 1 candidate values: {hard value, soft value} - Component 2 candidate values: {trpSpecific, resourceSpecific, both} 27. NR_pos_enh; 27-12; LOS/NLOS indicator for UE-based positioning assistance data; Support reception of the assistance data containing the LOS/NLOS indicator. 1.LOS/NLOS indicator type 2.LOS/NLOS indicator granularity; Component 1 candidate values:{softValue, hardValue} Component 2 candidate values: {resourceSpecific, trpSpecific, both} Need for location server to know if the feature is supported.; Optional with capability signaling.. |
| OPPO [8] | There is no need for support ‘both’ as candidate value for component 1 of LOS/NLOS indicator type. Similarly, there is no need to include ‘both’ as candidate value for component 2 of LOS/NLOS indicator granularity. If a UE can support resourceSpecific, it does not make sense for the LMF to provide trpSpecific configuration. Proposal 4: For FG 27-12: do not include ‘both’ as candidate value for component 1 and do not include ‘both’ in candidate value for component 2. |
| NTT DOCOMO, INC. [9] | - FG 27-12: LOS/NLOS indicator for UE-based positioning assistance data - Regarding component 1 candidate values, considering the common understanding of the AI 8.5.5 Note: “Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp.”, “both” can be reported. - Regarding component 2 candidate values, we prefer to keep “both”. |
| Nokia/Nokia Shanghai Bell [10] | - Component 1 definition should follow how 27-4-1 is going to be defined. |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] | - Support Component 1 - Support including “both” as value in component 2 27. NR_pos_enh; 27-12; LOS/NLOS indicator for UE-based positioning assistance data; Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity; No; LOS/NLOS indicator for UE-based positioning assistance data is not supported; Per UE; No; No; No; Component 1 candidate values: {softValue, hardValue, both} Component 2 candidate values: {resourceSpecific, trpSpecific, both} Need for location server to know if the feature is supported.; Optional with capability signaling.. |

| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | 1. Support of additional detected path timing reporting for K>2 additional paths for UE-assisted DL-TDOA 2. Support of RSRPP reporting for additional paths if UE supports FG 27-13a |  | No |  | Additional path reporting for UE-assisted DL-TDOA is not supported | Per UE | No | No | No | Component 1 candidate values: {4, 6, 8} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-13a | First path RSRPP reporting for UE-assisted DL-TDOA | 1. Support of RSRPP reporting for first path | 13-1 | No |  | First path RSRPP reporting for UE-assisted DL-TDOA is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of additional detected path timing reporting for K>2 additional paths for Multi-RTT 2. Support of RSRPP reporting for additional paths if UE supports FG 27-14a |  | No |  | Additional path reporting for Multi-RTT is not supported | Per UE | No | No | No | Component 1 candidate values: {4, 6, 8} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-14a | First path RSRPP reporting for Multi-RTT | 1. Support of RSRPP reporting for first path | 13-1 | No |  | First path RSRPP reporting for Multi-RTT is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-15 | Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP | 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of P/SPSRS Resources for positioning 3. Max number of P/SPSRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot |  | Yes |  | Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 4 candidate values: {1,2,4,8,16,32,64} Component 5 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-15; Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP; 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of SRS Resources for positioning 3. Max number of SRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot; Yes; Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 4 candidate values: {1,2,4,8,16,32,64} Component 5 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component Need for location server to know if the feature is supported; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-15b | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Different numerology between the SRS and the initial UL BWP is supported] - [SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: center frequenecy | 27-15 | Yes |  | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: FFS Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: FFS Component 6 candidate values: FFS Component 7 candidate values: FFS Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. [Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.] [Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP] [Note 4: If component 6 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.] [Need for location server to know if the feature is supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP; - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - Support of the same numerology between the SRS and the initial UL BWP - Support of operation with restriction on the BW: BW of the SRS includes BW of the CORESET#0 and SSB - Max number of SRS Resources for positioning - Max number of SRS Resources for positioning per slot - Support of the same centre frequency between the SRS for positioning and the initial UL BWP - SRS switching time; 27-15; Yes; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100} MHz for FR1 and {50, 100, 200, 400} MHz for FR2 Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 7 candidate values: {1,2,4,8,16,32,64} Component 8 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 10 candidate values: {0, 30, 100, 140, 200, 300, 500, 900} us for DL and UL respectively. Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. Note 2: Component 10 may correspond to the IE SRS-SwitchingTimeNR. Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-15c; Support of different numerologies between SRS for positioning and initial UL BWP for SRS configured outside initial UL BWP in RRC_INACTIVE state; Support of different numerologies between SRS for positioning and initial UL BWP; 27-15b; Yes; SRS for positioning should have the same numerology as the initial UL BWP; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-15d; Support of operation without BW restriction for SRS configured outside initial UL BWP in RRC_INACTIVE state; Support of the bandwidth of SRS not including the bandwidth of the CORESET#0 and SSB.; 27-15b; Yes; SRS for positioning should include the bandwidth of the CORESET#0 and SSB; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-15e; Support of different centre frequency between the SRS for positioning and the initial UL BWP; Support of different centre frequencies between the SRS for positioning and the initial UL BWP; 27-15b; Yes; SRS for positioning should have the same centre frequency as the initial UL BWP; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | In addition, the following agreements were achieved related to option 2 of SRS for positioning configuration in inactive state. Agreement - For Option 2 of SRS for positioning configuration, - The feature is supported at least for NUL in Rel.17 - The SRS for positioning is configured in the same band and CC as the initial UL BWP - Signaling details are up to RAN2 - The following is up to UE capability indication - Support of different SCS, CP type from the initial UL BWP - Support a different center frequency between the SRS for positioning and the initial UL BWP - Whether bandwidth of SRS for positioning may not include bandwidth of the CORESET#0 and SSB Working assumption For Option 2 of SRS for positioning transmission in RRC_INACTIVE, a UE capability for switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP is introduced - The capability is reported per band - The capability applies at least to TDD - FFS: FDD - The switching time value(s) are left up to RAN4 discussion - If the transmission of SRS for positioning with the switching time overlaps/collides in time domain with other DL reception or UL transmission at least for TDD, the SRS for positioning transmission is dropped in the symbol(s) where the overlap/collision occurs - Note: Transmission of SRS for positioning with the switching time covers the following example TDD cases: - “switching after SRS” (i.e., transmission of SRS + switching time) - “switching before SRS” (i.e., switching time + transmission of SRS). Based on the latest agreement, component 5 and 6 can be confirmed. For component 9, it should be changed to ‘different center frequency between the SRS for positioning and the initial UL BWP’. Then, considering ‘switching time’ was also agreed in the working assumption, a new component ‘switching time’ should be added in FG27-15b. In addition, for the ‘Note’ column, the following issues are discussed. - For Component 1 candidate values, the bandwidth should be limited to FG2-1‘Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC’ as the following. Therefore, similarly, for SRS bandwidth outside initial UL BWP, Table 5.3.5-1 in TS38.101-1/TS38.101-2 can be referred. 2. UE RF; 2-1; Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC; 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1; channelBWs-DL channelBWs-UL; BandNR; No; No; UE capability signalling shall follow RP-172832 (Per-band capability signalling, separately for DL and UL and for each SCS) Whether a bandwidth newly introduced in future is mandatory for UE shall be discussed case by case.; For FR1, all the bandwidths listed in TS38.101-1 v15.0.0 Table 5.3.5-1 for each band shall be mandatory with a single CC. The bandwidths listed in the slide #3 of R4-1805985 are mandatory with a single CC. 90MHz is optional for n41, n77, n78. For FR2, the set of mandatory CBW is 50, 100, 200 MHz.. - For Component 5 candidate values, we think if Note 3 is supported, candidate values are no need. - For Component 6 candidate values, we think if Note 4 is supported, candidate values are no need. - For Component 7/8 candidate values, we can refer to the values of FG27-15. - For Note 2, we don't see the need to leave it. - For Note 3 and Note 4, we are OK to leave them. - We support the note ‘Need for location server to know if the feature is supported’ Therefore, we propose - For FG27-15b, support the following marked by blue. 27. NR_pos_enh; 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP; - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Different numerology between the SRS and the initial UL BWP is supported] - [SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: different center frequency between the SRS for positioning and the initial UL BWP - Switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP; 27-15; Yes; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported; Per band; Component 1 candidate values: refer to Table 5.3.5-1 in TS38.101-1/TS38.101-2 Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: FFS Component 6 candidate values: FFS Component 7 candidate values: {1,2,4,8,16,32,64} Component 8 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. [Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.] [Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP] [Note 4: If component 6 is not signalled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.] [Need for location server to know if the feature is supported]; Optional with capability signaling. |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] | If the center frequency of SSRS outside initial UL BWP is different from that of the initial UL BWP, the UE may need to retune its RF chains for the reception and it will impact hardware implementation. Furthermore, the main motivation to support SRS out of initial UL BWP is to support larger SRS bandwidth for higher accuracy. In our understanding, the 1st figure is the most typical case. For this case, UE is naturally to support it. However, for the 2nd and 3rd figure, we think the cases are less popular compared to the 1st figure. Thus, a separate UE feature is suggested to differentiate the supporting of different cases. In summary, we suggest to support Component 9. Additionally, from the perspective of UE implementation, we also support to keep Component 5 and 6. Proposal 6: Support the following modifications for UE FG 27-15b (RED part) 27. NR_pos_enh; 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP; - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Whether to support Different numerology between the SRS and the initial UL BWP is supported] - [Whether to support SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: Whether to support different center frequenecy as the initial UL BWP; Component 1 candidate values: FFS{10, 20, 25, 50, 100, 200, 400} MHz Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: FFS{Support, Not support} Component 6 candidate values: FFS{Support, Not support} Component 7 candidate values: FFS {1,2,4,8,16,32,64} Component 8 candidate values:{1,2,3,4,5,6,8,10, 12, 14} Component 9 candidate values: {Support, Not support} Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. [Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.] [Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP] [Note 4: If component 6 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.] [Need for location server to know if the feature is supported]. |
| NTT DOCOMO, INC. [9] | - FG 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP - We prefer to keep component 5. - We prefer to keep component 6. - “FFS: center frequency” can be removed. - Regarding semi-persistent SRS transmission for outside initial UL BWP, we need to discuss whether FG 27-15a is necessary in prerequisite feature groups in addition to FG 27-15. - Regarding component 1 candidate values, we need to consider SupportedBandwidth information elements, and we think all values can be included as candidate values for component 1. - We think component 5 candidate value is not needed. - We think component 6 candidate value is not needed. - Regarding component 7, the following candidate values can be considered. - Candidate values: {1, 2, 4, 8, 16, 32, 64} - Regarding component 8, the following candidate values can be considered. - Candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} - “[Need for location server to know if the feature is supported]” can be removed. |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] | During RAN1#108e, two components were left in yellow for FG 27-15b. both component 5 and 6 may impact the required retuning time when the SRS needs to be transmitted outside of the initial BWP. if the numerology is the same and the SRS overlap with the initial BWP, retuning time could be smaller compared to when the UE needs to readjust numerology and move to the initial BWP. a separate FG for the case of different numerology and with SRS BW outside of coreset0 / SSB could thus be considered. 27. NR_pos_enh; 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP; - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Different numerology between the SRS and the initial UL BWP is supported] - [SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: center frequenecy; 27-15. Component 5 and 6 of FG 27-15b can be moved to a new, separate FG with 27-15b as a prerequisite. |
| Qualcomm Incorporated [12] | - Add the new components as agreed in the main session in the previous meeting: Working assumption For Option 2 of SRS for positioning transmission in RRC_INACTIVE, a UE capability for switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP is introduced - The capability is reported per band - The capability applies at least to TDD - FFS: FDD - The switching time value(s) are left up to RAN4 discussion - If the transmission of SRS for positioning with the switching time overlaps/collides in time domain with other DL reception or UL transmission at least for TDD, the SRS for positioning transmission is dropped in the symbol(s) where the overlap/collision occurs - Note: Transmission of SRS for positioning with the switching time covers the following example TDD cases: - “switching after SRS” (i.e., transmission of SRS + switching time) - “switching before SRS” (i.e., switching time + transmission of SRS) Agreement - For Option 2 of SRS for positioning configuration, - The feature is supported at least for NUL in Rel.17 - The SRS for positioning is configured in the same band and CC as the initial UL BWP - Signaling details are up to RAN2 - The following is up to UE capability indication - Support of different SCS, CP type from the initial UL BWP - Support a different center frequency between the SRS for positioning and the initial UL BWP - Whether bandwidth of SRS for positioning may not include bandwidth of the CORESET#0 and SSB. 27. NR_pos_enh; 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP; - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - Support of Different numerology between the SRS and the initial UL BWP - Support SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - Support a different center frequency between the SRS for positioning and the initial UL BWP; 27-15; Yes; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: NA Component 6 candidate values: NA Component 7 candidate values: {1,2,4,8,16,32,64} Component 8 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 9 candidate values: NA Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported. Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP Note 4: If component 6 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB. Note 5: If component 9 is not signaled, the UE only supports same center frequency between the SRS for positioning and initial UL BWP Need for location server to know if the feature is supported; Optional with capability signaling. |

| 27. NR_pos_enh | 27-15a | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-15a; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; 27-15; Yes; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | For this FG, we support the note ‘Need for location server to know if the feature is supported’. Then, we also support UE capability of semi-persistent SRS outside initial UL BWP. - For FG27-15a, support the note ‘Need for location server to know if the feature is supported’. - Support UE capability of semi-persistent SRS outside initial UL BWP. |
| ZTE [5] |  |
| China Telecom [6] | For the SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent, there is no difference for the LMF whether the feature is supported, so we don’t see the need for the location server to know the feature. Proposal 3： For the FG 27-15a, no need for location server to know if the feature is supported. The FG 27-15a can be modified as follows, 27. NR_pos_enh; 27-15a; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; 27-15; Yes; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP; Optional with capability signaling. |
| Samsung [7] |  |
| OPPO [8] | There is a separate UE feature for the semi-persistent SRS on initial UL BWP in RRC_INACTIVE. Thus, following the similar logic, it is beneficial to have a separate counterpart for the semi-persistent SRS outside the initial UL BWP in RRC_INACTIVE. Thus, we have the following proposal Proposal 7: Support the following modifications for UE FG 27-15a and add a new UE FG 27-15c (RED part) 27. NR_pos_enh; 27-15a; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP. 27. NR_pos_enh; 27-15c; Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; 27-15a 27-15b; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported. |
| NTT DOCOMO, INC. [9] | - FG 27-15a: Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS - “[Need for location server to know if the feature is supported]” can be removed. - We need a clarification on prerequisite feature groups of FG 27-15b. When it has been clarified, “FFS: outside initial BWP” can be removed. |
| Nokia/Nokia Shanghai Bell [10] | - No need for LMF to know the RRC state of the UE. |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] | 27. NR_pos_enh; 27-15a; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; 27-15; Yes; Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-15b; Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; 27-15; Yes; Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported; Optional with capability signaling. |

| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state - gNB | Same asRRC OLPC-SRS-Pos-r16 |  | Yes |  | OLPC for positioning SRS in RRC_INACTIVE state is not supported (gNB) | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-16; OLPC for positioning SRS in RRC_INACTIVE state - gNB; Same as RRC OLPC-SRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state; Yes; OLPC for positioning SRS in RRC_INACTIVE state is not supported (gNB); Per band; n/a; n/a; n/a; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] | Regarding UE feature, an LS [4] from RAN2 asks the following clarification related to FG 27-16 and 27-19. B). R1 27-16 and 27-19 R1 27-16 and 27-19 have a component description of ‘Same as RRC OLPC-SRS-Pos-r16’ and ‘Same as RRC SpatialRelationsSRS-Pos-r16’ respectively. It is unclear to RAN2 whether the pre-requisite in R1 27-16/27-19 should be srs-PosResources-r16 as in RRC OLPC-SRS-Pos-r16/SpatialRelationsSRS-Pos-r16 or should be “srs-PosResourcesRRC-Inactive-r17” (i.e. R1 27-15)). RAN2 would like RAN1 to clarify the pre-requisite used in the R1 27-16 and 27-19.. From our point of view, the description of ‘Same as RRC OLPC-SRS-Pos-r16’ in FG27-16 is to reuse the content of FG13-9 series for OLPC support, not the pre-requisite groups of FG13-9 series, which is also applied to FG27-19. Therefore, for the pre-requisite groups for both FG 27-16 and FG 27-19, we think basic feature groups in inactive state can be considered, e.g. FG 27-15, FG 27-15b, and FG 27-6 (for PRS related capability, e.g., OLPC support based on PRS). - The pre-requisite groups for both FG 27-16 and FG 27-19 can be FG 27-15, FG 27-15b, and FG 27-6 (only for PRS related capability, e.g., OLPC support based on PRS). |
| ZTE [5] | FG 27-16 and 27-19 have a component description of ‘Same as RRC OLPC-SRS-Pos-r16’ and ‘Same as RRC SpatialRelationsSRS-Pos-r16’ respectively. Based on RAN2 pre-meeting email discussion, it is unclear to RAN2 whether the pre-requisite of FG 27-16/27-19 should be srs-PosResources-r16 as in RRC OLPC-SRS-Pos-r16/SpatialRelationsSRS-Pos-r16 or should be “srs-PosResourcesRRC-Inactive-r17” (i.e. FG 27-15)). RAN2 would like RAN1 to clarify the pre-requisite used in the FG 27-16 and 27-19. Since the two FGs should be for SRS in RRC_INACTIVE state, we believe that “srs-PosResourcesRRC-Inactive-r17” (i.e. FG 27-15)) should be the prerequisite. Proposal 5: For FG 27-16 and 27-19, FG 27-15 should be the prerequisite. |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-16: OLPC for positioning SRS in RRC_INACTIVE state – gNB - We need to discuss the prerequisite FG since RAN1 received an LS requiring a clarification on FG 27-16 and 27-19 from RAN2 (R2-2204360). Considering the prerequisite of OLPC in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG. |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-16a | OLPC for positioning SRS in RRC_INACTIVE state – location server | Same as LPP OLPC-SRS-Pos-r16 |  | No |  | OLPC for positioning SRS in RRC_INACTIVE state is not supported (location server) | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Support of OLPC in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-16a; OLPC for positioning SRS in RRC_INACTIVE state – location server; Same as LPP OLPC-SRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state; No; OLPC for positioning SRS in RRC_INACTIVE state is not supported (location server); Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Support of OLPC in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signaling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-16a: OLPC for positioning SRS in RRC_INACTIVE state – location server - Considering the prerequisite of OLPC in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG. |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-17 | PRS processing in RRC_INACTIVE | Support of PRS processing in RRC_INACTIVE | [13-1, 13-2, 13-3, 13-4] | Yes |  | PRS processing in RRC_INACTIVE is not supported | per band | n/a | n/a | n/a | Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-17; PRS processing in RRC_INACTIVE - gNB; Support of PRS processing in RRC_INACTIVE; Yes; PRS processing in RRC_INACTIVE is not supported; per band; n/a; n/a; n/a; Note: UE supporting this feature shall support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement; Optional with capability signaling.. |
| CATT [3] | For this FG, we think its prerequisite feature groups should be 13-1, 13-2, 13-3, 13-4. Based on the above discussions, our proposal on FG27-17 as follows, Proposal 3: Adopt the following modifications marked as red colour to FG 27-17 based on the agreement in RAN1#108-e: 27. NR_pos_enh; 27-17; PRS processing in RRC_INACTIVE; Support of PRS processing in RRC_INACTIVE; 13-1, 13-2, 13-3, 13-4; Yes; PRS processing in RRC_INACTIVE is not supported; per band; n/a; n/a; n/a; Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement; Optional with capability signaling.. |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-17: PRS processing in RRC_INACTIVE - The prerequisite FG 13-2, 13-3 and 13-4 can be removed since these prerequisites can be covered by the note, while FG13-1 can be kept as the prerequisite since it is not covered by the note. This is similar handling as for FG27-18a/b/c, and the consistent handling would be preferable. |
| Nokia/Nokia Shanghai Bell [10] | - Add a component on SRS transmission in RRC_INACTIVE: - 2. Support of positioning SRS transmission in RRC_INACTIVE state. - Add 13-8 as pre-requisite (SRS resources). |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA |  | No |  | PRS measurement in RRC_INACTIVE state for DL-TDOA is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-18a; Support of PRS measurement in RRC_INACTIVE state for DL-TDOA - location server; Support of PRS measurement in RRC_INACTIVE state for DL-TDOA; 13-3, 13-3a, 13-3b, 13-6, 27-6; No; PRS measurement in RRC_INACTIVE state for DL-TDOA is not supported; per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD |  | No |  | PRS measurement in RRC_INACTIVE state for DL-AoD is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-18b; Support of PRS measurement in RRC_INACTIVE state for DL-AoD - location server; Support of PRS measurement in RRC_INACTIVE state for DL-AoD; 13-2, 13-2a, 13-2b, 13-5, 27-6; No; PRS measurement in RRC_INACTIVE state for DL-AoD is not supported; per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT |  | No |  | PRS measurement in RRC_INACTIVE state for for Multi-RTT is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-18c; Support of PRS measurement in RRC_INACTIVE state for Multi-RTT - location server; 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT; 13-4, 13-4a, 13-4b, 13-11, 27-6; No; PRS measurement in RRC_INACTIVE state for for Multi-RTT is not supported; per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state - gNB | Same asRRC SpatialRelationsSRS-Pos-r16 |  | Yes |  | Spatial relation for positioning SRS in RRC_INACTIVE state is not supported (gNB) | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-19; Spatial relation for positioning SRS in RRC_INACTIVE state - gNB; Same asRRC SpatialRelationsSRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state; Yes; Spatial relation for positioning SRS in RRC_INACTIVE state is not supported (gNB); Per band; n/a; n/a; n/a; Optional with capability signalling. |
| CATT [3] |  |
| Vivo [4] | Regarding UE feature, an LS [4] from RAN2 asks the following clarification related to FG 27-16 and 27-19. B). R1 27-16 and 27-19 R1 27-16 and 27-19 have a component description of ‘Same as RRC OLPC-SRS-Pos-r16’ and ‘Same as RRC SpatialRelationsSRS-Pos-r16’ respectively. It is unclear to RAN2 whether the pre-requisite in R1 27-16/27-19 should be srs-PosResources-r16 as in RRC OLPC-SRS-Pos-r16/SpatialRelationsSRS-Pos-r16 or should be “srs-PosResourcesRRC-Inactive-r17” (i.e. R1 27-15)). RAN2 would like RAN1 to clarify the pre-requisite used in the R1 27-16 and 27-19.. From our point of view, the description of ‘Same as RRC OLPC-SRS-Pos-r16’ in FG27-16 is to reuse the content of FG13-9 series for OLPC support, not the pre-requisite groups of FG13-9 series, which is also applied to FG27-19. Therefore, for the pre-requisite groups for both FG 27-16 and FG 27-19, we think basic feature groups in inactive state can be considered, e.g. FG 27-15, FG 27-15b, and FG 27-6 (for PRS related capability, e.g., OLPC support based on PRS). - The pre-requisite groups for both FG 27-16 and FG 27-19 can be FG 27-15, FG 27-15b, and FG 27-6 (only for PRS related capability, e.g., OLPC support based on PRS). |
| ZTE [5] | FG 27-16 and 27-19 have a component description of ‘Same as RRC OLPC-SRS-Pos-r16’ and ‘Same as RRC SpatialRelationsSRS-Pos-r16’ respectively. Based on RAN2 pre-meeting email discussion, it is unclear to RAN2 whether the pre-requisite of FG 27-16/27-19 should be srs-PosResources-r16 as in RRC OLPC-SRS-Pos-r16/SpatialRelationsSRS-Pos-r16 or should be “srs-PosResourcesRRC-Inactive-r17” (i.e. FG 27-15)). RAN2 would like RAN1 to clarify the pre-requisite used in the FG 27-16 and 27-19. Since the two FGs should be for SRS in RRC_INACTIVE state, we believe that “srs-PosResourcesRRC-Inactive-r17” (i.e. FG 27-15)) should be the prerequisite. Proposal 5: For FG 27-16 and 27-19, FG 27-15 should be the prerequisite. |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-19: Spatial relation for positioning SRS in RRC_INACTIVE state - gNB - We need to discuss the prerequisite FG since RAN1 received an LS requiring a clarification on FG 27-16 and 27-19 from RAN2 (R2-2204360). Considering the prerequisite of spatial relation in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG. |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-19a | Spatial relation for positioning SRS in RRC_INACTIVE state – location server | Same as LPP SpatialRelationsSRS-Pos-r16 |  | No |  | Spatial relation for positioning SRS in RRC_INACTIVE state is not supported (location server) | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Support of spatial relation in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-19a; Spatial relation for positioning SRS in RRC_INACTIVE state – location server; Same as LPP SpatialRelationsSRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state; No; Spatial relation for positioning SRS in RRC_INACTIVE state is not supported (location server); Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Support of spatial relation in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signalling. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] | - FG 27-19a: Spatial relation for positioning SRS in RRC_INACTIVE state – location server - Considering the prerequisite of spatial relation in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG. |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. 2. Supported resource set relationship for the target PRS resource and the associated subset [3. Support associated subset measurement reporting] |  | No |  | PRS subset association for DL-AoD is not supported by the UE. | Per UE | n/a | n/a | n/a | Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet} [Component 3 candidate values: {associated subset only, the target PRS resource and the associated subset}] Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-20; PRS subset association for UE assisted DL-AoD; 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. 2. Supported resource set relationship for the target PRS resource and the associated subset 3. Support associated subset measurement reporting including both cases of reporting subset only and of reporting the target PRS and the associated subset.; No; PRS subset association for DL-AoD is not supported by the UE.; Per UE; No; No; No; Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet} Need for location server to know; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] | Components 2 and 3 have been discussed in a couple of meetings and Components 2 has been supported, even though we are supportive of component 3, we can accept removing component 3 for the progress the above agreement was reached. - For FG 27-20, Component 3 can be removed. |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] | In our view, the component 3 is not needed. As in previous RAN1 agreement, the UE may report the RSRP measurement of PRS resources in the associated subset if the UE reports the RSRP measurement of the target PRS resource. But when the UE does not report the RSRP measurement of the target PRS resource, the UE still can report the RSRP measurement of PRS resources in the subset, which is a normal DL AoD measurement reporting. Thus, such case shall not be included in this FG. Proposal 2: Do not support component 3 in FG 27-20. |
| NTT DOCOMO, INC. [9] | - FG 27-20: PRS subset association for UE assisted DL-AoD - Support the current FG 27-20 (i.e. the brackets on the component 3 can be removed) - A consistent wording should be used as follows: - “UE assisted DL-AoD” can be changed to “UE-assisted DL-AoD” |
| Nokia/Nokia Shanghai Bell [10] | - Component 3 is not needed. |
| Ericsson [11] | The support of subset measurement reports is not a necessary component of the FG. The report of the subset is no different from reports from the main PRS. therefore we think the component #3 can be removed from 27-20. Component 3 of FG 27-20 can be removed. |
| Qualcomm Incorporated [12] | - Remove component 3 27. NR_pos_enh; 27-20; PRS subset association for UE assisted DL-AoD; 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. 2. Supported resource set relationship for the target PRS resource and the associated subset; No; PRS subset association for DL-AoD is not supported by the UE.; Per UE; n/a; n/a; n/a; Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet} Need for location server to know; Optional with capability signaling.. |

| 27. NR_pos_enh | 27-21 | PRS boresight direction for UE-assisted DL-AoD | Support of assistance data enhancement to indicate the boresight direction of a PRS resource for UE-assisted DL-AoD. |  | No |  | UE-assisted DL-AoD with boresight direction of each DL-PRS is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-21; PRS boresight direction for UE-assisted DL-AoD; Support of assistance data enhancement to indicate the boresight direction of a PRS resource for UE-assisted DL-AoD.; No; UE-assisted DL-AoD with boresight direction of each DL-PRS is not supported.; Per UE; No; No; No; Need for location server to know; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

| 27. NR_pos_enh | 27-22 | PRS beam pattern for UE-based DL-AoD | Support of PRS beam pattern for DL-AoD |  | No |  | UE-based DL-AoD with PRS beam pattern is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 27. NR_pos_enh; 27-22; PRS antenna beam pattern for UE-based DL-AoD; Support of PRS antenna beam pattern for DL-AoD; No; UE-based DL-AoD with PRS antenna beam pattern is not supported.; Per UE; No; No; No; Need for location server to know; Optional with capability signaling.. |
| CATT [3] |  |
| Vivo [4] |  |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

Others

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | We think that additional FGs to support more than one activated PRS processing window is needed. ### FG 27-xx 27. NR_pos_enh; 27-xx; Support of more than one activated PRS processing windows; 1. Number of supported activated PRS processing windows; One of {27-3-2a, 27-3-2b, 27-3-2c}; Yes; Only one activated PRS processing window is supported.; Per UE; No; No; No; Candidate values:{2, 3, 4}; Optional with capability signaling. We think that SP SRS capability may also be added for option 2 SRS transmission. ### FG 27-15f 27. NR_pos_enh; 27-15f; Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS; 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot; 27-15b; Yes; Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported; Optional with capability signaling. |
| CATT [3] | In the LS from RAN4 [1], RAN4 has discussed the threshold applied to the Rx timing difference between the serving cell PRS and PRS from a non-serving cell/TRP to determine the applicability of PRS measurements outside MG. RAN4 has agreed to introduce a UE capability to accommodate different UE implementations of PRS measurements outside MG. - Introduce UE capability for the maximum Rx timing difference in MG-less PRS measurement - Option 1: two values {CP length, 0.5 slot} - Other options are not precluded The threshold corresponds to the maximum Rx timing difference that the UE supports for PRS measurement outside MG. The FG 27-3-2(PRS processing window) should be the prerequisite feature group of this new FG. Based on the above discussions, we propose to introduce a new FG for the maximum Rx timing difference in MG-less PRS measurement as follows, Proposal 1: Introduce a new FG for the maximum Rx timing difference in MG-less PRS measurement FG 27-N based on RAN4’LS: 27. NR_pos_enh; 27-N; Maximum Rx timing difference in MG-less PRS measurement; 1. Support of maximum Rx timing difference in MG-less PRS measurement 2. The maximum Rx timing difference in MG-less PRS measurement; 27-3-2; No; The maximum Rx timing difference in MG-less PRS measurement is not supported and no assumption can be made on the maximum Rx timing difference in MG-less PRS measurement; per band; n/a; n/a; n/a; Component 2 candidate values: {CP length, 0.5 slot} Need for location server to know if the feature is supported; Optional with capability signaling. |
| Vivo [4] | In RAN4 LS [3], the following agreement regarding TEG timing error margin was achieved. - The framework of UE/TRP Rx TEG: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. - For UE that supports multiple Rx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Rx TEG is M, which means the timing error difference between the measurements within the same Rx TEG is within the margin M. - The applicability of reported UE Rx TEG is limited to the measurements contained within the measurement report in which the Rx TEG information is provided, and only to measurements that are tagged with the corresponding TEG ID. - The RRM accuracy requirements corresponding to the candidate timing error margin values {TE1, TE2, …, TEN} will be defined in Perf part. - The framework of UE/TRP Rx TEG can be also applied for UE/TRP RxTx TEG - Note: if additional issues are identified based on RAN1/2 progress, then this agreement can be revised.. It should be noted that UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF. So, from RAN1 perspective, for a UE, how to indicate the selected time error margin value to LMF should be discussed. From our point of view, UE can select Rx timing error margin before UE capability report. Based on the margin, The Rx timing errors can be grouped and the maximum number of Rx TEGs for each band can be determined. Then, UE can report supported number of Rx TEGs along with Rx timing error margin via UE capability signaling. In addition, regarding the type of Rx timing error margin, ‘per UE’ type can be supported according to the following RAN4 agreement. Agreements: - The same timing margin is used for all Rx TEGs per UE/TRP. Similarly, per UE type capability of RxTx timing error margin for RxTx TEG can be supported. For timing error margin of Tx TEG, we can wait for further RAN4 conclusion. - Support the UE capability of timing error margin for Rx TEG and RxTx TEG. - The capability type is per UE. |
| ZTE [5] |  |
| China Telecom [6] |  |
| Samsung [7] |  |
| OPPO [8] |  |
| NTT DOCOMO, INC. [9] |  |
| Nokia/Nokia Shanghai Bell [10] |  |
| Ericsson [11] |  |
| Qualcomm Incorporated [12] |  |

# Discussion/Approval Items during RAN1 #109-e — First Checkpoint

After review of contributions submitted to RAN1 #109-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #109-e.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 27-1-4a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously | 27-1-4 | No |  | No assumption can be made regarding multiple Rx TEGs measuring the same DL PRS resource simultaneously | Per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| vivo | OK |
| CATT | OK |
| Ericsson | ok |

# Issue 2: FG 27-2-1

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-2-1 | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP | 13-5 | No |  | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD is not supported | per band | n/a | n/a | n/a | Component 2 candidate values: 1, 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) [Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| vivo | The Note should be deleted. According to the following RAN4 agreement, same Rx branches are applied for PRS-RSRP and PRS-RSRPP, which means that there is a dependency between the two measurements. We support reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource. Agreements: - Same Rx branches as applied for PRS-RSRP measurement are used for path PRS-RSRP measurement. |
| CATT | Support. |
| Huawei, HiSilicon | We proposed to remove the note, because the note is already duplicated with an agreement, which can be implemented by RAN2. However, we have different understanding from vivo that for DL-AoD, it is possible that the main RSRP measurement is provided, but all the additional measurements are RSRPP-only. In this case, it is allowed for a specific PRS resource, RSRPP is provided, by RSRP is not. If removing the note will create such confusion, we would support to keep the Note. |
| Nokia, NSB | Unfortunately the note is quite confusing, and it doesn’t really belong to a UE FG, in our view. It should be removed. |
| NTT DOCOMO | Support |
| Vivo2 | Based on double-check agreement, we think the view of Huawei is right, and then we also should remove it and left to RAN2 Agreement For the reporting of first path DL-PRS RSRPP in DL AOD, - For the 1st measurement, the report includes DL PRS RSRP and optionally DL PRS RSRPP using absolute reporting - For additional measurement, at least one of the two following measurement is reported: - First path DL PRS RSRPP can be optionally reported using differential reporting with the first measurement of DL PRS RSRPP, - DL PRS RSRP can be optionally reported using differential reporting with the first measurement of DL PRS RSRP. |
| Ericsson | ok |

# Issue 3: FG 27-2-2

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a No | Yes FR2 only | n/a No | The candidate values are {16, 24} Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Need for location server to know if the feature is supported The maximum number of reported DL PRS RSRP in the capability signaling should be no less than the maximum number of reported DL PRS RSRPP of the first path in the capability signaling | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| Qualcomm | We don’t see the need to restrict it to FR2 only. Even in FR1, if we have 2 sets per TRP, each set having 8 PRS resources, we could have 16 resources in a TRP that the UE may be able to report. |
| vivo | Generally OK. Regarding the column of ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’, why change ‘n/a’ to ‘No’? |
| CATT | Support. |
| Huawei, HiSilicon | OK not to restrict it to FR2 as Qualcomm explained. On “capability interpretation for mixture of FDD/TDD and/or FR1/FR2”, maybe “n/a” is more appropriate. Given this column does not exist in Rel-15 or Rel-16, some guidance coiuld be useful. |
| Nokia, NSB | Unclear why it would be restricted to FR2 only. |
| Ericsson | No need to restrict this to FR2. We also think it is better to leave On “capability interpretation for mixture of FDD/TDD and/or FR1/FR2” column as “n/a”. |

# Issue 4: FG 27-3-1

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-1 | M-sample measurements | The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set | 13-1 | No |  | If the UE does not provide the capability, the UE is assumed to support M=4 only | per band | n/a | n/a | n/a | The candidate values are {1 [FFS others]} Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | In RAN4’s agreements, the number of sample = 2 is also possible. - Agreements: - Define low latency requirements the reduced number of samples: - Nsample= 1 if the condition under which AGC is not required is met - Nsample= 2 if the condition under which AGC is not required is NOT met As AGC issue has been discussed in RAN4, we suggest the following wording The capability to support reporting a measurement based on measuring M=1 or 2 samples (instances) of a DL PRS resource set; 13-1; No; If the UE does not provide the capability, the UE is assumed to support M=4 only; per band; n/a; n/a; n/a; The candidate values are {1 [FFS others]} Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning. |
| vivo | Based on the agreed feature list in last RAN4 meeting [R4-2206979]. It is better to use the same format as RAN4 that the candidate value can be {1, 2} and remove note 1 and“=1“ after M. 14 NR_pos_enh; 14-2; PRS measurement for reduced sample in RRC_inactive state; Capability of supporting reduced number of samples (M=1, 2) for PRS measurement in RRC_inactive state; [27-17]; no; The reduced number of samples (M=1,2) for PRS measurement in RRC_inactive state cannot be supported. The UE is assumed to support M=4 only.; Per UE; No; No; N/A; Optional with capability signaling. |
| CATT | We prefer to add M=2 samples in the component for this FG as follows, The capability to support reporting a measurement based on measuring M=1 or 2 samples (instances) of a DL PRS resource set. |
| Huawei, HiSilicon | OK to keep the same style as RAN4 as vivo indicated |
| Nokia, NSB | No need to add M=2. |
| Ericsson | following similar format as RAN4 as suggested by vivo is fine with us. |

# Issue 5: FG 27-3-2

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS | 13-1 | Yes |  | DL PRS measurement outside MG and in a PRS processing window is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE, details left to RAN2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window - The DL signals/channels from a certain CC, which overlap with DL PRS symbols in time are affected in FR1, - The DL signals/channels from a certain band, which overlap with DL PRS symbols in time are affected in FR2 Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | As we mentioned several times, both FG 27-3-2 and 27-3-3 should be basic features for PPW. Hence a note is needed, such as if FG 27-3-2 is reported, 27-3-3 should be reported as well. |
| Huawei, HiSilicon | We believe the proposal from ZTE is reasonable. In addition, as proposed in our paper R1-2103100, we suggest to break the FG into multiple one, each one corresponds to the support of a single PPW type. 27. NR_pos_enh; 27-3-2a; DL PRS measurement outside MG and in a type-1A PRS processing window - gNB. 27. NR_pos_enh; 27-3-2b; DL PRS measurement outside MG and in a type-1B PRS processing window - gNB. 27. NR_pos_enh; 27-3-2c; DL PRS measurement outside MG and in a type-2 PRS processing window - gNB. 27. NR_pos_enh; 27-3-4a; DL PRS measurement outside MG and in a type-1A PRS processing window - location sever. 27. NR_pos_enh; 27-3-4b; DL PRS measurement outside MG and in a type-1B PRS processing window - location sever. 27. NR_pos_enh; 27-3-4c; DL PRS measurement outside MG and in a type-2 PRS processing window - location sever. The FG 27-3-3 should be added as a component in each split FG reported to the LMF. We also think that adding a component “support of PPW configuration and activation/deactivation by MAC CE” is needed for the capability reported to gNB, which is without dedicated signaling. |
| Ericsson | We agree with ZTE’s proposal. Both FG 27-3-2 and FG 27-3-3 are part of the basic capability for PRS processing windows. So it does not make sense to report them separately. So we suggest either to merge 27-3-3 with 27-3-2 or to capture a note that they are reported together. |

# Issue 6: FG 27-3-3

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG – buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Type 1 – Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Type 2 – Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 4. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms Component 4 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support. If possible, we prefer to make the prerequisite as FG 13-1, and add a note, such as if FG 27-3-3 is reported, FG 27-3-2 should be reported as well. |
| Vivo | This FG is under discussion in AI 8.5, so we can wait for the conclusion before making the decision. Besides, we can not agree with component 2 directly, we don’t think this is a great way that the value of T to be defined as N+5, when it is actually used, the processing time is T-N. In actual, the processing time is also {4,5,6,8}, so, why don’t we just define the processing time directly? For us, we prefer reusing component 3 in FG 13-1 and discuss the candidate value in FG 27-3-3. In addition, in the RAN4#102-e meeting, the following agreement has been achieved that Tlast was only needed to consider the MGL if all of the PRS resources to be measured are available in the same MG occasion so that the latency can be significantly reduced. So, for us, we don’t agree with the description of type 1 in capability. Agreements in RAN4#102-e: Define Tlast as T+MGL when all of the PRS resources to be measured are available in the same MG occasion during Tavailabe. TS 38.133 h50 is the measurement duration for the last PRS RSTD sample in positioning frequency layer I, including the sampling time and processing time. If all of the PRS resources to be measured are available in the same MG occasion during Tavailabe, = +MGL. Otherwise, = +. |
| CATT | We prefer to wait for the related discussion in AI8.5 before agreeing this FG. |
| Huawei, HiSilicon | Agree other other companies, at least component 2 is still under WI discussion. |
| Ericsson | We prefer to report FG 27-3-2 and FG 27-3-3 together. Also, this FG is related to the length of the PPW. In Rel-17, the PPW will be configured by the gNB. Then, it seems this capability needs to be known to the serving gNB also. Hence, we propose to revise column 6 as ‘Yes’ instead of ‘No’. |

# Issue 7: FG 27-4-1

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity | one of 13-5,13-6, or 13-11 | No |  | LOS/NLOS Indicator for UE-assisted positioning is not supported | Per UE | n/a No | n/a No | n/a No | [Component 1 candidate values: {hard value, soft value[, both]}] Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Support |
| vivo | As far as we are concerned, if a UE supports LOS/NLOS indicator of soft value type, it will of course support the hard value 0 and 1. Therefore, the candidate values can be {hard value, both hard value and soft value}. But if majority can agree with the current proposal, we can compromise on the progress. Then, regarding the column of ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’, we wonder why change ‘n/a’ to ‘No’? |
| CATT | Support. |
| Nokia, NSB | We do not see the technical reason to require indication of component values here. |
| NTT DOCOMO | Support |
| Ericsson | Ok |

# Issue 8: FG 27-6

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot 4. Support of single -sample measurements in RRC Inactive state: The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set |  | No |  | DL PRS processing in RRC inactive state is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: The sample number M=1 does not account for the potential AGC sample | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | In our view, many features such as TEG, lower RX beam sweeping factor, LOS/NLOS indicator are also applicable for RRC inactive state. We don’t need to list all of them. If there is no clear description in the spec, the new features should be applicable regardless of RRC state. Hence, we prefer not to add component 4. |
| Qualcomm | After further checking, we withdraw this proposal, since the capability is already agreed in RAN4 and has been added in the RAN4 capability spreadsheet. Therefore, this Issue could potentially be closed. |
| Vivo | Regarding the component 4, we think it is duplicated with the agreed feature list in last RAN4 meeting [R4-2206979]. It is better to use the same format as RAN4, and how to capture the RAN4’s output is up to the editor. 14 NR_pos_enh; 14-2; PRS measurement for reduced sample in RRC_inactive state; Capability of supporting reduced number of samples (M=1, 2) for PRS measurement in RRC_inactive state; [27-17]; no; The reduced number of samples (M=1,2) for PRS measurement in RRC_inactive state cannot be supported. The UE is assumed to support M=4 only.; Per UE; No; No; N/A; Optional with capability signaling. |
| CATT | The new component 4 is not needed. |
| Nokia, NSB | Component 4 and corresponding note are not needed, and they would be part of RAN4 scope, as already mentioned by the proponent. |
| Ericsson | Agree with other that the additional next in red seems not needed. |

# Issue 9: FG 27-8

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning | 13-1 | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a No | n/a No | n/a No | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Regarding the column of ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’, we wonder why change ‘n/a’ to ‘No’? |
| Nokia, NSB | OK |
| Ericsson | We have similar question as vivo. Why change ‘n/a’ values to ‘No’ in the three columns? Could the proponents clarify the reason? |

# Issue 10: FG 27-10

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request for PRS measurements – gNB | 1. Support of using UL MAC CE to request measurement gap activation/deactivation for PRS measurements: The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG 2. Support of preconfiguration of MGs in RRC ignalling for PRS measurements: Each MG in the preconfiguration is associated with an ID | 27-11 | Yes |  | Using UL MAC CE to indicate measurement gap for PRS measurements to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | In the third column, ‘- gNB’ makes the sentence unreadable. |
| Huawei, HiSilicon | This is already adopted in the previous version to imply this FG is reported to gNB only (not to LMF) |
| Nokia, NSB | Agree with ZTE, this addition is not justified and it doesn’t add value. |
| Ericsson | OK to add the text in component 1. Regarding the “gNB” text, it should not be needed. All capabilities are for the gNB unless we mention that the capability is needed by the LMF. |

# Issue 11: FG 27-10a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements - location sever | support of low latency MG activation request for PRS measurements | [27-10] 27-11 | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | In the third column, ‘- location sever’ makes the sentence unreadable. FG 27-10 should be the prerequisite as well. The motivation of FG 27-10a is to tell LMF that the UE support FG 27-10, then LMF may not need to request MG from gNBs for the sake of overhead reduction. In other words, if FG 27-10 is not reported, why UE reports 27-10a? |
| Huawei, HiSilicon | We do not think any prerequisite FG is needed, since this FG is reported to the LMF, and linking it to the capability reported to gNB is not needed. |
| Nokia, NSB | Agree with ZTE comments above. |
| NTT DOCOMO | We have similar view to Huawei. |
| Ericsson | Similar views as ZTE. No need to add “location server” in the third column. The need is already mentioned in the text “Need for location server to know if the feature is supported” |

# Issue 12: FG 27-11

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation request for PRS measurements | 1. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID 2. Support of using DL MAC CE to activate/deactivate the MG for PRS measurements: The DL MAC CE for MG activation indicates the ID associated with the preconfigured MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG for PRS measurements is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | A typo, ‘Support of DL MAC CE based MG activation request for PRS measurements’ |
| Huawei, HiSilicon | Support vivo’s correction. |
| Ericsson | OK (also with vivo’s change) |

# Issue 13: FG 27-12

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity |  | No |  | LOS/NLOS indicator for UE-based positioning assistance data is not supported | Per UE | No | No | No | [Component 1 candidate values: {softValue, hardValue, both}] Component 2 candidate values: {resourceSpecific, trpSpecific[, both]} Need for location server to know if the feature is supported. | Optional with capability ignalled. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Same view as FG27-1-4. |
| CATT | Support |
| Nokia, NSB | Do not support, same view as with FG27-4-1 |
| NTT DOCOMO | Support |

# Issue 14: FG 27-15

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15 | Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP | 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of P/SPSRS Resources for positioning 3. Max number of P/SPSRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot |  | Yes |  | Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 4 candidate values: {1,2,4,8,16,32,64} Component 5 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability ignalled is intended for this component Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Support ‘P/SP’ in components 2 and 3. |
| CATT | Support |
| Nokia, NSB | Unclear why the change would be needed. Do not support |
| Ericson | Ok |

# Issue 15: FG 27-15b

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15b | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Support of Different numerology between the SRS and the initial UL BWP is supported] - [Support of SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: Support a different center frequenecy between the SRS for positioning and the initial UL BWP - Switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP | 27-15 | Yes |  | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: FFS - FR1 bands: {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100} - FR2 bands: {50, 100, 200, 400} Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: FFS Component 6 candidate values: FFS Component 7 candidate values: {1,2,4,8,16,32,64} FFS Component 8 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 10 candidate values: {0, 30, 100, 140, 200, 300, 500, 900} us for DL and UL respectively Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. Note 2: If component 9 is not signaled, the UE only supports same center frequency between the SRS for positioning and initial UL BWP [Note 2: Based on other ignalle UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.] [Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP] [Note 4: If component 6 is not ignalled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.] [Need for location server to know if the feature is supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Component 10 values are being discussed in Ran4. We sent an LS to RAN4 previous meeting. We suggest to keep the values as FFS: For RAN4. Unclear why Note 3, and Note 4 are removed. We suggest to keep them. |
| Vivo | Support ‘P/SP’ in component 7 and 8. Note 3 and Note 4 should be included as the explanation of component 5/6 if they are not signaled. |
| CATT | We prefer to keep Note 3 and Note 4. |
| Huawei, HiSilicon | We prefer to remove “P/SP” to have forward compabitlity if aperiodic SRS for positioning is supported in future releases, so that the total number of SRS can still use the same signaling (Rel-17). On component 5, 6, 9, we prefer to define the corresponding FGs with explicit signaling, so that FG reads - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - Support of the same numerology between the SRS and the initial UL BWP - Support of operation with restriction on the BW: BW of the SRS includes BW of the CORESET#0 and SSB - Max number of SRS Resources for positioning - Max number of SRS Resources for positioning per slot - Support of the same centre frequency between the SRS for positioning and the initial UL BWP - Switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP And we could have another three FGs to handle different numerologies, without BW restriction, and different centre frequency. |

# Issue 16: FG 27-15a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15a | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15b | Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  | Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | The index of ’27-15b’ above should be changed to ’27-15c’, since index of ‘27-15b’ appears 2 times. |
| Huawei, HiSilicon | Agree with vivo, a new suffix should be used. |
| Nokia, NSB | Agree with vivo, but also unclear why the new FG would be needed. On 27-15a it is unclear why LMF would need to know that. |

# Issue 17: FG 27-16

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: additional proposals are being discussed in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state - gNB | Same asRRC OLPC-SRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state |  | Yes |  | OLPC for positioning SRS in RRC_INACTIVE state is not supported (gNB) | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | The updated part seems unnecessary as the title of this FG clearly mentions RRC_INACTIVE |
| vivo | Firstly, we wonder the intention of ‘except that the feature is applicable to the SRS in RRC_INACTIVE state’. In addition, we suggest focusing on addressing the issues from RAN2 LS (R2-2204360) as follows. B). R1 27-16 and 27-19 R1 27-16 and 27-19 have a component description of ‘Same as RRC OLPC-SRS-Pos-r16’ and ‘Same as RRC SpatialRelationsSRS-Pos-r16’ respectively. It is unclear to RAN2 whether the pre-requisite in R1 27-16/27-19 should be srs-PosResources-r16 as in RRC OLPC-SRS-Pos-r16/SpatialRelationsSRS-Pos-r16 or should be “srs-PosResourcesRRC-Inactive-r17” (i.e. R1 27-15)). RAN2 would like RAN1 to clarify the pre-requisite used in the R1 27-16 and 27-19.. From our point of view, the description of ‘Same as RRC OLPC-SRS-Pos-r16’ in FG27-16 is to reuse the content of FG13-9 series for OLPC support, not the pre-requisite groups of FG13-9 series, which is also applied to FG27-19. Therefore, for the pre-requisite groups for both FG 27-16 and FG 27-19, we think basic feature groups in inactive state can be considered, e.g. FG 27-15, FG 27-15b, and FG 27-6 (for PRS related capability, e.g., OLPC support based on PRS). |
| CATT | We prefer to remove the text of “except that the feature is applicable to the SRS in RRC_INACTIVE state”. |
| Huawei, HiSilicon | Support. RAN2 has difficulty implementing the “same as …”, because they assume this capability applies to the SRS configured in Rel-16 as cited in the current Rel-16 TS 38.306. Otherwise, we may have to break this FG into multiple FGs, and basically rewrite everything again. |
| NTT DOCOMO | The red part may not be needed. We think [109-e-R17-UE-features] discussion (e.g. introducing the prerequisite FG) is enought to clarify FG 27-16. |

# Issue 18: FG 27-16a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: additional proposals are being discussed in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 27. NR_pos_enh | 27-16a | OLPC for positioning SRS in RRC_INACTIVE state – location server | Same as LPP OLPC-SRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state |  | No |  | OLPC for positioning SRS in RRC_INACTIVE state is not supported (location server) | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Support of OLPC in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | The updated part seems unnecessary as the title of this FG clearly mentions RRC_INACTIVE |
| CATT | We prefer to remove the text of “except that the feature is applicable to the SRS in RRC_INACTIVE state”. |
| Huawei, HiSilicon | Support. RAN2 has difficulty implementing the “same as …”, because they assume this capability applies to the SRS configured in Rel-16 as cited in the current Rel-16 TS 38.306. Otherwise, we may have to break this FG into multiple FGs, and basically rewrite everything again. |
| NTT DOCOMO | The red part may not be needed. We think [109-e-R17-UE-features] discussion (e.g. introducing the prerequisite FG) is enought to clarify FG 27-16a. |

# Issue 19: FG 27-17

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-17 | PRS processing in RRC_INACTIVE - gNB | 1. Support of PRS processing in RRC_INACTIVE 2. Support of positioning SRS transmission in RRC_INACTIVE state | [13-1,13-8 13-2, 13-3, 13-4] | Yes |  | PRS processing in RRC_INACTIVE is not supported | per band | n/a | n/a | n/a | Note: UE supporting this feature may shall support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | The new component 2 should not be here. FG 27-15 has reflected support of SRS in RRC_INACTIVE state. |
| Qualcomm | It is unclear the need of component 2, given that 27-15 and 27-15b is already reported the gNB in a per band fashion. Could the proponents clarify what more information is provided to the gNB by adding one more component in this FG? 13-8 doesn’t need to be a prerequisite either, since a UE may support DL only methods in RRC Inactive. |
| vivo | We don’t see the need to include ‘Support of positioning SRS transmission in RRC_INACTIVE state’, since the capability of SRS to gNB has already been agreed in other FGs. Correspondingly, FG 13-8 as prerequisite group should be deleted. |
| CATT | We prefer to remove the component 2 in this FG. |
| Huawei, HiSilicon | Adding the component 2 will mean that UE needs to support both components, and component-wise support/not support signaling is not allowed. In this sense, we do not support adding component 2. |
| Nokia, NSB | We prefer to keep component 2, but we will not insist in case we are the only company with that view. The name change is “- gNB” is confusing. |
| NTT DOCOMO | We prefer to remove the component 2. |

# Issue 20: FG 27-18a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA - location server | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | 13-3, 13-3a, 13-3b, 13-6, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for DL-TDOA is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Comment 1: 13-6 doesn’t need to be a prerequisite: In UE-based, there is no need the UE to report measurements. Comment2: We don’t see the need that 13-3a, 13-3b is prerequisite because, there is a note in those that says: “Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.”. Therefore, the UE can avoid reporting them, and still there will be no problem in understanding whis bands/BCs the UE supports. |
| Huawei, HiSilicon | Ok to remove 13-6 as Qualcomm explained. Ok to remove FG 13-3a and FG 13-3b. |
| ZTE | We think FG 13-3a and 13-3b should be prerequisite because both of them and 27-18a are per band granularity. LMF need to know the maximum number of PRS resources from FG 13-3a and 3b. |

# Issue 21: FG 27-18b

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD - location server | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | 13-2, 13-2a, 13-2b, 13-5, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for DL-AoD is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Comment 1: 13-5 doesn’t need to be a prerequisite: In UE-based, there is no need the UE to report measurements. Comment2: We don’t see the need that 13-2a, 13-2b is prerequisite because, there is a note in those that says: “Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.”. Therefore, the UE can avoid reporting them, and still there will be no problem in understanding whis bands/BCs the UE supports. |
| ZTE | We think FG 13-2a and 13-2b should be prerequisite because both of them and 27-18b are per band granularity. LMF need to know the maximum number of PRS resources from FG 13-2a and 2b. |

# Issue 22: FG 27-18c

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT - location server | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 13-4, 13-4a, 13-4b, 13-11, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for for Multi-RTT is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | We don’t see the need that 13-4a, 13-4b is prerequisite because, there is a note in those that says: “Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.”. Therefore, the UE can avoid reporting them, and still there will be no problem in understanding whis bands/BCs the UE supports. |

# Issue 23: FG 27-19

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: additional proposals are being discussed in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state - gNB | Same asRRC SpatialRelationsSRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state |  | Yes |  | Spatial relation for positioning SRS in RRC_INACTIVE state is not supported (gNB) | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | The same views as FG 27-16. |
| CATT | We prefer to remove the text of “except that the feature is applicable to the SRS in RRC_INACTIVE state”. |
| Huawei, HiSilicon | Support. RAN2 has difficulty implementing the “same as …”, because they assume this capability applies to the SRS configured in Rel-16 as cited in the current Rel-16 TS 38.306. Otherwise, we may have to break this FG into multiple FGs, and basically rewrite everything again. |
| NTT DOCOMO | The red part may not be needed. We think [109-e-R17-UE-features] discussion (e.g. introducing the prerequisite FG) is enought to clarify FG 27-19. |

# Issue 24: FG 27-19a

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Note: additional proposals are being discussed in [109-e-R17-UE-features] “Email discussion on incoming LS (R1-2205090) on updated Rel-17 RAN1 UE features list for NR by May 13 – Ralf (AT&T)”

| 27. NR_pos_enh | 27-19a | Spatial relation for positioning SRS in RRC_INACTIVE state – location server | Same as LPP SpatialRelationsSRS-Pos-r16 except that the feature is applicable to the SRS in RRC_INACTIVE state |  | No |  | Spatial relation for positioning SRS in RRC_INACTIVE state is not supported (location server) | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Support of spatial relation in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | We prefer to remove the text of “except that the feature is applicable to the SRS in RRC_INACTIVE state”. |
| Huawei, HiSilicon | Support. RAN2 has difficulty implementing the “same as …”, because they assume this capability applies to the SRS configured in Rel-16 as cited in the current Rel-16 TS 38.306. Otherwise, we may have to break this FG into multiple FGs, and basically rewrite everything again. |
| NTT DOCOMO | The red part may not be needed. We think [109-e-R17-UE-features] discussion (e.g. introducing the prerequisite FG) is enought to clarify FG 27-19a. |

# Issue 25: FG 27-20

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. 2. Supported resource set relationship for the target PRS resource and the associated subset [3. Support associated subset measurement reporting] |  | No |  | PRS subset association for DL-AoD is not supported by the UE. | Per UE | n/a No | n/a No | n/a No | Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet} [Component 3 candidate values: {associated subset only, the target PRS resource and the associated subset}] Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | We can accept to remove component 3 for the progress. |
| CATT | Support |
| Huawei, HiSilicon | We think adding component 3 without dedicated signaling does not harm. Maybe a Note can replace this component 3, such like Note: Support associated subset measurement reporting including both cases of reporting subset only and of reporting the target PRS and the associated subset is part of the FG, but no dedicated signaling is intended. |
| Nokia, NSB | Agree with vivo and CATT on removing component 3. We do not support the note from Huawei, it is not really needed. |

# Issue 26: FG 27-21

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-21 | PRS boresight direction for UE-assisted DL-AoD | Support of assistance data enhancement to indicate the boresight direction of a PRS resource for UE-assisted DL-AoD. |  | No |  | UE-assisted DL-AoD with boresight direction of each DL-PRS is not supported. | Per UE | n/a No | n/a No | n/a No | Need for location server to know | Optional with capability ignalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Regarding the column of ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’, we wonder why change ‘n/a’ to ‘No’? |

# Issue 27: FG 27-22

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-22 | PRS antenna beam pattern for UE-based DL-AoD | Support of PRS antenna beam pattern for DL-AoD |  | No |  | UE-based DL-AoD with PRS antenna beam pattern is not supported. | Per UE | n/a No | n/a No | n/a No | Need for location server to know | Optional with capability ignalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Firstly, OK to add ‘antenna’. Then, regarding the column of ‘Capability interpretation for mixture of FDD/TDD and/or FR1/FR2’, we wonder why change ‘n/a’ to ‘No’? |
| CATT | Support |

# Issue 28: New FGs

After review of contributions submitted to RAN1 #109-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following new FGs

| 27. NR_pos_enh | 27-23 | Support of more than one activated PRS processing windows | 1. Number of supported activated PRS processing windows | One of {27-3-2a, 27-3-2b, 27-3-2c} | Yes |  | Only one activated PRS processing window is supported. | Per UE | No | No | No | Candidate values:{2, 3, 4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-24 | Maximum Rx timing difference in MG-less PRS measurement | 1. Support of maximum Rx timing difference in MG-less PRS measurement 2. The maximum Rx timing difference in MG-less PRS measurement | 27-3-2 |  | No | The maximum Rx timing difference in MG-less PRS measurement is not supported and no assumption can be made on the maximum Rx timing difference in MG-less PRS measurement | per band | n/a | n/a | n/a | Component 2 candidate values: {CP length, 0.5 slot} Need for location server to know if the feature is supported | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | 1st capability: OK 2nd capability: Since this is being discussed in RAN4, RAN4 will send a capability to RAN2 directly; we could wait at this point, to avoid any conflicting agreements. |
| Vivo | We are OK to introduce FG27-23 with the following modifications. - The descriptions should be changed to ‘Number of supported activated PRS processing windows across all active DL BWPs’, which aligns with RAN1 agreement. Agreement - The PRS processing window is configured per DL BWP. - Processing type, to be selected from 1A, 1B and 2, will be provided associated with the PRS processing window if and only if multiple processing types per band in the UE capability signaling is supported. - No need to provide band ID and CC ID associated with the PRS processing window. - A single priority indicator is provided for a PRS processing window, which applies to all PRS within the PRS processing window for the corresponding DL BWP. - The maximum number of activated PRS processing windows per DL BWP is 1. - The maximum number of activated PRS processing windows across all active DL BWPs is 4. - The maximum number of activated PRS processing windows overlapping in time across all active DL BWPs is 1. - The prerequisite group should be changed to ’27-3-2’, since there is no ‘{27-3-2a, 27-3-2b, 27-3-2c}’ agreed in the feature list. Then, regarding the capability of ‘FG 27-24 Maximum Rx timing difference in MG-less PRS measurement’, the following feature list has already been agreed by RAN4 [R4-2206979], which can be captured by editor as reference. 14. NR_pos_enh; 14-3; PRS measurement without MG; Capability for the threshold used to be compared against with the Rx timing difference to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG.; [27-3-2]; yes; Per UE; No; No; N/A; The candidate threshold values: [CP length, half of slot]; Optional with capability signaling. |
| CATT | Support both the FG 27-23 and FG 27-24. The motivation for the introduction of FG 27-24 as follows, In the LS from RAN4 (R1-2203023), RAN4 has discussed the threshold applied to the Rx timing difference between the serving cell PRS and PRS from a non-serving cell/TRP to determine the applicability of PRS measurements outside MG. RAN4 has agreed to introduce a UE capability to accommodate different UE implementations of PRS measurements outside MG. - Introduce UE capability for the maximum Rx timing difference in MG-less PRS measurement - Option 1: two values {CP length, 0.5 slot} - Other options are not precluded The threshold corresponds to the maximum Rx timing difference that the UE supports for PRS measurement outside MG. Therefore, we propose to Introduce the new FG for the maximum Rx timing difference in MG-less PRS measurement (FG 27-24 above) based on RAN4’LS. |
| Huawei, HiSilicon | OK with the change from vivo and the corresponding prerequisite FGs. |
| Ericson | Ok to introduce the first capabiltly FG 27-23. For proposed FG 27-24, we share similar view as vivo. |

# Discussion/Approval Items during RAN1 #109-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 3 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 27-1-4a

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously | 27-1-4 | No |  | No assumption can be made regarding multiple Rx TEGs measuring the same DL PRS resource simultaneously | Per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – no need to comment in support of the proposal – please leave table empty unless there are concerns]

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 2: FG 27-2-2

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | Yes | n/a | The candidate values are {16, 24} Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Need for location server to know if the feature is supported The maximum number of reported DL PRS RSRP in the capability signaling should be no less than the maximum number of reported DL PRS RSRPP of the first path in the capability signaling | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – no need to comment in support of the proposal – please leave table empty unless there are concerns]

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 3: FG 27-3-2

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS | 13-1 | Yes |  | DL PRS measurement outside MG and in a PRS processing window is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE, details left to RAN2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP A UE that supports FG 27-3-3 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | We want to add the following Note under component 2 (fourth column) Note: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the FG, but no dedicated signaling is required. |
| CATT | Support |
| ZTE | Support |

# Issue 4: FG 27-3-3

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band A UE that supports FG 27-3-2 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| CATT | Support |
| ZTE | OK |

# Issue 5: FG 27-10

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request for PRS measurements | 1. Support of using UL MAC CE to request measurement gap activation/deactivation for PRS measurements: The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG 2. Support of preconfiguration of MGs in RRC ignalling for PRS measurements: Each MG in the preconfiguration is associated with an ID | 27-11 | Yes |  | Using UL MAC CE to indicate measurement gap for PRS measurements to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Issue 6: FG 27-10a

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-10] | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | The prerequisite FG should be removed. Again it is not needed for LMF and gNB to identify the FG dependency with each other, nor is it typical for RAN2 to reference the capability across specfiications (TS 38.306 and TS 37.355). |
| ZTE | Support. FG27-11 should be the prerequisite as well. The logic is, UE first support FG 27-11, and 27-10, then it is meanlingful to report this FG to LMF. LMF may not need to further request MG from gNB for signaling overhead reduction. This prerequisite is from UE perspective. We don’t see any issue. Perhaps, it is better to send LS and clearly let them confirm this. |

# Issue 7: FG 27-11

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation request for PRS measurements | 1. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID 2. Support of using DL MAC CE to activate/deactivate the MG for PRS measurements: The DL MAC CE for MG activation indicates the ID associated with the preconfigured MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG for PRS measurements is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Issue 8: FG 27-17

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-17 | PRS processing in RRC_INACTIVE | Support of PRS processing in RRC_INACTIVE | [13-1, 13-2, 13-3, 13-4] | Yes |  | PRS processing in RRC_INACTIVE is not supported | per band | n/a | n/a | n/a | Note: UE supporting this feature may shall support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Issue 9: FG 27-18a

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA - location server | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | 13-3, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for DL-TDOA is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Issue 10: FG 27-18b

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD - location server | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | 13-2, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for DL-AoD is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Issue 11: FG 27-18c

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT - location server | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 13-4, 13-11, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for for Multi-RTT is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Issue 12: FG 27-22

Proposed Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-22 | PRS antenna beam pattern for UE-based DL-AoD | Support of PRS antenna beam pattern for DL-AoD |  | No |  | UE-based DL-AoD with PRS antenna beam pattern is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know | Optional with capability ignalling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[This proposal is stable and ready for agreement – no need to comment in support of the proposal – please leave table empty unless there are concerns]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | We prefer to change the “PRS antenna beam pattern” in the three columns into “TRP beam/antenna information” as follows, in order to match the agreement in below: 27. NR_pos_enh; 27-22; TRP beam/antenna information for UE-based DL-AoD; Support of TRP beam/antenna information for UE-based DL-AoD; No; UE-based DL-AoD with TRP beam/antenna information is not supported.; Per UE; n/a; n/a; n/a. Agreement From the RAN1 perspective, for the TRP beam/antenna information to be optionally provided by the LMF to the UE for UE-based DL-AoD: - The LMF provides the quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - Note: the peak power per angle is not provided - Note: up to RAN3 to decide how the TRP beam information is provided to the LMF for both UE-assisted and UE-based - Send an LS to RAN2/RAN3 to decide on the signaling details |

# Issue 13: FG New FGs

Proposal: Introduce the following new FG

| 27. NR_pos_enh | 27-23 | Support of more than one activated PRS processing windows across all active DL BWPs | 1. Number of supported activated PRS processing windows | 27-3-2 | Yes |  | Only one activated PRS processing window is supported. | Per UE | No | No | No | Candidate values:{2, 3, 4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK |
| Huawei, HiSilicon | Support. |
| CATT | Support |

# Discussion/Approval Items during RAN1 #109-e — Third Checkpoint

Based on the comments/questions/suggestions received by the second checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 4 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | FG 27-3-2: We want to add the following Note under component 2 (fourth column) bedause there is no UE feature description on the capability to receive PPW activation. 27. NR_pos_enh; 27-3-2; DL PRS measurement outside MG and in a PRS processing window; 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS Note: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the FG, but no dedicated signaling is required.. FG 27-15b We prefer to add the following component based on RAN1 working assumption. It was commented in the first round that RAN4 is discussing the candidate values, but at least we believe the the following change could be adopted as the placeholder, which can be updated once RAN4 fixed the details of values. 27-15b; Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP; - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Support of Different numerology between the SRS and the initial UL BWP is supported] - [Support of SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: Support a different center frequency between the SRS for positioning and the initial UL BWP - SRS switching time. |
| Qualcomm | We are OK with both proposals from Huawei, HiSilicon shown above |

# Issue 1: FG 27-3-3

Please report on the latest status in the maintenance session and whether/how component description (2) and its candidate values can be progressed.

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band A UE that supports FG 27-3-2 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Based on the current discussion on ePos-05, we suggest the make the following change. Note that in Rel-16, the following Note is captured in LPP. NOTE: When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38. 214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS resources within P, if - N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and - the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and - the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46]. 27-3-3; DL PRS Processing Capability outside MG - buffering capability; 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 3. Duration of DL PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it; 27-3-2; Component 1 candidate values: {Type 1, Type 2} Component2 candidate values: . a) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8, N+10, N+15, N+20} ms] Component 4 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note 1:The (N, T) UE capabilities are interpreted as legacy (N, T) in FG 13-1, and the UE is expected to receive the PRS within the PRS processing window and but the processing of the received PRS may be outside a PRS processing window. Note 2: The (N2, T2) UE capabilities are interpreted such that the UE is capable of measuring up to N2 ms PRS within a PPW and is capable of completing the PRS processing within the PPW in T2-N2 ms after the N2 ms PRS. Note 3: UE shall support either or both of component 2 and component 3. Note 4: A UE shall declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band A UE that supports FG 27-3-2 must indicate this FG is supported; Optional with capability signaling. 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Max number of DL PRS resources that UE can process in a slot under it |
| ZTE | We are fine with the updated 27-3-3 from Huawei. Regarding the note, we are also fine except for the following bullet. - the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46]. |
| Qualcomm | Support the compromised proposal by Huawei, HiSilicon |
| CATT | OK with the revision from Huawei, which is a compromised solution discussed in [ePos-05]. |
| vivo | we can not agree with component 3 directly, and don’t think this is a great way that the value of T to be defined as N+5, when it is actually used, the processing time is T-N. In actual, the processing time is also {4,5,6,8,…}, so, why don’t we just define the processing time directly? In addition, we have provided our concern about note 2 in the main session, and then, we wonder the value T in component 3 needs to be defined as “N+x”, we prefer - the value can be T: {N+4, N+5, N+6, N+8, N+10, N+15, N+ 20} ms] - note 2 can be: The (N2, T2) UE capabilities are interpreted such that the UE is capable of measuring up to N2 ms PRS within a PPW and is capable of completing the PRS processing within the PPW in T2-N2 ms after the N2 ms PRS. |

# Issue 2: FG 27-10a

Please indicate your preference between the following two alternatives:

- Alt. 1: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-10], 27-11 | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alt. 2: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-10] | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Alt.2 We can accept Alt.1 for the sake of progress if the following note is added. Note: RAN1 understands that FG 27-10a is intended only for the LMF to know, and that the current prerequisite FGs of FG 27-10a are capabilities only for the gNB to know. It is up to RAN2 to decide whether such a FG dependency is meaningful from signaling description perspective, and whether and how it can be captured in RAN2 specifications. |
| ZTE | Alt 1. We are OK with the note for Alt.1 added by Huawei, this is what we suggested in the last round comment. |
| Qualcomm | Alt. 1 and OK with the note from Huawei, Hisilicon |
| CATT | We can live with Alt.1 with the additional note above. |

# Discussion/Approval Items during RAN1 #109-e — Fourth Checkpoint

Void

# Conclusion

The following was agreed during RAN1 #109-e in this agenda item.

## Final Agreements by the First Check Point

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously | 27-1-4 | No |  | No assumption can be made regarding multiple Rx TEGs measuring the same DL PRS resource simultaneously | Per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | Yes | n/a | The candidate values are {16, 24} Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Need for location server to know if the feature is supported The maximum number of reported DL PRS RSRP in the capability signaling should be no less than the maximum number of reported DL PRS RSRPP of the first path in the capability signaling | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS | 13-1 | Yes |  | DL PRS measurement outside MG and in a PRS processing window is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE, details left to RAN2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP A UE that supports FG 27-3-3 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band A UE that supports FG 27-3-2 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request for PRS measurements | 1. Support of using UL MAC CE to request measurement gap activation/deactivation for PRS measurements: The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG 2. Support of preconfiguration of MGs in RRC ignalling for PRS measurements: Each MG in the preconfiguration is associated with an ID | 27-11 | Yes |  | Using UL MAC CE to indicate measurement gap for PRS measurements to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation request for PRS measurements | 1. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID 2. Support of using DL MAC CE to activate/deactivate the MG for PRS measurements: The DL MAC CE for MG activation indicates the ID associated with the preconfigured MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG for PRS measurements is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-17 | PRS processing in RRC_INACTIVE | Support of PRS processing in RRC_INACTIVE | [13-1, 13-2, 13-3, 13-4] | Yes |  | PRS processing in RRC_INACTIVE is not supported | per band | n/a | n/a | n/a | Note: UE supporting this feature may shall support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA - location server | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | 13-3, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for DL-TDOA is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD - location server | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | 13-2, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for DL-AoD is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT - location server | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 13-4, 13-11, 27-6 | No |  | PRS measurement in RRC_INACTIVE state for for Multi-RTT is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:  Introduce the following new FG

| 27. NR_pos_enh | 27-23 | Support of more than one activated PRS processing windows across all active DL BWPs | 1. Number of supported activated PRS processing windows | 27-3-2 | Yes |  | Only one activated PRS processing window is supported. | Per UE | No | No | No | Candidate values:{2, 3, 4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Final Agreements by the Final Check Point

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | Support of low latency MG activation request for PRS measurements | [27-10], 27-11 | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported Note: RAN1 understands that FG 27-10a is intended only for the LMF to know, and that the current prerequisite FGs of FG 27-10a are capabilities only for the gNB to know. It is up to RAN2 to decide whether such a FG dependency is meaningful from signaling description perspective, and whether and how it can be captured in RAN2 specifications. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 2a. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE . 2b. Duration of DL PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE . 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  | DL PRS measurement outside MG and in a PRS processing window is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2a candidate values: - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms [Candidate 2b component values: a) N2: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T2: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS : 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS : 60kHz, 120kHz Need for location server to know if the feature is supported Note 1:The (N, T) UE capabilities are interpreted as legacy (N, T) in FG 13-1, and the UE is expected to receive the PRS within the PRS processing window and but the processing of the received PRS may be outside a PRS processing window. The (N2, T2) UE capabilities are interpreted such that the UE is capable of measuring up to N2 ms PRS within a PPW and is capable of completing the PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured PRS resource(s) inside the PPW, to the end of PPW is not smaller than T2 ms [Note 3: UE shall support either or both of component 2a and component 2b] Note 4: A UE may shall declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band A UE that supports FG 27-3-2 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS : Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH /PDSCH /CSI-RS - State 2: PRS is lower priority than all PDCCH /PDSCH /CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH /PDSCH /CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH /CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH /PDSCH /CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH /PDSCH /CSI-RS | 13-1 | Yes |  | DL PRS measurement outside MG and in a PRS processing window is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE , details left to RAN2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE ) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP Note: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the FG , but no dedicated signaling is required. A UE that supports FG 27-3-3 must indicate this FG is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15b | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - Support of Different numerology between the SRS and the initial UL BWP - Support of SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - Support a different center frequency between the SRS for positioning and the initial UL BWP - Switching time between SRS Tx and other Tx in initial UL BWP or Rx in initial DL BWP | 27-15 | Yes |  | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: - FR1 bands: {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100} - FR2 bands: {50, 100, 200, 400} Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 7 candidate values: {1,2,4,8,16,32,64} Component 8 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 10 candidate values: {100us, 140us, 200us, 300us, 500us} Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. Note 2: If component 9 is not signaled, the UE only supports same center frequency between the SRS for positioning and initial UL BWP Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP Note 4: If component 6 is not signalled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Final Agreements by GTW

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15b | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Support of Different numerology between the SRS and the initial UL BWP is supported] - [Support of SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: Support a different center frequency between the SRS for positioning and the initial UL BWP | 27-15 | Yes |  | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: FFS - FR1 bands: {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100} - FR2 bands: {50, 100, 200, 400} Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: FFS Component 6 candidate values: FFS Component 7 candidate values: {1,2,4,8,16,32,64} FFS Component 8 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. Note 2: If component 9 is not signaled, the UE only supports same center frequency between the SRS for positioning and initial UL BWP [Note 2: Based on other ignalle UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.] [Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP] [Note 4: If component 6 is not signalled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB] [Need for location server to know if the feature is supported] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15a | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  | Support of Positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15c | Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  | Positioning SRS transmission in RRC_INACTIVE state outside initial BWP with semi-persistent SRS is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Need for location server to know if the feature is supported | Optional with capability signaling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting 2. Supported resource set relationship for the target PRS resource and the associated subset [3. Support associated subset measurement reporting] |  | No |  | PRS subset association for DL-AoD is not supported by the UE. | Per UE | n/a No | n/a No | n/a No | Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet} [Component 3 candidate values: {associated subset only, the target PRS resource and the associated subset}] Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity | one of 13-5,13-6, or 13-11 | No |  | LOS/NLOS Indicator for UE-assisted positioning is not supported | Per UE | n/a No | n/a No | n/a No | [Component 1 candidate values: {hard value, hard+soft value[, both]}] Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-1 | M-sample measurements in RRC_CONNECTED | The capability to support reporting a measurement based on measuring M=1 or 2 samples (instances) of a DL PRS resource set | 13-1 | No |  | If the UE does not provide the capability, the UE is assumed to support M=4 only | per band | n/a | n/a | n/a | The candidate values are {1 [FFS others]} Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity |  | No |  | LOS/NLOS indicator for UE-based positioning assistance data is not supported | Per UE | No | No | No | [Component 1 candidate values: {softValue+ hardValue, hardValue, both}] Component 2 candidate values: {resourceSpecific, trpSpecific[, both]} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-2-1 | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP | 13-5 | No |  | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD is not supported | per band | n/a | n/a | n/a | Component 2 candidate values: 1, 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) [Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

- R1-2202929, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2203100, Rel-17 UE features for NR positioning, Huawei/HiSilicon

- R1-2203429, Remaining issues on Rel-17 UE features for NR Positioning enhancements, CATT

- R1-2203533, Discussion on UE features for NR positioning enhancements, vivo

- R1-2203621, Discussion on UE features for Rel-17 positioning, ZTE

- R1-2203653, Remaining issues on UE features for Rel-17 NR positioning enhancements, China Telecom

- R1-2203880, Discussion on  UE features for NR positioning enhancements, Samsung

- R1-2203962, UE features for NR positioning enhancements, OPPO

- R1-2204360, Discussion on Rel-17 UE features for NR positioning enhancements, NTT DOCOMO, INC.

- R1-2204590, On UE features for NR positioning enhancements, Nokia/Nokia Shanghai Bell

- R1-2204944, Views on NR positioning enhancements UE features, Ericsson

- R1-2205002, UE features for NR positioning enhancements, Qualcomm Incorporated