3GPP TSG-RAN WG1 #109-e							            R1-2204860

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.2.2.1

Source:	AT&T

Title:	Evaluation of AI/ML for CSI feedback enhancements

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to the following objectives on evaluation methodology for the various use cases under consideration

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

This contribution focuses on evaluation methodology and key performance indicators for CSI enhancements specifically related to the CSI enhancements sub-use case presented in [2].

# Evaluation Methodology

For the evaluation of AI/ML based algorithms for the approved and proposed sub-use cases, the group agreed to reuse the statistical models used by 3GPP for LLS and SLS simulations, as a first step for algorithms evaluations.

For the MU-MIMO sub-use case we presented, where the AI/ML training and inference are in the network, and the user adjusts its CSI feedback based on actions and policies from the network, we believe that a system level simulation should be used for evaluation as a first level, the SLS will use channel models from 38.901 for UMa, UMi, and InH deployment scenarios.

Proposal 1: Consider an SLS evaluation for the CSI feedback enhancements sub-use cases

In addition to the SLS evaluations, field trial data can be optionally used to test and evaluate the proposed algorithm and framework. Such data however is very hard to share between various companies. To overcome this problem, data sets from testbeds can potentially be used, pending agreement on the right format and platform to share them. Another possibility would be to construct data sets for training and validation testing, depending on the sub-use case considered. A third possibility would be to use publicly available test beds. The data set can act as a common ground for baseline evaluation and performance evaluation of the proposed algorithms.

Observation 1: It is very challenging to share field test results among companies.

Observation 2: Depending on the sub-use case, a data set can be constructed using common assumptions for training and algorithm evaluation.

Observation 3: Depending on the sub-use case, testbed data can be shared or a public data set can be used for training and algorithm evaluation.

For the system level evaluations on the sub-use case presented, the CSI feedback assumptions are to be evaluated depending on the proposed algorithm. Each company should report its feedback assumptions and overhead used in the evaluations.

Proposal 2: CSI feedback assumptions should be reported per company and overhead should be evaluated.

For the SLS evaluations, a single cell and a multi-cell environment are important. For the single cell simulation, the CSI enhancements and AI/ML algorithm can be compared against the baseline, for a given gNB, irrespective of out-of-cell interference. The focus here would be to show the performance improvements for a given local environment, with different user locations, user separations, different loads and usage scenarios.

Proposal 3: Consider both a single-cell and multi-cell SLS evaluation scenarios

On the traffic model, depending on the sub-use case, full buffer simulations can be considered as a first step to evaluate the proposed algorithms and enhancements against the baseline. FTP models can be judicially considered as well, as long as they take the particular pain points of the use cases into account. For MU-MIMO for example, an FTP model with long overlaps over time to increase the probability of user selection and scheduling would be beneficial.

Proposal 4: The traffic model should be used to highlight the performance improvements of the sub-use cases considered.  A full buffer model can be considered as a first step.

Example SLS evaluation assumptions are presented in Table 1.

Table 1 SLS Assumptions for CSI enhancements

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD or TDD, OFDM |
| Scenario | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz, Indoor) are not precluded. |
| Frequency Range | 4GHz |
| Inter-BS distance | 200m |
| Channel model | According to the TR 38.901 |
| Antenna setup and port layouts at gNB | - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 41 dBm |
| BS antenna height | 25m |
| Modulation | Up to 256QAM |
| Number of RBs | 1272 for 30 kHz SCS |
| Simulation bandwidth | 40/100 MHz with 30kHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation as a baseline |
| MIMO layers | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback | Feedback assumption for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms Feedback assumptions for CSI enhancements need to be reported |
| Traffic model | Full buffer Other FTP model is not precluded. |
| Traffic load (Resource utilization) | Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |
| Evaluation Metric | - Average throughput, 5% throughput - Average probability of success - Average number of MU transmissions (when applicable) - CSI feedback overhead - Ratio between throughput and CSI feedback overhead - Training latency - Inference latency |

# Key Performance Metrics

To evaluate the performance of the AI/ML algorithm and CSI feedback enhancements against the baseline, we should take into account, in addition to the performance of the algorithm in terms of throughput, coverage and probability of success, the CSI feedback overhead, as well as the latency incurred and the complexity of the AI/ML algorithm.

For the MU-MIMO sub-use case, we propose to compare average and cell edge throughput, probability of MU-MIMO transmissions, against the baseline MU-MIMO scheme. Furthermore, we propose to include the feedback overhead, and the ratio of the CSI feedback overhead to that of the throughput, to better assess the performance improvements we get from using the AI/ML algorithm.

Proposal 5: the ratio of CSI feedback overhead to throughput should be used as a KPI to evaluate the sub-use cases

Another important metric to consider is related to the training and inference latency, and its effect on the system performance.

Proposal 6: The training overhead and latency as well as the inference latency should be included as a KPI to evaluate the sub-use cases

# Conclusion

In this contribution, we presented our views on the evaluation methodology for CSI feedback enhancements for AI/ML. We made the following observations and proposals.

Observation 1: It is very challenging to share field test results among companies.

Observation 2: Depending on the sub-use case, a data set can be constructed using common assumptions for training and algorithm evaluation.

Observation 3: Depending on the sub-use case, testbed data can be shared or a public data set can be used for training and algorithm evaluation.

Proposal 1: Consider an SLS evaluation for the CSI feedback enhancements sub-use cases

Proposal 2: CSI feedback assumptions should be reported per company and overhead should be evaluated.

Proposal 3: Consider both a single-cell and multi-cell SLS evaluation scenarios

Proposal 4: The traffic model should be used to highlight the performance improvements of the sub-use cases considered.  A full buffer model can be considered as a first step.

Proposal 5: the ratio of CSI feedback overhead to throughput should be used as a KPI to evaluate the sub-use cases

Proposal 6: The training overhead and latency as well as the inference latency should be included as a KPI to evaluate the sub-use cases

# Reference

[1] RP-2213599 Study on AI/ML for Air-Interface

[2] R1-2204861 CSI feedback for AI/ML aided MU-MIMO user selection and pairing, AT&T, RAN1#109e.