3GPP TSG-RAN WG1 #109-e							            R1-2204863

e-Meeting, May 9th-20th, 2022

Agenda Item:	9.2.3.2

Source:	AT&T

Title:	System performance aspects on AI/ML for beam management

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. One of the identified use cases include:

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

For each of the use cases, one of the objectives is to

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

This contribution focuses on enhancements for reduced overhead/latency and increased accuracy for beam management, specifically focusing on the need for consideration of system performance aspects on AI/ML for beam management.

# System Performance Aspects for AI/ML Beam Management

One of the key 5G NR physical layer features used especially in higher frequency bands is the use of beamforming at both the base station and UE to improve coverage and manage inter-cell interference. As a result, beam management features have been extensively studied and developed over several releases. Fundamental to most analysis of beam management techniques is the trade-off between overhead of the beam measurement/reporting (e.g. via beam sweeping at the transmitter and/or receiver) and incremental improvement in system KPIs (e.g. RSRP, SINR, etc.) due to refinement of the beams selected on a given link.

In static scenarios, this trade-off can be mitigated via careful planning during the deployment phase which is analogous to the antenna optimization techniques used in lower bands to optimize parameters such as downtilt or panel orientation. However, in dynamic environments the optimal parameters may change over time and in the context of a large number of narrow beams typically used in higher bands, beam management overhead, specifically the latency of the beam selection, becomes a UE-specific optimization problem which is challenging to do in a fully offline manner and requires real-time feedback and control loops in the network.

As a result, when considering applying AI/ML techniques to beam management, the requirements on the location of the data used for training and inference generation may be scenario dependent and both centralized (e.g. across multiple gNBs) and decentralized (e.g. a single gNB or UE) need to be evaluated in terms of both the performance benefits (e.g. improved accuracy or reduced latency) and overhead constraints (e.g. data interval for collection and volume).

Proposal 1: Both centralized (e.g. across multiple gNBs) and decentralized (e.g. a single gNB or UE) approaches for AI/ML based beam management should be evaluated.

# Conclusion

In this contribution, we discussed system performance aspects for AI/ML beam management. We made the following observations and proposals.

Proposal 1: Both centralized (e.g. across multiple gNBs) and decentralized (e.g. a single gNB or UE) approaches for AI/ML based beam management should be evaluated.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface.