3GPP TSG RAN WG1 #109-e                                                                                        R1-2204868

E-Meeting, May 9th – 20th, 2022

Agenda item:	9.4.1.1

Source:	Robert Bosch GmbH

Title:	Discussion on channel access mechanism for sidelink on unlicensed spectrum

Document for:	Discussion

# Introduction

NR sidelink was initially developed for V2X applications based on automotive requirements. However, due to growing interest in other industrial areas, expansion towards NR sidelink in unlicensed spectrums has considered in Rel-18 WI to unlock more industrial and commercial use cases [1]. The current WID [1] is approved with the following objective:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this TDoc, we are focusing on Sidelink channel access mechanisms for unlicensed spectrum for FR1. We assume that sidelink unlicensed access is considered for both Mode 1 and Mode 2 resource allocation. Additionally, we are also proposing some possible considerations for sidelink physical channel design based on our proposed channel access mechanisms.

# Sidelink channel access mechanisms for unlicensed spectrum

Sidelink over unlicensed spectrum is very important for multiple use cases including commercial use cases, industrial IoT, and (again) automotive use case. For automotive sectors, SL-U can unlock new range of automotive use cases, e.g.:

- Sidelink unlicensed access additionally for private premises: e.g., automated valet parking (AVP), charging-stations, supply-chain distribution areas, coordinated maneuvers during fairs and events, etc.,

- Offloading already congested licensed and ITS spectrum for: extended sensor sharing, fully automated driving, etc..

For industrial IoT use cases, sidelink in unlicensed spectrum (at least in controlled environments) can be a key enabler for IIoT direct communication (UE-to-UE) in order to reduce latency, compared to data delivery through a gNB. Wherein traffic can be offloaded to unlicensed spectrum using sidelink.

As stated in the WID [1], the channel access mechanisms need to reuse NR-U specified mechanisms in Rel.-16. Additionally, Rel-17 introduces some new enhancements to NR-U enabling URLLC/IIoT application on unlicensed spectrum. During the current sidelink evolution new phase, we need to consider adopting NR-U features specified in Rel-16 and (also, whenever possible) Rel-17 to the sidelink framework.

In the following section, we propose some possible sidelink channel access mechanisms in FR1 unlicensed spectrum.

## Applicability of LBT for sidelink

Due to the regulations of unlicensed spectrum in some regions, NR-U specified a listen-before-talk (LBT) before transmission to achieve fair coexistence with Wi-Fi and other uncoordinated NR-U stations. Therefore, an NR-U station is only allowed to transmit data after a successful LBT, where its transmission can occupy a certain channel occupancy time (COT). In NR sidelink, resource reservation depends on sensing and resource exclusion procedure. The sensing results are compiled from decoding valid sidelink control information with reservation slots/frequencies. Additionally, SL sensing procedure has multiple sensing window durations  (depending whether the traffic in periodic or aperiodic) and different sensing window capabilities, i.e., a short sensing window placed directly before candidate resources for evaluating selected/reserved resources. However, this is not operating exactly like LBT and will not be able to replace it.

The LBT procedure (as in NR-U) relies on energy detection, which does not decode any reservation information. Therefore, sidelink in unlicensed spectrum needs to consider both, sidelink sensing (as in Rel-16 and beyond) and LBT being adopted for sidelink. Hence, it is important to study how to consider LBT and sensing simultaneously for SL in unlicensed spectrum.

Objective 1: Both LBT and sensing (e.g., as in Rel.16) are mandatory for operating sidelink in unlicensed spectrum

Proposal 1: Study how to consider LBT and sensing mechanisms, simultaneously, for SL in unlicensed spectrum

NR-U considered different channel access mechanisms/equipment for unlicensed spectrum based on required traffic, namely load-based equipment (LBE) and frame-based equipment (FBE). The latter, FBE, may allow COT sharing among connected devices with different LBT categories. For instance, the initial transmission (initiated by a UE) should use a regular listen before talk, e.g., Cat-4. Later, if other UE(s) are sharing the initial initiated COT, the transmission of the of UEs may safely use Cat-2 or Cat-1 (immediate transmission), i.e., depending on transmission gap.

In contrast to NR-U, sidelink needs to pursue communication directly between UEs. Therefore, it is important to study the applicability of channel access mechanisms, designed for both LBE and FBE, for the sidelink communication in unlicensed spectrum. The study should consider sidelink different resource granting type including dynamic and configured and the different communication types, i.e., unicast, groupcast and broadcast.

Objective 2: The applicability of unlicensed channel access mechanisms, for both LBE and FBE, need to be evaluated for SL-U

Proposal 2: Study suitability of LBE and FBE for sidelink channel access mechanisms

Proposal 3: For UE sharing a COT initiated by another UE, study the following:

- Possible switching gap durations between transmitting UEs sharing the initiated COT

- Whether one or more LBT categories are needed

Figure 1: UE1 shares its sidelink initiated COT with UE2 and UE3 assuming various switching gap sizes depending on, e.g., UE capability

## LBT impacts on resource allocation

### Impact on resource reservation

For SL-U, it is important to study how to adapt dynamic and configured resources (SPS-like) transmission based on unlicensed access mechanisms. For example,  FBE can be considered for configured resources in SL and, in contrast, LBE can be considered for dynamic grants/on-shot transmission. For dynamic transmission, slot aggregation may be considered to extend the transmission time to the actual COT. Aggregating the transmission slots in this case guarantees fairness with other LBT-based technology and may reduce the number of times the UE is requested to do LBT/access the channel.

Objective 3: FBE may be considered for configured resources while LBE may be suitable for dynamic resources.

Proposal 4: Study the impact of unlicensed access on sidelink resource allocation including:

- The required LBT and LBT categories for sidelink dynamic and periodic reservations

- Whether slot aggregation is beneficial for LBE scheme

### Impact on sidelink HARQ feedback

In Sidelink transmission, HARQ may have different timeline and different reservation options. However, at least for HARQ feedback scheduled with a limited switching gap, study whether the COT (initiated by the TX UE) can be shared by the UE sending the HARQ feedback.

Proposal 5: Study the impact of SL-U on HARQ timeline and whether/when HARQ feedback can share the COT initiated by TX UE

Figure 2: a) With sharing UE1 COT for a group of UE HARQ feedback; b) UE1’s feedback is sent in separate initiated COT by UEs interlacing the nominal frequency of a HARQ slot

## Adapting time/frequency resource for SL-U

### Frequency-domain impact: resource pool with interlace structure

NR-U is designed to possibly obey an occupied channel bandwidth (OCB) and the power spectral density (PSD) requirements mandated by regulatory. For example, according to ETSI regulations [4], the OCB is the bandwidth containing 99 % of the power of the signal, which should be between 80% and 100% of the nominal channel bandwidth, e.g., 20 MHz in 5GHz band. In order to cover the total nominal bandwidth, NR-U proposed an interlace structure to adapt the waveform to this regulation in addition to the legacy (contiguous) option.

Sidelink resource pool design should also fulfil OCB and PSD requirements whenever they are mandated. Therefore, the sidelink resource pool design, i.e., subchannel design in frequency, need to be adapted in this case. It is also important to study whether interlace structure can be mapped to subchannel design while allowing multiple UEs to share a sidelink slot.

Objective 4: Unlicensed spectrum access for sidelink may mandate covering >= 80% of a nominal frequency

Objective 5: Interlace structure in frequency has been considered for Uu transmission in NR-U

Proposal 6: Study the impact on sidelink resource pool and subchannel design to fulfil the required occupied channel bandwidth (OCB)

### Impact on frequency-domain: Wideband operation and impact on BWP

Sidelink in unlicensed spectrum has been motivated as an alternative design that can extend the bandwidth beyond, e.g., ITS and licensed bands. Therefore, it is important to consider the wideband operation of unlicensed access. NR-U has also specified channel access mechanism for expanding the transmission beyond the nominal frequency. Accordingly, a contiguous subband-aggregation mechanism was introduced in NR-U with a per-subband LBT option(s), wherein only successful (contiguous) LBT subbands are aggregated as a wide band operation.

Similarly, sidelink channel access mechanism in unlicensed spectrum need to consider a wideband starting with NR-U mechanism. This could be even considered without waiting for the outcome of carrier aggregation. For example, wideband operation may define a single SL BWP composed of all only successful (contiguous) LBT subbands. Accordingly, the resource pools need to be configured for the new wider BWP. It is also important to study the impact on physical channel design in adaptive wider unlicensed bandwidth.

Figure 3: SL-U wideband operation with expandable SL BWP definition and, possibly, aggregated slot transmission

Objective 6: Wideband operation for SL-U will fulfil the motivation to enhance SL data rate on unlicensed spectrum

Objective 7: Wideband operation for SL-U should be even considered without waiting for the outcome of carrier aggregation in Rel-18 using, e.g., a single adaptable BWP

Proposal 7: For SL-U wideband operation, study:

- Different LBT options

- SL-BWP adaptation to expandable transmission wideband

- the impact on physical channel design and allocation

### Impact on time-domain: flexible slot

Since LBT (at least Cat-4) may have a variable contention window size, starting from a sidelink slot boundary may not be always guaranteed. Therefore, a flexible starting option (after the slot boundary, i.e., form an arbitrary OFDM symbol) need to be studied for SL-U.

Figure 4: Impact of LBT on starting position after a slot boundary and, possibly, aggregated slot transmission

Objective 7: Due to LBT variable contention window, a fixed slot boundary may not be guaranteed.

Proposal 8: For SL-U with, at least, LBT Cat-4 study whether flexible slot starting is possible and/or configurable

# Sidelink on unlicensed access physical channel design

This discussions in this section may also cover some aspects in AI 9.4.1.2 (Physical channel design framework).

It is clear that all previous possible sidelink channel access mechanisms for unlicensed bands will induce an impact on physical sidelink channels, e.g.,:

- It is important to study whether PSCCH/2nd stage SCI convey COT and/or wideband operation information to other UEs, at least for FBE is considered,

- It is very crucial to study whether allocation of PSCCH, PSSCH, and PSBCH are following an interlace structure,

- It is important to discuss multiple LBT options for physical channels and, especially, whether allocation of PSBCH is limited to CAT-2 LBT,

- There is a need to study where to allocate PSCCH/2nd stage SCI in wideband option.

Objective 7: All proposed SL-U channel access mechanism may impact sidelink physical channel desing.

Proposal 7: Study the following:

- Whether PSCCH or 2nd stage SCI convey COT and/or wideband operation information

- Whether allocation of PSCCH, PSSCH, and PSBCH follow an interlace structure

- whether it is sufficient to consider CAT-2 LBT for PSBCH allocation

- where to allocate PSCCH/2nd stage SCI in wideband option

# Conclusions

In this contribution the following proposals have been made:

Proposal 1: Study how to consider LBT and sensing mechanisms, simultaneously, for SL in unlicensed spectrum

Proposal 2: Study suitability of LBE and FBE for sidelink channel access mechanisms

Proposal 3: For UE sharing a COT initiated by another UE, study the following:

- Possible switching gap durations between transmitting UEs sharing the initiated COT

- Whether one or more LBT categories are needed

Proposal 4: Study the impact of unlicensed access on sidelink resource allocation including:

- The required LBT and LBT categories for sidelink dynamic and periodic reservations

- Whether slot aggregation is beneficial for LBE scheme

Proposal 5: Study the impact of SL-U on HARQ timeline and whether/when HARQ feedback can share the COT initiated by TX UE

Proposal 6: Study the impact on resource pool design and subchannel design to fulfil required occupied channel bandwidth

Proposal 7: For SL-U wideband operation, study:

- Different LBT options

- SL-BWP adaptation to expandable transmission wideband

- the impact on physical channel design and allocation

Proposal 8: For SL-U with, at least, LBT Cat-4 study whether flexible slot starting is possible and/or configurable

Proposal 9: Study the following:

- Whether PSCCH or 2nd stage SCI convey COT and/or wideband operation information

- Whether allocation of PSCCH, PSSCH, and PSBCH follow an interlace structure

- whether it is sufficient to consider CAT-2 LBT for PSBCH allocation

- where to allocate PSCCH/2nd stage SCI in wideband option

# References

[1] RP-220300, “WID revision: NR sidelink evolution,” OPPO, 3GPP TSG RAN Meeting #95e, March 2022

[2] TR 38.889, Study on NR-based access to unlicensed spectrum, Rel-16

[3] RP-190706, “Revised WID on NR-based,” 3GPP TSG RAN Meeting #83, Mar. 2019

[4] EN 301 893, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU,” V2.1.1, ETSI, May, 2017