3GPP TSG RAN WG1 #109-e                                                                                        R1-2204869

E-Meeting, May 9th – 20th, 2022

Agenda item:	9.5.1.3

Source:	Robert Bosch GmbH

Title:	Views on potential solutions for SL positioning

Document for:	Decision

# Introduction

In RAN#94-e, Rel-18, sidelink positioning was discussed within the NR positioning enhancement WI description in [1]. Additionally, the RAN-level study item “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases” also considered sidelink as one of the radio interfaces in these use cases [2]. In the end, the work item description “Study on expanded and improved NR positioning” was approved in RAN#94-e mentioning the following points related to sidelink positioning:

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

This TDoc proposes potential solutions for sidelink positioning. It is structured in four sections. The first aspect discussed are potential positioning methods. Second, the reference signal design is discussed. The third and fourth aspect are the combination of Uu-based positioning complementing sidelink positioning as well as general consideration with respect to sidelink positioning.

This TDoc mainly focusses on potential solutions explicitly for sidelink positioning. However, we are also supportive to the other positioning enhancements discussed in the WID. Hence, we will highline proposal with an implicit impact also on increased accuracy, integrity and power saving.

# Potential Solutions for SL positioning

In our understanding, sidelink positioning should be specified with a unified design to consider various use cases including automotive, commercial, IIoT, and public safety. At least automotive and public safety requirements are readily available in [3]. These requirements include very challenging relative lateral and longitudinal positioning accuracy that may not be achievable with Uu positioning only. Therefore, sidelink positioning is a crucial solution for enhancing precise positioning.

Figure 1. depicts an exemplary scenario for sidelink positioning in an automotive setting. Please note that sidelink UEs can either be mobile users like trucks, cars, bikes, 2-wheelers, pedestrians etc., but also connected infrastructure like a road-side unit (RSU). In turn, different positioning methods (cf. Section 2.1) might benefit from a sidelink positioning setting involving a static anchor (e.g. RSU) and non-static, moving UEs (e.g. car). For V2X use cases, road-side units (RSU) equipped on fixed road infrastructure can be used as fixed anchor points that assists absolute and/or relative sidelink positioning. Given the fact that RSU can also be of gNB types, Uu positioning and Sidelink positioning co-existence is foreseeable. Therefore, it is useful to study both standalone sidelink positioning (absolute or relative (include ranging)) and combined sidelink + Uu positioning mechanisms.

In contrast, to enable VRU protection including powerful VRU micro-mobility devices (e.g., eBike, 2-wheeler, etc.) or pedestrian UE, relative positioning or pure ranging between two UEs via sidelink can be beneficial. The next subsection, cover in detail the different notions on positioning and the relevant positioning methods.

Figure 1: An exemplary sketch of sidelink positioning using static and non-static anchor points in an automotive context.

Objective 1: Sidelink positioning will be used to conduct the positioning for different vehicular applications

## Study different positioning methods

In general, the term positioning can be interpreted in multiple ways. First, one can distinguish absolute or relative positioning. However, the terms relative positioning and ranging are sometimes used interchangeably.

Therefore, it is important to consider the right definition to design potential solutions for sidelink positioning, we copy here the definition in TS 22.261 :

- Ranging: refers to the determination of the distance between two UEs and/or the direction of one UE from the other one via direct device connection.

- Relative positioning: relative positioning is to estimate position relatively to other network elements or relatively to other UEs.

According to the definition above, it is important to identify which positioning method fits each dedicated technique mentioned above, e.g., study whether:

- TDOA (at least anchor point relevant): is suitable for absolute and relative positioning

- RTT (ToF): is relevant for ranging techniques for determination of the distance between two UEs

- AOA/AOD: is relevant for both ranging techniques and relative positioning as defined above.

Additionally, some of the positioning methods requires accurate coordination with the measured references, e.g., TDOA. Therefore, it is also important for selecting the positioning method to consider the deployment scenario, e.g., coordination between UEs/anchor-points and the involvement of static and non-static anchor points. For example, carrier-phased-based methods are known to work more robust if at least one anchor point is stationary.

In turn, the selection of the right positioning method highly depends on the target way of positioning (relative, absolute) and the mobility of the involved UEs (static, non-static, mixed). Hence, we propose to keep the L1/L2 layer impact being minimum and independent of sidelink positioning signal design. As a consequence, ranging could be the key physical measurements in sidelink positioning.  Based on further available information and time-synchronisation etc. both relative and/or absolute positioning can be conducted afterwards.

Proposal 1: For studying sidelink positioning method (e.g., TDO, RTT, AOA/D, …) consider the relevant positioning technique (e.g., relative, ranging, or absolute) and the deployment scenario.

Proposal 2: For studying sidelink positioning method (e.g., TDO, RTT, AOA/D, …) keep L1/L2 layer impact being minimum and independent of sidelink positioning signal design.

## Study different sidelink positioning reference symbol design

The approved WID in [2] provides also an objective to study sidelink reference signals positioning (SPRS) design and allocation. In our view, it is important that SPRS design/allocation considers at least the following aspects:

- Reuse NR Uu SPRS design as much as possible

- Previous NR release studied Uu positioning reference signals design being able to achieve a high positioning accuracy. Therefore, in our view, taking Uu PRS design as starting point is beneficial.

- SPRS allocation, scheduling, and impact of resource allocation:

- Sidelink communication has many potential use cases in V2X applications. Previous releases should be able, at least, to decode a valid sidelink control information without doing any assumption on sidelink capabilities. This is important for sensing and resource reservation

- Study SPRS allocation with periodic reservation and/or dynamic allocation. In particular, interference with existing SL communication and/or other sidelink messages could appear and should be avoided.

- Sidelink positioning might be performed in the unlicensed band based on progress in the sidelink evolution work item. Mechanisms should be studied to schedule sidelink positioning potentially also in the unlicensed band.

- For SPRS signaling procedures, e.g., SPRS request and response, consider fitting to different casting types, i.e., unicast, groupcast, and broadcast

Proposal 3: Study SPRS design, allocation and signalling considering the following aspects:

- Reusing NR Uu SPRS design as much as possible

- Impact of sidelink synchronization method and sidelink broadcast channel

- Impact on sidelink control channel design including coexistence to old releases

- SPRS scheduling impact on SL resource (re-)selection / reservation,

- SPRS allocation with interference avoidance mechanism

- SPRS allocation with periodic reservation and/or dynamic (on demand) allocation

- SPRS request and response should consider the different casting types, i.e., unicast, groupcast, and broadcast

## Sidelink positioning complimenting Uu-based positioning

In [2], it was mentioned the objects that Uu and sidelink positioning combination needs to be studied. For us, it is important to understand if combination is a relevant RAN discussion, e.g., for RAN2 or RAN1. In our understanding, the combination may not introducing any RAN impact, e.g., when the combination is considered at higher layers (e.g., at automotive sensor fusion). In this case, Uu and sidelink positioning may co-exist without impacting RAN operations. Thereby, it is useful to study sidelink positioning (e.g., relative/ranging) and Uu positioning as two additional “independent” localization sources supporting the vehicles’ local sensors in critical situations. One important assumption could be that both measurements are independently fed to higher layers.

Proposal 4: Study possible mechanisms combing sidelink and Uu positioning and study whether this combination requires any RAN impacts.

## Sidelink positioning considering NLOS

Sidelink positioning can extend the vehicles/VRUs sensing capabilities beyond line-of-sight (LOS) conditions, especially, when the LOS for the vehicle’s local sensors is blocked. Therefore, considering also NLOS sidelink positioning techniques will potentially enhance the localization of SL capable objects, e.g., vehicles and VRUs at these blocked areas. Therefore, a possible could be to study sidelink positioning techniques in both LOS and non-line-of-sight (NLOS) conditions.

Proposal 5: Study sidelink positioning techniques considering both LOS and NLOS conditions.

## Additional potential solutions

### Carrier phase based positioning:

As mentioned in the introduction, carrier phase based positioning is not considered as a 3GPP positioning method at the moment (in Uu-based positioning). In our understanding, certain scenarios might benefit from supporting such carrier phased positioning approaches. In particular, if RSUs, serving as static anchors, are considered.

Proposal 6: Study carrier phase based positioning for sidelink positioning techniques

### Accuracy, integrity and power efficiency of sidelink positioning

Positioning enhancements for Rel-18 [2] includes a special part dedicated for enhancements of accuracy, integrity and power efficiency. In this section, we show the motivation and possibilities of considering this part of sidelink positioning.

Regarding integrity, the trustworthiness of the position is crucial for most V2X applications. Herein, we believe that sidelink positioning may introduce a new notion of implicit integrity as, e.g., verifying the positions retrieved from the network or other sources. Similarly, due to the limited communication range of sidelink by design, the reported position of UEs in the neighbourhood could be verified. Therefore, the implications of sidelink positioning on positioning integrity should be evaluated.

Proposal 7: Study the impact of sidelink positioning to improved positioning integrity of, e.g., Uu-based positioning

The other aspects, including  positioning accuracy and power efficiency, can also be considered for sidelink positioning. For example, Sidelink in Rel-17 already specifies some enhancements including sidelink resource allocation for power saving UE, namely partial sensing and sidelink DRX. Therefore, a power efficient SPRS design may need to consider DRX/partial sensing introduced for sidelink. Additionally, even for full sensing UEs, i.e., UEs with no sidelink DRX/DTX, the SPRS placement, allocation and design should guarantee efficient power consumption.

Proposal 8: Study whether SPRS design should consider both full sensing UEs and partial sensing/DRX capabilities introduced with Rel-17.

Finally, sidelink carrier aggregation is going to be standardized in Rel-18 [10], therefore, further enhancements to sidelink positioning accuracy may utilize carrier aggregation. In this case, after SPRS proper design,  bandwidth aggregation of SPRS can be used to enhance positioning accuracy. For simplicity, adjacent carrier aggregation can be assumed.

Proposal 9: Study enhancing Sidelink positioning accuracy using S-PRS bandwidth aggregation (will be introduced in Rel-18).

# Conclusion

For Rel.-18 sidelink positioning study phase, the following are proposed:

Proposal 1: For studying sidelink positioning method (e.g., TDO, RTT, AOA/D, …) consider the relevant positioning technique (e.g., relative, ranging, or absolute) and the deployment scenario.

Proposal 2: For studying sidelink positioning method (e.g., TDO, RTT, AOA/D, …) keep L1/L2 layer impact being minimum and independent of sidelink positioning signal design.

Proposal 3: Study SPRS design, allocation and signalling considering the following aspects:

- Reusing NR Uu SPRS design as much as possible

- Impact of sidelink synchronization method and sidelink broadcast channel

- Impact on sidelink control channel design including coexistence to old releases

- SPRS scheduling impact on SL resource (re-)selection/reservation,

- SPRS allocation with interference avoidance mechanism

- SPRS allocation with periodic reservation and/or dynamic (on demand) allocation

- SPRS request and response should consider the different casting types, i.e., unicast, groupcast, and broadcast

Proposal 4: Study possible mechanisms combing sidelink and Uu positioning and study whether this combination requires any RAN impacts.

Proposal 5: Study sidelink positioning techniques considering both LOS and NLOS conditions.

Proposal 6: Study carrier phase based positioning for sidelink positioning techniques

Proposal 7: Study the impact of sidelink positioning to improved positioning integrity of, e.g., Uu-based positioning

Proposal 8: Study whether SPRS design should consider both full sensing UEs and partial sensing/DRX capabilities introduced with Rel-17.

Proposal 9: Study enhancing Sidelink positioning accuracy using S-PRS bandwidth aggregation (will be introduced in Rel-18).

# Introduction

[1]	RP-212666, “Moderator’s summary for [RAN94e-R18Prep-06] on Expanded and improved positioning,”  TSG RAN Meeting #94-e, Moderator: Intel.

[2]	RP-212706, “New WI: Further NR positioning enhancements,” TSG RAN Meeting #94-e, Source: Intel

[3]	3GPP TR 38.845, “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”, Rel-17

[4]	3GPP RP-210040: "Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases," (source: 5GAA).

[5]	Bosch Mobility Solutions, Link.

[6]	RP-210903, “Revised WID on NR Positioning Enhancements,” Intel Corporation, CATT, March, 2022

[7] 	3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains; Rel-16

[8]	TR 37.885, “Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR,” Rel-15.3.0

[9] 	RP-212701, “New WID: MIMO Evolution for Downlink and Uplink,” CMCC, RAN#94-e

[10]	RP-212704, “New WI: Further enhancements for NR sidelink,” Samsung, RAN#94-e